Time Control Techniques

TEN PERCENT RULE:

-Apply 10% of your planned IAS to existing IAS

-Hold for 10 minutes

-You will gain/lose 1 minute

INCREMENTAL METHOD:

-Divide planned IAS by 6

-Apply this to planned airspeed for 1 minute

-You will gain/lose 10 seconds

CONSTANT GROUNDSPEED:

-Flight plan at a constant groundspeed

-Maintain that groundspeed

-Works best with 210-220 kts groundspeed

SCNS ETA:

-Turn away from courseline until SCNS ETA equals your pre-planned turnpoint time

-Can be used to cut off legs without much effort

-Works well if you are early

INCREMENTAL METHOD:

-Increases/decrease planned IAS by same number of seconds late/early

-Hold this correction for the time (in seconds) equal to flight planned IAS

WHIZ WHEEL METHOD:

-Set planned number of minutes under the planned IAS

-Read airspeed corrections on outside scale for number of minutes off time (inside scale)

WHIZ WHEEL METHOD #2:

- Set index on the flight planned IAS

-Determine air distance flown for a given amount of time

-With air distance determined, slide new amount of time desired under the air distance

-Read number under index

-This will be your new indicated airspeed

TURNING EARLY/LATE:

-At 210 KIAS, you travel 3.65 NM per minute or 1 NM in 16 seconds

Time Control Techniques (CONT)

S-TURNS:

-Turn 30o off course, hold for 1 minute

-Turn back 60o, hold for 1 minute

-Turn to original course

-You will lose 16 seconds

Turn 45o off course, hold for 1 minute

Turn back 90o, hold for 1 minute

-Turn to original course

-You will lose 30 seconds

-Turn 60o off course, hold for 1 minute

-Turn back 120o, hold for 1 minute

-Turn to original course

-You will lose 45 seconds

TEN KNOT RULE OF THUMB:

-Increase/decrease groundspeed by 10 knot from planned

-You will gain/lose 1 second for every 1 NM traveled

-Will get you within a second or two everytime

CUT-OFF:

-On turns of 90o or greater

-Draw a line from planned cut-off point to course interception point (C to D)

-Draw a line perpendicular to the cut-off course (A to B)

-Convert the distance AB to time at current ground speed such that if the AB distance were 3.65 NM, you would gain one minute by flying from C to D

-On turns of less than 90o, do the same as above only this time the AB distance is half

-Then convert time

-This will become your time gained

A

D

C

B