- General

- Length 54’ 1.67”

- Rotor dia 48’ 3.2”

- Turning Radius 35’

- Mast tilted 5 deg for

- Deice: 2.1 psi, 21 lb/hr

- Cabin Heat: 1.4 psi, 14 lb/hr

- Max GW 9500 lb

- Min blade tip ground clearance 7’ 9”

- Safety wire .021” copper

- WSPS 90% coverage

- HIT check: +-20 writeup, +-30 don’t take

- Emer freq UHF:243, VHF 121.5, FM 40.50

- Wait 25 mins after Bat off to taxi

- Inertial reels lock 2-3 G decal

- 4 first aid kits in cabin

- Min crew 1 pilot

- Bleed Air

  - Particle Separator

  - cabin Heating

  - Deice

  - Start fuel purge nozzles

  - Left pneumatic pump

  - Oil cooling fan

  - Bleed band actuators

- 40 sec energize time, 3 min cooling, 3 attempts in 1 hr

- 1 psi= 200 lb, 5 kias, 100 ft/min

- Cal factor: +-1 from 60cal=+-.5 psi

- <40 F/5 C: icing of engine air inlet screens expected

- Restart takes approximately 1 min

- Refueling; > 50’ from aircraft if not involved

- # thermo couples?

- Plan T/O LND +/- 15min

- Contact ops sup +/- 30 min T/O, +15 land

- Fireguard: inside tip path, in sight of pilot

- Approach: 12º NH tail/skids contact gnd same time

- WSPS contact ground 10 º NL

- Weight/Balance/TOLD

- Class 2 aircraft

- Cabin floor rated 100lb/ft2

- Cargo hook 4000 lb

- Lateral CG limit +-5”

- CG>140 term approach 5’ hover

- <=8600 lb, CG 130-144

- 8600-9500 CG 134-143

- Passengers 200lb

- Combat troops 240 lb

- Paratroopers 260 lb

- Litter patients 265 lb

- Max TOLD conditions used ifΔ: 200lb, 5 ºC, 500’ PA

- 11 troops, 6 litters

- (11-208) recal weight balance if +500 lb

- Arrive destination 250 lb reserve (VFR/IFR)

- Canned form dated w/in 1 year

- Fuel

- 5 cells

- 208.5 gallons, 206.5 usable

- 1404 lbs JP8

- <125 psi refueling

- 3 115V transmitters

- JP8 settles 1hr/ft (29” 2.5 hr)

- JP8 weighs 6.7 lb/gal

- Low level light 185+-55

- 8-25 psi (pumps)

- Overspeed gov maintains rpm +-50

- Droop Cam comp keeps rpm +-40

- 22 atomizers/4 starter nozzles

- 8-12% N1 primary fuel introduced

- 28-32% N1 secondary fuel

- GOV switch emer lose 6-8 psi

- Most for CG at 70 gal

- Hydraulics

- Reservoir: 5.3 pints

- Total: 10 pints

- Filter pops 70+-10 psi

- MC light 500 psi +-100

- MC light extinguish 800+-100 psi

- Pump 1000 lb +-25

- Px relief 1175 psi

- Electrical

- GPU: 7.5 kw, 600 amps min

- Battery: NiCad, 24 V, 34 amp-hr

- 14V by 10% N1 at Start

- Over Voltage relay trips field relay 31-33V

- Idle detent switch on 28 VDC bus

- Segment lights 28 VDC

- Main Fuel switch 28 VDC ess bus

- Generators: 30 V 300 amp

- AC power 28V (via transformer) or 115V (via inverters)

- Segment lights after “main fuel ON”

  - Engine Fuel Pump

  - Engine Oil Press

  - XMSN Oil Press

  - DC Generator

  - INST Inverter

  - HYD Pressure

- 28 VAC

  - Fuel Px

  - Engine Oil Px

  - XMSN Px

  - Torque

  - CI

- 115 VAC

  - AI

  - Fuel Qty

  - J2 Compass

- Powertrain

- 1125 SHP at 50 psi cal

- 20.4:1 reduction

- Oil <79 º C bypasses cooling

- 110 º C max oil temp

- 3 bypass (2 clogged, 1 thermal)

- Engine

- T53-L-13 engine

- 1400 SHP

- 3.22:1 RPM reduction btwn N2/output shaft

- Compressor increases pressure 7:1, 5 stage

- Oil <79 deg C bypasses cooling

- IGVA failure closed, limit 20-25 psi

- Air Restart: Open throttle 6400-6600 @ 85 N1

- Overspeed

  - 6900 rpm

  - 101.5%

  - >6700 rpm, >15 psi, >3 sec

- N1 Gearbox

  - Starter Generator

  - Oil pump

  - FCU

  - Tach gen

- N2 Gearbox

  - Torquemeter boostpump

  - Overspeed Gov

  - Tach gen

- Rotor System

- Tail rotor 90 º gearbox 2.6:1 reduction

- Preconed 2 ¾ º

- Hub spring contact 4 º flapping

- Hub spring removed< -5 F/-20 C

- Autorotate RPM range 294-339

- Low light at 6100-6300, 300-310; N2/Nr tach fail

  - If both: audio and light

- Light (high): 329-339

- Delta hinge 35 º off center trunnion

- 15% N1, blades turning

- W!: slopes >5 º contact control stops

- LTRE factors

  - High gross weight

  - Improperly rigged t-rotor

  - High PA/temp

  - Adverse winds

  - Low N2/Nr rpm

  - OGE hover

- Left velocities or left Xwind 5-35 KIAS cause Tail rotor work in own rotor wash

- LTRT sideslip may become uncontrollable above 30-40 KIAS

- Aerodynamics

- TFE: 10-20 kias

- Airspeed unreliable< 20 kias

- ETL: 16-24 kias

- Stall region 25%, Driving 45%, Driven 30%

- Settling w/power: >800 fpm, <30 kias

- Cabin doors secure >50kias

- <50 kias 1 door open/1 closed

- Airspeed indicator 124 kias max

- 10 º slope sides and NH, 7 º NL

- 30 knot wind/flight limit lateral and aft

- Start limit 30 knots/15 gust

- IR scoop suppressor, 140 lb

- Rotor blade erosion protection coating + 1 psi hover, +2psi cruise

- 2 types Turbulence:

  - Orographic (wind 15/20 knots+ steep slope causes wind to blow off slope instead of down)

  - Convective (norm<2000’ but up to 8000’)

- 3 types drag

  - Parasite-nonlift surfaces

  - Profile-blades

  - Induced-lift producing surf

- Max autorotational range 98 kias

- Max endurance/min rate autorotational descent 63 kias

- Max r/c: 54-56 KIAS

- Maneuvers

- All ops 500’ AGL except T/O, landing, training req

- SEF

  - 700’ 60 kias

  - 400’ recovery options

  - Pow Recovery: climb by 200’ w/ 60 kias

- Autorotation

  - 70 kias min, <3000 fpm

  - Aligned w/in 45º of wind if 15 knots, else 90º

  - Hovering: wind w/in 15 º, 4’

  - 180 Auto: 800’, aligned w/landing area 150’, 80 kias in turn

  - St ahead: 500’ AGL, 90 kias, steady state by 100’ AGL

  - Low Level: 50’ AHO, 40 KIAS descent

  - > 180 º pow recovery by 250’

  - Flare 75-100’

  - Pow Recovery Termination: landing attitude 25’, pow pull 15-25’, term 4’ AGL 0-15 groundspeed

- Marginal Pow T/O:  Max Pow at 5’ hover, 50’ obstacle, term 50 KIAS

- Max Per T/O:  Hover pow +6psi, 100 ft obstacle, term 70 kias

- Slide on Landing: TD 30 kias

- Traffic Pattern: DW  500’ AGL 90 KIAS; Base 300’ AGL 70 KIAS

- Hover 3-5’; Turn <90 º in 4 sec

- Unusual attitude: >1000’AGL, 30 º bank, 20 º NH, 10º NL

- Team sortie: wind limits 20 knots, gusts 10

- Steep appch: start 30 kias 45 º

- Instruments

- VOR needle +-  4

- Altimeter +- 75’ (11-217), +-70’ (local reg)

- Radar Alt 0-3’ on ground

  - Test 1000’+-100

- Instrument Dep

  - SID

  - Obstacle Dep Proc

  - Div Dep

  - ATC (radar vectors)

  - MAJCOM approved

  - VCOA

- Weather to file:

  - min for low compatible appch at dep airfield

  - min for low compatible appch at dest airfield

  - Can half vis no less than ¼ nm or 1200 RVR

  - COPTER only can’t decrease vis

  - Ceiling not required unless not straight in

  - TEMPO does not apply

- Straight in

  - course guidance can’t x centerline over rwy

  - +/- 30 º of centerline?

  - <=500 fpm

- Alt necessary

  - WX +/- 1 hr TEMPO or prevailing <700/1

  - Weather reporting

  - RADAR only

  - Unmonitored navaids (A/NA)

  - GPS only

- Airfield can be alt

  - WX +/- 1 hr TEMPO or prevailing <700/1or 500/ ½ + lowest compatible whichever’s greater

  - Weather reporting

  - RADAR only

  - Unmonitored navaids (A/NA)

  - GPS only

  - VFR exception:+/- 1 hr VFR descent from enr alt to appch/ldg

- MEA: MOCA + Min Rec Alt

- MCA: Min Crossing Alt

- MOCA: obstacle clearance

  - 1000’ obstacle clnc w/in 5 nm, 2000’ mtns

- OROCA: same as MOCA but off route

- ORTCA

- MEF: Max Elev Fig:  Sectionals, MSL, highest obstacle in quadrangle

- Estalished: ½ full scale def ILS VOR, +-5º NDB

- Weather

- VFR requires 700’ ceiling, 2 sm vis

  - Night/NVG, Xcountry, team: 1000/3

- Avoid thunderstorms 5 nm

- Special VFR recovery 300-1/2 day 500-1 night (CAIRNS)

- Mod turb not recommended based on acft>12500

- Light/trace icing ok if blade erosion protection coating and polyurethane tape not installed

- Class B: 3 sm, clear of clouds

- Class C/D/ Class E (<10K): 5,3,2,1

- Class E/G >10K: 5,1,1,1

- Class G<1200A:

  - Day ½, clr clds, airspeed to see/avoid

  - Night 1 sm, clr clds, airspeed to see/avoid

- Class G 1200><10K:

  - Day 1 sm, 5,2,1

  - Night 3,5,2,1

- SVFR required for controlled airspace if <1000’ /3sm

- Cairns provide WW/TWA if wx w/in 15nm

  - AWA if 60 nm

- NVG, return 30 min prior to fcst<1000/3

- Marginal VFR: 1000/3-3000/5

- Marginal IFR: <500/1

- -OVC not ceiling

- Navigation

- 1000’ obstacle clnc w/in 5 nm, 2000’ mtns

- Mtns: 500’ in ½ nm

- TPC: 500’ contour/ 698’ obstacle error

- JOG: 100’ contour/ 298’ obstacle error

- +/- 2 min TOT

- 20 min nav, 3 TP, 60 deg turns or less, 20 nm max

- IP 3-12 nm 30 º turn or less

- Reaction time 3-4 sec

- Remotes

- ½ nm parallel powerlines

- 1/3 nm clear of tower and guy wires

- 300’ AGL High recon 50 KIAS

- 50’ AHO Low recon 50 KIAS

- 500’ slant range farm buildings/chicken houses

- <25’ rotor clearance jump call clr, 10’ min

- 10’ min skid clearance

- Appch callouts above 300’ every 100’ below 300’ every 50’

  - Alt, airspeed, sink, torque

- Appch monitor:

  - VVI, Nr, Torque

- Bubble effect: veg, high DA, skc=8-15 C

- Pow check: OGE+3

- Pow margin<5 psi recalc TOLD

- Unrest escape: OGE

- Rest escape: OGE+3

- Low Level

- Cross ridgeline 45 º

- Don’t turn upwind prior to 50 KIAS

- Downwind 100-200’AHO 80 KIAS

- 45 º: rotor tip below ldg gear

- L/L is 100’ AHO-500’ AGL

- Stay 200’ AHO over NOE boxes

- Call transient RT w/in ½ nm/200’ AHO

- Call inbound RT 3-6nm

- Normally, do not plan L/L over or into an NOE box unless there is intent to land at an RT within that box

- Formation

- Primary purpose

  - Mutual support/control

  - Increased lift capacity

- Min separation 1 RD

- >3 acft consider 2 smaller elements

- 1-3 RD >500 AGL

- 3-6 RD<500 AGL

- Fluid Trail: 30º L/R aft

- Fixed Trail: directly behind, stacked slightly high

- Staggered: Even # L/R 30-45º, Odd # behind 1

- Combat cruise: 500’ 3-9, 45 deg optimum

- Taxi: 100’ min spacing MR-TR

- Wing T/O: 5 sec after radio call

- Delayed T/O:  Norm delay 5 sec, lead: 70 KIAS 500 fpm

- Overshoot: 1000’ separation

- 5 acft max

- Lost wingman “execute, mts/nmts, hdg, speed, MSA”

- Lost Visual Non mountainous

  - Turn 10* position # º, 30 sec(after alt), MSA+2*position#

- Lost Visual Mountainous

  - Speed =prev acft-10 (3 min aft alt), MSA+2*position#

- Lost visual contact: >=500 fpm climb

- NVG

- 11-208

- All crew use same type of NVG, all on/off

- Ambient illum levels (EMI)

  - NVGs best above 5%

  - No training <5% EMI

  - IR filter required for search or ldg light <20% EMI

  - <20% EMI=marginal ambient light

- Min alt 500 AGL in surveyed LL

- Min alt 200 AHO surveyed NVG LL route

- Must have white ldg or searchlight operable

- Position lights steady bright during flight

- Anticollision may be off, position lts dim for term ops if hazard for crew

- Must have NVG –compatible light source

- Min RT size 3 rd

- Approach

  - 30 KIAS  by 200’ AGL

  - Last 100’  300 fpm

- Scanner primary duties

  - Obstacle clearance

  - Hover alt reference

  - Drift detection

- 217V2

- Cone cells:

  - Day, color, centered at fovea

- Rod Cells

  - Night, low-light, gray, outside fovea

  - Require rhodopsin (most inc in first 30 min)

- Focal vision: central 2 degrees

- Ambient vision (peripheral): subconscious, special orientation

- Dark adaptation: 30-45 min norm

  - Bright light >1 sec impair night vision

  - Recovery 5-45 min

  - After NVG recovery 5-8 min

- Types of vision

  - Photopic: cone cells, color

  - Scotopic: rod cells, poor resolution, loss of color

  - Mesopic: intermediate photopic/scotopic

- Visible light 400-700 nm

- NVGs visible and near IR (700-900nm)

- Illuminance: light strikes object

- Luminance: light reflected

- Albedo: ratio light strike/reflected

- Contrast: luminance difference of 2 objects

- Moon primary source of natural illumination

  - Depends on moon angle/lunar phase

- Lunar month approximately 29 days

- Stars provide 20% illuminance moonless night

- Skyglow: 1 ½ hrs after sunset/ ½ hr prior sunrise

- Ionization provides 40% illuminance moonless night

- Components image intensifier tube

  - Objective lens: focus photons onto photocathode

  - Minus Blue filter

  - Photocathode: starts intensification process

  - Microchannel plate: 1 e in ->1000 out

  - Phosphor screen: converts e visible light

  - Fiberoptic inverter: reorients image

  - Diopter lens: focuses image onto retina

- Gain: ratio input/output, controlled by ABC

- NVG field of view 40 º

- Field of regard: specific to aircraft

- Binocular cues: up to 200 m

  - Stereopsis, vergence, accommodation

- Monocular cues

  - Size constancy, motion parallax, linear perspective

- Dynamic Visual Cues

  - Static cue motion, Optical flow, Peripheral vision motion

- Circadian Trough between 0300-0600

- Fatige types: acute, cumulative, circadian

- 6º civil twilight

- 12º nautical twilight

- 18º astro twilight

- EENT:  End of Evening nautical twilight