CREW COORDINATION / MISSION STUDY GUIDE

1.  INTRODUCTION

P / CP / N – Training requirements / Seating

N - Route study

N / CP - Form 70/  Stick Diagram with times, waypoint #’s, and bullseye

2.  CREW COORDINATION

GROUND

P - Walk around, SKE , Nav aids

CP - Radios & Nav aids, Have Quick, Secure Voice, Mode IV.

N - Mark load (date & callsign), Check chutes

P - Crew brief time / Stations time

PRIOR TO TAXI

P / CP -  plan (especially DZ coord.),  Predicted CDS flaps  & SKE timing

SKE – CP - Parameters - (set for invertent wx penetration)

DEPARTURE

FCI  prior to departure

N – (Wing) call “timing, 10 sec, 5 sec, ready ready hack” (phantom spacing?)

N - Combat Entry , Non-flying pilot briefs the drop card

CP - all ATC radios / P – interplane

CP / N - Abort calls (CP: ATC / N: Interplane)

SKE -CP - ADI: Radial/DME;  T.O. in SKE unless leading

ENROUTE

CP / N - Positive transfer of Navigation

CP / N -  Call out objects by clock position and distance (walk me to it)

CP – Identify items or terrain features on centerline along flightpath

P / CP ADI: VIS - both fly in SCNS

N / CP - Chart read during off-course maneuvers, AVOID Restricted Areas,

- Avoid Class C / D airspace; use CTAF / Traffice advisories as req.

- Timing

CP - Leg times

N – Time from update points /  TOT control (TOA?)

N – Airspeed changes –vs- off course maneuvering (Update MSA)

P / CP / N – Nav 4-5 page, Airdrop page?

- Turn Point Brief  (5 NM)

N - Course (predicted heading) / Distance / MSA /  Night Altitudes

- VIS Turns

concur on TP /  TALK ME ON TO IT / Nav can pass the turn call off

Turn call example: “Ready, Ready, Right turn heading 245”

On rollout – SCNS XTRK / Winds & Drift/ Leads time status

- SKE Turns

SKE -P / CP – Pilot not flying will VERBALIZE ALL PREPS

SKE -N - I get 5 sec and E preps as well as SD thru escape (advise on preps)			SKE -CP - give intrack distance at 30 secs

SKE -P - will work for position until 5 sec and make time adjustment

SKE -P / CP - pilot not flying watch PPI and call trends / directive calls

SKE -P / CP - Turn call example: “Ready, Ready, Right turn heading 245”

SKE -N - On rollout – Advise XTRK / Winds & Drift / Lead’s time status

SKE -N - Save new heading on RFCI

THREATS

-If notified via ABCCC or AWACS (AUTHENTICATE)

N / CP - identify location of threat while the other navigates then compare

N / CP - Formulate a plan // let everyone know

-Pop up threats

N / CP – SCNS coordination and techniques

N / CP - Mark new threat on chart

CP - Inform Formation  & ABCCC

P – Discuss evasive maneuvers and crew coordination

RUN-IN

N - Hit IP upwind on drift killed heading (TP?)

CP - Make sure we have DZ freq. set / Range clearance – if required.

P / CP - Memorize key terrain features  //  micro navigate

SLOWDOWN - location / signal / what will SCNS read at our SD?

-Recheck drift and winds (>3° during Vis = intrack with DZ)

N  -  After winds, UPDATE CARP ASAP - Notify the crew! (CP update safety box)

N / CP - Check wind limits for drop

P / CP / N -  Positive DZ identification  - (1st notify /  2nd notify w/ clock and dist.)

N / CP – Check predicted GS -vs- actual GS and update usable DZ time			SKE - N - Mark 50 foot addition to drop alt. by element & Drop alt. AGL

SKE - N - DZ entry point, what will our SCNS read

SKE - CP - Set secondary control panel when ___?

DROP

N - put me where you want me (Be Assertive, I should respond to you)

P – Tend to listen to CP when they speak due to safety and drop clearance

P / CP / N - “NO DROP  PARAMETERS / SAFETY BOX”

CP - “Two handed” for all drops

N / CP – Verbally verify Block Letter!

N – Call five seconds on time and not early. (GPS backup techniques)

CP - CDS, after “Load clear” if I don’t say “Flaps 50%”- ADVISE IMMED.

Malfunctions - CP - red light / configuration /  (70%) / alt & airspeed / radio call

N - racetrack / salvo -- get us there and run all checklists for subsequent drops

SKE - N –Make sure we get advisories (DN PREP)   (CP help)

SKE -CP –cleared to reset secondary control panel / advise

SKE - P / CP / N - “NO DROP  PARAMETERS / SAFETY BOX”

ESCAPE

Heading / altitude / airspeed

N – Acceleration

SKE - N / CP – Time for turn if on the wing

RECOVERY

N - Combat Exit  , Descent 

Type / altitude / approach seperation

CP – Ensure we get clearance for the next route – if required

N – OH & DW, hack 12 seconds for turn and 15 secs for landing dist.

SKE - Navaids – ADI Mode Sel Switch after final turn within 18 DME (hold list)

SKE - CP – Cleared to set multiples of 6, 12 or 18 off of lead during separation

3. GENERAL

Back each other up / Keep SA / HAVE FUN / Bus Time___?

REMINDERS:

-1 /  FLIGHT PLANS / NOTAMS / SECRETS / PUBS / 280s / BRICK / MEALS