# TC-12 Walk-Around Gouge

This is an unofficial guide produced to increase standardization among instructors.  If there are other items that you expect your fam students to know, please forward to Brian Sanford so we can all teach the same stuff.

Interior

Emergency lights

Total:	5

- Location:  Cabin door, emergency escape hatch,    cockpit, two in cabin

- Power source:  rechargeable batteries

Must be turned off with aircraft battery on

Oxygen system

Limits: 1000 psi local, 1500 cross country, 49 cu ft capacity

Location:  aft of rear bulkhead

Noisy ramp check

What are you actually checking?

- That the standby pumps and firewall valves will operate normally on the hot battery bus

Fire extinguishers: 2

Check for normal pressure (green arc) and inspection currency

Exterior

Flaps

- Position indicator senses off of right flap

- Purpose of hinge bonding braids

- Grounding wire to avoid welding hinge with lightning strike

- Purpose of red painted area

- To visually check from cockpit, danger area for hands, see rubs if out of alignment

Main Gear

- Fire extinguisher pressure around 500 psi

- Strut extension 5.5 inches

- Tire pressure 60 – 64 psi

Wing

- Static wick purpose to dissipate static electricity

- Stall strip purpose to force wing root to stall before tip

- Right side:  Battery scoop and drain

Engine Compartment

Left side

- Acceleration bleed valve:  Low 66 –71 %

- Engine driven fuel boost pump and oil scavenge pump on same shaft

- Air conditioner compressor and synchro-phaser (right engine only)

- Igniter box

Right side

- Oil limits: cold – 3 quarts low, hot – 2 quarts low

If below, motor starter only for 15-20 seconds

Point to:

- Acceleration bleed valve:  High 88-92%

- Ice vane manual cable, and electric motor

- Oil pressure and temperature transmitters, filter

- Py air filter

- EPA tank and pump

- Cables:  power, condition, and prop lever

- Oil to fuel heat exchanger

- High pressure engine driven fuel pump: 850 – 1000 psi

- Fuel control unit

- Fuel flow transmitter

- Flow pack

- Fire extinguisher nozzles

- Fire detection sensors