FLT PLANNING WX REQRMTS

Departure Aircraft

Is departure weather at or

above authorized ceiling

and visibility landing

minimums?

Departure Alternate

Required

If departure weather is:

- RVR > 16(1/4)- or -

- RVR > 12 at approach end

- RVR > 10 at departure end

May use any two consecutive

RVR readouts.  Must have

centerline lighting.

- THEN -

A departure alternate is:

- REQUIRED within 30 mins

with reported & forcast WX >

200-1/2(RVR 24) or appraoch

mins, whichever is higher for

one hour after takeoff.

- OR -

- REQUIRED w/in 2 Hrs with a forecast > 500-1 above

approach minimums but not <

700-2 for a precision or 800-2

for a non-precision approach

(ETA + 1 hour)

NOTE: Must be able to

maintain MEA to alternate if

an engine fails.

NO

DEPARTURE

ALTERNATE NOT

REQUIRED

YES

ISLAND OR REMOTE

FORECAST WEATHER

Winds within limits

corrected for RCR.

If precision approach is

available, ceiling and

vis may  temporarily

or intermittently drop

below non-prec mins

(excluding ASR) but

not below prec mins.

If non-prec apprch is

available (except ASR)

prevailing ceiling & vis

must be > published

minimums.

IS DESTINATION AN

ISLAND OR REMOTE?

YES

NO

DESTINATION

APPROACHES

Straight-in approaches

may be used vis only if fuel

permits.

Circling approaches

require both ceiling

and visibility.

Is a published approach

available?

File IFR to a point enroute

wher VFR conditions

exist, or to a published

apprch, descend to VFR

then VFR to destination.

NO	YES

REMINDER:

Check NOTAMS &

TCN for changes

DOES DESTINATION REQUIRE TWO ALTERNATES?

Is forecast weather (prevailing or intermittent)

below visibility approach minimums?

- or -

Does wind exceed limits corrected for RCR?

DOES DEST WX

REQUIRE AN

ALTERNATE?

Alternate required if

the worst WX (ETA

+ 1 hr) is forecast to

be less than:

Ceiling - 3000 ft

Visibility - 3 miles

or 2 miles

above lowest

minimums.

NO		           YES

ALTERNATE REQUIRED:

Worst forecast wx ETA

+ 1 hr must be: at least

1000ft or 500ft above

lowest approach minimums.

- AND -

Visibility of 2 miles or 1 mile

above lowest approach

minimums.

YES

IS RADAR REQUIRED

FOR THE APPROACH?

NO			           YES

NO ALTERNATE

REQUIRED

IS DESTINATION OUTSIDE CONUS?

IS FLIGHT INTRA-THEATER AND EXCEED THREE HOURS?

NO

NO		YES		     YES

NO