C-130 BRAKING RESTRICTIONS

(Ref Sect VII, Dash-1)

1.	If runway available does not exceed CFL by min of 300’,

brakes should be cooled for 10 min between a normal

landing and takeoff, even if brakes only “checked” on

landing.  Utilize full rwy and reversing on rollout to

minimize brake use.

2.	If full anti-skid braking is used for landing, the gear should

be left extended after an immediate subsequent t/o for a min

of 15 min prior to retraction/braked landing. Do not

terminate a mission (if able) after a full anti-skid braking.

3.  A partially braked landing is defined as a smooth 3 sec brake

application with steadily increasing brake pedal pressure

initiated at 90 knots. Multiple partially braked landings must

not exceed 3 sec brake applications, and gear must remain

extended in the pattern.  No tailwind can be present and

weight is limited to 130,000 lb.

4.  After any full anti-skid braking operation above 130,000 lb,

brakes should be cooled for 65 min (dual disc) and 30 min

(single disc) prior to further a/c movement.  Multiple anti-

skid landings are restricted to below 130,000 lb and 15 min

airborne cooling time.

5.  Peak temps occur in the brake assembly approx 1-5 min

and in the wheel and tire assemblies approx 20-30 min after

max braking operation.  Ground personnel should approach

from fore or aft when inspecting the brakes.  Set opposite

brake or chock nosegear only if gnd evac’ing.

6.  Do not taxi/tow the a/c for at least 15 min after overheated

brakes are cooled.