Hanks Gouge

UC-12B

Engines

PT6A-41 - 				type

850 -					horsepower

1 -					N1 turbine

2 -					N2 turbines

4 -					compressors - 3 axial, 1 centrifugal

-65 F (-53 C) - 				coldest operating temp

10sec -					max time for light off

37,500 rpm - 				compressor speed at 100% N1

38,100 rpm -				compressor speed at 101.5% N1

38,500 rpm -				compressor speed at 102.6% N1

9 to 1 -					compressor ratio

15 to 1 -					gear reduction ratio ( power turbine to propeller gear)

450 psi -					engine fire ext. pressure at 70 F

2.5 lbs -					Halon agent

3 per engine - 				fire sensors

8 -					fuel probes each side, 1 in auxiliary tank

Starting limits -				40sec on, 60sec off, 40sec on, 60sec off, 40sec on, 30min off

88 to 92 -				high pressure bleed valve opens

66 to 71 -				low pressure bleed valve opens

25 % -					percent of air that burns

Two high pressure bleed valves used to prevent compressor stalls

if stick closed compressor stalls may occur at low power settings

if stick open infected engine bleeding air - high fuel flow and ITT

Torque and ITT limits max power

if its cold and your low will be torque limited

if its hot and your high will be ITT limited

PCL goes to cambox the to fuel control unit

Prop lever goes to primary governor

Condition lever goes to fuel control unit

High pressure bleed valve on right side of engine

Low pressure bleed valve on left side of engine

Two Halon fire extinguishers - can read pressure in wheel well, 2 more one under copilot seat and one in aft section

Fire extinguisher is on hot battery bus

Can relight engine if absolutely necessary after using extinguisher

Shut first generator off during start to make use of another path and not blow fuse, after initial amps have got engine going 12%, gen. can be brought back on line and through the fuse

Autofether system -			1 both power levers above 90% N1

2 autofeather switch arm

3 410+-50 - de-arm good engine

4 260+-50 - autofeather sets in through overspeed gov. electrically

Test position bypasses step 1&2

If need to feather must fuel cutoff, Idle will bring N1 below 90%

Left engine is critical engine

Autopilot

245k/.47m -				max speed

17000 ft -				max altitude without yaw damp

200 ft -					min alt. for Autopilot

Do not use yaw damper below 200ft AGL or during Takeoff & Landings

Disengagement must be tested satisfactory prior to flight of intended use

Propeller

2000 rpm -				max normal operation

1900 rpm for 1 min - 			max reverse - above 1900 rpm

2200 rpm for 5 sec - 			max transient

Above 2080 rpm - 			indicates failure of primary and secondary governors, torque limit 1800 ftlbs

1600 - 2000 rpm -			primary governor range (controlled by pilot)

1750 - 1850 rpm - 			prop rpm for ILS interference

2080+-40 rpm -				overspend governor activation (104% selected rpm)

2120 rpm -				fuel topping governor activation (106% rpm) part of primary Gov. use P3 air called Py air

1600 - 1640 rpm -			primary governor check

900 - 1100 rpm -				low idle

+-30 rpm -				prop sync limit

90% N1 -				autofeather armed (power lever position)

260+-50 ftlb -				autofeather energized

+18 degree -				low pitch stop (approx.)

+5 to 18 degree -				beta range

+5 degree -				zero thrust

-9 degree -				max reverse

+90 degree -				feather

If primary gov. fails two settings -		core valve fails up - oil goes out - prop feathers

core valve fails down - oil goes in - prop pushed lower setting - overspeed

Prop Sync - 				places blades 40 degrees out from one another

master prop is left slaved prop right

nose gear actuator switch controls prop sync light

center the system by shutting it off

right engine may not feather if secures in flight with prop sync still on

Add power, increase torque, decrease oil press, increase blade angle

Remove power, decrease torque, increase oil press, decrease blade angle

Prop test switch tests over speed governor

Prop reverse not ready light when gear is down and both props are not full forward

Fuel

Jet A, Jet A1, Jet B, JP - 4, JP - 5, JP - 8, & commercial 80/87 - 115/145 may be used

195 gallons -				max each main wing tank

193 gallons -				useable each main wing tank

79.5 gallons -				max each auxiliary tank

79 gallons -				max useable each auxiliary tank

247 lbs -					nacelle low light - aprox. 30 min flight time

3% -					max permissible error in fuel gauge system

31,000 ft -				max alt. with aviation gas (standby pumps operative)

20,000 ft -				max alt. with aviation gas ( standby pumps inoperative)

10 hours -				max time engine driven fuel pump with out boost pump (suction lift, can not crossfeed)

40 sec -					motive flow valve energized after engine start

15 - 45 sec -				engine flameout when firewall shutoff valve closed

100 ft -					no fueling distance from airborne radar equipment

300 ft-					no fueling distance from ground radar equipment

386 gallons -				main system useable

158 gallons -				auxiliary system useable

8 -					fuel drains (7 can be seen)

2 -					fuel vents - heated vent and siphoned main vent

14 -					fuel nozzles - 7 start and 7 secondary

2 -					number of igniters (3 & 9 position)

549 gallons -				total max capacity

544 gallons -				total usable

1000 lbs -				max imbalance

0 - 265lbs (in the yellow arc) - 		do not takeoff range

150 hours - 				limitation for engine on aviation fuel

10 psi -					low fuel pressure switch

70 F/ 21 C -       				fuel oil heater starts to close (closes off oil )

90 F/ 32 C -    				fuel oil heater fully closed

30 psi -					engine driven boost pump output

850 - 1000 psi -				main fuel pump output

If there is fuel in the aux. tank and transfer light on after pressing override - negative flow valve does not work

There is a fuel drain collector system with collector tank with an EPA pump and flip switch

Crossfeed from nacelles tank to engine on opposite side, primarily use for single engine operation, switch turns on standby pump in nacelles tank of opposite side

Fuel flow indicator DC powered

Can not suction crossfeed

The engine driven boost pump provides fuel to primary pump and used to receive fuel from the auxiliary tank

ITT

400 - 750 C - 				normal operating range

750 C - 					max continuos

1000 C for 5 sec - 			max starting

660 C - 					max low idle

725 C -					climb/cruise

700 C -					recommended climb/cruise

750 C for 1 min -				max reverse

850 C for 10 sec - 			max transient

Measured between N1 and N2 turbines - 8 thermal couples

ITT self powered

Torque

400 - 2230 ftlb - 				normal operating range

2750 ftlb for 5 sec - 			max transient

2230 ftlb -				max at 2000 rpm (max torque for other setting is same I.E. 1900 is the same)

1800 ftlb limit -				prop speed at or above 2080 rpm

1100 ftlb -				prop speed below 1600 rpm

1100 ftlb -				oil pressure between 60 & 85 psi

410+-50 ftlb -				autoignition

260 +-50 ftlb -				autofeather

Torque 26VAC powered

Turbine Tachometer ( N1 )

52 - 101.5% - 				normal operation range

52 - 55% - 				low idle, 52% min

70 - 73% - 				high idle (NATOPS states aprox. 70%)

88% for 1 min - 				max reverse

101.5% -				max cont./normal cruise

85% - 					min to maintain press on descent (single engine)

75% -					min to maintain press on descent (dual engine)

50% -					starter off (engine start)

102.6% for 10 sec - 			max transient

Above 102.6% - 				engine change

12% -					min stabilized N1 for start

15% -					consider GPU start

17% -					anticipate hot start (15 for test)

Oil

14 qts -					max capacity

9 qts -					oil tank capacity

5 qts -					oil measured on dip stick

2 qts -					low for hot engine

3 qts -					low for cool engine (can motor engine 15-20 sec to get within limits)

10 - 99 C - 				normal operating temperature

99 C -					max temperature

-40 C - 					min start

-40 - 99 C - 				low idle

-40 - 99 C - 				high idle

0 - 99 C - 				reverse

100 - 104 C for 5 min - 			transient

60 to 85 psi -				tolerate to end of flight 1100 ftlb limit

60 psi - 					min

85 - 105 psi - 				caution range below 21000ft

85 - 135 psi - 				normal operating range above 21000ft

105 - 135 psi -				normal operating range below 21000ft

200 psi - 				max oil pressure

+- 10 psi -				acceptable oil fluctuations

Main oil pump driven by accessory gear box

Oil pressure and temperature are DC

Anti-ice / De-ice

6 sec -					inflation time of wing de-ice boot

4 sec -					inflation time of horizontal stabilizer boot

-40 C -					min temp for surface de-ice boot

30 sec -					cycle time for prop heat

2 min -					completed cycle for prop heat

14 - 18 ampers - 				normal operating range for prop heat

15 C -					max temp for using ice vanes

15 sec -					max time for ice vanes to extend/retract

5 C or less -				temp require use of ice vanes

The pitot tubes are on the corresponding sides

If there is a hole in a ice boot pneumatic press. will be low

The leading edge of engine inlet is heated by exhaust gas - we have not control over this

If ice vanes are extended there will be a decrease in torque and increase in ITT and loss of about 15% range

Fuel heat consist of heated fuel vent, fuel control heat, fuel oil heat exchanger

Prop heat consists of two thermal blankets on each prop, each draws 5to6 amps 14to18 norm total, loss of one blade around 12amp

Airspeed

245 KIAS/ .47M - 			max airspeed

86 KIAS(91 KCAS) - 			min single engine control speed (clean, dead eng. windmilling, good eng. takeoff power,5)

104 KIAS - 				min safe single engine airspeed (clean dead eng. windmilling)

115 KIAS -				best single engine angle of climb (clean, bad engine feathered, good engine at 2000 rpm)

121 KIAS - 				best rate of climb - single and dual engine (clean,dead eng. featherd, 2000 rpm)

80 - 155 KIAS -				full flap normal operating range

155 KIAS - 				max full flap operating speed

200 KIAS - 				max approach flap speed

170 KIAS - 				max turbulent air penetration -maneuvering speed

4 KIAS -				max allowable difference between pilot and copilot airspeed

75 KIAS -				stall speed landing configuration 12,500 lbs

85 KIAS -				recommended touchdown speed gear up

85 KIAS -				min ground control speed

85 KIAS -				stall speed approach flaps 12,500 lbs

99 KIAS -				stall speed no flaps 12,500 lbs

100 KIAS -				ditch speed flaps full down

100 KIAS -				best two engine angle of climb

105 KIAS -				ditch speed approach flap

111 KIAS -				maneuvering speed flaps down

120 KIAS -				min single engine recommended speed on final until landing assured

139 KIAS -				max tire speed (ground)

140 KIAS -				best glide speed

140 KIAS -				min speed for icing

140 KIAS -				min speed for windmilling start

140 KIAS -				recommended speed for downwind landing pattern

160 KIAS -				max speed for manual extension of ice vanes

181 KIAS -				maneuvering speed flaps up

182 KIAS -				max landing gear extension speed

164 KIAS - 				max landing gear operating speed (max gear retraction)

13,500 ft -				max diving speed decrease 5 K for each 1,000 ft above (automatic)

155 KIAS -				climb airspeed from sea level to 10,000 ft

135 KIAS -				climb airspeed from 10,000 ft to 20,000 ft

125 KIAS -				climb airspeed from 20,000 ft to 25,000 ft

115 KIAS -				climb airspeed from 25,000 ft to 31,000 ft

Environmental

1000 psi -				min oxygen for local flight

1500 psi - 				min oxygen for X-country

0 - 6.1 psi - 				normal operating pressure range

6.1 +- .1 psi - 				max pressure differential (both safety valve and out flow valve open)	rate, alt., 6.1

-.1 psi -					min pressure differential ( both safety valve and out flow valve close)

4.0 psi - 					max cracked windshield innerpanel

2.8 to 5.9 -				normal indication for gyro suction

4.3 to 5.9 -				normal indication for sea level to 15,000 ft on gyro suction

0 - 					max cracked cabin window

3900 ft -					cabin pressure at 20,000 ft

9900 ft -					cabin pressure at 31,000 ft

12,500 ft -				altitude warning ann. light - 10 crew mask drop

200 - 2000 ft per min -			range of the cabin control selector

Aprox. 6 sec -				starboard flow valve opens after left one after take off

61 % -					min N1 for air conditioner

49 CU FT -				oxygen supply cylinder

1850+-50 psi -				max oxygen cylinder pressure at 70 degree F (changes +-3.5 psi W/deg)

12 - 20 psi -				pneumatic pressure normal range

20 psi -					pneumatic pressure max range

75% N1 - 85% single eng. -		required to maintain pressurization

18,000 BTU -				max BTU output of AC at 70% N1 - forward unit

14,000 BTU -				max BTU output of AC at 70% N1 - rear unit

32,000 BTU -				total output of AC at 70% N1

Pressurization will start immediately after takeoff from one engine and after 6 sec the second engine will kick in

Windshield heat will screw up the magnetic compass

While using the prop deice 14 to 18 amps will be indicated, if circuit breaker poped can still use in manual position, while keep both

Should have 1/2 in build up of ice before using the boots, pneumatic pressure inflated 16 - 20 - norm 12 - 20

With single engine anitice/deice still work

Do not use the windshield wipers on a dry day

Electrical

27.5 - 29 VDC - 				generator normal range

28 V/250 amp - 				generator specification

.3 to .6 -					normal load range = to 30 to 60 % or 75 to 150 V(25Vper mark)

1 VDC - 				max volt differential between generators

10% - 					max load deferential between generators

100% - 					max generator load (sea level to 31,000 ft)

85% - 					max generator load (ground operation)

24 VDC/ 34 amps -			battery -20 cells 1.2V each ( 34 amp in 1 hour or 1 amp for 34 hours)

22 VDC -				min for battery start

20 VDC -				min for APU start

115/26+-5 VAC/400+-10HZ -		max inverter limits (600 amps, single phase) -inverter light when 37V or below

30V/300 amps -				continuos load for APU

1000 AMPS for .1 sec -			peak load for APU

325 amp -				isolation limiters protecting generator buses

4 -					Dual fed busses

2 -					VHF radios

1 -					UHF radio

6 sec -					battery charge caution ann. light

52 % -					min rpm N1 - generator load 0 to 70% (no AC)

55 % -					min rpm N1 - generator load 70 to 75% (no AC)

60 % -					min rpm N1 - generator load 75 to 80% (no AC)

65 % -					min rpm N1 - generator load 80 to 85% (no AC)

Standby pump  are powered by Hot battery bus and sub busses 4

The battery caution light comes on after start and should extinguish within 2 to 5 min - if not do load meter check

Battery is used for - start, emergency power, and shock dampening

The APU connects on the right so when doing an APU start you would start the left engine first

Can reset all but the feeder circuit breakers - short is indicated

If current limiter fails the load will remain the same unless something on one of the generator busses is on

Loss of a generator causes the other sides load to double up to 1.0 then battery will pick up the rest

Two reasons for split load - loss of generator and generator communication box not communicating

The inverter light is on the 115VAC circuit, to test if 26VAC circuit is good must check torque

The generator control box control - high V, low V, reverse current, and regulation of polarity

Items on the hot battery buss are - 		L & R standby fuel pump - dual powered

L & R firewall fuel shutoff valves - dual powered

Door latch inspection light

L & R engine fire extinguishers

external power sensing

entry lights

battery relay

cabin cargo door sensing

baggage compt. light

RMI ‘s are repeaters of opposite side

Must have avionics master switch on for radios to work

If avionics master switch fails, pulling the circuit breaker will get it back

Fuel flow and oil, oil temp/press are DC ( total electrical failure includes battery)

N1, ITT and Prop rpm are self powered

Standby gyro and copilot turn needle are vacuum powered - will still have for IMC in the event of total electrical failure

RMI, HSI, Alt and torque are AC powered

AC points - DC tunes

AC lies - DC dies

Airframe

54 ft 6 in -				wing span

43 ft 10 in -				length

14 ft 6 in -				height ( at rest )

12,500 lbs - 				gross weight, 13500lbs (may be authorized)

10,400 lbs - 				max zero fuel

12,590 lbs - 				normal category taxi

13,590 lbs - 				restricted category taxi - max ramp weight

12,500 lbs - 				normal category takeoff

13,500 lbs -				restricted category takeoff

12,500 lbs - 				normal category landing

12,500 lbs - 				restricted category landing

200 lbs per square foot - 			max cargo section on seat tracks

100 lbs per square foot - 			max cargo section between seat tracks

410 lbs - 				max baggage compartment

300 lb- 					max airstair door - 1 person at a time - light for check position is on battery bus

181.0 FOR - 196.4 AFT -			C.G. limits below 11279, (185.0 FOR at 12500 & 188.3 FOR at 13500)

Emergency door opens inside and outside unless locked then just inside

Altitude

31,000 ft - 				max operating range

20,000 ft - 				max emergency operation with aviation gas and standby boost pump inoperative

25,000 ft - 				max operation unpressurized cabin

17,000 ft - 				max operation with inoperative yaw damp

10,000 ft -				max alt. with cracked cabin window

20,000 ft -				max alt. for windmilling air start

5000 ft -					min alt. for stalls

4000 ft-					min alt. for practicing single engine (fuel cutoff or prop feather)

200 ft -					min alt. for single engine waveoff

200 ft -					min alt. for yaw damp.

200 ft -					min alt. for auto pilot

Acceleration

3.17 G - 				max positive clean

-1.27 G - 				max negative clean

2.0 G - 					max positive full flap

2.0 G -					normal positive

0.5 G -					positive acceleration that will result in loss of oil pressure

+2 to 3 G deceleration -			shoulder harness will lock - Seat will move up, down, forward and aft

2.0 G for .01 sec -			emergency lights will activate from a shock impulse along the longitudinal axis

3 to 7 G -				activation of ELT from impact - good for 48 hours, self powered

Flight Controls

Must be flared landing

300 fpm - 				max sink rate

25 K - 					max x-wind component

10 K - 					max x-wind component icy runway

5 K - 					max x-wind component soft field

40 K -					ground speed for reverse ( reduce prop erosion )

75 K -					when structural damage could occur while secured

139 KIAS (160 mph) - 			max tire speed

14 degree -				approach flaps (40%)

34 degree - 				down flaps (100%)

60 psi -					differential pressure for actuation of rudder boost

3 - 6 degree -				max asymmetry of flaps - can get a split between sides

4 - 12 K above stall -			stall warning (up,app.,down)

4 - 6 sec -				landing gear extension (NATOPS states aprox. 6 sec.)

5 - 7 sec -				landing gear retraction (NATOPS states aprox. 6 sec.)

79 - 82% N1 -				landing gear warning horn activation (can only silence if flaps are up or approach)

70 -					number of strokes that may be required to emergency extend gear

3 in -					min nose strut extension

5.5 in -					min main strut extension

48 degrees -				max deflection of nose wheel either direction - 96 degrees total

14 degrees -				nose wheel deflection with use of nose wheel steering only

25 up / 17 down - 			degree of aileron deflection

20 up / 14 down -				degree of elevator deflection

15 either -				degree of ruder deflection

55 - 60 psi -				nose wheel tire pressure

60 - 64 psi -				main wheel tire pressure

1 US PINT -				hydraulic brake system capacity

100 fpm -				recommended sink rate for ditching

75 ft -					max alt. difference between field elevation

40 ft -					max alt. difference between pilot and copilot

10 persons -				max capacity of life raft

15 -					max number of toiler flushes before servicing

14 hours -				max crew day with Autopilot

12 hours -				max crew day without Autopilot

1000 ft prior -				alt. warning of alt set - system DC powered

300 +-50 ft -				alt. alert sounds if deviation is greater than

100 +-20 ft -				Radalt. test

50 to 2450 ft -				Radalt. operation - system DC powered

50 to 2450 ft -				GPWS operation

25 lbs -					rudder pedal pressure from rudder boost

200 NM -				max range of radar

+_ 12 degree -				tilt up and down of radar

45 degrees either side -			horizontal tilt of radar

6 ft -					min distance for ground personal with radar on - do not turn on while fueling

Min crew -				training - pilot and copilot

transport mission - pilot, copilot, and crewman ( may be waived by CO)

functional check flight - pilot, copilot, and crewman as appropriate

Aileron trim tab is on left aileron - trim as adjusted

Elevator trim tab is on both sides - trim as adjusted, split to reduce chance of runaway, pilot can override

Ruder trim is antiservo

You must shut the yaw dampener off for takeoff and landing because the servo will bind the rudder

For the rudder boost to work both bleed air switches must be open or environmental/off

The two reason for light in the gear handle are - warning system activated and for a test

The flap limiter switch is on the right inboard flap

The landing gear are high floatation gear and stick out 5 in

Three area DME is located is pilot copilot HSI and the Sierra window also containing ground speed and time - only if to station

Pilot HSI on NO 1 system and RMI on NO 2 system

You will get an AP disconnect light if you trim against system, copilot disconnects, or you pull the circuit breaker

The tacan contain 126 channels and two bands X/Y making 252 total

squat or WOW switches-			Left - PAASS

opens safety valve (can tell if not work by door push button against bellows)

closes ambient air solenoid

minimizes heat  on stall heater

disables stall warning

disables pressure control

Right - GHD

disables landing gear motor circuit

disables flight hour meter

activates solenoid operating downlock hook

When using the alternate air static source the pilot is only affected

The altitude encoder is on the pilot side

The pilots turn needle is DC powered

Cannot manually raise gear

Only the VOR 2 can be set up on both sides

The ADF will park CCW at the 90 position if loss of signal accrues

Prohibited Maneuvers

Abrupt control movements

Unusual G loading

Bank angle greater than 50 degrees - normal category

Bank angle greater than 35 degrees - restricted category

Pitch attitude greater than 30 degrees above or below the horizon

Intentional spins

Actual practice landing with a prop feathered or engine secured

Single engine waveoff below 200 ft agl

shutting down an engine for practice below 4000 ft