# CAT II APPROACH

## DOWNWIND – 100% APP + 60 KTS

- AFCS ON, ALT HOLD & HORZ NAV ON

- TEST RADAR ALTIMETERS

- APPROACH CHECK FOR CAT II APP

- ADVISE APP CON OF  CAT II APP

- NSP VOR/ILS AND HDG SELECTED

- SELECT AND TUNE ILS FREQ

- HSI ILS COURSE SET

BASE – 100% APP + 20, 170KTS MAX

- BEFORE LANDING CHECK COMPLETED

- 40% FLAPS, GEAR DOWN

DOGLEG – 100% APP + 20, 170 KTS MAX

- NSP, DESELECT HEADING

- 170 KTS, BUT NOT LESS THAN APP + 20

- START AUTOLAND CHECKLIST

LOCALIZER CAPTURE – 100% APP + 20 MAX

- VERT NAV ON

- CRAB ANGLE CHECKED 4 DEG DIFF,

### IF NOT CHANGE CP’S TO = PILOTS

- FLAPS LANDING APPROX ¼ DOT BELOW GS

## GLIDE SLOPE INTERCEPT

- X-CHECK GS ALTITUDE AT FAF/OM

- TIMING IF REQUIRED TO MAP

- AUTOLAND PUSHBUTTON DEPRESSED,

### ON GS ABOVE 700’ RA

- GREEN LIGHTAUTOLAND CHECK COMP

# CAT II APPROACH

## AT 1000’ RA

- VERIFY CONFIGURATION

BY 500’ RA

- AUTOLAND LIGHTS ON

- CROSSCHECK RADAR ALTIMETERS

- CROSS CHECK ALTIMETERS

- CHECK ADI DEVIATION INDICATORS

- NO WARNING FLAGS OR LIGHTS

- CHECK AFCS INDICATIONS

- CHECK AIRSPEED/RATE OF DESCENT

- CHECK CRAB ANGLE ON ADI’S

200’ RA

- CP CALL 100’ ABOVE

- CHECK AIRSPEED + /- 5 KTS

- CHECK ILS + / - ½ DOT

- CP OUTSIDE

DH

- LAND OR GO-AROUND

- DISCONNECT AUTOPILOT/ATS