CDS

Pre Slowdown (LM)

- Personnel seated forward of load

- Checks CDS arming switches (245) are in normal position

- Checks forward barrier (BSA for 5001 lbs loads or 10Klbs chains)

- Checks ADS ramp support arms connected

- Checks rigging (including retriever cable and knife)

- Removes vertical restraint (if no CVR)

- Red light - On

- Don’s helmet & restraint harness

Slowdown

- Open ramp and door (if doesn’t open from front, LM climbs over

- loads to his control panel)

- Checks static line retriever cable compression spring

- Arms CDS switch (checks other in normal position)

Drop and Completion of Drop

- “Green Light”

- Static line retreiver rewinds for 3 seconds

- Gates are cut by knife

- LM sees gate cut then de-arms CDS switch

- LM calls “Load Clear” or the condition

- NOTE: If gate fails to cut, LM can manually cut if prebriefed.

- Red Light - On, resets flaps to 50%

- LM manually retrieves the static lines

- LM clears Engineer to close ramp and door

No-Drop

- LM Acknowledges

- De-arms CDS switch

- Clears closing of the ramp and door

- Accomplished completion of drop checklist

NOTE: If drop reattempted, begin with pre slowdown checklist

EMERGENCY

- Extend additional flaps and lower the nose

- Maintain Drop airspeed and AGL

- Avoid flying over or upwind of water or built up areas

- Raise aft anchor cable (if required), Close ramp and door

- Secure load

- Completion of drop checklist