# CIRCLING APPROACH

- VISUAL FLIGHT MANEUVER

- USED TO ALIGN THE AIRCRAFT WITH THE LANDING RUNWAY

- 40% FLAPS WITH GEAR AT 100% APP + 20, UNTIL THE FINAL TURN

- CIRCLING MDA AND WX MINIMUMS USED ARE FOR THE RUNWAY TO WHICH THE APPROACH IS BEING FLOWN, NOT NECESSARILY THE RUNWAY OF INTENDED LANDING

- APPLIES ONLY TO NON-PRECISION APPROACHES

- CIRCLING IS NOT COMPATIBLE WITH PRECISION APPROACH PROCEDURES

- DO NOT PLAN TO CIRCLE FROM A PRECISION APPROACH

- CONTROLLER WILL SPECIFY THE DIRECTION OF THE CIRCLE WITH RESPECT TO THE RUNWAY OR THE AIRFIELD

- OBSTRUCTION CLEARANCE AREAS ARE DETERMINED BY AIRCRAFT CATEGORY

- AT MDA DETERMINE IF THE CEILING AND VISIBILITY ARE SUFFICIENT FOR THE CIRCLING APPROACH

- MDA IS THE MINIMUM ALTITUDE, A HIGHER ALTITUDE MAY BE MAINTAINED THROUGHOUT THE MANEUVER

- IF WX PERMITS, YOU MAY FLY THE CIRCLING APPROACH AT A HIGHER ALTITUDE THAN THE MDA UP TO VFR TRAFFIC PATTERN ALTITUDE

- MANEUVER TO KEEP AS MUCH OF THE AIRPORT ENVIRONMENT IN SIGHT AS POSSIBLE

- DESCEND BELOW CIRCLING MDA ONLY WHEN THE AIRCRAFT IS IN A POSITION TO INTERCEPT A NORMAL GLIDE PATH AND EXECUTE A NORMAL LANDING

- IF ANY DOUBT THAT THE AIRCRAFT CAN BE MANEUVERED TO THE LANDING RUNWAY EXECUTE A MISSED APPROACH

- MISSED APPROACHTURN TOWARDS THE RUNWAY AND EXECUTE THE MISSED APPROACH PROCEDURE FOR THE RUNWAY TO WHICH THE APPROACH WAS FLOWN