COMBAT OFFLOAD BRIEFING GUIDE

1.  Review Combat Offload Checklist, Airfield Diagram In-Flight

2.  Run After Landing Ops Stop Checklist

3.  Place Nose Into Wind If Able During Dirt Ops

4.  Set Parking Brake

5.  Co-Pilot: Brakes, Radios, Interphone, Taxi & Landing Lights (night)

6.  Safety Observer:  On Headset at Steps (FS 245), Use PA Mic at FS 245

7.  Engineer:  Monitor Engines, Lighting (night)

8.  Loadmaster:  Brief Drivers, PAX, Form-F, Call Ready for Taxi

9.  Loadmaster Must Remain in Interphone Contact with Crew

10.  Loadmaster Only One Authorized to Work Dual Rail Locks

11.  Run Before Takeoff & Line-Up Checklists

# IMC REJOINS

- Set Leader Number to Join at End of Formation.

- Enable All Formation Slot Numbers.

- Set Range Switch at 16 x 1000.

- Approach Formation from 1000 ft. Above/Below Formation Altitude.

- Establish Radio Contact / Confirm SKE Frequency and Leader’s Slot #.

- When Formation appears on PPI, check MASTER LOST Ind. Extinguished.

- Identify Appropriate Element Leader.

- Upon Positive Identification, Lead Completes FCI Check Prior to Rejoin.

- Maintaining 1000 ft. Altitude Separation, Close to Position.

- When Stable in Position and Last Formation Aircraft is Positively Identified, Request Rejoin Clearance from Lead.

- When Cleared to Rejoin, Climb/Descend to Formation Altitude.