DEPARTURE PLANNING

CONDITION					RULE

T

A				The gross weight must never exceed

K				that which would result in a critical

E				field length in excess of runway

O				available.

F

F

| CLIMBOUT IMC |  |
| --- | --- |
| No minimum climb rate specified in the departure procedure: | Aircraft must be capable of climbing 152 feet per nautical mile on three engines. |
| A minimum climb rate is specified in the departure procedure: | Aircraft must be capable of climbing at the minimum climb rate specified in the departure procedure on four engines plus clear vertically all obstacles in the climb out path on three engines. |
| CLIMBOUT VMC |  |
| If climbout cannot be made using the above criteria and the weather is VMC (3 miles visibility minimum) the AC may elect to use a see and avoid concept for obstruction clearance. Do not use see and avoid at night unless in the AC’s opinion adequate illumination exists to positively identify the obstacle(s). Even when obstacles are not a factor, the aircraft must be capable of climbing at a rate of at least 200 feet per minute on three engines at obstacle clearance speed or Vmca. |  |