# DUAL TR FAILURE

## WITH NO LOSS OF AC POWER

PILOT INDICATIONS

- SLAT INDICATORINTRANSIT

- FLAP INDICATOR11 O’CLOCK POSITION

- GEAR INDICATORSINTRANSIT

- HIS,ADI,BDHIWARNING FLAGS IN VIEW

- NO ANNUNICATOR OR MASTER CAUTION LIGHTS

- AFCSINOPERATIVE

- NORMAL AND ALTERNATE PITCH TRIMINOPERATIVE

BUSSES LOST

- MAIN DC #1 AND #2

- AVIONICS DC #1 AND #2

- ISOLATED DC

- AVIONICS ISOLATED DC

FE INDICATIONSBATTERY LIGHT AND ISO DC OFF

INSTRUMENT POWER SWITCH TO EMERGENCY RESTORE 2 ISOLATED DC BUSES

LOSE NAV AC BUS #1

- AUTOPILOT

- BDHI’S

- INS#3 EXITATION

- TACAN’S

- ADF’S

SYSTEMS AFFECTED

- LANDING GEARBUTTON DOWN

- THRUST REVERSERSINOPERATIVE

- CANNOT DISCONNECT CSDMUST SHUT DOWN ENGINE TO DO