# EMERGENCY BOGIE ROTATION

- MLG EXTENDS TO DOWN AND SIDE BRACE LOCKED (RED WHEELS) BOGIE DOES NOT ROTATE

- FLAPS 40%, 100% APP + 30KTS

- MLG MAY FAIL TO ROTATE ABOVE 185 KTS WITH NORMAL HYDRAULIC PRESSURE AVAILABLE

- MLG WILL NOT ROTATE ABOVE 170 KTS USING ACCUMULATOR PRESSURE

- AFT MLG CAN BE ROTATED USING THE EMERGENCY CONTROL SWITCHES

- FWD MLG CAN BE ROTATED USING THE EMERGENCY OUTBOARD POSITIONING BUTTONS

IF YOU LEAVE THE EMERGENCY EXTEND SWITCHES IN THE EXTEND POSITION, NORMAL BRAKES, ALTERNATE BRAKES AND CASTOR WILL NOT BE AVAILABLE

BRAKE SYSTEM CHECK (INFLIGHT) 3-115

- DO AFTER EXTENDING THE MLG WITH THE EMERGENCY EXTEND SWITCHES

- DO AFTER A MLG CB IS OPENED

- DO AFTER USING THE MLG RETRACTION PROCEDURE IN SECTION 2C

- DO NOT DO A BRAKE CHECK

## EMERGENCY BRAKES SELECTED

- IF BOTH #1 AND #4 HYDRAULIC SYSTEMS ARE LOST

BOGIE PITCH ANNUNICATOR LIGHTS

- ONE LIGHT FOR EACH AFT MAIN GEARANNUNCIATOR PANEL

- COMES ON WHEN BOGIE PITCH IS GREATER THAN 15 DEGREES NOSE DOWN OR 11 DEGREES NOSE UP AND THE SIDE BRACE IS LOCKED

- MAY COME ON WITH THE MLG UP AND LOCKED TO WARN THAT AN ERRONEOUS SIDE BRACE LOCKED SIGNAL EXISTS AND STRUCTURAL DAMAGE COULD OCCUR IF THE GEAR HANDLE IS PLACED IN THE DOWN POSITION