# EMERGENCY GEAR EXTENSION

## EMERGENCY MLG EXTENSION – NORMAL HYD PRESS NOT AVAILABLE

- FLAPS 40%, 100% APP + 30 KTS

- LANDING GEAR MAY FAIL TO ROTATE WITH AIRSPEED GREATER THAN 185 KTS

- LANDING GEAR WILL NOT ROTATE AT SPEEDS GREATER THAN 170 KTS USING ACCUMULATOR PRESSURE

- EMERGENCY EXTEND SWITCH TO EXTEND BARBER POLE WITHIN 5 SEC

- IF #1 AND #4 HYDRAULIC SYSTEMS ARE DEPRESSURIZED THE FWD MLG USE THE LEFT APU ACCUMULATOR AND THE AFT MLG USE THE RIGHT APU ACCUMULATOR

- WITH LOSS OF HYDRAULIC PRESSURE #1 AND #4 INSURE APU ACCUMULATOR PRESSURE OF 3000 PSI

- ACCUMULATOR CAN BE PRESSURIZED WITH A HAND PUMP

- ALTERNATE HYDRAULICS, #1 FOR FWD MLG AND #4 FOR AFT MLG, TO UNLOCK DOOR LOCKS AND ROTATE THE BOGIE’S USING THE EMERGENCY BOGIE ROTATION ACTUATORS

- ELECTRICAL POWER TO LOWER THE GEAR IS SUPPLIED BY THE AFT MAIN AC #1L FWD MLG, AC #2R FWD MLG, AC #3L AFT MLG, #4R AFT MLG

EMERGENCY NLG EXTENSION – NORMAL HYD PRESS NOT AVAILABLE

- FLAPS 40%, 100% APP + 30

- EMERGENCY SWITCH TO EXTENDBARBER POLE WITHIN 10 SEC

- WITH LOSS OF HYDRAULIC SYSTEMS #1 AND #4, HYDRAULIC PRESSURE IS SUPPLIED BY THE CREW ENTRY DOOR ACCUMULATOR

- CREW ENTRY DOOR ACCUMULATOR MUST HAVE 2500 PSI FOR SUCCESSFUL EXTENSION

- ALTERNATE HYDRAULIC PRESSURE FROM #4 SYSTEM IS USED TO UNLOCK THE EMERGENCY DOOR LOCK ACTUATORS, THE EMERGENCY GEAR LOCK ACTUATOR TO UNLOCK THE GEAR AND TO LOCK THE DRAG BRACE IN THE EXTEND POSITION

- ELECTRICAL POWER TO THE EMERGENCY EXTEND MOTOR SUPPLIED BY FWD MAIN AC NO2