# EMERGENCY SWITCH ON LIGHT

- WILL COME ON WHEN NLG EMERGENCY EXTEND SWITCH IS PLACED TO EXTEND

- FROM AN UP AND LOCKED CONDITION WHEN THE FIRST NLG DOOR REACHES THE FULLY OPENED POSITION

- IF THE NLG RH FWD NORMAL DOOR LOCK ACTUATOR IS IN THE UNLOCKED POSITION WHEN THE SWITCH IS PLACED TO EXTEND, THE EMERG SWITCH ON LIGHT WILL COME ON WHEN THE NLG IS DOWN AND LOCKED

- THE LIGHT WILL REMAIN ON UNTIL THE EXTEND SWITCH IS RETURNED TO NORMAL POSITION

- THE LIGHT WILL ALSO COME ON WHEN ANY OF THE MLG EMERGENCY EXTEND SWITCHES ARE PLACED TO EXTEND, WHEN THE AFFECTED MLG IS DOWN AND LOCKED AND THE CROSSWIND POSITIONING COLLAR IS ENGAGED

- THE LIGHT CAN ALSO COME ON WITH THE NLG UP AND LOCKED, TO WARN THAT AN ERRONEOUS NOSE GEAR DOOR OPEN SIGNAL EXISTS WHICH COULD CAUSE STRUCTURAL DAMAGE IF THE EMERGENCY EXTEND SWITCH WERE TO BE OPERATED

BEFORE EXTENDING THE NLG OR USING THE EMERGENCY NLG EXTENSION – EMERGENCY EXTEND LIGHT ON PROCEDURE (3-113) HAVE THE FE GO TO MADAR AND FOLLOW THE ROUTINE

FE WILL HAVE THE SCANNER CHECK THE LIGHTS