SYSTEMS TRAINING

Revised 18 FEB 99

All information in this handout was taken from the 1C-130E(H)-1 and Lockheed manuals. If some limitations or operation appears to be different this was a Lockheed limitation. Consult your -1 if a question arises. Information in this handout is for training only and is not to be used for systems operation or to

replace any manuals or operating instructions.

# GROUND TRAINING 01- 03								Notes

A. Characteristics of the Fuel System

1. Components of the fuel system

a. Tanks

1) Main tanks - integral type                	Outbd tanks have 3 baffles and a percolator tube to prevent excessive surge 															pressure on the bulkheads during wing-down and slide-slip maneuvers.

2) Auxiliary tanks - bladders				3 cells (Red, blue, and green dye to detect leaks)(2 drains).

3) External tanks							2 types of tanks.

a) Lockheed - 							Removable tailcone (pump access).

b) American -							Coupling bands (pump access).

4) Fire retardant foam						In all fuel tanks (dark gray).

5) Pump switches							All main, external , aux  & dump pump switches when turned to the ON

position 115 V, 3 phase, 400 HZ Ac power is supplied to the pumps. The 																switch also operate DC circuits for crossfeed valves, dump valves, etc.

b. Boost pumps									3 phase AC powered pumps.

1) Main tank pumps						15-24 psi. (Internal thermal switch 375F, if opened pump is to be replaced)

a) Low pressure switch				8.5 psi. Pressure comes from the fuel heater strainer.

b) location								RH Aft side of  engine nacelle.

2) Auxiliary tank pumps					28-40 psi. (Internal thermal switch 375F, if opened pump has to be

replaced)

a)Tank empty light						23 psi.

b) location								Switches, # 2 & 3 dry bays.

3) External tank pumps					28-40 psi. (Internal thermal switch 375F, if opened pump has to be

replaced)

a) Tank empty switch					23 psi.

b) location								Switches, # 2 & 3 dry bays.

4) Power sources							Main tanks -  # 1- LH AC.

# 2 -ESS AC.

# 3 -MAIN AC.

# 4 -RH AC.

AUX tanks -  Main AC.

Ext. tanks -   LH Fwd pump- LH AC.																									       		    			 RH Fwd pump- RH AC.

AFT pumps- Main AC.

5) Surge box								Located in the aft inbd corner of each main tank. Four flapper valves in 																	the flow bottom allow fuel to flow into the box, and two in the top to allow

fuel to out when the box is full. The boost pump is also located in the surge																box, this is to ensure a continual supply of fuel for the boost pump demand.

6) Pressure transmitter / location	AC inst. & fuel control bus, # 3 dry bay, fwd end.

c. Quantity Indicating System

1) Probes										Capacitance  probes, Mains in top of wing, with new wings.

2) Quantity indicators / power sources		AC Inst. & Fuel control bus, press to test provides ground, indicator  																		move toward zero. Totalizer connected to each indicator. When tank

quantity CB is pulled amount is subtracted from totalizer.

3) Balance limits								Outbd to inbd - 500/1000, wing to wing 1500, Except Aux, External

1000, Aux one full other empty if other tanks symmetrical.

4) Dipstick usage									Stick has marking for inbd and outbd mains. Aux and Ext cannot be dipped.

5) Auxiliary magnetic sight  gauge				Underside of wing, reads 500-5900 in 500 lbs increments.

d. Vent system

1) Wrap around										# 2 & 3 Mains and Aux

2) Float - controlled vents / TCTO 1229		#1 & 4 Mains, Note Ext vent system in fwd top of tanks

3) Anti - siphon loop								Vent system for the external tanks. Vent line is in the front of the tank and 																runs  through the pylon between the tank and wing to the aft section of the 																wing .

4) Atmospheric vents								To equalize pressure at all times.

a) location										Note location on bottom of wing on walk around and schematic.

b) Fuel tank fires								Can be indicated by white soot coming from vents.

Note: Do not confuse with oily looking spot around vents with JP-8 fuel.

5) Water removal system / vent tanks			Low points of tanks, Boost pump, ejectors, nozzles TCTO-1039.

emptied											Aids in max usable fuel, positive fuel supply, Vent tank vent system

e. Crossfeed manifold

1) Tank to engine 									For all takeoffs, unless main pump inoperative.

2) Gravity feed										Can be used when pump fails on main tanks only, -1 restrictions.

3) Crossfeed operation							Used to allow aux and external fuel to be utilized for flights.

4) Crossfeed, bypass, separation, 				Crossfeed separates left and right wing fuel system, Bypass used

and primer valves								if aux or external crossfeed fails.

a) Operation											Press to deplete pressure in crossfeed manifold.

b) Power source									ESS DC.

c) Location											Crossfeed separation valve center dry bay; primer valve in  # 1 																			dry bay, drains to # 2 main tank; bypass valves  in # 2 & 3 dry bays.

f. Dump system											Two types, (61-2358 / 62-1866 w / o X valves)

(63-7764 and up with X valves)

1) Manifold

a) Dump mast									Located on each wing tip

b) X- valves										Located in # 1 & 4 dry bays, valve has tag with “X” for identification.

2) Pumps											 	8 Dump pumps

a) Power source								Main AC bus,( normally # 3 generator).

b) Operation

1 Fuel remaining in tanks					2100 each outbd, 1800 each inbd.

2 External /auxiliary crossfeed			Go through bypass  valves. (Discuss schematic)

valve failure

3 ATC and -1 restrictions					Notify ATC of dumping, 5000’ above terrain, not in a circle.

4 Alternate method / pump or 			Go through bypass valves. (Discuss schematic)

valve failure

5 Inoperative indicator						Start dumping first, confirm visual dumping. (verify with loadmaster)

6 Dumping chart							Section 3 in -1 for different  # of operating pumps.

g. Refuel, Defuel, and ground transfer

1) Engineer duties- 55-130 chapter 6 & 10

NOTE

Students not required to memorize refuel, defuel or ground transfer procedures. All items are addressed in the

job guide( including grounding) and MUST be used for all fueling operations IAW 55-130.

2) Dash one manifold schematic in -1			Compare the two different types of systems

a) Integral part of the dump manifold			Incorporates refuel, defuel and dump manifold in one(63-7764 and up)

b) Need for X- valves							To prevent dumping fuel during refuel / defuel, ground transfer

3) Over the wing capabilities						IAW 1C130 B 2-2, section 3 (note when using JP5 & 8)

4) SPR panel

a) Power Source								Fuel totalizer gage, 115 V AC, Inst. And Eng. Fuel Cont.

b) Switches										Main DC

c) Indicators										115 V AC, Inst. And Eng. Fuel Cont.

d) Servicing										IAW 1C130 B 2-2, section 3, Also discuss JP-8 fuel quantity in tanks.

h. Fuel management

1) All areas in the -1 of section 7 will be covered and the student will  be expected to have proficiency knowledge  of				 correct configuration  for the  following:

a) Takeoff and landing							Tank to engine for takeoff, and X-feed from aux or external for landing.

X feed separation to be closed for all  T/O and landings.

b) Touch and go								Can be on X-feed from aux or external.

c) Stop and go									Can be on X-feed from aux or external.

d) Full stop and taxi back						Tank to engine.

e) Fuel usage

1) Long range								Use Aux first

2) Short range								Use Ext first

f) Fuel tank trimming							Discuss different ways

i. Limitations                                           			SEE LIMITATIONS SHEET ALSO (IN BACK)

FUEL DISTRIBUTION

Symmetrical tanks										1000 lbs. maximum

Wing – to – wing											1500 lbs. maximum

Outboard – to – inboard									500 – 1000 more outboard

External tanks											1000 lbs.

Aux tank diff												No restrictions (If mains are symmetrical)

FUEL SYSTEM

Aux/ Ext tank pump pressure							28 – 40 psi

Main tank pump pressure								15 – 24 psi

Low – pressure light										8.5 psi

Tank – empty light										23 psi

j. Emergency procedures								IAW section 3 of -1

NOTES

## B. Characteristics of Primary AC electrical System

1. Sources

a. Engine generators							4 / rated at 115/200 V 400 CY 3 PH 40 KVA

b. ATM generator								1 / rated at 115/200 V 400 CY 3 PH 20 KVA, 30 KVA with cooling

c. External AC power							200/115 V, 3 PH, 400 HZ, 40 KVA, ABC phase

2. Components and  location

a. Generators

1) Engine-driven							Aft side of reduction gear box

Max load 1.05

2) ATM										Above the LH MLG wheel well

a) Cooling - with fan					Max load 1.0

b) Cooling- W/O fan- inflight         	Max load 1.0

c) Cooling- W/O fan- ground  	    	Max load .66

3) Power distribution

a) LH AC									# 1 generator

b) Essential AC							# 2 generator

c) Main AC								# 3 generator

d) RH AC									# 4 generator

4) Disconnector's							K -1 / K-9 Relays (Aft side 245)

b. AC Distribution System

1) Generator control panel			5 under flight station.

2) Contactor relays / function			7 relays in each panel.

1. Generator control relay- controls circuit for exciter field.

2. Overvoltage relay- trips GCR when average 3 ph voltage is 131/135 v.

3. Differential protection relay- trips GCR when feeder line occurs .

4. Lockout relay- Prevents cycling until reset is used.

5. Auxiliary control relay- Turns on GEN OUT light  when energized.

6. Time delay relay- Allows 1.8 sec delay when undervoltage sensed.

7. Undervoltage relay- Open when one phase drops below 70 volts.

3) Frequency sensitive relays		 Aft side 245 (5 ea.)(Prevents use of generator in low freq. output).

ATM FSR has extra wire connected to start circuit to hold ATM on line when

accomplishing self contained start when freqs drop below 368 cps.

1. Opens at 368 cps or less.

2. Closes at 380 cps or more.

4) Generator contactors				10 ea, heavy duty contactors automatically connect four AC buses to the 																 operating generators.

5) AC power distribution				K1 - K4, Generator 1,2,3,& 4 generator contactors

6) Bus tie contactors					K5 - K8, Bus tie contactors, K10 - ATM, always powers ESS AC bus if on.  						/ bus switching  priority

c. External AC power

1) Contactor									Aft side 245 (K-9) (10 heavy duty relays K-1/K-10)

2) Phase sequencing relay				Aft side 245  Check for A, B, C phase sequence

Ext. Power can't be applied if not correct.

d. Switches, Controls, and indicators

1) Controls

a) Overhead electrical control		5 control switches, Reset, Off, On, Trip Positions,

panel									5 Gen Out lights / 4 AC bus off indicator lights

b) Generator control panels			5 under flight station on electrical rack

2) Switches

a) Generator								5

b) AC External power					1 switch, auto off if any Gen switch turned on.

c) AC voltmeter, frequency meter, 	When selected monitors Gen volts, freqs & load.

phase selector, and frequency

and voltage selector

d) Generator disconnect				28 V ESS DC power (Hold Gen disc switch 1-2 seconds

(some airplanes)

e) Generator disconnect test			Two position switch	 (OFF, TEST GEN DISC)

(some airplanes)					Checks all 4 disconnect circuits fusible links

3) Primary AC indicators

a) Generator out indicator light		4  / 28 V ESS DC indicator lights, Illuminates when power indicator relay

opens, Generator contactor opens, Engine in LSGI, or Generator

disconnected, or Low voltage output.

b) Bus off indicators					4 relays, rectifiers change AC to DC for relay operation

c) AC external power -on				Requires proper phase sequence.

indicator light / switch				E pin supplies interlock voltage through EXT AC interlock CB to FSR

d) Generator disconnect fired 		When fired melts fuse portion of plunger, drops down into the disconnect			       		indicator light (some airplanes)	 mechanism and shears shaft of the generator.

C. Secondary AC  Electrical System

1. General description								2 DC inverters. 28 volt dc motor driven generators.

2. AC Instrument and Engine Fuel				Under flight deck.

Control System

a. AC instruments inverters					3 phase but wired to produce single phase

115 V AC, 400 CPS, 2500 ampere unit

b. Regulator's									Motor speed determines output frequency and regulated by

controlling motor field current

c. Inverter power relay							Connects to the ESS DC bus when motor reaches 80 volts

# 1 power relay energizes

d. Automatic function							Changes from DC to AC if inverter malfunction occurs

e. # 1 step down transformer					26 V AC power for # 3 & # 4 engine oil and gear box oil pressure,

fuel pressure indicator, booster hydraulic and emergency brakes.

f. # 2 step down transformer					26 V AC power for # 1 & # 2 engine oil and gear box oil pressure,

hydraulic pressure indicators for normal brake, utility, and ramp,

utility & booster rudder boost hydraulic pressure.

g. Single phase AC								115 V AC for fuel temperature control (TD	 system), fuel flow,

engine torque meters, fuel quantity indicators, LOX indicator,

TIT indicators, Standby power failure relay.

3. Copilots AC Instrument Inverter System		     Power for pilots & copilots gyro system

a. Copilots AC instrument inverter			ISOL DC power to operate a motor to produce 115/115 V AC

400 cycles 3 phase, 250 volt ampere unit.

b. Copilot's inverter power relay				Inverter under flight deck , relay behind LH AC CB panel.

c. Transformer									Located behind LH AC CB panel

4. Switches, Controls and Indicators

a. Switches										2 switches

1) Copilot's AC Instrument				3 position switch , inverter, off ,standby.

2) AC Instrument and Engine Fuel		3 position switch , inverter, off ,standby.

control

3) Voltage and Frequency Selector		"A, B" Monitors CP inst. bus in DC position.

"C" monitors AC inst. bus in DC position.

4) Phase selector							A B/C

b. Indicators

1) Voltage and Frequency meter		      Monitors  115V AC 400 HZ.

2) Select power-out lights					Isol DC powered, come on if AC operation is lost.

Cross check OFF flags in TIT indicators and Gyros.

D. DC Electrical System

1. General description								Normal power is 4 TR units. Changes 3 phase AC to 27.5 volts DC.

Current between 10-200 amperes.

2. Sources

a. Battery											Used for starting GTC and Emergency power for DC system.

24 V DC battery, SCNS battery same, New maintenance free batteries.

b. External DC power							28 Volt DC 400 ampere, 2 EXT power relays in battery compartment

c. ATM generator								Powers ESS AC bus which will power ESS DC

d. Engine driven generator					Normal operation # 2 & # 3 generators will power ESS & Main DC buses

3. DC components and locations

a. Battery relay									Pilots lower CB panel, Current limiter below relay (200 ampere )

b. Touchdown switch							Auxiliary touchdown switch controls DC bus tie control ,GTC control

c. Reverse current relays						The RCR is a combination of relays, allows current in one direction

as a normal relay, but opens the circuit with sufficient flow of reverse

current. 200 ampere relays .

d. Transformer - Rectifiers	(TR) 			200 volt ac phase to phase voltage units. Two ESS and Two Main.

Powered from ESS AC and Main AC buses, 3 CBs for each TR from

respective bus. 4 bleeder resistors to keep positive load on TR to prevent  															      overheating.

e. DC buses										Main, ESS, Isol, Battery  Buses connected by RCRs.

Main to ESS is a 600 ampere relay, ESS to ISOL is a 300 ampere relay

4. Switches, Controls and Indicators				Switches, Controls and Indicators are used to operate, control, and monitor

DC bus components.

a. Description

b. Switches

1) Bus Tie									Two position switch 28V Isol DC powered. Connects Isol and ESS DC bus

on the ground, and allow the battery to power all DC buses.

2) DC power					                    3 position switch OFF, BATTERY, EXT POWER. When in battery position

Battery relay closes to power Isol Dc bus. When in EXT PWR position

2 relays in battery compartment are closed to power Main DC bus.

OFF position will open all relays.

3) Voltmeter and Bus Selector			5 position switch, Checks DC voltage on Main, ESS, Battery, INS Battery

and INS Battery Bus				.

c. Indicators

1) Loadmeters								(4 ea) 1 for each TR unit. Reading taken from shunts and used  with bleeder 															 resistors to monitor load of TR. ( Note location of bleeder resistor  and CB,s.

under flight deck).

2) Bus-off indicator lights					(3) 1 for each DC bus, 28 V Isol DC power

3) Isolated DC bus on battery on 		Illuminated when the Isolated DC bus becomes disconnected from the

indicator relay							Essential DC bus. (RCR failure)

4) External DC power indication			28 V DC green light on when power source in correct polarity

NOTES

Locate

External power fuse on the aft side of sta. 245.

Remove External power prior to changing the fuse if blown.

“E” pin voltage use from power unit if fuse blown.

D.C current limiters for external power on RH side of CP side CB panel.

E. Characteristics of the Engine System

1. Major power plant assemblies					     T56A-7(RPM 13,820)(3,755 SHP)(Max torque 19,600-4200 SHP)

a. Power section 									Single entry

1) Compressor assembly						14 stage

a) Air inlet housing								Anti-iced inlet scoop/guide vanes, oil cooler scoop, torque shaft.

b) Stage and flow								14 stage axial flow compressor (125 psi, 600 F).

c) Diffuser										14 stage bleed air from compressor for aircraft systems and flow

diffuser into combustion section.

d) Acceleration bleed valves					8 bleed valves, 5th and 10th stage. (4 on each stage)

e) Pressure and temperature					Fuel flows into combustion chamber and burns, increases temperature

and thereby the energy of gases.

2) Combustion assembly							Discuss the 2 minute cool down time for nozzle's coking IAW -1 section 2.

a) Nozzles										(6) 1 per burner can.

b) Burner cans									(6) run around tube connects burner cans for ignition.

c) Ignition plugs									(2) located in # 2 & 5 burner cans, Powered from the Ignition exciter, which

is powered by 28 V ESS DC.

3) Turbine assembly

a) Stages											4 stage turbine.

b) Thermocouples								(18) 3 per burner can. Indications sent to TIT indicators and TD amp.

22 per thermocouple indication.(Discuss turbine life by lower TIT settings.)

4) Accessory drive housing

a) location										Forward section of compressor.

b) Component's									Fuel pump, oil pump, speed sensing control(16-65-94% RPM),

speed sensitive valve (94%RPM).

b) Torquemeter assembly

1) Two concentric shafts						Outer shaft serves as reference point, inner shaft transmits power from

power section to reduction gear.

2) Torquemeter magnetic pick up			Measures torsional deflection of load of the inner shaft

AC inst. and fuel cont. bus power	.

c) Reduction gear assembly

1) Reduction gear train						2 stage, 13.54 / 1. (13,820-1021) engine speed, propeller shaft speed

2) Accessories									Generator, Hydraulic pump, Oil pump, Starter, Tach generator (GHOST)

3) Safety coupling								For decoupling power section from reduction gear if negative torque

in excess of 6000 “ lb.

4) Prop brake									Held disengaged at 23% rpm by oil pressure. Engages to prevent reverse

rotation . Springs engage below 23% to engage brake.

2. Engine Systems

a. Oil System									Type oil MIL-L 23699, can mix half tank capacity with 7808 in emergency.

1) Oil tank

a) Capacity								12 gallons, 7.5 expansion space.

b) Low oil warning						4 gallons remaining.

c) Servicing								RH side of engine. Note, do not service engine oil when cold or engine has 															not operated for an extended period of time. Oil may of drain into the lower 																part of accessory drive and servicing without motoring may cause engine to 															be	overserviced which could damage oil tank or oil system components.

2) Engine										Engine oil pressure 50-60 psi,  10 psi (20 psi excursion)

a) Pressure pumps						Oil feeds from the tank into the gearbox and power section of the engine.

b) Scavenge pumps	The engine driven oil pump pumps through a filter from the oil tank, then to the engine components, (3) scavenge pumps pick up the oil, then it flows to the oil cooler where it is cooled then returned back to the oil tank.

c) Temp bulbs							28 V DC power source. temp Bulb for engine oil temp indicator located in 																supply line below oil tank.

3) Oil cooler 									Oil cooler motor 28 V ESS DC, Located in lower portion of nacelle.

Discuss LORI oil coolers and temperature when full open.

a) Thermostat							Thermostatic element located in oil cooler senses oil temp

and regulates oil flow through oil cooler. Bypasses at 75C

b) Auto									In auto oil temp is maintain at 60 -85C. Oil temperature thermostat

located in scavenge oil return line on outlet side of oil cooler, controls

the position of the oil cooler flap.

c) Manual								If auto fails, use open, closed, and fixed to position oil cooler flap to maintain

oil temp limits. With oil cooler full open in flight oil temp should remain at

approx. 51-57C (full bypass). Oil cooler regulator will be in full bypass at this time.

4) Purpose of fuel heater 					Warms the fuel to prevent  ice from forming in the strainer. Capable of

raising and strainer the temperature of the fuel from a -70F at the inlet to a 															minimum outlet	temperature of + 34F. A 200 mesh screen wire strainer 																is provided in the outlet section to collect solid particles that may  be in the 																flowing fuel.

5) Reduction gear box						Independent  dry-sump oil system supplied from engine oil tank.

a) Pressure pump						Gear-type positive displacement pump driven by reduction gear accessory 																drive Filter is a disc type, with bypass valve. Gearbox oil pressure 150-250 																psi,  20 	psi (40 psi excursion). Pump has pressure adjusting valve (115%) 															inc. only.

b) Scavenge pumps					(2) Nose and scavenge. Scavenge system has a pressure relief valve,

relieves at 220-250 psi . 2 magnetic plugs to check for resistance.

b. Engine Fuel System

1) Fuel heater and strainer					Oil to fuel heat exchange, fuel warmed to prevent icing in the strainer.

A 6 PSID	 bypass incorporated in the strainer assembly.

2) Fuel pump assembly						Contains centrifugal boost pump, two gear driven pressure pumps,

(primary and secondary ) and a high pressure fuel filter.

3) Paralleling valve								Solenoid-operated valve. At 16% RPM this valve places high pressure

fuel pumps in parallel operation. A fuel pressure switch (secondary pump

switch) closes when secondary fuel pump pressure increases to 140-160

psi. Gives warning of possible primary pump failure. See on crossfeed from

aux or external fuel source in section 3 in -1.

4) Fuel control									Primary fuel metering device in the system. Provides 20% more than is 																needed. Flyweight limits engine RPM to 103.5%. Provision for LSGI

operation. Sense's compressor inlet temperature and pressure.

5)Temperature datum valve					Final metering device in system. Meters fuel to keep TIT at a desired value.

Temperature limiting (below 94% RPM)

Temperature scheduling (Above 94% 	RPM and  65 throttle travel)

6) Fuel nozzles									(6) Directs spray pattern into burner cans

7) Manifold										Flexible hose assembly located at diffuser section

8) Drip valve										During start energized closed and held closed by fuel pressure. Drains 																	manifold during engine shutdown when pressure drops below 8/10 psi.

c. Start System

1) Electrical control								28 V ESS DC

a) Condition levers							Feather, Ground stop, Run, and Airstart.

b) Interlock relay	If oil control CB (Fire Shutoff Valves) is out, or fire handle pulled, the engine start control is inoperative. Located on the RH aft side of 245 fwd of the RH static line retriever motor.

c) Starter button								Push button switch to control starter control valve solenoid. Light in button

indicates ground circuit complete for valve operation.

2) Air sources									GTC, External air cart, other running engines	.

3) Bleed air valve								Valve in horse collar controlled by switch on engineers overhead panel.

(28 V. ESS DC operated)

4) Starter control valve							Solenoid controlled, air operated to regulate starter RPM. ( 28 V. ESS DC 																operated)

5) Starter											(3) types, Bendix, Hamilton standard, Airesearch (v clamp).

6) Fuel and ignition								Controlled by speed sensitive control at 16 and 65 % RPM.

a) 16 % switch								At approx. 2200 RPM, Ignition ON, Fuel enrichment valve open, fuel cutoff 																valve opens (Geneva lock), paralleling valve closed, drip valve closed.

Held closed by fuel pressure.

b) 65 % switch								At approx. 9000 RPM Ignition OFF, drip valve de-energized, paralleling

valve opens.

c) 94 % switch								At approx. 13000 RPM TD changes from 50% to 20% take.

(start limiting to normal limiting)

7) Speed sensitive valve						Located on accessory drive on engine.

a) Below  94%								Open when RPM drops below 94%.

b) Above  94%								Closed when RPM increases above 94%.

8) Power sources								14th stage bleed air off diffuser. (Filter on pressure line)

14th stage greater pressure than 5th and 10th.

d. Temperature datum									The TD system compensates for variations in fuel heat value and density,

engines,	and fuel control system control system characteristics. located 																between fuel control  and nozzles.

1) Start limiting									(830C) Valve reduces fuel flow up to 50% during start,

20% above 94% rpm by returning the excess fuel to the fuel pump

2) Normal limiting								(977C)

3) Temperature controlling

a) 65 degrees TLA							(Throttle range 65-90) If there is a difference the TD control signals the TD

(throttle lever angle)						valve to increase or decrease the fuel flow to bring the temperature back on

schedule. Cross over switch in coordinator at 65 position.

b) Desired vs. Actual TIT					The TD control compare the two. In the temp controlling  range (65-90) if 																	there is a difference, the TD control signals the TD valve to increase or 																		decrease fuel flow	to bring the temperature back on schedule. In the 																		temperature	limiting range (0-65)	the TD control acts only when the limiting 																temperature is exceeded at which time it signals the TD valve to decrease 																	fuel flow. The TD valve is located between the fuel control and the fuel

nozzles.

c) Temperature controlling check	Used to check TD system for malfunctions. ( See section 7 in -1)

4) Locked										With the TD control valve switch positioned to locked, this locks the motor 																(to hold a stable position of the needle). With throttle above 65 throttle

position, the TD valve bypass control needle will be locked in the position 																when the switch was placed to LOCKED. Overtemp protection is provided. 																Eliminates the crossover bump when throttle is moving through the 65

position, and can provide symmetrical power by correcting for rich or lean 																fuel control scheduling. Any specific fuel flow trim correction applied in  65-															90 throttle range  can be locked into the TD valve.

5) Null- fuel control capabilities				With switch in NULL position, AC power is removed from the amplifier. The 															TD valve bypass needle will be in NULL position, bypassing a fixed

percentage of the total fuel being supplied by the fuel control. During NULL 															operation	temperature must be controlled manually by  throttle lever  re																	positioning. 20% bypass and all fuel metering is then accomplished by fuel 																control.

e. Throttles and condition levers

1) Throttles

a) Reverse									 0 - 18

b) Ground idle								18  (LSGI 9 - 30)

c) Flight idle									34 (governing 34 - 90)

d) Crossover									65

e) Takeoff                                          	90

2) Condition lever / switches

a) Ground stop								Effective only on the ground . Circuits for stopping engine are completed 																through the landing gear touch down switches.

b) Run										Effective only on the ground and inflight, run position is used for starting

engines and for subsequent operation.

c) Air start									This position is used for inflight restarting of engines. Holding the condition

lever in Airstart completes the same circuits that are used for starting on the

ground.

d) Feather									This position is used for emergency shut down inflight and on the ground.

Fuel is shut off by  mechanical linkage to the fuel control and with an

electrical	circuit. The propeller also receives a mechanical an electrical

signal to feather.

f. Alternate methods of engine shutdown / starting

1) Buddy start									See -1

2) Windmill taxi start									and

3) Airstart													     55-130	(11-C130)

4) Cruise engine shutdown												            Requirements

NOTES

F. Characteristics of the Engine and GTC Fire, Overheat, and Suppression System.

1. Detection											Engine nacelle, Horse collar area, and GTC compartment. Consist of  a

sensing element, control box, indicators, and wiring to furnish control

and power to the systems.

a) Fire

1) Inconel tubing							Sensing element, serves as a protector for an internal thermistor core. 																	Senses high rise in temperature or fire.

2) Amplifier									28 V DC power source.  Control box located in fwd section of each dry bay 																for	the engines and in the LH MLG wheel well behind the mud guard for the 															GTC.

3) Indicators									2 steady red lights in the "T" handles.

4) Test function								When the test relay is energized contacts of this relay  ground the sensing 																element inner core.

b) Nacelle overheat							The nacelle encloses the area around the engine. Normally, area provides 																adequate heat dissipating and thus prevents heat build up. Engine

malfunctions or adverse operating may cause temperature to rise

excessively.

1) Detector's	Temperature sensing element . It functions as a switch to bring together  two contact points

a) Quantity								6 connected in parallel

b) Temperature							300F

2) Indicators									1 Master and 4 individual on copilots inst. panel.

3) Test function								The test only checks the wiring and lights, not the detectors.

c) Turbine overheat							If the temperature rises above preset value the detector  will close

creating a ground to the flasher unit and  the lights in the fire handle

illuminate.

1) Detector's	Temperature sensing element . It functions as a switch to bring together  two contact points.

a) Quantity								4 detectors connected in parallel.

b) Temperature							 700F (371C)

2) Indicators									2 flashing lights in "T" handles, keyer unit used to produce flashing signal.

3) Test function 								The test only checks the wiring and lights, not the detectors. Do not hold the 															test switch longer than 30 seconds, any longer could damage keyer unit.

Wait 1 minute before testing again.

d) Circuit protection							28 V ESS DC.

2. Suppression / extinguishing					Pressure 600 - 640 / 70F day.

a) Fire handles									5 handles located on fwd overhead panel.

1) Engine / GTC							When any one handle is pulled, it arms the fire extinguishing system and 																activates the fire isolation system.

2) Items controlled							Directional control valves , and routes current to the agent discharge switch.

3) Priority										Directs agent to last fire handle pulled.

b) Extinguishing

1) Components and operation

a) Bottles	2 bottles located in the upper LH MLG wheel well area. The agent CB or DB(may be used when CB is not available) in each bottle are serviced to

approx. 19 LBS each CB or 22 LBS. each DB. DB is HALON 1202 type

agent. Pressure 600 - 640 / 70F day.

b) Firing mechanism / squid			An explosive charge called a squib is packed into a threaded bolt and 																	screwed 	into the bottle. The explosive charge is set off electrically through a 															switch in the flight station after the fire handle is pulled.

c) Two way check valve				Installed at the junction point between the two bottles. Serves mainly to pre																vent agent fired from one bottle from discharging into other bottle.

d) Directional control valves 			Located in # 2 & # 3 dry bays, 1 in the upper LH wing root area, and 1 below 															the two extinguisher bottles.

2) Controls

a) Fire handle							When fire handle is pulled the agent discharge switch and  fire directional 																valves are energized  through the fire extinguisher control valve assembly.

b) Agent discharge						When the switch is activated an explosive charge is set off . The expanding

gases force the circular knife through the copper diaphragm and the agent  															is released into the tubing and through the discharge outlet.

c) Power source						28 V DC, Battery bus.

NOTES

Discuss the location of the bottles and the directional control valves in LH wheel area and #2 & 3 dry bays.

G. Characteristics of the Propeller System

1. Major propeller components

a) Barrel assembly / purpose					The three main functions of the barrel assembly are

(1) Retain the blades.

(2) Transmit engine torque to the propeller blades.

(3) Provide a positive method of retaining the propeller to the reduction gear

assembly propeller shaft.

b) Pitch lock assembly							located in the barrel assembly. Consist of a stationary pitch lock ratchet ring,

a pitch lock rotating ratchet, a pitch lock valve and flyweight assembly

1) Purpose									Prevents the blades from decreasing pitch if overspeeding occurs,

or hydraulic pressure is lost.

2) Operation

a) Normal								The stationary and rotating lock rings are held disengaged by propeller oil 																pressure which is controlled by the pitch lock regulator assembly. In order

that pitch lock action does not interfere with normal reversing and

unfeathering of the propeller the pitch lock rings are mechanically held apart 															by cam action when blade angle is below 25 and above 55.

b) Pitchlock operation					If oil pressure is lost, the two ratchet rings engage. When engaged the

propeller	pitch can still be increased to allow feathering. When an

overspeed of approx. 103% is sensed by flyweights in the pitch lock

regulator assembly, oil pressure	is removed to allow the two pitch lock 																	ratchets to engage and prevent a decrease in blade angle. To release the 																pitch lock, the overspeed must be corrected to restore oil pressure, and the 															blade angle must be increased above 2 to disengage the ratchets.

If a  prop is operated pitched locked,

1. TD valve switch for affected engine LOCKED.

2. Engine bleed air switch CLOSED.

3. Establish 96-98% rpm with throttle and / or airspeed.

4. Upon reaching a suitable landing area attain 150 KTAS (if possible).

5. Upon reaching 150 KTAS or a point where 96% rpm cannot be maintain

with throttle , whichever occurs first shut down the engine IAW ESP.

c) Pitchlock range						Operates in the alpha (flight) range when blade angle is between 25 and 																55.

C.	Dome assembly 							Mounted in the forward section of the barrel assembly. Contains the 																		mechanism	for changing propeller blade angle.

1) Pitch

a) Piston									When the piston moves forward blade angle decreases , if the piston moves 															aft blade angle increases.

b) Rotating and stationary cams		The cams are assembled as unit. The aft end of the cam is splined to the 																barrel. Two sets of ball bearings support the rotating cam inside the

stationery cam. Gear teeth on the aft end of the rotating cam, mesh with the 															blade segment 	gears.	 The direction of track slopes in the rotating cam is 																opposite to the direction of track slopes in the stationery cam.

2) Low pitch stop							Located in the dome . Has three low pitch stop levers, a moveable wedge, a 															servo valve, a servo piston, springs, and oil transfer tube.

a) Purpose								Is a safety device used to prevent the propeller from going into the beta 																	range when aircraft is inflight. It mechanically stops the dome piston from 																decreasing blade angle below 23.25 degrees unless stop levers are

retracted.

b) Operation

1 Engaged - flight	The piston moves forward, rotating the cam to decrease blade angle. As the 		piston	 moves forward, it slides over the stop levers. When the piston moves

to position that the lips of the stop levers engage, the stop levers stop the

piston. To further decrease blade angle , the wedge must be retracted.

Decrease pitch is ported to the servo valve around the transfer tube. This	pressure is approx. 100 PSI in the flight range but not high enough to unseat 	the  servo valve.

2 Released - ground					In the beta range, the throttle control rotates the alpha shaft, and unseats 																the backup valve to increase pressure across the governor metering valve. 															The highest valve of either increase or decrease pressure is ported to the																back of the low pressure relief valve. When the stop levers stop the																		movement of the piston, the pressure builds up in the decrease line. A

pressure of 240 to 280 PSI  unseats the servo valve and moves the piston.																This allows the levers to retract and the piston can move the propeller 																	blades toward the reverse pitch.

d. Control assembly							Non rotating unit mounted on the rear barrel half extension.

1) Pump housing							Located in the lower part of the control assembly. The rotating propeller

contains 16 qts. Of oil while the prop shaft/ control cavity and the housing																assembly plumbing hold 2 to 3 qts. Total system capacity is 26 Qts.

a) Pump's								Main pressure, standby, main scavenge, aux pumps.

b) Pressurized sump (AFT)			Has 3 gear type positive displacement pressure pumps. Has approx. 6 qts of 															fluid.

c) Atmospheric sump (FWD)			Contains the gear driven main scavenge pump, the auxiliary electrically																	driven scavenge pump, and the sump relief valve. Holds approx. 1 qt of																	hydraulic fluid. The sump relief valve maintains 15 to 20 PSID pressure																	differential in the pressurized sump.

d) Low oil warning system				Prop low oil warning light is a float controlled actuated switch in the

control assembly . When oil quantity drops to approx. 2 qts below normal																in the pressurized sump the prop low oil warning light for that prop and the

prop low oil quantity light will illuminate.

e) Servicing								IAW the applicable maintenance technical order. (Discuss procedure)

2) Valve housing							Contains various cams, valves, and switches which control the flow of

hydraulic fluid to the propeller pitch changing mechanism.

a) Pilot valve, speeder spring,		The pilot valve is controlled by the mechanical action of the flyweights

opposing and fly weights	the force of the speeder spring. When the

propeller is in an on speed condition, the pilot valve meters sufficient fluid to

the increase pitch or forward side of the dome assembly piston to overcome															the centrifugal twisting moment and occurs, the maintain the required blade

angle. When an overspeed condition flyweight force overcomes the speeder															spring force, and the pilot valve	moves to	increase the flow to increase

blade angle and cause the propeller to slow down. If the propeller slows

below governed speed , the force of the speeder spring overcome the force

exerted by the flyweights, and the pilot valve meters fluid to the aft side of																the dome assembly piston to decrease blade angle and allow the propeller 																to increase speed.

b) Feather valve, solenoid valve,			When the condition lever is moved to the feather position the pilot valve is																and actuating valve ported to increase pitch, and the feather valve moves to

the feather position. The condition lever also actuates a switch in the control

pedestal, completing a circuit to the holding coil of the propeller feather																	override button. When the button pulls in it completes a circuit to energized

a feather solenoid and the auxiliary pump motor. The feather actuating																	valve and feather solenoid valve route fluid to position the feather valve and															pilot valve for propeller feathering.

c) Pressure cutout switch				When the propeller blades reaches feather angle, a pressure buildup occurs 															and actuates a pressure cutout switch in the control assembly, which																	opens the holding circuit for the propeller feather override button.

(Discuss which one you can fly without and why) Pressure cutout switch opens at approx. 600-800 psi.

d) Blade angle control

1 Flight								Alpha range	(34- 90) range is from flight idle to full power. Propeller blade 															constant angle changes are controlled by propeller governor which

maintains RPM within limits.

2 Ground								Beta range, (0-34) In this range, the propeller functions as a  controllable 																pitch propeller.

e. Spinner assembly						Encloses the dome, control assemblies, in a stream line housing.

1) Airflow									Air inlet in the nose of the spinner admits cooling air.

2) Cooling air							Cooling air enters the inlet and passes over the dome assembly, Barrel 																	assembly, and control fins assembly. Air is exhausted out through

the vents in the cowling in the engine nacelle.

2. Propeller Governing System

a. Ground range								Beta range  (0-34) on the coordinator assembly.

1) Blade angle								In this range the propeller functions as a controllable pitch propeller. During 															ground operation, changes in throttle position mechanically affect the fuel

flow and the propeller blade angle.

2) Rpm control								The propeller does not control RPM within the ground range. The speed 																servo assembly along with the speed flyweight in the fuel control adjust fuel

for blade angle change in the ground range.

b. Flight range									Alpha range (34- 90) on the coordinator assembly.

1) Mechanical governing

a) Blade angle							Propeller governing is accomplished by the action of the flyweight speed																flyweights opposing the force of the speeder spring.

b) Rpm control							When the propeller is in an onspeed condition, the pilot valve meters

sufficient fluid to the increase pitch or forward side of the dome piston to

overcome the centrifugal twisting moment and maintain the required blade																angle. When an overspeed condition occurs, the flyweight overcomes the 																speeder spring force, and the pilot valve moves to the increase the flow to																the increase pitch side of	the piston  to increase blade angle and cause the															propeller to slow down. If the	propeller slows below governed speed, the																force of the speeder spring overcomes the force exerted by the flyweights,																and the pilot valve meters fluid to the dome assembly piston to decrease																blade angle and allow the propeller to	increase speed.

2) Normal governing						The speed of the propeller is controlled by the propeller governing system																within the flight range of the throttle lever maintain constant RPM.

a) Blade angle							In flight, changes in throttle position mechanically affect fuel flow and the 																propeller governor regulates blade angle, maintaining constant engine 																	speed.

b) Rpm control							The principal function of the propeller governing system is to maintain

constant 	engine operating RPM.

c) Synchrophaser						This is the main component of the synchrophasing system, which maintains 															a desired blade phase relationship between a master propeller and the three

other propellers (slave propellers) . It also reduces the noise level and																	vibration in the airplane.

1 Speed stabilization				The propeller mechanical governor maintains a constant speed in the flight

(alpha) range. However, with the propeller governor control switch in MECH

GOV, throttle changes cause the governor to overspeed or underspeed

while trying to compensate for changes in power. With the propeller																		governor control switch in normal, the stabilization circuit stabilizes the

mechanical governor.(This signal comes from the TACH generator)

2 Throttle anticipation				During normal governing operations, the throttle anticipation circuit stabilizes 															the propeller during fast throttle movement. It functions only in the alpha																	(flight) range during normal governing or synchrophasing mode of operation.

(This signal comes from the Potentiometer located in the valve housing)

3 Synchrophasing					The synchrophaser uses signals from an anticipation potentiometer and an

engine driven tach generator to keep engine speed constant during and

after throttle movements. Alpha shaft rotation, in the propeller control

assembly, drives an anticipation potentiometer. Signals from the anticipation

potentiometer are routed to the synchrophaser unit which electrically

modifies this signal into a or decrease pitch signal that is transmitted to the																two phase motor in the speed servo bias assembly. The synchrophaser																	anticipates changes in engine speed due to throttle movements and keeps																the engine from overspeeding or underspeeding during throttle movement.

4 Reindexing						Corrects for an out of sync condition. (See reindexing procedure in -1)

(See procedures in back of book)

5 Switches and controls			Synchrophaser switch, propeller resynchrophaser switch are on the flight

control pedestal, sync trim control knob (disconnected on solid state																		syncrophasers),and propeller governor control switches ( Normal / Off) on

copilots side shelf, fuel governing switches on aft end of overhead  control 																panel.

6  Power source						115 V ESS AC single phase and 28 V ESS DC . The AC voltage is changed															to	operating voltages necessary for the amplifiers by a power supply in the 															synchrophaser assembly. The DC voltage is used for relay control and

interlock.

c. Negative Torque System

1) Causes of negative torque				When the propeller tries to drive the engine section.

2) NTS operation

a) Reduction gear box / NTS			Positions the mechanical linkage of the feather actuating valve and the 																	feather actuator rod valve when a negative torque condition exist.

b) Valve housing						Contains various cams, valves, and switches which control the flow of

hydraulic fluid to the propeller pitch changing mechanism.

1 NTS plunger						Actuated by the gear box  plunger when the propeller attempts to drive the 												springs when engine. A ring gear in the gear box moves forward against the

plunger as a result of a torque reaction generated through helical splines. The

plunger pushes against a cam in the signal assembly to actuate control

linkage connected to the valve housing. This increases blade angle to

relieve the condition, except if the throttles are below the flight range. A cam

moves the actuator away from the NTS plunger rendering the system 																	inoperative to prevent the propeller from receiving a negative torque at high

landing speeds when throttles are moved toward reverse.

2 Feather valve and				The feather valve and NTS check system consist of a feather valve and NTS

check switch						check switch, four lights, four NTS relays, and a feather valve switch and																NTS switch in each propeller control assembly. In the valve position it

completes the light circuits from the ESS DC bus through the lights and

contacts of each NTS check relay to the feather valve switch in each

propeller control assembly. If the feather valve is manually positioned for

feathering the propeller, it completes a circuit to ground the corresponding																light. This indicates the feather valve in position to feather the propeller.

In the NTS position it completes two circuits. One is completed from the

ESS DC bus through each indicator light to a set of contacts in each NTS

check relay. The other from the ESS DC bus through the coil of each NTS																check relay to the	NTS check switch in the propeller control assembly.																	When a negative torque condition exist, the engine NTS plunger actuates a																linkage which closes the NTS switch. The relay will remain energized as																long as the feather valve and NTS check switch is in the NTS position.

3 NTS range							-1260 (+ - 600) inch lbs.

4 Safety coupling					Provided to decouple the power section from the reduction gear if a negative 															torque applied to the reduction gear exceeds approximately -6000 inch LBS,

a valve much higher than required to operate the NTS system.

d. Feathering System

1) Electrical

a) Fire handle							When a fire handle is pulled out it closes DC circuits to operate valves 																	which isolate the engine as follows.

1. The shutoff valve on the Engine fuel control is closed.

2. The engine oil shutoff valve is closed.

3. The firewall fuel shutoff valve is closed.

4. The firewall hydraulic shutoff valves are closed.

5. The engine bleed air is shutoff.

6. Engine starting control circuits are de-energized.

7. The propeller is feathered.

8. Positions the fire extinguisher system control valves.

9. Arms the extinguishing agent discharging switch.

b) Condition lever						A condition lever, one for each engine is the primary control used for engine 															starting and stopping and propeller feathering and unfeathering.

c) Aux feather motor					The aux feather motor is used to feather and unfeather the propeller inflight . 															It is also used for static operation of the propeller on the ground. It operates 															when the condition lever is placed in either  the air start or feather position.																It also operates when the fire emergency handle is pulled.

115 V ESS AC MOTOR 3 PHASE, ESS DC Conterolled.

2) Mechanical / condition lever				When moved to the feather detent, mechanical linkages transmit motion to 															the engine mounted coordinator and from the coordinator to the to the

propeller and to the shutoff valve on the engine fuel control. Switches are

also actuated by the lever as it is pulled aft which results in the following.

1. The propeller receives a feather signal mechanically and electrically 																			positions the feather solenoid valve.

2. The fuel shutoff valve on the engine fuel control  is closed both 																					mechanically and electrically.

3. The propeller Auxiliary pump is turned on, providing pressure to feather the 																	propeller.

4. The nacelle preheat system remains operable only when the aircraft is on

the ground ( if equipped).

3) 86 degree switch	The 86 switch is used to prevent pressure surges in the propeller hydraulic system which could cause the pressure switch to open before the propeller is fully feathered. This a safety feature to ensure a continuation of the feathering

action. When the propeller reaches a pitch angle of 86 degrees, the beta shaft has rotated to reposition the pressure cutout switch cam and open the switch. Pressure cutout switch completes the feathering cycle. The 86 switch is required to be operational for flight.

4) Pressure cutout switch					The pressure cutout switch assembly will supply a parallel ground to the

holding coil of the feather override switch. The ground will be broken when the prop has reached full feather position (92.5), a mechanical stop prevents any further travel. Fluid pressure builds up to approximately 600 to 800 PSI  the pressure cutout contacts open, feather override solenoid opens and stops the Auxiliary pump. The 86  switch is used to prevent pressure surges in the propeller hydraulic system  which may cause the pressure switch to open before the propeller is fully feathered.

5) Feather override switch				When a condition lever is positioned to the feather position or a fire

emergency handle is pulled a circuit is completed through the feather relay																to the feather override solenoid switch on the copilots side panel . This is

energized closed to power the aux pump motor relay /auxiliary motor. When															the feather cycle is completed the pressure cutout switch opens which  de-																energizes the feather 	override  switch which opens the auxiliary pump motor 															relay .

6) Feather check							A static check of the feather valve uses linkage from the FEATHER position

of the condition levers. The throttle lever is set below flight idle with the																	engines not operating and the FEATHER VALVE AND NTS CHECK switch																placed in the VALVE position. The NTS check light switch light should

illuminate immediately. This indicates the feather valve linkage has been 																moved by the condition lever.

7) Circuit protection						28 V ESS DC for the relays and switches, CB’s on copilots CB panel.

115 V ESS AC 3 PH for the aux motor, CB’s on pilots side CB panel.

NOTES

H. Characteristics of the Gas Turbine Compressor

1. Component's										Used for operation of the ATM and supplying bleed air manifold pressure for

engine starting, air conditioning or operation of other pneumatic systems.

a. Compressor and turbine					Located between the accessory section and the turbine.

1) Stages										2 stage centrifugal- type compressor driven by a single stage turbine. The 																	turbine drives the compressor and the GTC accessories which are the, starter																motor, oil and fuel pumps, an oil cooler and fan, and a governor.

2) Compressor flow	When operating at full speed, part of the compressed air is discharged into the power turbine to support combustion, and the remainder is available as pneumatic air.

b. Load control valve / operation				The load control valve controls the  amount of air which can be bled from the 																GTC.	Valve will only open when RPM is 95% or above. The rate valve  will not															allow loading of the GTC faster than it can supply the increase demand for air.

c. Controls and circuit protection				Fire handle must be in and door open for normal GTC operation.

1) Control switch							A 3 position rotary switch (OFF, RUN, START) located on the GTC control																panel	is used to energize a self holding starter relay for the GTC starter.																The relay  will remain closed until 35% RPM or until starter switch is moved																to the OFF position. The switch is spring loaded to the RUN position when

released from START.  Switch power is 28 V. ISOLATED DC.

2) Bleed air valve switch					A 2 position toggle switch (OPEN, CLOSED) controls the normally closed 																solenoid-operated bleed air valve. Applying bleed air load to the compressor

is prevented by the 95% speed switch which completes the circuit to the 																bleed air valve only after operating speed is reached. 28 V ISOLATED DC.

3) Door switch and light					The 2 position toggle switch (OPEN-CLOSED) GTC door switch and the																door warning light are location the GTC control panel . The door warning																light illuminates when the door is not fully closed. 28 V ISOLATED DC for																the door and 28 V MAIN  DC for the light.

4) Fuel and Ignition system				Fuel for the GTC is supplied from any fuel tank (normally #2) through the

crossfeed manifold.

a) Oil pressure switch					During the start cycle, when oil pressure reaches approx. 3 psi the fuel and

ignition circuits are energized by oil pressure.

b) Acceleration limiter					Located in line between the acceleration valve and the compressor discharge 																port. Energized closed from 0 to 35% through the start and ignition relay. From 															35 to 95% it is de-energized open until bleed air switch is placed open. 																		Acceleration from 35 to 95% is dependent on fuel pressure and compressor 																discharge pressure. If tailpipe temperature reaches 1250 F the

overtemperature thermostat opens and bleeds off compressor discharge																pressure. As compressor discharge drops, more fuel bypasses through the

acceleration (with lower fuel flow to the fuel nozzle).

c) 35%, 95%, 110% switches				As the GTC starter starts to rotate oil pressure builds up. When it reaches 2 														to 3 psi, the oil pressure switch makes contact, energizing the fuel shutoff to														open, and the ignition is energized to on. When the RPM 	reaches 35% the														35% switch breaks the ground to the starting relay, starter is de-energized . 														The accelerator limiter valve is de-energized open, and start light																		extinguishes, and ignition is also stopped at 35% RPM. At 95% RPM  the

95% switch makes contact, illuminating the ON SPEED light. An overspeed

set at 110% will close the fuel shutoff valve to prevent overspeeding.

d.) Oil system	The oil system is positive-pressure, dry- sump type. Oil gravity feeds from the tank to the oil pump assembly(oil cluster). A motor operated shutoff valve in the supply line is controlled by the GTC fire handle.

Oil enters the oil cluster through a check valve that has a 2 psi differential to 															prevent from gravity feeding back into the oil cluster when GTC is not

running. The valve is held open by mechanical	link when operating, if the

pump loses its prime, the gravity flow check valve can be opened manually

the primer button.

1) Quantity									Tank capacity is 3 QTS., total system 4 QTS.(Shutdown at 1 qt low)

2) Oil cooler									The oil cooler located between the scavenge pumps and the oil tank. Air is 																forced through the oil cooler by a fan mounted on the accessory drive

housing and exhausted out of the ducting above the GTC exhaust tailpipe.																The flow also aids in the cooling of the GTC compartment during GTC																	operation. A	thermostatic bypass directs oil through the cooler or bypasses																depending on the oil temperature.

3) Servicing									The oil tank is located in the cargo compartment below the utility reservoir

just forward of the left hand wheel well. The tank is vented to atmospheric

pressure. The tank has baffling to separate the oil from the air as it returns																from the scavenge pumps and also strengthens the tank to withstand the																pressure 	differential when the aircraft is pressurized.

2. Operation

a. Start / duty cycle								1 Minute on / 4 Minutes off, continuos operation.

b. Warm up

1) Cold weather operation					4 Minutes of operation without load if temperature is 32 F and below

2) Normal									1 Minute of operation if temperature is above 32 F.

c. Use

1) Applying a load							A full load for the compressor is 126 PPM at 54 PSI absolute. The overload

thermostat perverts the overloading of the GTC . Bleeding off to much air

would not leave enough cooling air for the turbine section and the tailpipe

temperature would go up. When the tailpipe temperature reaches 1200 F,

the overload thermostat opens bleeding off the pressure in the open side of																the valve, the bleed valve partially closes limiting the amount of bleed.

2) Bleed air system check						Minimum Pressure 35 psi / pressure 30-15 psi in no less than 8.5 seconds.

3) Engine start								Bleed air manifold is pressurized from the GTC for engine start.

d. Shutdown / cooling time					1 Min. Cool down time prior to shutdown

### NOTES

Do not open the GTC filler cap inflight, oil tank is vented to the outside atmosphere.

GTC current limiter (200 amp) and starter relay located behind the lower forward pilot’s CB panel.

(Same size current limiter as the cargo winch)

I. Characteristics of the Bleed Air / Anti-icing Systems

1. Bleed air sources

a. Engine bleed air 								14th stage bleed air, approx. 125 psi at 600 F temperature.

b. GTC bleed air								Small jet engine located in LH landing fairing, approx. 54 psi at 435F.

c. External air cart								Comparable to the GTC for bleed air source.

2. Bleed Air System

a. Bleed air ducts								Consist of stainless steel ducts extending across the wing outbd of #1 to 																outbd of # 4 engine. Directs compressed air to the pneumatic operated

systems.

b. Duct insulation								Used to insulate the ducts to prevent high temperature from being

transmitted to aircraft structure. (Fuel tanks, wiring in leading edge, etc.)

c. Bleed air valves

1) Check valves								Total of 12 used in system to control flow of air from pressure sources to 																pneumatic components .

a) Engine									1 in each engine nacelle to prevent loss of pressure into the compressor of

an engine that is not running. (Flapper type spring loaded closed)

b) GTC									Located in duct leading from GTC to prevent loss of manifold pressure into

the GTC compressor.

c) External cart							Located in duct leading from ground compressor inlet to prevent loss of 																	manifold pressure into the external air connection when engines are

operating.

2) Engine bleed air valves					4 each, 1 located in the aft of the fire wall in each nacelle(horse collar)

3) Wing isolation valves					2, located in each side of the fuselage just outbd of the point where the

manifold enters the fuselage. Used in an emergency to isolate manifold

pressure from the LH or RH wing.

4) Power sources							28 V ESS DC

d. Urinal drain ejectors							Bleed air flows out the urinal drains anytime the manifold is pressurized to 																prevent drains from freezing inflight. (Normal system pressure bleed)

e. Bleed air pressure gage					Used to monitor pressure in the manifold. ( reads approx. 6 psi lower with 																flight deck air conditioner on)

3. Wing and empennage anti-icing 				The leading edge anti-icing system is divided into 6 sections for anti-icing

purposes. 2 sections each  wing , 2 sections for the empennage. Each

consist of a shutoff valve, ejectors, and control components.

a. Nozzle's										The nozzles are part of the ejector assembly. As air flows from the nozzles it

mixes with ambient air within the leading edge. Air circulates in the leading

edge excessive pressure is exhausted into the dry outbd bays .

b. Shutoff valve									6 shutoff valves which are pneumatically actuated and electrically controlled

to control the flow of air from the bleed air system to the ejectors where it is

ejected into the nozzles.

c. Mercury thermal switch						The mercury thermal switch opens the valve control circuit to prevent 																	overtemperature in the leading edge. (6)

d. Temperature bulbs							Temperature bulbs are resistance- type that sends a signal to the indicators

on the overhead panel. Placed so they sense temperature of the air in the

leading edge area.

e. Thermostats									Bimetallic thermostats close light circuit on rising temperature.

f. Indicators										6 leading indicators, one for each section of the anti-icing system.

Indicator ranges:

INOPERATIVE Approximately 75 F and below																								NORM OPERA RANGE	 Between 75 F and 200 F

OVERHEAT  Approximately 200 F and above.

g. Overheat lights								7 overheat lights on the right side of the overhead panel that 																				illuminate when temperature exceeds 200 F (215 F with replacement 																	thermostats).

i. Power source 									28 V ESS DC

j. Operation										SEE -1 restrictions

1) Ground									Will not be used to remove ice from the surfaces. Can be used for test only

no longer than 30 seconds. Monitor gages and overheat lights when

operating.

2) Inflight										Regulation of the temperature within the leading edge is achieved

automatically by thermostatic control of valves to keep the temperature in																the leading edge between  158 F-180 F. Warning lights illuminate if the

temperature exceeds approximately 200 F or more.

(215 F with replacement thermostats).

4. NESA windshield anti-icing					Heated windows, 3 front windshields, 2 side windshields, and 2 lower on the 																pilots side lower. NESA heat reduces the thermal shock and the possibility of

cracking the windshield .

a. Windshield panels							Total of 23 windows in the cockpit, 9 heated

b. Thermistors									Serves as a temperature sensing element for the automatic control circuit.

A thermistor is a temperature sensitive resistor.

c. Transformers									(2) High voltage AC power is supplied to the windows from the transformers

that steps power up to 290 volts in normal and 450 volts in high.

d. NESA windshield switches /				Control circuits of the system are energized from the MAIN DC bus by two 					operation									switches on the overhead nesa control panel. In NORMAL or HIGH  DC is 																supplied to two automatic control boxes, and  four power relays.

1) Normal, high, and cold start			When Normal is selected the normal power relay is energized  when heat is 															required for the windshields. In HIGH position the high power relay is

energized when heat is required, use HIGH only inflight when ice forms																	faster than normal can remove it. Cold start buttons are used to provide																	manual control of the windshield heating to raise the windshield

temperature gradually from extremely cold temperatures. If temperature of   the windshield panels is below -43 C (-45F) the control systems do not function automatically. Cold start buttons operate when control switches are in the normal position. Pressing the buttons apply AC power to the windows when the buttons are depressed for 5 seconds on, 10 seconds off until the

temperature of the windshield is above -43 C (-45F).

2) Caution for turning off system			Panels feel excessively hot.

Electrical arcing is observed in one of the panels.

One of the panels containing thermistors is not heating.

( could cause other  panels in the same system to overheat).

3) Power sources							115 V LH AC operated, 28 V Main DC controlled.

NOTES

Verify wing and empennage shut off by observing the temperature gages and the overheat lights.

Holes located in the bottom of each wing tip allow bleed air to exit the leading when the system is operating. This allows bleed air to flow to the leading edge wing tip so the entire leading edge will be heated.

Just because engine instruments indicate they closed one may not have closed all the way.

J. Characteristics of the Engine Propeller Anti-ice / Deicing Systems

1. Engine anti-icing system						NOTE: When system automatically comes on power changes will be

noticed as TORQUE, TIT  and FUEL FLOWS  will be affected.

a) Protected areas								Engine intake, oil cooler scoop, compressor inlet , and torque shaft.

b) Air flow sources								Two systems are provided for engine inlet air duct anti-icing. One routes

bleed air from the bleed system to passages in the engine inlet duct and oil 															cooler scoop. The other routes air from the compressor diffuser section of

the engine to passages in the compressor inlet vanes.

c) Valves											The engine inlet air duct and oil cooler anti-icing  is shutoff by a solenoid

valve which is energized closed. The air flows when the valve is de-

energized open. The guide vane anti-icing airflow is controlled by two

pressure actuated valves, which are controlled by a single solenoid valve.																When the solenoid valve is energized, the pressure actuated valves shut off															the airflow, and when the 	solenoid valve is de-energized, the pressure-																	actuated valves open. Both valves are termed fail-safe, meaning that anti-																icing is provided when the system power supply is lost.

d) Controls										The electrical controls are interconnected with the ice detection system so																that the duct anti-icing can be turned on automatically when the detection

system senses icing.

1) Switches									Four switches located on the overhead anti-icing control panel  have two

positions, OFF-ON. With the switch in the ON position anti-icing can be 																	operated manually or automatically.

2) Ice detectors								Ice detection system is used as an automatic control for turning on the																	engine inlet air duct  anti-icing , and the propeller anti-icing and de-icing																	systems.	Probes in # 2 and # 3 intakes if they become iced over and the

prop and engine anti-icing master switch is in the AUTO position the																		detector units trigger a control relay.

e. Operation

1) Manual									To manually operate the systems position the prop and engine anti-icing																system master switch to MANUAL and the engine inlet air duct anti-icing																switches to ON.

2) Auto										To allow the system to operate automatically by the ice detection system, 																position the prop and engine anti-icing system  master switch to AUTO,

and the engine inlet duct anti-icing switches to ON.

3) Reset										To shut  system off while leaving it subject to automatic control, position the 																prop and engine anti-icing system  master switch to RESET and release to 																	AUTO.

f. Circuit protection								28V ESS DC.

2. Propeller Anti-icing / Deicing System			System is interconnected with the ice detection system to automatically 																	come on when icing is detected.

a. Protected areas								Spinner (3 sections), propeller afterbody,  and blade heaters.

1) ABC phases								"A"  phase power controls  anti-icing for the forward section of the spinner																	and the propeller afterbody.

"B" phase power controls de-icing for the spinner middle and rear section.

"C" phase power controls de-icing for the blades.

2) Anti-icing Vs deicing					Anti-icing is continuos heat type system, to where de-icing is controlled 																	through a deicing timer heating only one propeller at a time. The heating

elements are heated 15 seconds during each one minute cycle.

3) Timer										The de-icing timer is a 28 V.  ESS DC motor driven, rotary switch, with eight  															positions	. The switches close in sequence of 15 seconds. All four blades of

the selected propeller are de-iced simultaneously, 15 seconds on then 45 																seconds off. The timer cycles from # 1 propeller to # 2,# 3,# 4 and back

to # 1	propeller in 60 seconds, then cycle is repeated.

b. Control

1) Switches									The PROP & ENG ANTI-ICING MASTER switch on the control panel has

three positions AUTO, MANUAL, and RESET.  Four propeller ice control																	switches, one for each propeller, are provided on the control panel  for the																	propeller	de-icing.

2) Ice detectors, brush block, 			Ice detectors located in # 2 and # 3 intakes detect icing conditions. The

and deicing ring							brush block is located on the control assembly is used to transmit electrical 															power through  the slip rings on the de-icier contact ring holder assembly.

Each contact brush assembly touches the contact rings on the blade to form															an electrical circuit for the blade heating element.

c. Operation

1) Manual									When the PROP & ENG ANTI-ICING MASTER switch is placed to the																	MANUAL	position, and an individual propeller ice control switch is turned on,

28 V ESS DC energizes the spinner nose power relay. Phase “A” AC power

is applied through the contacts of the relay to continuously anti-ice the																	spinner nose and afterbody.

2) Auto										In the AUTO position, an ice detection system automatically turns on 																	the propeller anti-icing and de-icing system when ice is detected.(Prop ice																control switches have to be on to operate)

3) Ground use

a) Engines running						The engines must be running to dissipate heat to prevent damage to the

heating elements. Never operate the system for more than 2 cycles while on															the ground.

b) Engines not running					Do not use if engines are not  running. Trouble shooting by maintenance

only for ground operation  w /o engines running.

4) Blade de-ice below limits				If blade deicing ammeter falls below 65 amperes, do not fly into icing

conditions. Normal  reading 65-90 amperes	.

d. Circuit protection								115 V.  RH AC bus for blade heaters and spinner front and afterbody.

Discuss the no ice light coming on during LSGI or during engine start.

NOTES

K. Characteristics of the Air Conditioning / Pressurization Systems

1. Air Conditioning System normal operation

a. Flight deck air conditioning unit

1) Flow regulator							30 PPM with engines and 15 PPM with GTC operation.

2) Flight deck refrigeration shutoff   		The flight station has a manual override feature to permit continued

valve override							operation or shut down in case of a malfunction. A “T” handle located under

the nav table has 3 positions (override- handle down, neutral - center																	position, and off- handle up) to manually operate the flow regulator .

3) Heat exchanger	The refrigeration unit is made of two stages of cooling, the first stage is air to the heat exchanger, and the second stage is a high speed turbine cooling.

a) Turbine								The turbine speed varies from 10,000 to 85,000 RPM’s depending on the

weight of the air applied. In the process of driving the turbine, the air under 																goes expansion. Expansion and loss of energy are used to drive the turbine

which causes the air temperature to drop. The turbine can be capable of																discharging temperature below freezing.

b) Fan									The fan is driven by the turbine which provides a load to provide efficiency

and prevent overspeeding of the turbine. The compressor fan picks up some

of the cooling air in the heat exchanger, compresses the air, and directs it to

a group of nozzles located at the heat exchanger exhaust. This creates a jet

pump effect to increase the flow of cooling air through the heat exchanger.

4) Temperature control valves			The valve is actually two valves controlled by a single motor. The valve 																	determines how much passes through the heat exchanger. The position of

the turbine bypass determines how much air is directed to the cooling

turbine. In auto approximately 5 minutes are required for the valve to travel

from one extreme position to the other. In manual operation it will travel

from full cold to full hot in approximately 4 minutes and full hot to full cold in

approximately 35 seconds.

5) Water separator							Removes 70-80% of the moisture condensed in the process of refrigeration.

(Has no ice screen) Fog may be seen coming from vents when a cold 																	temperature has been selected.

6) Anti-icing control box					Regulates temperature going through  the water separator for anti-icing of

the water separator. Two thermostats provide signals to the control box to

add more or less heat to prevent freezing.

7) Windshield defogging					Provided to direct heat for windshield defogging. Has two manually operated 																handles to position valves in the ducting to pilots and copilots windshields.

Note: In extreme cold weather operation the cooling enhancers in the																	ducting should be closed to direct heat to the windows to warm the windows															to prevent cracking	of the glass due to the cold temperatures.

8) Temperature control box				Three thermostats in this circuit provide input signals to a computer logic

circuit in the temperature control box. The logic circuit then determines the																direction in which the temperature control valve will drive.

a) Cabin thermostat					Located on the RH AFT side of the overhead panel. Operates in AUTO only

to provide the desired temperature selected.

b) Blower									Circulates air temperature over the cabin thermostat to regulate the

temperature to the desired setting.

9) Foot warmers

a) Pilots foot warmer					Manual control valves for directing heat to the pilots and copilots foot area.

b) Navigator's foot warmer			Manually controlled "T" handle at navigator's table directs heat to nav feet 																area.

10) Auxiliary vent valve					Allows outside ram airflow in flight when AUX VENT is selected. When 																	selected valve will not open until differential  pressure is below .28 PSID

(0.6 HG) to prevent the differential pressure from collapsing the air																		conditioning low pressure ducts.

b. Cargo compartment air conditioning		70 PPM unit.

unit

1) Temperature control box -				Independent of the cargo compartment temperature control system.

floor heat

2) Underfloor heat							Controlled by a two position switch (OFF, ON) on the overhead air

conditioning control panel. When the aircraft is operating at high altitudes,																where outside air temperature is very low, the cabin temperature tends to																decrease. This decrease occurs because warm air rises to the top of the

cargo compartment. To correct this condition, underfloor heating is provided.

a) Shutoff valve							Opens when the switch is placed ON. In the event of an overheat of about

180 F a thermoswitch will de-energize the solenoid , and the valve will 																	close.

b) Thermostats							Modulates the diverter valve to maintain a temperature between 70F to

80 F. This is controlled through the more heat and the less heat relays.

c) Recirculating fan						Operates when the under floor heat switch is placed ON to ensure proper 																circulation of the air entering the cargo compartment from the overhead																	ducts. The fan automatically turns on when the underfloor heat switch is

positioned to ON. Fan power is 115 V LH AC bus which is ESS DC

controlled from a switch on the overhead control panel.

3) Temperature control valves			The valve is actually two valves controlled by a single motor. The valve 																	determines how much passes through the heat exchanger. The position of

the turbine bypass determines how much air is directed to the cooling

turbine. In auto approximately 5 minutes are required for the valve to travel

from one extreme position to the other. In manual operation it will travel

from full cold to full hot in approximately 4 minutes and full hot to full cold in

approximately 35 seconds.

4) Temperature control box				Three thermostats in this circuit provide input signals to a computer logic

circuit in the temperature control box. The logic circuit then determines the																direction in which the temperature control valve will drive.

a) Cabin thermostat					Located on the LH mid way  between the utility reservoir and the crew door																in the cargo compartment. Operates in AUTO only to provide the desired

temperature selected.

b) Blower									Circulates air temperature over the cabin thermostat to regulate the

temperature to the desired setting.

5) Ice control screen						As turbine discharge air temperature drops below 32 F, the moisture on the

screen freezes. As the ice builds the turbine speed slows down, the

temperature rises melting the ice, the cycle repeats holding turbine

discharge temperature slightly above freezing.

6) Water separator							The water separator consist of a large hollow housing and a cyclonic - type

condenser. As air passes through the bag, the moisture is condensed into

droplets of water. Guide vanes are positioned on the inside of the cone

shaped frame causing the air to swirl. The swirling motion throws the																		droplets against the side of the housing where they drain by gravity to an																overboard drain. The water separator removes 70 to 80% of the moisture																condensed in the process of the refrigeration. A pressure relief valve in the

nose of the condenser opens if condenser bag becomes clogged with ice of 															foreign material.

7) Auxiliary vent valve						Allows outside ram airflow in flight when AUX VENT is selected. When																	elected valve will not open until differential  pressure is below .28 PSID

(0.6 HG) to prevent the differential pressure from collapsing the air																		conditioning low pressure ducts.

c. Differences between the Flight Deck and the Cargo Compartment Air Conditioning Systems

1) Flight  Deck System						Supplies a maximum of 30 PPM through the distribution ducts.

15 PPM if AIR COND - GTC if selected.

a) Windshield defogging				One difference between the two systems is all air passes through the water

separator for the flight station. If aux vent is selected ram air is prevented

from being directed on the windshields for defogging.

b) Pilots foot warmer					Operational during aux vent and normal operation for pilot and copilot foot 																area.

c) Navigators foot warmer				Operational during aux vent and normal operation for nav and crew bunk 																area.

2) Cargo  Compartment System /		Supplies a maximum of 70 PPM through the distribution ducts. A butterfly

Under floor heat							valve maintains a constant airflow at all altitudes to remain at 70 PPM																	airflow. The underfloor diverter valve can provide up to 43 PPM of air to the

floor heat supply ducts.

2. Pressurization system normal operations	Pressurizing the aircraft means raising the air pressure inside the aircraft

higher than outside atmospheric pressure.

a. Outflow valve									Pneumatically positioned by pressure signals from the pressure controller.

Can also electrically position a motor. It exhaust air into the atmosphere

through a louver in the RH side of the aircraft.

The major components of the out flow valve.

1. A butterfly valve and housing .

2. A mechanical linkage system.

3. An actuator diaphragm.

4. A jet pump.

5. An electric motor actuator.

b. Cabin pressure controller					Automatically controls the cabin pressure.

1) Isobaric chamber						The isobaric control system positions the outflow valve to maintain a

constant cabin pressure.

2) Differential chamber						The differential controls system positions the outflow valve to vary the cabin

pressure altitude when max differential pressure is reached (15.16" HG).

3) Rate of climb chamber					The rate control system positions the outflow valve to maintain a constant

rate of cabin pressure change up to the isobaric altitude selected. Knob

settings are:

MIN rate 30-200 FPM, MID 900 FPM, MAX 1600-2900 FPM

c. Safety Valve									The safety valve is electrically controlled pneumatically opened in a non-

pressure condition or for emergency depressurization. Normally closed but

will open to relieve pressure if positive differential pressure reaches

15.9 " HG or higher or if the negative pressure reaches -.76 "hg.

d. Check valve									A hinged flap installed in the cargo compartment air conditioning and 																	pressurization system to prevent a rapid loss of cabin pressure in the

event of failure of the air recirculating duct system. Allows inward flowing air, but will close if inward pressure is lost.

e. Cabin pressurization controls

1) Air conditioning Master Switch		See chart in section 1 of the 1C-130E(H)-1.

a) OFF

b) AIR COND AUTO PRESS

c) AIR COND MAN PRESS

d) AIR COND NO PRESS

e. AIR COND GTC

f. AUX VENT

2) Cabin pressure controller				Consist of a cabin differential pressure gage, a rate of climb indicator, a

cabin altitude selector knob, a rate selector knob, and an altitude selector 																indicator.

3) Manual pressure control switch		A 3 position (Increase, OFF, Decrease) toggle switch on the air conditioning 															control panel. The switch is powered when the AC master switch is in the

AIR COND MAN PRESS position. Increase turns the actuator toward closed, decrease turns it toward the open position. After switching from AUTO to MANUAL the pressure  control switch must be held to increase or decrease for 40 seconds to gain control of the outflow valve.  (28 V ESS DC).

4) Emergency Depressurization 			Door is located in the center escape of cargo compartment which can be 						door / handle						released	by a "T" handle on the LH  AFT side of overhead control panel.

Complete depressurization takes approx. 15 seconds after handle is pulled.

5) Emergency Depressurization			A guarded switch located on the overhead control panel is used to open the 					Switch								safety valve in the LH Aft side of the cargo door. After switch is activated it

takes approx. 30 seconds for aircraft to depressurize.  (28 V Battery Bus).

6) Pressurization test valves				For ground use only by maintenance. Safety wired open.

f. Cabin Pressurization System				Located on the overhead AC and pressurization control panel and the

Indicators									copilots inst. Panel.

1) Differential pressure gage				Indicates differential pressure in inches of mercury.

2) Cabin rate of Climb Indicator			Indicates the rate of change of cabin altitude in feet per minute.

3) Cabin altimeter							Indicates cabin air pressure altitude within the range of 0 to 50000 ft.

g. Pressurization chart							See chapter 4 in –1

NOTES

L. Characteristics of the Oxygen System

1. Oxygen System components / location

a. Converter										25 liter converter located in the RH side of the nose wheel well.

b. Heat exchanger								Increases the temperature of the oxygen delivered to the regulators and

insures any oxygen still in liquid form will be changed into gaseous form.

c. Regulators									A diluter-demand automatic pressure breathing regulator is Installed at each															crew members station, 2 on the crew bunk and 4 in the cargo compartment.

2. Portable oxygen bottles							4 bottles (2 in cockpit and  2 in cargo compartment with smoke mask) when

charged to 400 PSI can supply a 30 minute supply of oxygen.

3. Recharge hoses									Located at each portable bottle station and  used to recharge the portable 																bottles when needed to approx. 300 PSI..

4. Switches, controls, and  indicators

a. Oxygen regulator components

1) Visual flow indicator						Indicates flow and rate through regulator.

2) Regulator pressure indicator			Indicates pressure on system.

3) Supply lever								Turns oxygen supply ON / OFF to the regulator.

4) Diluter lever								Two position lever may be used to shut off the air port manually and allow

the regulator to deliver pure oxygen.

5) Emergency toggle lever				Oxygen is supplied at a slightly higher pressure. The test mask position is 																the same except the lever is spring loaded back to the normal position.

b. Liquid Oxygen quantity Indicator			Located on the copilot's instrument Panel lower RH side. It is a capacitance

type quantity indicator which monitors the total liquid oxygen available in the 															converter. Power source 115 V. AC INST. AND ENG FUEL CONT.

c. Press to test switch							Allows functional checking of the indicator when button is depressed.

d. Oxygen low level warning light			A warning indicator light next to the oxygen quantity indicator  that																		illuminates when liquid oxygen level drops below 2.5 liters. 28 V ESS DC

e. Oxygen shutoff valve	Located on the aft side of station 245 RH side is used as a means to shut              off	oxygen from the converter to the main distribution lines.

NOTE:  (Verify ON position in knob window  when checking during preflight)

5. Operation											For normal operation the supply lever is placed to the ON position and the																	diluter	lever set at 100% position.

6. Circuit protection / power supply				Quantity Indicator 115 V AC INST. AND ENG FUEL CONT.

Low level warning light 28 V ESS DC

7. Servicing											Servicing of the converter is accomplished by maintenance personnel.																	Portable oxygen bottles can be serviced by the crew members as needed																from the	recharger hoses.

8. MCR 55-130 requirements						See 55-130 chapter

9. Oxygen duration charts							See duration charts in section 4. Discuss oxygen use when all crew																		members are using oxygen.

## M. Characteristics of the Hydraulic System

1. Utility Hydraulic System normal				Utility pressure supplied from the #1 & #2  engine driven pumps is directed

operations										to the utility system flight control boost packs, flap drive,  main and nose

landing gear normal brakes, and nose wheel steering.

a. Reservoir / Servicing						The reservoir is a 3.2 gal capacity vented  to the interior atomsphere and																can be serviced inflight if needed. MIL-L 83282 fluid is used. 21 spare quarts															are carried on board aircraft if needed in an emergency.

b. Suction boost pump							Located below the reservoir to ensure adequate supply of hydraulic fluid to the

engine driven pumps to prevent cavaition of the engine driven pumps.

1) Pressure output							70-110 PSI (0-20 GPM) Low pressure light comes on at 20 PSI.

2) Circuit protection						115 V. ESS AC 3 phase motor. ESS DC controlled.

3) Suction Boost Pump switch			A two position (ON/OFF)switch on the copilots lower instrument panel is

28 V ESS DC controlled.

4) Thermal protectors						11 amp internal thermal switch opens if the pump overheats and will reset

after the pumps cools.

5) Low pressure warning light			Located on the copilots lower instrument panel  and will illuminate if the 																	pressure drops below 20 PSI.

6) Primer check valve						Allows flow of hydraulic fluid to the engine driven pumps when operating. 																Prevents the gravity flow of fluid back to the reservoir when system is not

operating. Located in the flap well area inbd side life raft tub.

c. Hydraulic pump								Located on #1 & #2 engine reduction gear box.

1) Displacement							Each pump is a variable delivery demand type. The pumps not only supply a 															volume of hydraulic fluid but also controls pressure within the hydraulic

system. Capable of providing hydraulic fluid under pressure (3000 PSI) to

the different components requiring hydraulic pressure.

(2900 - 3200 PSI normal system pressure)

2) Pressure filter							10 micron non bypassable type located on the RH side of the engine. Is																	used to prevent system contamination of fluid to the system components.

3) Shutoff valves							1 motor and 1 solenoid type requiring 28 V ESS DC for operation.

The motor shutoff valve is a gate type that has an internal switch to remove 															power to prevent the motor from burning out. The solenoid actuated valve is

spring loaded to the open position and energized closed.

4) Hydraulic Pump switch					This a two position (ON / OFF) switch, 1 for each pump located on the

copilot instrument panel, when turned off closes the  shutoff valves in the																dry bay. The switches are normally in the ON position to prevent fluid from																being	trapped in between	the shutoff valves, which could cause valve

damage. 28 V ESS DC

5) Run-around circuit						Created when the pump switches are turned off. This feature is provided to

prevent damage to the engine-driven pumps that would otherwise result from lack of fluid and overheating.

d. Pressure regulation

1) Low pressure warning switch			Down stream of the solenoid shutoff valve and prior to the one way check 																valve.

2) Low pressure warning light	Located between the solenoid shutoff valve and prior to the one way check valve.Light comes on if engine pump output pressure drops below approx. 1000 PSI.

3) Circuit protection						28 V ESS DC

4) One-way check valve					Also called the isolation check valve. Located down stream of the solenoid 																shutoff valve and prevents the other pump pressure from entering the

pressure line. Indication would be light going out on engine start of engine 																not operating.

5) Utility hydraulic pressure gage		Located on the copilots instrument panel, one for both pumps. Xmitter

located on hydraulic panel next to the reservoir. 26 V AC INST. AND ENG																FUEL CONT. BUS (fuses protected) ( # 2 inst. pwr transformer )

6) Pressure relief valve					Provides protection from system overpressures (3450 psi)

7) Accumulator								Provides pressure reserve and damping effect during demand  and pressure

fluctuations. Preload of the accumulator is 1500 +- 100 PSI.

e. Ground Test System						A ground test valve is provided to pressurize the utility hydraulic system with

the auxiliary system pressure without running engines. A single nine port 																valve provides supply, return, and case drain functions. The valve is for

ground use only, can not be used inflight since the controls are located it the															LH wheel well fairing and must be positioned manually.

2. Booster Hydraulic System normal			Booster pressure supplied from the # 3 & # 4  engine driven pumps is																	directed to operate the booster system flight control boost packs only.

a. Reservoir / Servicing						The reservoir is a 2.0 gal capacity vented  to the interior atomsphere and																can be serviced inflight if needed. MIL-L 83282 fluid is used. 21 spare quarts															are carried on board aircraft if needed in an emergency.

b. Suction boost pump							Located below the reservoir to ensure adequate supply of hydraulic fluid to

the engine driven pumps to prevent cavaition of the engine driven pumps.

1) Pressure output							70-110 PSI (0-20 GPM) Low pressure light comes on at 20 PSI.

2) Circuit protection						115 V. ESS AC 3 phase motor. ESS DC controlled.

3) Suction Boost Pump switch			A two position (ON/OFF)switch on the copilots lower instrument panel is

28 V ESS DC controlled.

4) Thermal protectors						11 amp internal thermal switch opens if the pump overheats and will reset

when the pump cools.

5) Low pressure warning light			Located on the copilots lower instrument panel  and will illuminate if the 																	pressure	drops below 20 PSI.

6) Primer check valve						Allows flow of hydraulic fluid to the engine driven pumps when operating. 																Prevents the gravity flow of fluid back to the reservoir when system is not

operating. Located in side the aircraft next to the booster reservoir.

c. Hydraulic pump							Located on #3 & #4 engine reduction gearbox.

1) Displacement							Each pump is a variable delivery demand type. The pumps not only supply a 													volume of hydraulic fluid but also controls pressure within the hydraulic

system.Capable of providing hydraulic fluid under pressure (3000 PSI) to the

different components requiring hydraulic pressure.

( 2900 - 3200 PSI normal pressure)

2) Pressure filter							10 micron non bypassable type located on the RH side of the engine. Is																	used to prevent system contamination of fluid to the system components.

3) Shutoff valves							1 motor and 1 solenoid type requiring 28 V ESS DC for operation.

The motor shutoff valve is a gate type that has an internal switch to remove 															power to prevent the motor from burning out. The solenoid actuated valve is

spring loaded to the open position and energized closed.

4) Hydraulic Pump switch					This a two position (ON / OFF) switch, 1 for each pump located on the

copilot instrument panel, when turned off closes the  shutoff valves in the																dry bay. The switches are normally in the ON position to prevent fluid from																being	trapped in between	the shutoff valves, which could cause valve

damage. 28 V ESS DC

5) Run-around circuit						Created when the pump switches are turned off. This feature is provided to

prevent damage to the engine-driven pumps that would otherwise result from lack of fluid and overheating.

d. Pressure regulation

1) Low pressure warning switch			Down stream of the solenoid shutoff valve and prior to the one way check 																valve.

2) Low pressure warning light			Located between the solenoid shutoff valve and prior to the one way check

valve. Light comes on if engine pump output pressure drops below approx. 															1000 PSI.

3) Circuit protection						28 V ESS DC

4) One-way check valve					Also called the isolation check valve. Located down stream of the solenoid 																shutoff valve and prevents the other pump pressure from entering

the pressure line. Indication would be light going out on engine start of

engine not operating.

5)Booster hydraulic pressure gage		Located on the copilot's instrument panel, one for both pumps. Xmitter

located on hydraulic panel next to the reservoir. 26 V. AC INST. AND ENG																FUEL CONT. BUS (fuses protected) ( # 1 inst. pwr transformer )

6) Pressure relief valve					Provides protection from system overpressures (3450 psi)

7) Accumulator								Provides pressure reserve and damping effect during demand  and pressure

fluctuations. Preload of the accumulator is 1500 +- 100 PSI.

3. Auxiliary hydraulic System normal operations

a. Reservoir / servicing						The reservoir is a 3.4 gal capacity vented  to the interior atomsphere and																can be serviced inflight if needed. MIL-L 83282 fluid is used. 21 spare quarts															are carried on board aircraft if needed in an emergency.

b. Hydraulic pump								A electric motor driven hydraulic pump used for normal operation of the

ramp door,  emergency brakes when selected, emergency extension of the

nose landing gear and provide a means of operating the components of the

utility hydraulic system when systems are tied together.

1) Three-phase electrical motor			Fluid cooled  motor, an ESS DC controlled relay located behind the pilots

CB panel is used to power the aux pump from the ESS AC bus.

( discuss the new type relays that are sealed)

2) Pressure displacement					The pump is a variable- volume output type which will maintain

approximately 3000 PSI  output pressure.

(2900 - 3300 PSI normal pressure)

3) Pump switches (2)						1 ON / OFF toggle switch located on the copilots instrument panel and

another located on the ramp control panel controls the aux pump relay

located behind the pilots CB panel. Both energize the same relay.

4) Turning the pump off					Wait a minimum of 10 seconds after the pump is turned OFF, to allow the																accumulator to discharge, before turning pump back on. Failure to do this																will over load the motor and open the  pump CB’s.

5) Power source							Motor, 3 phase 115 V ESS AC, aux pump relay  28 V ESS DC control for the

aux pump.

6) Handpump								A handpump in the system provides an operational source of system

pressure for ground or inflight operation. A direct reading gage located near 															the handpump shows system pressure.

c. Pressure regulation

1) Auxiliary hydraulic gages				2 gages, one located on the copilots instrument panel  powered  by 26 V AC 															from the AC INST. AND ENG FUEL CONT. BUS (FUSES)  by use of a																	remote transmitter, and the other one is a direct reading gage located in the															cargo compartment near	the handpump.

2) Emergency brake indicator			Will indicate hydraulic pressure from the aux pump or the handpump

operation.

3) Accumulator								Provides pressure reserve and damping effect during demand  and pressure

fluctuations. Preload of the accumulator is 300 PSI.

4) One-way check valves					2 - one way check valves are installed, one in the pressure line leading from

the aux pump and one leading from the handpump. The check valves allow 															handpump pressure to operate the system when the handpump is operated															and the electric pump is off.

d. Ground test valve							A ground test valve is provided to pressurize the utility hydraulic system with

the auxiliary system pressure without running engines. A single nine port 																valve provides supply, return, and case drain functions. The valve is for

ground use only, can not be used inflight since the controls are located it the															LH wheel well fairing and must be positioned manually.

e. Ramp and door							Ramp and door operation can be accomplished by the aux pump or

handpump. (Ground operation) IAW 1C-130E(H) -1

For cargo door operation with the aux pump (Verify ramp and door controls														are in neutral first) (CLOSE, NEUTRAL, OPEN) locate the	toggle switch, turn

the aux pump switch ON, move and hold toggle switch tothe OPEN position

until door is full open and in the uplock. To close the cargo door (On (E)

model aircraft prior to AF 72-1288) hold  the door switch to OPEN position

and while holding switch pull the manual door release handle until the uplock

is released,release the toggle switch to NEUTRAL and door will close, re

lease manual release handle. (On aircraft after AF 72-1288 and (H) models

use close position to close cargo door).

For ramp operation a	three position ( RAISE, NEUTRAL, LOWER) toggle switch will be positioned to LOWER to open the ramp. To raise the ramp, move the switch to the RAISE position. Both switches are spring load back to the neutral position.For handpump operation of the cargo door make sure the aux pump is not	operating, locate the three position (OPEN, NEUT,CLOSE) cargo door selector valve, select OPEN on the selector valve and operate the handpump until the cargo door is in  the uplock, return selector to NEUT. To close the cargo door select OPEN again, using the handpump, pump up the pressure until the door weight is off the uplock, pull the manual door release handle, move selector to NEUT and door will close. Return selector to neutral position, and door will close. For handpump  operation of the ramp a six position selector ( 6N, 1, 2, 3N, 4, 5, and back to 6N) knob is used. (Rotate knob CW position only to prevent seal damage inside valve).Position 1 unlocks the ramp locks, position 2 lowers ramp, position 3N is neutral, position 4 raises ramp, position 5 engages ramp locks, and 6N is a neutral position.

(See –1 ,section 1  for more detail operation on using handpump.)

NOTES

Total system capacities of the hydraulic systems

Utility

Booster

Auxiliary

Note 11-258 for oil storage bins and location/ Quantities. (Weight and balance book on acft.)

MISC:

Spare fluids (See 11-258)(in back of weight and balance book on aircraft)

22 qts engine oil MIL-L- 23699

22 qts hydraulic fluid MIL-H-83282

(See page 1-2 in 1C-130E(H)–1 for more info on emergency oil and fluid use)

M. Characteristics of the Flap / Flight Control System

1. Flap System normal operations				Normal operation is hydraulic pressure from the utility system, when the flap 															handle is positioned to the flap setting selected with the flap handle.

a. Introduction

1) Description								The flaps are fowler type, located under the trailing edge of the wings. As

the flaps are extended , the wing area is increased, which produces more lift

which improves aircraft attitude control at low airspeeds. Time for full

extension is between 8-15 seconds, and 10-15 seconds for full retraction. at

100% flaps form a 35  angle  with the wings.

2) Flap motor                               	Utility hydraulic system pressure used to power a reversible  hydraulic motor 															for flap operation.

3) Units activated for flap operation		Hydraulics motor, a cam actuated microswitch follow up mechanism, torque

tubes, gearbox, and drive screw  assemblies.

b. Emergency flap brake valve/				The Emergency flap brake is a  (28 V DC Main DC) solenoid operated					circuit protection							hydraulic valve. In its de-energized	position, hydraulic pressure passes

through it to the flap selector valve. In the event of a flap torque tube failure																or coupling coming apart, the asymmetric brake is applied.

c. Flap lever										Flap handle positions the flap quadrant in the pedestal, also includes a pulley

and lever, a friction brake, and two switches. The first switch allows high

rudder boost at flap settings below 15%, the other switch is at the 70%

setting. ( At 70% and below on the flap handle, horn will not silence until

gear is down and locked).

d. Flap drive control unit						Consist of a ring cam, which is moved by the cable system, and two

switches. One of the switches is closed for up travel and the other is closed

for down travel. As the flap lever is moved the up or down switch is closed

by the rotating cam, which energizes the flap control valve.

e. Wing flap control valve 						A solenoid operated (28 V Main DC) directs hydraulic pressure to the up or 															down side of the flap motor for extension 	or retraction of flaps. Has two

override buttons marked raise and lower for use in case of electrical failure.

f.  Flap brake									Each time the raise or lower solenoid valve is actuated the flap brake is

released allowing the flap drive motor to operate for flap operation.

g. Flap motor									The hydraulic motor is a 9 piston, reversible motor.

h. Asymmetrical sensing switches			There are 3 switches and 3 cams located in each of the asymmetry brakes.

The cams are timed to prevent any 2 switches from being ‘MADE” at the

same time as long as the flaps are symmetrical. One of the switches on each side is closed at all times. When 2 of the switches are made in series the asymmetrical  flap brake control valve will energize.

1) Location									A wing flap asymmetry brake is splined-coupled to the outermost jackscrew

actuator on each side of the aircraft.

2) Function									The asymmetrical flap brake control valve, when energized, removes

pressure from the flap control valve and directs pressure to the flap

asymmetry brakes, this locks the torque tubes. In the energized position

the valve also connects the pressure port of the flap control valve to return.

This relieves hydraulic pressure on the spring applied flap brake to lock the

drive assembly, allowing the brake to lock the drive assembly.

i. Emergency flap brakes						Two  emergency flap brakes, one located at each of the outbd flap drive

gearboxes.  They are spring loaded released and hydraulic pressure applied

from pressure supplied from the emergency flap brake valve. Can only be

manually released on the ground.

j. Manual shift handle							A "T” handle located on the LH fwd MLG wheel well is used to shift from

hydraulic to manual drive in case of an emergency.

k. Handcrank									The handcrank can be used to manually extend the flaps by rotating the stub

next to the “T” handle. (SEE EMERGENCY PROCEDURE IN SECTION 3)

After manually extending the flaps install the pip pin in the stub to prevent

the flaps from rising due to the air load on the flap surface.

l. Position indicator

1) Location									Located on the copilots instrument panel.

2) Transmitter								Located on the bottom of the flap drive control unit. Transmits a signal to the

indicator in the cockpit in 10 % increments. Flaps gage should read

0%  0%, 50%   8%, 100%  7% for the different flap settings.

3) Control power							28 V Main DC (CB located on the Aft junction box)

m. Rudder boost pressure					High rudder boost is available at flap settings below 15%. The switch is

located in the center pedestal below the flap handle on the quadrant..

2. Flight Controls Systems Normal Operations

a. Flight Control Systems Introduction / Rudder System

1) Introduction

a) Flight control systems				The flight controls include main surface control systems, which are aileron,

rudder, and elevator systems and tab control systems.

Control is by cables, pushrods, bellcranks, and torque tubes.

b) Flight control boost packs			The boost packs are mechanical systems with hydraulic boost. Pressure

is supplied from the utility and booster hydraulic systems and from the

aux  system on the ground.

2) Rudder System							The rudder booster assembly is a single tandem type hydraulic actuating

cylinder which furnishes most of the force  to activate the 	rudder.

a) Controls movement about vertical axis

b) Receives utility & booster pressure

c) Diverter valves & pressure reducers

During normal operation, fluid supplied at approx. 3000 PSI pressure 																	routed by solenoid- controlled, normally de-energized diverter valves.

(fail safe to the low side)( located on the rudder pressure reducer control 																panel) The panel also has a pressure relief valve set at approx. 1600 PSI

to prevent overpressure in low rudder boost.

1 Rudder Booster Pressure 		The transmitter switch is 28 V ESS DC powered located on the rudder 								Switch							reducer control panel in the aft  LH and RH cargo door area.

2 Flaps extended below 15%		For landing and takeoff when flaps are extended below 15% the pressure

will read 3000 PSI. A micro switch in the flap quadrant	is activated when the															flap handle is positioned to power the diverter valve to the high pressure 																side or de-energized it to the low pressure side.

3 Flaps retracted above 15%		Pressure reduced to approx. 1100 - 1400  PSI (1300) at cruise This is					done to prevent excessive loads on the vertical stabilizer and rudder

hinges points.

d) Pressure gages						Two  pressure transmitters transmit pressure to two pressure indicators 																on the copilot's instrument panel. The gages are powered from the #1

instrument transformer (booster) and the # 2 instrument transformer 																		(utility). Gages will indicate pressure prior to it entering the boost pack so

normal and reduced pressures will be indicated depending on the flap

setting above or below 15% flap handle positioning.

e) Shutoff valves						2 booster shutoff valves, normally open, are used to isolate the rudder 																	boost packs (utility or booster) in case of an hydraulic leak on the boost 																	pack.

1 Booster Shutoff Switches		Two  guarded (OFF/ON) switches energize normal off solenoids to isolate																the boost pack selected. A light above the switches indicates the circuit is 																activated, not that the valve closed. After a boost pack is isolated a																		hydraulic check valve in the return line prevents return pressure from																	other	operating systems from entering the boost pack.

2 Circuit protection					28 V. ESS DC (CP lower CB panel)

3) Aileron System							The aileron booster is a single tandem type actuating cylinder which 																furnishes most of the force to actuate the ailerons.

a) Controls movement about Longitudinal axis

b) Operated by pilots or copilots control wheel

c) Control valve							Controls fluid flow in and out of the actuator from inputs from the 																	control wheels.

1 Location							Located on the boost pack aft side.

2 Applies utility pressure			Pressure from the utility and booster systems position the cylinders

and booster pressure			from inputs from the control wheels to the control valve.

d) Actuating cylinder					Single tandem type

e) Booster shutoff switches			2 booster shutoff valves, normally open, are used to isolate the aileron 															boost packs (utility or booster) in case of an hydraulic leak on the boost 															pack.

1 Control switches					Two  guarded (OFF/ON) switches energize normal off solenoids to isolate														the boost pack selected. A light above the switches indicates the circuit is 														activated, not that the valve closed. After a boost pack is isolated a																hydraulic check valve in the return line prevents return pressure from															other operating systems from entering the boost pack.

2 Circuit protection					28 V. ESS DC (CP lower CB panel)

f) Pressure reducers					Two pressure reducers, one for utility and one for the booster  system 															reduce hydraulic pressure down to 2050 PSI as not to exceed structural

limitations of the aircraft. (external tank pylons).  A Pressure relief set at

2300 is used as a backup in the event of pressure reducer failure.

4) Elevator System							The elevator booster assembly has dual actuating cylinders connected to 															the booster  assembly output lever that operates the elevator  control

surfaces. 3000 PSI  pressure used for actuator operation .

a) Controls movement around lateral axis

b) Operated by pilots or copilots control column

c) Boost pack

1 Location							Located on the aft bulkhead in the cargo compartment near the rudder 																unit.

2 Cylinders operate in sync		The assembly is controlled mechanically by the fore and aft movement of 															the yoke in the flight station. Each cylinder has a single piston actuator 																and are connected as a single unit to work as single unit.

d) Boost pack selector valve			The selector valve receives inputs, from the control column fore and aft

movement through cables, to flow hydraulic fluid to the actuating cylinder

to position the elevator surface to the input position selected.

e) Shutoff valves							2 booster shutoff valves, normally open, are used to isolate the elevator 																boost packs (utility or booster) in case of an hydraulic leak on the boost 																	pack.

1 Control switches				Two  guarded (OFF/ON) switches energize normal off solenoids to isolate														the boost pack selected. A light above the switches indicates the circuit is 														activated, not that the valve closed. After a boost pack is isolated a																hydraulic check valve in the return line prevents return pressure from															other operating systems from entering the boost pack.

2 Circuit protection				28 V. ESS DC (CP lower CB panel)

NOTES

N. Characteristics of the Trim Tab System

1. Trim Tab System

a. Introduction									Trim tabs are provided on the control surfaces to aid in trimming airplane 																	during flight.

b. Aileron trim tabs								The ailerons have two trim tabs, LH is adjustable with an electric motor,

and the RH one is fixed and ground adjustable to compensate for inherit 																	unbalance about the longitudinal axis of the airplane.

1) Location									Trailing edge of the aileron assembly.

2) Function									Lateral trim is obtained through the operation of a trim tab on the LH aileron

by a switch on the flight control pedestal.

c. Aileron trim tab actuator					The actuator is screwjack AC motor operated by 115 V  ESS AC single

phase power and controlled by 28 V. ESS DC power from  a switch on the

flight control pedestal. Tab movement is controlled by moving the switch																right or left, switch is spring loaded to the off position . Two control relays 																are energized by the switch. (LOCATED ON UPPER LH SIDE OF 245)

d. Elevator trim tabs							The elevators have two electrically operated adjustable trim tabs.

1) Location									Trailing edge of the elevator assembly.

2) Function									Adjust for nose up and nose down trim by sliding thumb switches located on

the pilots and copilots control wheels in normal or by a switch located on the															flight control pedestal in emergency.

e. Elevator trim tab actuator						The motor is operated by 115 V  ESS AC single phase power and controlled 														by	28 V ESS DC power. Tab movement is controlled by moving the sliding 															thumb switches forward or aft and is spring loaded to off position . Two dual														control DC relays are energized by the switches for tab movement. In the															emergency position 28 V. ESS DC controls the tab movement from  a																switch the flight control pedestal. Tab movement is controlled by moving the														switch on forward or aft, switch is spring loaded to the off position . The AC															and DC motors are both in the motor housing together and connected by 															flexible shafts that transmit twisting  motion to 90 gear boxes and																	screwjacks for trim tab positioning.

f. Rudder trim tab								1 trim tab

1) Location									Trailing edge of the rudder

2) Function									Adjust for yaw.

g. Rudder trim tab actuator					The motor actuator is operated by 115 V  ESS AC single phase power.

h. Trim Tab Control Panel					Located in the flight control pedestal.

I. Trim Tab Thumb Switches					Located on the LH pilots and RH copilots control wheels.

j. Trim Tab Position Indicators				3 position indicators located on pilots inst. panel, powered by MAIN DC from 															the aft junction box.

2. Switches, controls, and Indicators

a. Trim tab switches and controls

1) Rudder Trim Control					3 position switch, Nose LT, Nose RT, and spring loaded to the off position.

Indicator reads 0-20 LT and   0-20 RT in 5 increments.

2) Aileron Trim  Control					3 position switch, LH wing UP and LH wing DN and spring loaded to the off 															position. Indicator reads 0-20 UP and  0-20 DN in 5 increments.

3) Elevator Trim Control					(2) 3 position thumb switches and (1) 3 position Emergency operation

switch, both spring loaded to the off position.  Indicator reads 0-25 UP and																0-25 DN in 5 increments, actual trim tab movement is limited by switches																to 6 nose up and 25 nose down. Mechanical stops are set at 8 nose up 																and 27 nose down.

4) Elevator Tab Power					A 3 position toggle switch, located on the flight control pedestal on the

Selector Switch							center console. In normal position 115 V ESS AC is the power source

for tab operation, in emergency position 28 V ESS DC power operates

the trim tab, and in the off position all circuits are de-energized to the

elevator trim tab system.

5) Thumb Switches						Two thumb switches on the pilots and copilot's control wheels operate the

elevator trim tabs in the normal position.

6) Elevator Emergency 					A 3 position switch on the center control pedestal operates the elevator

Trim Tab Switch						trim tabs in the emergency position.

7) Circuit protection						115 V. ESS AC motor operation, 28 V. DC controlled in normal, and

28 V. ESS DC in emergency position.

NOTES

O. Characteristics of the Landing Gear  System / Brake System

1. Main Landing gear								The main landing gear consist of four strut -wheel assemblies paired in

tandem configuration and connected by a drag strut.

a. Strut assembly								The strut assembly is serviced with hydraulic fluid and air pressure which

act together to absorb the shock of landings and while taxing cushions the																aircraft to prevent structural damage. An axle  on the lower section of the																strut  for is the attaching point of the wheel  and brake assembly.

b. Torque strut									The torque strut connect the two strut assemblies together to maintain 																	wheel alignment.

c.  Retracting mechanisms

1) Hydraulic motor							A 9 cylinder fixed displacement, reversible hydraulic motor is used to

operate each of the MLG retracting mechanisms. Direction of rotation is																	determined by	fluid flow, and this is determined by the positioning of the

landing gear selector valve. The motor is mounted on the forward side of the 															single speed gear box.

2) Main landing gear gearbox			The gear box incorporates a normal drive feature, an emergency manual

drive feature, and a hydraulically released, spring applied unlock brake.  For

normal operation, a sliding clutch connects the motor drive gear to the

output  gear.

3) Gearbox	(90)							(2) 90 gearboxes driven by the main landing gearbox are connected to																	together 	by a horizontal torque tube that rotate vertical torque tubes.

4) Vertical torque shaft					The vertical torque tubes rotate the jackscrew which raises  or lowers the

gear by means of a ballnut connected to the strut lower flange.

5) Ball screw

1 Ball nut									The ballnut is connected to the strut lower flange and as the jackscrew

rotates raises or lowers the gear.  The vertical tracks	keep the strut aligned															as it travels 	up and down. When the gear reaches the full up position a

pressure reducer slows the gear speed down to prevent the gear from																	slamming into the upper bumper stop, which consist of ring spring which																softens the impact of the ball nut as it reaches its limit of travel. When

lowering the gear, it contacts the lower bumper stop which consist of a 3																piece ring spring which softens the impact of 	the ball nut as it reaches its 																limit of travel.

2 Down and locked Vs unsafe		Discuss and show the down and locked visual indications. The ball nut 																	contacting the lower bumper stop is the -1 procedure. Discuss the main																	landing gear tie down procedure with chains and the new tie down device.

6) Friction washer							The friction washer is designed to hold the MLG down when utility hydraulic 															pressure is off the system. The weight of the MLG compresses a washer 																between the cap nut and the flange above the washer to  prevent the

jackscrew from rotating.

7) Drag pins									Two dowel  pins extend from the bottom of the strut lower flange and fit into 															holes in the shelf bracket when the MLG extends. the dowel  pins, tracks 																and shoes take the fore, aft and side loads of the landing and takeoffs.

8) Main Landing gear doors				The MLG doors operate from linkage attached to aft strut.

2. Nose Landing System

a. Nose landing gear actuator				The NLG actuator is a combination drag strut and hydraulic actuating

cylinder incorporating an internal lock.

1) Internal lock								The internal  lock is a cylindrical segment (lugs) that slide in radial holes in

the piston head.

2) Lock indicator piston					Movement of the lock piston forces the segment locks outward and extends

the lock indicator piston.

3) Retracting operation					During operation pressure applied to the up port causes the lock piston to

move from between the lock segments and retract the lock indicator.

4) Down and lock indicator				Extends out of the end cap to provide a visual down and locked indication

and also contacts the microswitch to provide a down and locked indication

on the control panel on the copilot's instrument panel.

b. Nose gear uplock assembly

1) Retracted									When retracted, the uplock link of the nose gear forces the latch guide

upward and the latch jaws engage the uplock link.

2) Ball detents								Ball detents inside the actuator housing wedge from between the fingers

keeping the jaws locked.

3) Extension									For extension, hydraulic pressure moves the actuator wedge from between

the fingers, allowing the uplock to release.

4) Manual release T handle				Located next to the copilots seat for emergency extension of the NLG, with

all hydraulic pressure removed.

c. Nose gear door linkage					Connects to the trunnion on the NLG, opens and closes the FWD and AFT 																doors	with retraction and extension of the NLG.

3. Nose wheel steering

a. General description							Used to steer the aircraft on the ground by pilot input to the control wheel by

use of cables, pulleys, chains, and actuators.

b. Components and their functions

1) Steering control valve 					Spring loaded to the neutral position, ports fluid to the steering actuators.

2) Steering actuators						2 are located on the aft side of the NLG strut supply power for turning the 																strut.

3) Torque arms								2 torque arms connect the strut to the steering actuators. Disconnected for 																towing to prevent damage to the actuators since the aircraft can be turned

shorter during towing. (Always check pin scissors for proper installation.)

4) Strut piston								Acts as a shock absorber for the NLG during taxi, takeoff, and landing.

5) Landing gear centering				The NLG uses an internal cam when the weight of the aircraft is off the

gear to center the NLG. (Arrow in cockpit should be centered in flight).

c. Limitations									120 turn arc with torque arms connected and up to 180, 90  from center

with arms disconnected (towing). (See weight limitations)

d. Operation									The airplane is steered during taxi by directional control of the nose wheel.

The steering control valve is positioned by the input signals from the pilot as

the steering wheel is rotated right or left of center. The control valve directs

fluid to the actuators to turn the nose tires RT or LT. Orifices in the steering

cylinders provide snubbing to dampen oscillations of the nose wheel and to

prevent shimmy.

4. Main landing gear operation

a. Landing gear selector operation			Positioning of the 2 position (UP, DOWN)landing gear handle on the

copilot’s instrument panel directs the gear actuating mechanism  to raise or 															lower the nose and main landing gears.

b. Landing gear selector valve 				When the lever is moved to  the down position, The NLG uplock is released

down position								the MLG brake locks that hold the gear in the up position are released,	and															the NLG and the MLG extend.

.

c. Main landing gear down operation		When the landing gear handle is positioned to the down position, it enters a																detent to  hold the gear handle down. After landing, the touchdown switch

de-energizes the landing gear lever release solenoid, engaging a

mechanical locking device to hold the gear lever down.

d. Landing gear selector valve 				When the landing gear handle positioned in the UP position, a solenoid																	operated selector valve directs pressure to move the gear UP.

e. Main landing gear up operation			When the landing gear handle is positioned to the up position, a solenoid																operated selector valve directs pressure  from the utility hydraulic system to

release the nose gear downlock and the landing gears retract.

f. Landing gear selector valve 				When the gear is full UP, the up limit switches de-energize the solenoid					neutral position								valve to the neutral position, fluid flow stops, brakes are spring applied to																hold the gear UP.

5. Nose gear operation

a. Down operation								When the landing gear handle is placed down hydraulic pressure through 																the landing gear selector valve ports fluid to the NLG actuating cylinder ,

the nose downlock indicator piston, and the nose gear uplock. Pressure is

also available for nose wheel steering.

b. Up operation									When the landing gear handle is placed UP hydraulic pressure through the 																landing gear selector valve ports fluid to the NLG actuating cylinder , the

nose downlock indicator piston, and the nose gear uplock.

c. Emergency operation						A manually operated NLG emergency extension valve located in the cargo

compartment just aft of 245 on the LH side can be used to extend the NLG

by use of the Auxiliary pump or the handpump. If no hydraulic pressure is

available a handle left of the copilots seat can be used to extend the gear.

(See section 3 in the -1)(Gear can only be lowered) Nose wheel steering

will not be available during this procedure.

6. Normal / Emergency Brake System

a. Normal brakes								Pressure supplied from the utility hydraulic system.

1) Check valves							Allows pressure to be trapped in the line if operating pressure is lost. This

keeps a preload on the accumulator to allow more than one brake

application if need in an emergency.

2) One way restrictor						Installed on the normal brake accumulator and provides a priority of pump

flow to the brake assemblies during anti-skid demands.

3) Accumulator / brake applications		100 cu in, Air charged to increase supply of hydraulic pressure in case of

pressure failure,  preload of 1500  100 psi,  allows about 2 applications if																anti-skid is turned off, 1 application if not. 2 brake applications at 2900 psi,

one @ 2250 psi.

4) Pressure transmitter					26 V. AC, AC INSTRUMENT AND ENG FUEL CONT. BUS  (FUSES)

# 1 & 2 inst. pwr transformers. Located in the nose wheel well RH side

under the LOX converter. Sends a signal to the pressure indicator on the 																copilots instrument panel .( NORMAL BRAKES)

5) Brake selector valve					When normal brakes are selected the normal brake selector valve is																		de- energized to open and the emergency brake selector is energized 																	closed.

6) Dual brake control valves				Located beneath the flight station floor, outbd of the flight control columns.																The valve on the LH side is for the LH brakes and the RH one for the RH																brakes. Both can be actuated by either the pilots or copilots brake pedals.

7) Hydraulic fuse							(2 fuses, one for each brake) The hydraulic fuse is a valve that blocks																	pressure 	when a  predetermined pressure drop and fluid loss occurs.

Without this protection the failure of a hydraulic component downstream of

the of the valve could cause complete loss of fluid in the system. Located in															the top of the wheel well above the bulkhead fittings.

8) Shuttle valve								Located on the brake assembly, allows the system with the higher pressure															will cause the shuttle valves to shift as necessary to provide pressure to the 															brakes.

9) Parking brake shutoff valve			The parking brake is applied by depressing the brake pedals,  and pulling																the "T" handle on the pilots side. When the parking brake is set with power

ON the airplane the anti-skid switch ON, a valve is positioned to block the

return port of the anti-skid valve. This prevents rapid leakage and

subsequent release of the pressure used fro setting the parking brake.

b. Anti-skid System							The anti-skid system prevents skidding of wheels when too much brake																	pressure is applied during airplane deceleration's.

1) Transducer								4 Transducers, one in each axle assembly controls the braking action 																	through the anti-skid valves when the wheel begins to approach a  skid 																	condition.

2) Anti-skid control valve					One located above the booster reservoir on the right forward side of the RH 															wheel well, and the other is on the LH hydraulic panel fwd of the utility

hydraulic panel.

3) Anti-skid control box					Uses four modules to sense wheel speed and make necessary inputs to the

anti skid control valve for proper brake pressure modulation.

4) Anti-skid Test panel					26 V AC INSTRUMENT & ENG FUEL CONTROL BUS power.

A three position switch (FWD, OFF, and  AFT) and four green indicator

lights ( LEFT FWD, RIGHT FWD, LEFT AFT, AND RIGHT AFT).

5) Anti-skid operation						The system prevents the skidding of the wheels when too much brake																	pressure	is applied during deceleration. This is accomplished through a																	brake releasing system controlled by the signals from wheel speed

transducers mounted in the axle of each MLG.

6) Positive locked wheel 					The transducers form part of an electrical circuit which prevents landing with 					protection circuit					the brakes on, and which releases the brakes in case of a locked condition.

7) Landing with brake pedals				Do not land with brake pedals depressed. Wheel Tire and MLG damage

depressed 								could occur in case of anti skid failure.

8) Test function								When the test is placed to the fwd or aft  position 26 V, 400 CY power is																	applied is to the anti skid control box simulating a skid condition. When the																switch released to off, the selected lights should illuminate momentarily.																	Wait at least 3 seconds between fwd and aft test position to prevent

erroneous test indications. Illumination of the lights indicate that the anti																	skid control box would of have responded to an actual condition.

c. Emergency Brakes							Used when normal brakes are inop or during an emergency.

1) Hydraulic source 						Pressure supplied from the auxiliary hydraulic system.

2) Check valves							Allows pressure to be trapped in the line if operating pressure is lost. This

keeps a preload on the accumulator to allow more than one brake

application if need in an emergency.

3) Accumulator /							50 cu in, Air charged to increase supply of hydraulic pressure in case of

brake applications						hydraulic pressure failure,  preload of 300 psi, allows about 1 brake

application.

4) Pressure transmitter					26 V. AC, AC INSTRUMENT AND ENG FUEL CONT. BUS  (FUSES)

# 1 & 2 inst. pwr transformers. Transmitter located in the nose wheel well 																LH side forward of the aircraft battery compartment box. Sends a signal to																the pressure indicator 	on the copilots instrument panel . (EMERGENCY 																BRAKES)

5) Brake selector valve					When emergency brakes are selected the emergency brake selector valve																is de-energized to open and the normal brake selector is energized closed.

6) Dual brake control valves				Located beneath the flight station floor, outbd of the flight control columns.																The valve on the LH side is for the LH brakes and the RH one for the RH																brakes. Both can be actuated by either the pilots or copilots brake pedals.

7) Hydraulic fuse							(2 fuses, one for each brake) The hydraulic fuse is a valve that blocks																	pressure 	when a  predetermined pressure drop and fluid loss occurs.

Without this protection the failure of a hydraulic component downstream of

the of the valve could cause complete loss of fluid in the system. Located on

the drag strut on the upstream side of the brake shuttle valve.

8) Shuttle valve 							Located on the brake assembly, allows the system with the higher pressure															will cause the shuttle valves to shift as necessary to provide pressure to the 															brakes.

NOTES

Discuss gear components, pork chop fittings, brake shuttle valves, hydraulic lines, brake fuses, normal/emergency, ETC.

d. Use of wheel brakes						See section 7 in 1C 130B -1

It is absolutely necessary that the airplane brakes be treated with respect. When landing or taxing conditions

permit use reverse thrust as much as possible to maintain safe taxi speeds and thus minimize brake wear and

heat build up in the brake/ wheel/ tire assembly.

Taxing: 										a. Pilots must ensure brakes are not dragged to control taxi speed.

b. Reverse thrust and LSGI are the primary means of controlling taxi speed.

Stop the aircraft if oil temperatures are excessive, cool the oil

temperatures, continue the taxi, repeat if necessary.

c. Use brakes as little as possible for turning the airplane on the ground.

d. Do not taxi into crowded areas if overheated brakes are suspected or 																	known.

Landing:										a. The full landing roll and propeller reversing should be used to minimize

brake wear.

b. After landing where brakes are not used and only checked during ground

roll allow 10 minutes cooling time preceding the next takeoff to account

for brakes during taxi. This is required because CFL increases due to the

brakes being above ambient temperature.

1. Caution when applying brakes with a tailwind.

2. Note, If runway available exceeds CFL by 300 ft. the 10 minute cooling

time may be omitted.

c. If landing ground roll must be minimized, the following procedures provide

maximum braking offset.

1. Immediately after touchdown, lower the nose and smoothly apply

steady increasing brake pressure until maximum pedal travel is 																			achieved.

2. If a full anti skid braking  is used for landing, the gear should be left 																		extended after an immediate takeoff for a minimum of 15 minutes

before the gear or another brake landing is attempted.

3. Unless operational required, a mission sortie should not terminate with

a landing requiring full anti skid braking.

4. If a short field landing requiring full antiskid braking is followed by a																		engine running off load (I.E. brake being used and set) heat may build																	up in the tire/ wheel/ brake assembly. Minimize the brake use, keep																		the personnel clear of the wheel well area, be prepared to evacuate

aircraft if overheating is indicated. WARNING: Failure to cool brakes

could result in tire explosion or wheel well fire.

d. If conducting a series of full anti skid braked landings the minimum

airborne cooling interval between landing is 15 minutes. To operate at

this minimum gross weight is limited to 130,000 lbs. or less, landing gear

extended in the pattern, no tailwind factor.

e. A partially brake landing is defined as a 3 second brake application with

steady brake pressure at approximately 90 KIAS.

1. The landing interval during stop and go or full stop taxi back profiles is 																	limited by the time required to fly a normal traffic pattern.

2. When conducting a series of partially braked landings at minimum 																		intervals, full	 brake applications must not be exceed 3 seconds,

gear to remain extended during traffic pattern, gross weight limited to

130,000- lbs or less, and no tailwind factor.

3. When conducting a series of touch and go landings, no minimum

interval  for brake cooling time is required.

f. Regardless of the braking technique used, the following precautions will

minimize brake wear and should be observed.

1. If antiskid is inoperative use extreme care when applying brakes

during touchdown and before applying reverse thrust, or anytime

there is a considerable lift on the wings.

2. Heavy pressure can lock the wheels.

3. WARNING: After any full antiskid braking operation above 130,000

lbs.(aborted takeoff, eng out/ flap landing ETC.) assure adequate

brake/ tire cooling prior to further operation.

4. Approximately ground cooling time is 65 minutes.

5. Main wheels have 3 fusible plugs installed on the inside of the wheel

assembly. When wheel temperature rises above to above 390 F the

plugs core melts, allowing tire to deflate at a safe rate.

Parking: 	If brake failure indicated, or used excessively, or hot, request the fire department crash crew to inspect 							the tire and brakes as a fire is possible.

- All other personnel stay clear 300 ft.

2. If conditions require to be close to the tire/ wheel assembly approach

from the fore or aft only.

3. Peak temperatures occur in the brake assembly in approximately

1-5 minutes and in the tire and wheel assembly 20-30 minutes after

a  maximum braking.

4. If hot brakes are suspected or known one side set the opposite 																			brake, chock the nose gear and proceed with the ground

evacuation. If both brakes are hot chock the nose gear only, and

proceed with the ground evacuation.

5. Do not taxi or tow for at least 15 minutes after overheated brakes.

6. Record in the AFTO forms 781 if hot brakes are known / suspected.

Digital flight recorder (DFDR)

The DFDR unit located overhead at FS 891, WL 270  (tail section) records data from a flight data acquisition unit in the cargo compartment FS 330 (hog trough). The recorder has a  tape capacity is 25 hours of airplane systems and flight data.

The unit receives signals from the following systems:

ENGINES: 				PROPELLER:			   HYDRAULIC:						COMMUNICATION:

Tachometers	  		Synchrophaser			   Low pressure warning		 	Intercommunication system

Torquemeters			Low oil warning	 			   							 	      UHF # 1 & 2

Low oil warning											LANDING GEAR:			  	 VHF # 1 & 2

Condition levers		FLIGHT CONTROLS:			Landing gear position			   HF # 1 & 2

Throttles 					Flap position

Fire warning				Elevator position			MANEUVER LOAD DATA:		FLIGHT DATA:

Nacelle overheat		Rudder  position			Three axis accelerometer			Airspeed

Altitude

To test the recorder a  guarded switch (same switch as the CVR test switch) above the nav table is used to bypass the external  power relay  which allows the DFDR to be tested as the CVR is being tested. When internal aircraft  power is applied to the DFDR system, the recorder starts operating.  A  monitor light comes on if the recorder stops recording.

Power source 115  V. ESS AC &  28 V. Main DC for the monitor light.

Cockpit voice recorder (CVR)

The CVR is a continuous operating magnetic tape device that receives	signals from the pilots, copilots, engineer, and cockpit area microphone. The cockpit area microphone located above the pilot's head is dedicated to the fourth channel of the voice recorder. The tape is capable of recording for 30 minutes, recording beyond 30 minutes are erased. The recorder is equipped with an underwater located beacon assembly that is battery powered, water sensitive activated, and radiates a  pulsed signal into the surrounding water. A test switch (button) controls a circuit within the recorder. When pressed a 600 HZ tone is applied to the four channels which can be checked with a headset plugged into the headset test jack to test the aural indication and to test the meter for a visual go-no-go indication. To test the recorder a  guarded switch above the nav table is used to bypass the external  power relay  that allows the CVR to start operating when the external power is disconnected.

Power source 115  V. ESS AC.

(See1C-130E(H) –1 section 2 on test operation of CVR)(page 2-22/23)

e. Section VII limitations

LIMITATIONS - 1

PWR  SECTION OIL PRESSURE

Max (start & warm up)					100 psig

Maximum allowable			  			60 psig

Normal					 				50  60 psig

Minimum (LSGI)				 			50 psig (Normal at 100% RPM)

Minimum allowable 						Below 50 psig if normal at 100% RPM( See note, section V)

Allowable fluctuation			 		 10 psig from mean

GEAR BOX OIL PRESSURE

Max (start & warm up)					 Above 250 psig

Maximum allowable						 250 psig

Normal									 150  250 psig

Minimum (LSGI)				 			 50 psig (Normal at 100% RPM)

Minimum allowable						 130 mission accomplishment ( See note, section V)

Allowable fluctuation			 		 20 psig from mean

OIL QUANTITY

Maximum									12 Gallons

Normal									4  12 Gallons

Minimum									4 Gallons

OIL TEMPERATURE

Maximum ground (30 min)				85 100 C

Maximum flight    ( 5 min)				85 100 C

Normal									60   85 C

Minimum 									40 C

Minimum (for start)		            		 40 C  MIL-L- 23699

Minimum (feather check)				20 C

ENGINE TORQUE

Oil temperature below 0 C.				Minimum

Oil temperature below 0				 40 C.		4500 in-lbs

Maximum allowable						19,600 in-lbs

Maximum continuous					18,000 in-lbs

NTS action 								1260  600 in-lbs

Propeller reversing check				1000 in-lbs

Minimum (for runup) 					8000 in-lbs

Maximum (10 kt wind  @ 30)			7000 in-lbs

Torque comparison check				200 in-lbs increase

LIMITATIONS - 2

STARTING TIT

Maximum (for enrichment)				100 C

Maximum (normal start)					200 C

Abnormal torching						Inspect

Normal starting TIT						750830 C

Max during start							830 C

720 C or below							Stop start, Maintenance action prior to flt.

720 – 750 C								Record 781,Temp controlling check

830 850 C								Record in 781

850 965 C 							Stop / start, record in 781

Exceeds 965 C							Stop / start, inspect

Exceeds 977 C (5 sec)					Inspect before flight

Exceeds 1070 C (momentary)			Inspect before flight

ENGINE TIT

Takeoff maximum (5 min)				961 – 977 C

Military (30 min)							977 C

Climb										932 C

Maximum continuous					932 C

Crossover									740 – 780 C

Normal limiting							977 C

ENGINE RPM %

Maximum for lightoff					35%

Maximum for oil pressure				35%

Low speed ground idle					69% – 75.5%

Normal ground idle						94% – 102%

Maximum reverse						96% – 106%

Flight idle									92.5% – 100.5%

Normal									98%  102%

Starter disengagement					60%

Allowable flux							 .5%

AC ELECTRICAL

Frequency range							380  420 cps

Minimum voltage						110 volts

Maximum voltage						125 volts

Maximum AC load			1.05%

LIMITATIONS - 3

DC ELECTRICAL

Maximum voltage						30 volts

Minimum									25 volts

Maximum DC load						1.03%

Battery voltage (normal)					24 volts, 36/31 amps

Battery voltage (minimum)				21 volts

Battery voltage (SCNS)					21 volts

PROPELLER DE–ICING

Minimum load							65 amperes

Maximum load							90 amperes

Timing cycle								15 sec. on, 45 sec. off

Maximum ground use					Two cycles

BLEED AIR SYSTEM

GTC pressure output						35 psi minimum

GTC bleed air check						30 – 15 psi NLT 8.5 sec.

Engine bleed air check					65 – 35 psi NLT 10 sec.

PRESSURIZATION

Maximum allowable						15.8 in. HG

Aux vent valve open						.28 psid or .6 in. HG

Safety valve open						– .76 to 15.9 in. HG

Windshield crack 1– pane				10 in. HG

Windshield crack 2– pane				10 in. HG

Cargo portal crack 1– pane				10 in. HG

Cargo portal crack 2– pane				Zero pressure

LEADING EDGE ANTI–ICING

Maximum ground use					30 sec. (maintenance only use)

Inoperative range							Below 75 F

Normal range								75 – 200 F

Overheat range							Approximately 200 F

Radome ground use						Do not operate

WINDSHIELD ANTI–ICING

Maximum OAT ground check			+ 27 C

Cold  start use							– 43 C

Cold start cycle							5 sec. on, 10 sec. Off (Switches in normal)

LIMITATIONS - 4

AUXILIARY HYD PRESSURE

Maximum allowable						3500 psi

Normal									2900 – 3300 psi

EMERGENCY BRAKE PRESSURE

Max allowable							3500 psi

Normal									2900 – 3300 psi

1 brake application remains				2900 psi

NORMAL BRAKE PRESSURE

Max allowable							3500 psi

Normal									2900 – 3200 psi

2 brake application remains				2900 – 3200 psi

1 brake application remains				2250 psi

UTILITY / BOOSTER HYD PRESSURE

Max allowable							3500 psi

Normal 									2900 – 3200 psi

RPM for indication						On speed

RPM for normal pressure				On speed + 30 seconds

LSGI										2550 psi

RUDDER BOOST PRESSURE

0 – 15 % flaps (maximum)				1600 psi

15 – 100% flaps (maximum)			3500 psi

0 – 15 % flaps (normal)					1100 – 1400 psi

15 – 100% flaps (normal)				2900 – 3200 psi

0 – 15 % flaps (caution)					1400 – 1600 psi

HYD ACCUMULATOR PRELOAD

Utility / Booster system					1500  100 psi

Auxiliary system							300 psi

Normal brakes							1500  100 psi

Emergency brakes						1000  100 psi

OXYGEN SYSTEM

Full service								25 liters

No-flow pressure							270 – 455 psi

Continuous use							270 – 340 psi

Regulators cockpit						6

Regulators cargo area					4

LIMITATIONS - 5

FUEL DISTRIBUTION

Symmetrical tanks						1000 lbs. maximum

Wing – to – wing							1500 lbs. maximum

Outboard – to – inboard					500 – 1000 more outboard

External tanks							1000 lbs.

Aux tank diff								No restrictions

FUEL SYSTEM

Aux/ Ext tank pump pressure			28 – 40 psi

Main tank pump pressure				15 – 24 psi

Low – pressure light						8.5 psi

Tank – empty light						23 psi

AIRSPEEDS

Thunderstorm penetration				Stall +65 kts (NMT 180)

Windmill taxi start						100 knots, 4000  remaining

10% flaps									220 knots

50% flaps									180 knots

100 % flaps								145 knots

Ramp and door open						150 knots

Cargo door open							185 knots

Air deflectors doors open				150 knots

Recommended for airstart				180 knots or less

Airstart, without NTS					130 knots below 5000

Landing gear / lights						165 knots

Windshield wipers						180 knots

Bailout, crew door						150 knots

Maximum bank angle					60

Maximum bank angle with flaps 		45

TAXI OPERATIONS

Crosswind taxi (normal)					30 knots of 90 wind

Crosswind taxi (maximum)				60 knots of 90 wind

Turn limit 155,000 or above 20 kts		20

Inboard prop clearance					5 ft., 9 in.

Outboard prop clearance					6 ft., 5 in.

Taxi speed								Brisk walk

Gross weight, taxi (max)					155,000 lbs.

Tire rotation speed (nose)				139 knots

Tire rotation speed (main)				174 knots

LIMITATIONS - 6

ENGINE GROUND OPS

LSGI range								9 – 30 throttle

LSGI downshift TIT (max)				850 C

MAXIMUM WEIGHTS

Maximum Taxi							155,000 lbs.

Maximum Takeoff						155,000 lbs.

EWP Takeoff								175,000 lbs.

Recommended Landing					155,000 lbs.

EWP Landing								175,000 lbs.

Normal Landing							130,000 lbs.

C.G. limits (normal)						15% – 30%

MISCELLANEOUS

GTC starter limit							1 min on, 4 min off

Airstart light-off RPM					30% normal / 40% must

Crew door jettison						3.1 in. HG

ELECTRICAL SYSTEM

A/C generator capacity					40 KVA

ATM generator capacity					20 KVA (30 KVA  w/fan)

Generator rating							115  VAC, 3 phase, 400 cps

Generators needed (4 buses)				Any two (engine)

Auxiliary AC power sources				Isolated DC bus

Pilots inverter power						Essential DC bus

DC power sources						Battery, 4 TR units

RH AC bus power						# 4 engine generator

LH AC bus power						# 1 engine generator

Main AC bus power						# 3 engine generator

Ess AC bus power						#2 engine / ATM generator

Battery bus items							AC external power

Alarm bell

DC voltmeter

Emer exit light extinguish

Emer depressurization

ELT reset

Fire extinguisher system

Isolated DC  bus - on batt

Jump lights

SKE battery

LIMITATIONS - 7

AIRCRAFT GENERAL

Wing span							132 ft., 7 in.

Length of aircraft						99 ft., 6 in.

Height of aircraft						38 ft., 6 in.

Stabilizer span						52 ft., 3 in.

Cargo compartment					41 L X 9 H X  10 W

CARGO EQUIPMENT

Tiedown  ratings (wall)				5000 lbs.

Tiedown  ratings (floor)				10,000 lbs.

Tiedown  ratings (capped)			25,000 lbs.

Ground troops 						92

Paratroopers							64 + equipment

Litters									74 + 2 attendants

EMERGENCY EQUIPMENT

Fire extinguishers					4 Halon hand-held

First aid kits 							23 total ( 2 in cockpit)

Hand axes								2 (minimum)

Life rafts								4 ( 20 person)

Crew seat inertial reel				2-3  G’s

Smoke mask/oxygen bottles 		4 with A-21 regulators

Passenger oxygen kits (POK)		3 (minimum)

Emergency escape breathing device (EEBD) 5

Escape rope							3 (1 per escape hatch)

Emergency exit lights				7 (1 per exit)

Life preserver unit (LPU)			1 per crew member)