# ENGINE START MALFUNCTIONS

## SHEARED STARTER SHAFT

- BUTTON POPS OUT APPROXIMATELY 1 SEC

- REMOVE AIRPYLON BLEED VALVE

- DEPRESS STARTER BUTTON

- IF BUTTON STAYS INSHEARED SHAFT

- IF BUTTON POPS OUTBAD HOLDING COIL

CSD FAIL

- DISCONNECT

- MX IF AVAILABLE

- TO FLY WITHCHECK OIL QUANTITY / MAX 50 HOURS OF FLIGHT

HOT START

- INDICATIONSTIT SURPASSES N2

- STOP START / CLEAR ENGINE / MX (PROBABLY FUEL CONTROL)

HUNG START

- INDICATIONSSTALL AT 55% N2 / TIT RISING

- STOP START / CLEAR ENGINE

- ATTEMPT 2ND  START WITH AUG AIR VALVE CLOSED

- IF ENGINE HAS HISTORY OF HUNG STARTS USE 20% FOR RUN

- IF SECOND START FAILSMX ATTEMPT 3RD START WITH AIR AUG CLOSED AND 20% FOR RUN

HIGH FUEL FLOW

- INDICATIONSGREATER THAN 800 PPH PRIOR TO IGNITION

- STOP START / CLEAR ENGINE \ MX (PROBABLY FUEL CONTROL)

NO IGNITION

- STOP START / CLEAR ENGINE / CHECK CB’S

- TRY 2ND START USING AIRSTART POSITION

NO FUEL FLOW

- CHECK WITH FE (POSSIBLE BAD INDICATOR)

- IF NOT INDICATOR

- STOP START / CHECK CB’S / MX (FUEL SHUTOFF BAD ?)

NO N1 OR N2 RPM

- N2 CHECK WITH FE (POSSIBLE BAD INDICATOR)

- N1 CHECK WITH SCANNER FOR ROTATION / WAIT 5 MIN

- IF NO N1 IN 5 MIN STOP START / CALL MX

# ENGINE START MALFUNCTIONS

## PAGE 2

NO OIL PRESSURE BY N2 RPM (LIGHT REMAINS ON)

- CHECK WITH FE / STOP START / MX

STARTER BUTTON NOT OUT BY N2 RPM (46 +/- 3)

- PULL OUT BUTTON MANUALLY

STARTER VALVE OPEN LIGHT NOT OUT BY N2 RPM

- STOP START / MX

HOLDING COIL FAILURE

- HOLD BUTTON IN MANUALLY FOR START

NO LIGHT IN STARTER BUTTON

- CONTINUE WITH START / BURNED OUT BULB

ENGINE CLEARING PROCEDURE

- MOTOR ENGINE FOR 30 SEC TO CLEAR FUEL