ENGINE RUN-UP BRIEFING GUIDE

NOTES:

1.  Select an Area Free of FOD

2.  Check Restrictions on Power Settings

3.  Head Aircraft within 30° of Wind Direction for > 7,000 in-lbs torque

when Wind > 10 kts

4.  Run-Up Symmetrical Engines

ACTIONS:

1.  Pilot:  Nosewheel Parking Brake - Centered & Set

2.  Co-Pilot:  Yoke Off Stop, Guard Brakes, Eyes Outside

3.  Pilot & Engineer:

a.  Ground Idle RPM - Note Torque

b.  Throttles - Flight Idle - Note Torque

c.  Throttles to 8,000 in-lbs, Check Normal & Mech Governing

d.  Check Engine Instruments

e.  Throttles - Flight Idle - Note Torque (200 in-lbs Increase)

f.  Throttles - Ground Idle - Note Torque Decrease (Positive Decrease 			in Torque, if not, Ground Stop)