ENGINE RUNNING ON/OFFLOAD BRIEFING GUIDE

1.  Applicable items of After Landing Checklist

2.  Engines - Low Speed Ground Idle (check SKE & RADAR)

3.  Flaps & Air Deflector Doors - as req’d

4.  Face into Wind (Dirt Ops)

4.  More than 10 PAX - Ramp & Door (less - crew door)

5.  Vehicles/Cargo - 300’ straight, 50’ prior to turn

6.  Night Operations:		Landing Lights - as req’d

Wing Tip Taxi Lights - On

7.  Safety Obs (FS 245):	PA - On

Heads up on Cargo/PAX moving

Crew use HOT MIC

8.  Start with Before Takeoff Checklist

REVERSE TAXIING BRIEFING GUIDE

1.  Brief crew on direction of turn out

2.  Pilot and Co-Pilot feet on floor

3.  Do not use brakes

4.  LM keep talking, if stop talking, stop backing

5.  Turn towards 1 & 2/3 & 4, more turn/less turn

6.  Try to put aircraft on taxi line if able

7.  Clear all obstacles by 25’

8.  If night, ensure proper illumination

8.  Countdown 5-4-3-2-1

9.  Questions