LANDING GEAR EXTENSION USING EMERGENCY EXTEND SWITCHES

- LANDING GEAR MAY NOT ROTATE ABOVE 185 KTS

- USE 40% FLAPS AND 100% FLAP APPROACH SPEED + 30 KTS

- MLG SHOULDINDICATE IN-TRANSIT WITHIN 5 SEC

- NLG SHOULD INDICATE IN-TRANSIT WITHIN 10 SEC

- BOTH SHOULD TAKE APPROXIMATELY 3 MIN TO EXTEND

- WHEN ACCUMULATOR PRESSURE IS USED FOR BOGIE ROTATION MLG WILL NOT ROTATE ABOVE 170 KTS

- NEVER DO A INFLIGHT BRAKE CHECK WHEN USING EMERGENCY BRAKES WITH LOSE OF NORMAL AND ALTERNATE BRAKING

- IF MLG EMERGENCY EXTEND SWITCH IS LEFT EXTENDED NORMAL AND ALTERNATE BRAKES, AND CASTORING ARE NOT AVAILABLE

- IF NLG EMERGENCY EXTEND SWITCH IS LEFT EXTENDED NORMAL NOS WHEEL STEERING WILL NOT BE AVAILABLE

HYDRAULIC SYSTEM OVERPRESSURE  (3-147)

- 3400 PSI

- DO NOT DEPRESSURIZE ANY OF THE ENGINE DRIVEN HYDRAULIC PUMPS IN THE AFFECTED SYSTEM

- DO NOT PULL THE FIRE HANDLE AS PUMP FAILURE MAY OCCUR, AND DUMP FLUID ON A HOT ENGINE

- SHUTDOWN THE ENGINE WITH THE PRECAUTIONARY ENGINE SHUTDOWN CHECKLIST

IF SHUTDOWN IS NOT A IMMEDIATE OPTION:

- REDUCE POWER TO A MINIMUM

- PLACE PTU ON FOR ADJACENT SYSTEM

- DEPRESS ONE OR MORE PUMPS ON ADJACENT SYSTEM

- MONITOR ALL SYSTEMS FOR FLUID LOSS

- MONITOR HYDRAULIC SYSTEM RESERVOIR TEMP, OVER 225 DEGREES FLUID BREAKDOWN OCCURS, RECONSIDER SHUTDOWN

ENGINE OVERHEAT  (3-8, 3-38)

- INDICATED BY FLASHIN OVEWRHEAT LIGHT ON PILOTS CENTER INSTRUMENT PANEL

- FLASHING FIRE LIGHT ON ENGINEERS CONSOLE

- CHECK FOR HIGH TIT, POSSIBLE NACELLE BLEED AIR DUCT FAILURE

- IF NACELLE BLEED AIR DUCT FAILUREPRECAUTIONARY ENGINE SHUTDOWN

- OVERHEAT ACCOMPANIED BY:

- BLEED DUCT OVERHEAT

- FSS FIRE WARNING

- ACCOMPLISH PYLON FIRE CHECKLIST

- IF REAL OVERHEAT, RETARD THROTTLE TO IDLE

-     IF LIGHT STAYS ONPRECAUTIONARY ENGINE SHUTDOWN

- LIGHT OUT, RETURN THROTTLE TO PREVIOUS POSITION

- CHECK WHEN LIGHT COMES ON

- LIGHT ON AGAIN, PRECAUTIONARY ENGINE SHUTDOWN

POSSIBLE TO LEAVE RUNNING IF GREATER HAZARD EXISTS, POWER SETTING BELOW WHERE LIGHT COMES ON MAY BE USED

ENGINE OVERHEAT ON THE GROUND, FIRE HANDLE THE ENGINE