HE

Pre Slowdown (LM)

- Personnel seated forward of load

- Checks CDS arming switches (245) are in normal position

- Removes emergency restraint on loads to be extracted

- Checks right hand locks to ensure proper setting

- Checks ADS ramp support arms connected

- Inspects loads and extraction sysytem for proper rigging

- Unlocks left hand locks for loads to be dropped

- Red light - On

- Don’s helmet & restraint harness

Slowdown

- Open ramp and door (if doesn’t open from front, LM climbs over

- loads to his control panel)

- Engineer ensures ramp and door open light is illuminated for entire drop

- (no light means no drop)

- NOTE:  For the ADS to work, the aircraft must be airborne, ADS ramp

- support arms connected, and ramp and door opened to the airdrop position

Drop and Completion of Drop

- “Green Light” Co-pilot turns on green light and presses ADS button

- Extraction chute is released and swings aft into airstream

- NOTE: LM is positioned by T-handle to pull when he hears and sees green

- light if extraction chute didn’t release

- NOTE: LM standing by to pull right hand release handle (emergency position)

- if load fails to extract with extraction chute out

- LM calls “Load Clear” or the condition

- Red Light - On

- LM clears Engineer to close ramp and door

No-Drop

- LM Acknowledges

- Clears closing of the ramp and door

- Restrains load

- Accomplished completion of drop checklist

NOTE: If drop reattempted, begin with pre slowdown checklist

HE

EMERGENCY

- Maintain Drop airspeed and AGL

- Avoid flying over or upwind of water or built up areas

- LM runs malfunction checklist and tells pilot when complete

- If right hand locks release prior to green light

- Apply emergency restraint (from front to rear)

- Raise aft anchor cable support (if required)

- Close ramp and door

- Relock platforms (if unable, secure with aircraft tiedowns)

- If extraction chute fails to release mechanically, or falls on the ramp

- Apply emergency aft restraint (from front to rear)

- Raise aft anchor cable support (if required)

- Close ramp and door

- Relock platforms (if unable, secure with aircraft tiedowns)

- If load fails to extract with single extraction chute outside the aircraft

- Apply emergency aft restraint (from front to rear)

- Cut extraction line aft of the attachment point (forward-deploys chutes)

- Raise aft anchor cable support (if required)

- Close ramp and door

- Relock platforms (if unable, secure with aircraft tiedowns)

- If load fails to extract with multiple 28-foot outside the aircraft

- Pull right rail control handle to emergency

- If fails to extract, pull simul handle full forward (ensures left locks are out)

- NOTE: Pull for 10-20 seconds to compensate for lock loading/binding

- If still fails to extract, land. Forced landing likely.