C-130E OPS LIMITS

| ENGINE TORQUE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max Allow. T/O |  | 19,600 / 5 min. |  |  |  |  |  |  |  |
| Max Allow. Flight |  | 19,600 / 30 min. |  |  |  |  |  |  |  |
| Max Continuous |  | 18,000 in-lb |  |  |  |  |  |  |  |
| Oil Temp < 0 |  | Min Torque |  |  |  |  |  |  |  |
| Oil Temp 0-40 |  | 4,500 |  |  |  |  |  |  |  |
| NTS Action |  | -1,260  600 |  |  |  |  |  |  |  |
| Allow. Revers Diff. |  | 1,000 |  |  |  |  |  |  |  |
| ENGINE RPM |  |  |  |  |  |  |  |  |  |
| Ignition by |  | 35% |  |  |  |  |  |  |  |
| Oil Press Ind by |  | 35% |  |  |  |  |  |  |  |
| LSGI |  | 69 - 75.5% |  |  |  |  |  |  |  |
| Normal Gnd Idle |  | 94 - 102% |  |  |  |  |  |  |  |
| Max Reverse |  | 96 - 106% |  |  |  |  |  |  |  |
| Flight Idle on Gnd |  | 92.5 - 100.5% |  |  |  |  |  |  |  |
| Normal in Flight |  | 98 - 102% |  |  |  |  |  |  |  |
| Starter Disengage |  | 60% |  |  |  |  |  |  |  |
| Starter Duty Cycle |  | 1 On,1 Off/1 On, 5 Off/1 On,30 Off |  |  |  |  |  |  |  |
| STARTING TIT |  |  |  |  |  |  |  |  |  |
| Max Start Enrich |  | 100 |  |  |  |  |  |  |  |
| Max Norm Start |  | 200 |  |  |  |  |  |  |  |
| Max during Start |  | 830 (exc peak @ 94%) |  |  |  |  |  |  |  |
| Continue Start but Write-up |  | 830 - 850 (exc peak @ 94%) |  |  |  |  |  |  |  |
| Stop Start, Write-up and 1 Retry |  | 850 - 965 (exc peak @ 94%) |  |  |  |  |  |  |  |
| Abnormal Torch |  | Stop Start, Write-up, MX |  |  |  |  |  |  |  |
| Stop Start, Write-Up, and Overtemp Ins. |  |  |  |  |  |  |  | 965 |  |
| Cold Start requiring MX & Write-up |  |  |  |  |  |  | <720 |  |  |
| Cold Start requiring Temp Cont |  |  |  |  | 720 - 750 |  |  |  |  |
| ENGINE TIT |  |  |  |  |  |  |  |  |  |
| Green Arc |  | 200 - 932 |  |  |  |  |  |  |  |
| Takeoff Range |  | 961 - 977 |  |  |  |  |  |  |  |
| Takeoff Max/Time |  | 977 / 5 Min |  |  |  |  |  |  |  |
| Military / Time |  | 977 / 30 Min |  |  |  |  |  |  |  |
| Climb Continuous |  | 932 |  |  |  |  |  |  |  |
| Overtemp Insp. Req |  | 977 for 5 sec or 1070 |  |  |  |  |  |  |  |
| Crossover |  | 65 Throttle/ 76020 |  |  |  |  |  |  |  |
| Max Downshift TIT |  | 850 |  |  |  |  |  |  |  |
| FUEL SYSTEM |  |  |  |  |  |  |  |  |  |
| Outbds > Inbds |  | 500-1000 |  |  |  |  |  |  |  |
| Symmetrical Tanks |  | <1000(exc Aux) |  |  |  |  |  |  |  |
| Wing-to-Wing |  | <1500(exc Aux) |  |  |  |  |  |  |  |
| Aux Max Diff |  | 1 empty, 1 Full if others are even |  |  |  |  |  |  |  |
| Main Fuel Boost Pump Press/Light |  | 15 - 24 psi / 8.5 psi |  |  |  |  |  |  |  |
| Aux/Ext Fuel Boost Pump Press/Light |  | 28 - 40 psi / 23 psi |  |  |  |  |  |  |  |
| OIL TEMP |  |  |  |  |  |  |  |  |  |
| Normal Range |  | 60 - 85C |  |  |  |  |  |  |  |
| Max Ground/Time |  | 85-100/30 Min |  |  |  |  |  |  |  |
| Max Flight/Time |  | 85-100/5 Min |  |  |  |  |  |  |  |
| Oil Cooler Flaps |  | 78 - 82C |  |  |  |  |  |  |  |
| Min for Takeoff |  | 40 and increase |  |  |  |  |  |  |  |
| Min for Start |  | -40C |  |  |  |  |  |  |  |
| ENGINE OIL PRESSURE |  |  |  |  |  |  |  |  |  |
| Normal Range |  | 50 - 60 psi |  |  |  |  |  |  |  |
| Max(Start/Warmup) |  | 100 psi |  |  |  |  |  |  |  |
| LSGI Minimum | <50 psi if Norm @ 100% |  |  |  |  |  |  |  |  |
| Flux |  | 10 psi |  |  |  |  |  |  |  |
| GEARBOX OIL PRESSURE |  |  |  |  |  |  |  |  |  |
| Normal Range |  |  |  | 150 - 250 psi |  |  |  |  |  |
| Max(Start/Warm-up) |  |  |  | 250+ psi |  |  |  |  |  |
| Inflight Min to complete msn |  |  |  | 130 psi |  |  |  |  |  |
| Low Speed Gnd Idle |  |  |  | 50 psi |  |  |  |  |  |
| Flux |  |  |  | 20 psi |  |  |  |  |  |
| BLEED AIR |  |  |  |  |  |  |  |  |  |
| GTC Min Press |  | 35 psi |  |  |  |  |  |  |  |
| Eng Min Press |  | 70 psi |  |  |  |  |  |  |  |
| GTC Bleed Leak  |  | 30-15 psi in  8.5 sec |  |  |  |  |  |  |  |
| Eng Bleed Leak  |  | 65-35 psi in  10 sec |  |  |  |  |  |  |  |
| OIL QUANTITY |  |  |  |  |  |  |  |  |  |
| Capacity range |  | 4 - 12 gal / 19.5 total |  |  |  |  |  |  |  |
| Low Light |  | 4 gal |  |  |  |  |  |  |  |
| Allow Burn Rate |  | 1 quart / hr |  |  |  |  |  |  |  |
| HYDRAULIC PRESSURE |  |  |  |  |  |  |  |  |  |
| Norm Press |  | 2900 - 3200 psi |  |  |  |  |  |  |  |
| Aux Press |  | 2900 - 3300 psi |  |  |  |  |  |  |  |
| Max Pressure |  | 3500 psi |  |  |  |  |  |  |  |
| LSGI |  | 2550 psi |  |  |  |  |  |  |  |
| Aileron Boost Press |  | 2050 psi |  |  |  |  |  |  |  |
| Low Rudder Boost |  | 1100-1400 psi |  |  |  |  |  |  |  |
| Low Rud. Bst Caut. |  | 1600 psi |  |  |  |  |  |  |  |
| Low Press Light On |  | <1000 psi |  |  |  |  |  |  |  |
| Suct Boost Light On |  | <20 psi or 11 ampere |  |  |  |  |  |  |  |
| Norm Accumulator |  | 1500 psi  100 |  |  |  |  |  |  |  |
| Aux Accumulator |  | 300 |  |  |  |  |  |  |  |
| Press not to turn off HydSwitches |  |  |  |  | >3450 psi |  |  |  |  |
| Press to perform ESP |  | >3900 psi |  |  |  |  |  |  |  |
| Good Press if Suct Boost Pmp fails |  |  |  |  | >2500 psi |  |  |  |  |
| Norm 1 Brake App |  | 2250 psi |  |  |  |  |  |  |  |
| Norm Brake Accum |  | 1500 psi  100 |  |  |  |  |  |  |  |
| Emer 1 Brake App |  | 2900 psi |  |  |  |  |  |  |  |
| Emer Brake Accum |  | 1000 psi  100 |  |  |  |  |  |  |  |

OPS LIMITS  / BOLD FACE

| PRESSURIZATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Gauge Range | -1.2 to 15.8 in/HG |  |  |  |  |
| Safety Valve Open | -0.76 to 15.9 in/HG |  |  |  |  |
| Nominal | 15.16 in/HG |  |  |  |  |
| Cockpit (1or2) or Cargo (1Only) Cracked Pane |  |  |  | 10 in/HG |  |
| Cargo Pane Cracked (2pane) |  |  |  | 0 in/HG |  |
| Rate Cont Min/Max | 30-200/1600-2900 fpm |  |  |  |  |
| AC ELECTRICAL |  |  |  |  |  |
| Voltage Range | 110 - 125 VAC |  |  |  |  |
| Frequency Range | 380 - 420 Hz |  |  |  |  |
| Max Load in Flight | 1.05 |  |  |  |  |
| Gen Capacity | 40 Kva |  |  |  |  |
| ATM Capacity | 20 KVa / 30 KVa below 104F |  |  |  |  |
| ATM Load | 0.66 / 1.00 below 104F |  |  |  |  |
| DC ELECTRIC |  |  |  |  |  |
| Voltage range/Norm | 25 - 30 / 28 VDC |  |  |  |  |
| Max Load | 1.03 |  |  |  |  |
| Battery Voltage/Min | 24 VDC / 21 VDC |  |  |  |  |
| CP Inverter Pwr | 250 Va |  |  |  |  |
| AC I&EFC | 1500/2500 Va |  |  |  |  |
| LEADING EDGE ANTI-ICE |  |  |  |  |  |
| Max Ground Ops | 30 Sec |  |  |  |  |
| Normal range | 75 - 200 F |  |  |  |  |
| Control valve range | 158 - 180F |  |  |  |  |
| PROP DE-ICING |  |  |  |  |  |
| Load Range | 65 - 90 Amps |  |  |  |  |
| Timing Cycle | 15 sec on/ 45 sec off |  |  |  |  |
| Max Gnd Ops | 2 Cycles (2Min) |  |  |  |  |
| PROPS |  |  |  |  |  |
| Prop Brake Engage | 23% |  |  |  |  |
| Prop Brake Diseng. | Starter Torque |  |  |  |  |
| Feather Blade Angle | 92.5 |  |  |  |  |
| Reverse Blade Angle | -6 to -8 |  |  |  |  |
| Pitchlock Range | 25 to 55 |  |  |  |  |
| Pitchlock Engages | 103% or loss of press |  |  |  |  |
| Pitchlock RPM Desired |  |  | 96 - 98% |  |  |
| Unlock Pitclock | Increase 2 |  |  |  |  |
| Low pitch stop | 23 |  |  |  |  |
| Low pitch stop  | > 200 increase |  |  |  |  |
| Prop Oil Qty | 26 qts |  |  |  |  |
| Prop Oil Sump/Light | 6.5 qts / 2 qts Low |  |  |  |  |
| Safety Coupling | -6,000 |  |  |  |  |
| Aux Feather Limits | 1 Min On, 1 Min Off, 2 min in 30 min |  |  |  |  |
| Static Feather  | 25 sec/ 6 sec pull feather overide button |  |  |  |  |

| OXYGEN SYSTEM |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fully Serviced | 25 liters |  |  |  |
| Low Light | 2.5 liters |  |  |  |
| Press No Flow | 270 - 455 psi |  |  |  |
| Press Continuous | 270 - 340 psi |  |  |  |
| AIRSPEEDS |  |  |  |  |
| Max Speed | 0.64 Mach |  |  |  |
| Severe Turbulence Penetration | 65 above Power-Off Stall not over 180 |  |  |  |
| Flaps10 / 50 / 100 | 220 / 180/ 145 |  |  |  |
| Cargo Door Only | 185 |  |  |  |
| Ramp and Door | 150 |  |  |  |
| Air Deflector Doors | 150 |  |  |  |
| Windshield Wipers | 150 |  |  |  |
| Paratroop | 150 |  |  |  |
| Paratroop up and Locked |  |  | 250 |  |
| Gear and Lights | 165 |  |  |  |
| Airstart | 180 Kts / 200 TIT |  |  |  |
| Airstart (No NTS) | 130 Kts / below 5000’ |  |  |  |
| Max Bank Angle | 60 / 2 g |  |  |  |
| Max Bank w/Flaps | 45 / 1.4 g |  |  |  |
| Tire Rotation Nose | 139 |  |  |  |
| Tire Rotation Main | 174 |  |  |  |
| MISCELLANEOUS |  |  |  |  |
| Turn Limit above 155,000 or 5-20 Kts |  |  |  | 20 |
| LSGI range | 9 - 30 |  |  |  |
| Reverse Range | 0 - 18 |  |  |  |
| Ground Idle detent | 18 |  |  |  |
| Flight Idle Gate | 34 |  |  |  |
| Beta Throttle Range | 0 - 34 |  |  |  |
| Alpha Throttle Rnge | 34 - 90 |  |  |  |
| Temp. Limiting | 0 - 65 |  |  |  |
| Temp. Controlling | 65 - 90 |  |  |  |
| GTC Starter Limit | 1 on 4 off ( cycles) |  |  |  |
| Cold Wx Ops | 32F or 0 C |  |  |  |
| Hot Wx Ops | >95F or 35C |  |  |  |
| T/O Weight/EWP | 155,000 / 175,000 |  |  |  |
| Max Landing Wt | 155,000 @ 300 fpm |  |  |  |
| Normal Ldg Wt | 130,000 @ 540 fpm |  |  |  |

ENGINE SHUTDOWN PROCEDURE

1.     CONDITION LEVER - “FEATHER” 	(CP)

2.     FIRE HANDLE - “PULLED” 	(CP)

- AGENT - “DISCHARGED” 	(CP)

(FOR FIRE OR NACELLE OVERHEAT)

GTC EMERGENCY SHUTDOWN

1.  FIRE HANDLE - “PULLED”  	(E)

2.  AGENT - “DISCHARGED”   		(E)

(FOR FIRE)

FUSELAGE FIRE/SMOKE AND FUMES ELIMINATION

1.  OXYGEN - “ON, 100 PERCENT” 	(P)

MULTIPLE ENGINE POWER LOSS/RPM ROLLBACK

- MAIN TANK BOOST PUMPS - “ON” (E)

- MAIN TANK CROSSFEED VALVES - “CLOSED” 			(E)

- PROPELLER GORVERNOR CONTROL SWITCHES - “MECH GOV” (CP/E)

- TEMPERATURE DATUM CONTROL VALVE SWITCHES  - “NULL” 	(E)

SECTION 1

DIMENSIONS				ENGINES

Wing Span 		132’ 7”		14 stage axial-flow compressor

Length			99’ 6”		6 thru-flow combustion chamber

Height			38’ 6”		4 stage turbine

Stabilizer Span		52’ 8”		Acceleration bleed valves @ 5th

Cargo Compartment		    	       and 10th compressor stages

Length		41’		Diffuser at 14th stage taps off

Width (min)	10’ 3”		       125 PSI, 600 F bleed air

Height (min)	9’		Accessories:  “SSOFF”							Speed Sensitive Control

REDUCTION GEAR ASSEMBLY		Speed Sensitive Valve

Reduction Ratio (13.54 to 1)			Engine Oil Pump

13,820 RPM to 1,021 RPM	     	Fuel Control Unit

Accessories: “GHOST”				Fuel Pump

AC Generator

Hydraulic Pump			ENGINE FUEL CONTROL

Reduction Gear Oil Pump		Fuel Control Metering Section

Engine Starter			     puts out 120% of fuel reqd

Tachometer Generator		18 thermocouple probes

Each has two junctions: (TIT/TD)

SPEED SENSITIVE CONTROL		Temperature is ave of all jcts

16% - Fuel Shutoff Valve Opened	  	Open Junction: 3.5 lower

Ignition Relay Energized (PIED)	Eroded Hole: 7.5 lower

Paralleling Valve Opens	Missing Probe Tip:  22.0 lower	      Ignition Exciter

Fuel Enrichment Valve Opens	94% - TD Start/Normal Limiting

Manifold Drip Valve Closes	          TD valve 50% to 20% take

65% - Ignition Relay Deenergized		Also @ 94% Speed Sensitive

Drip Valve is held closed by 		Valve opens to close the

pressure				Accel Bleed Valves

TEMPERATURE DATUM (TD) FUNCTION CHART

|  | TD SWITCH POSITION |  |  |
| --- | --- | --- | --- |
|  | NULL | LOCKED | AUTO |
| Above Crossover | 20% Bypass No Temp | Normal Limiting ( 900 TIT) 20% Bypass + Up to 20% Take | Temp Controlling 20% Bypass + Up to 20% Put or Take Throttle position schedules a TIT |
| Below Crossover | Limiting | Holds fixed correction if locked above crossover |  |
| Below 94% RPM |  | Start Limiting(830) | 20% Bypass + Up to 50% Take |

CONDITION LEVERS

RUN - Closes a switch that places the engine fuel and ignition system under 	the control of the speed-sensitive control.  The ice detection systems 	are engergized for #2 and #3.

AIR START - Closes same switch as RUN but also closes a switch to turn on 	the auxiliary feather motor to provide pressure to unfeather the prop.

GROUND STOP - Closes the electric fuel shutoff valve.  Must be on ground.

FEATHER - Mechanically and electrically energizes Feather Solenoid Valve

and closes the Fuel Shutoff Valve,also energizes the Aux Feather motor

ELECTRONIC PROPELLER GOVERNING

Speed Stabilization - Prevents RPM hunting by changing spring tension

Throttle Anticipation - Changes compression on spring in anticipation of 	rapid throttle movements

Synchrophasing - Keeps the props turning at the same speed and maintains a 	constant rotational position of blades to limit noise and vibration.  	Senses electrical pulses to compare then changes spring compression

FUEL SYSTEM

|  | Without Foam |  | With Foam |  |
| --- | --- | --- | --- | --- |
| Tank | Hard Struts | Soft Struts | Hard Struts | Soft Struts |
| Outboard | 8385 | 8772 | 7956 | 8323 |
| Inboard | 7735 | 8092 | 7345 | 7684 |
| Auxiliary | 5915 | 6188 | 5616 | 5875 |
| External | 8840 | 9248 | 8385 | 8772 |
| Total | 61750 | 64600 | 58604 | 61308 |

Fuel Dumping - Roughly 400 lbs/min per pump, and it leaves approximately 7,800 lbs (2100 outboard, and 1800 inboards) or about 1.5 to 2 hours of fuel.

All pumps are 3 phase AC and Valve are operated by 28 VDC motors.

ELECTRICAL SYSTEM

LH AC

RH AC

Radio Circuits

Main AC

SKE

Upper Main AC Dist Panel

Curr Limiters for Main, RH, LH AC &AC, 9 Combat wired CB for Ess AC

PILOTS			           COPILOT

“LRAFBI”	                       “MRSEAL”

AC Inst & Eng Fuel Cntrl

FUSES

for gyros &

press ind.

Ess AC

&

Battery

SKE

Ess AC & Isol

9 Comb Cbs & Trim

Ess DC

Start & Ign Control

Ess DC

Eng

Inst

Ess DC

Aircrft

Lndg Gear Cntrl

Main DC

Lights

SKE & Flaps

LH AC		ESS AC		MAIN AC	RH AC

#1 Main FBP	#2 Main FBP	#3 Main FBP	#4  Main FBP

Fwd LH Ext FBP Aux Hyd Pmp	Aux FBP	Fwd RH Ext FBP

Galley		Aux Feath Mtr	Aft Ext FBP	Anti-Ice

NESA Heat	Suct Bst Pmp	Radar

Underfloor Heat	Trim		SKE		All of the primary AC

“Crew Comfort”	Compass	“Dump Bus”	are 115/200 Volt

SCNS				400 Cycle 3 Phase

Autopilot

CP Instruments 		AC Instruments and Engine Fuel Control

Flight Directors		Torquemeter	Powered by: Inverter from ESS

ADIs			TIT		DC & also ESS AC

Powered by:		Fuel Flow	Has auto-switching from

Inverter from Isol DC &  	LOX Qty	Ess DC to ESS AC

Txfrmr from Ess AC	Fuel Qty		115 V / 400 cycle / single phase

115 Volt 400 Cycle	#1 Txfrmr	#2 Txfrmr

3 Phase			“1 FEB 34”	Anti-Skid Test

Fuel Press	1&2 Oil Press

EmerBrake Press

Boost Hyd Press		“122-2-122”

3&4 Oil Press	    2      2      2	 1      2

Both are 26V	    1                                2

Battery 			Isolated		Essential DC	Main DC

“FAAJIVEEES”		GTC Start&Cont	Valves		Lights

Fire Ext.			ATM Control	Oil Temp	Flap Ctrl Valve

Alarm Bell		Interphone	Oil Qty		Norm Brakes

AC Ext Pwr		UHF (Man)	Oil Cool Flaps	SKE

Jump Lights		Gen Control	Oil Low Qty Lts	Static Line Ret

Isol DC Bus 		Bus Off Ind	SCNS		Winch

on Bat Light		CP Pitot Ht	Fire Detect	Anchor Cable

Voltmeter (Bat)		CP Flood Lts	Ignition Control	Air Def Doors

Emer Depress		Bus Tie		Control for:

Emer Lt Ext				Aux Hyd Pump	Aft FuseJctBox

ELT					Suct Bst Pump	Ramp & ADS

SKE Bat					Aux Fth Motor	Tab & Flap Ind

Emer Elev Trim	More Lights

All DC are 28/24 Volts

HYDRAULIC SYSTEMS

Reservoir Quantities - Utility - 3.2 gal, Booster - 2 gal, Auxiliary - 3.4 gal

Ground test valve will connect the auxiliary system to the utility system.  Located behind the left wheel well on the outside.  Left open if all the utility gauges show pressure when you turn on the aux hyd pmp.

There is approx. 1 gal of fluid that can be trapped in the run-around circuit.

The Auxiliary hydraulic pump is an AC motor with a DC control (2 CBs)

All trim tabs are three phase Ess AC motors with Ess DC control (2 CBs) and Emergency elevator trim is Ess DC.

Trim travel for elevator - electrically 6o, 25o / mechanically 8o, 27o

Flap Travel time - 8-15 secs  and 10-15 secs 

100% flaps = approximately 35o

The flap brake is spring closed and hydraulically opened, the emergency flap brake hydraulically closed and mechanically opened (on ground)

Gear Travel Time - 19 secs or 

Gear has a brake that hold the gear up is spring applied and hydraulically released or mechanically with the emergency engaging handle.

Touchdown Switch	Touchdown Relay		Aux Relay

Ground Stop		Ldg Gear handle lock	GTC Control & Door

Dumpmast Shutoff		Airdrop Release		DC Bus Tie

SKE prox warning		Ramp/Door Controls	SCNS tests

Nacelle Preheat		    in cockpit		Strobe Light

Anti-Skid/Wheel Brakes

Defensive System Jettison

Landing Gear Warning Horn - Throttle 5 of F.I., gear up (silenceable)

Flaps >70%, gear up (non-silenceable)

If there is a power failure then both brake systems can supply pressure, the one with greater pressure will actually do it.

The Anti-Skid inop light will illuminate when anti-skid is inop, parking brake set, or emergency brakes selected.

FIRE AND NACELLE OVERHEAT SYSTEMS

Handles:	Fire				GTC

Bleed air valve			Bleed air valve

Engine Start Control deenergized	Extinguish valve positions &

Extinguish valve positions & agent	  agent armed

armed				Fuel shutoff valve

Fuel shutoff valve (fuel control &	Oil shutoff valve

firewall)			Door switch disarmed

Start circuits deenergized

Hydraulic shutoff valve (firewall)

Oil shutoff valve

Prop feathers

Agent: Approx 19lbs per bottle will be emptied into the nacelle or GTC compartment.  Located in  left wheel well.  It will go wherever the last handle was pulled.  The valves are held open and will drain power from battery.  Test will only test the continuity not the detectors.  Bottles are approx 600 psi.

Turbine Overheat: (Flashing Light) 371C(700F) 4 detectors mounted aft of firewall

Nacelle Overheat: 149C(300F) 6 detectors mounted forward of firewall

Fire: (Steady Light)Tubing detects high temperature on engine and GTC

SECTION II

GENERAL

When gear, flaps, or ground idle appear on checklist, the action must be coordinated with the pilot prior, if the airplane is moving.

When altimeters appear then all applicable crewmembers will state the setting and/or reading.

PREFLIGHT CHECK/COCKPIT CHECKLIST

After the first flight, on the same day, and no maintenance is required the pre-flight checklist does not have to be reaccomplished. (if maintenance is required then preflight only the applicable items).

Away from home station, ground time less than 24 hours, with the same crew and aircraft, only thruflight items (asterisks) need be accomplished.

Once the Cockpit checklist is accomplished then someone must remain with the aircraft, if the airplane does not fly then do Engine Shutdown and Before leaving the Airplane checklists.

BEFORE STARTING ENGINES CHECKLIST:

When arriving at the airplane crews should check/set oxygen, pubs, interior lights, and clocks.  You can check any other item that is powered now but turn unnecessary equip off prior to starting engines.  P/CP will be on Hot Mike at the starting of the Before Starting Engines Checklist.

STARTING ENGINES

Start sequence:	RPM indication (after 5 seconds LM call no rotation)

Fuel flow - approx 300 pph or enrichment then 300 pph

Ignition - by 35%, if not but FF then motor engine to 25%, 			if ATM is off and GTC stops or battery relay is 			open then go to Feather.

Oil Pressure - Engine and gearbox by 35%

Hydraulic Pressure - positive indication by on speed

Parallel - Sec. Fuel Pmp Press Light by 65%

Starter - Release @ 60%

Series - Sec. Fuel Pmp Press light out/ drop in TIT

@ approx 65%

Peak TIT

If stalled start, then stop start and motor engine to 25%.

During low air density condition (high altitude/High Temp >28C) if smooth acceleration and TIT in limits then allow 70 seconds for engine on speed, do not have starter engaged for more than 60 seconds.

Prop Low Oil Light during start or taxi - Go to Ground Stop not Feather.

CROSSWIND TAXIING

With 4 engines with xwinds of 0-30 knots you can use nosewheel and rudder, if the xwinds are 30-60 you need aileron, diff power, and diff braking as well.

REVERSE TAXIING

Use forward thrust to stop aircraft, do not use brakes.

Monitor oil temperature closely, and taxi forward 5 feet to realign mains.

TAXI

Skidding of nosewheel can occur during turning on wet surface or aft CG.

Avoid reversing on unprepared surfaces and excessive or prolonged braking.

If popping noise occurs when shifting from normal to LSGI, get MX.

Place throttles toward Flight Idle in LSGI to assist in cooling when oil temp is high.

Turns with brakes locked are prohibited, avoid braking in turns.  If you must stop then record in 781.

The FE may start Taxi checks after completion of Before Taxi automatically.

Check Emer brakes on start taxi, and select norm brakes and check them when in uncongested area.

Minimum of 170’ required for 180 turn with nosewheel @ 60.

Head the a/c within 30 of wind direction if torque is >7000 in/lbs and wind velocity is >10 knots.

Radiation Hazard areas - 	HF antenna = 5’

Radar hazardous to personnel = 40’

No explosives or fuel = 300’

BEFORE TAKEOFF

You can perform this check while taxiing if uncongested and outside vigilance is maintained.

Make sure both pilots are not in INS on the Flight Dir at same time (IMC).

LINEUP

Start this checklist immediately prior to or while taxiing on runway.

NORMAL TAKEOFF

Rolling takeoffs are preferred but if you need charted TOLD then static.

If an engine Torque, TIT, FF are not similar or do not match throttle position them perform a Temperature Controlling check before takeoff.

If >19,600 torque then record in 781 the value and runway/Wx conditions.

Forward pressure on yoke will aid in steering if aft CG or asymmetric power.

REDUCED POWER TAKEOFF

Set 900 TIT will be used for most takeoffs unless higher is required.

Use 932 for takeoffs and climbs to cruise altitudes and formation takeoffs.

MAX EFFORT TAKEOFF AND OBSTACLE CLEARANCE

Use max power and static takeoff.  When airplane lifts off retract the gear and adjust angle to obtain obstacle clearance speed.  After clearing the obstacle slowly retract the flaps while maintaining altitude and accelerate to best climb speed.  Minimum flap retraction speed is obstacle clearance + 10.

CROSSWIND TAKEOFF

Pilot maintains directional control with nose wheel steering and diff power, and the copilot keeps wings level with ailerons.  After liftoff the line of flight should be aligned with the runway until crossing airfield boundary.

AFTER TAKEOFF

Raise the gear and flaps on command of pilot without calling for checklist.  Then review them when checklist is called for.  Turn off  “Hot Mike” when not required for crew coordination.

Retract the flaps normally at takeoff speed + 20, but minimum is obstacle clearance.

Accelerate to 150 is staying configured in pattern and 170 if clean in the pattern.

Climb speed: 180 to 10,000’ / 170 to 15,000’ / 160 to 25,000’.

DESCENT

Maximum range descent is accomplished at flight idle, gear and flaps up @ max L/D.  Provides a moderate rate of 1500 FPM for an enroute letdown.

A penetration descent is made at flight idle, gear and flaps up @ max L/D until 20,000’ then at 250 KIAS.

A rapid descent can be accomplished at flight idle, gear and flaps up @ max speeds, or at flight idle, gear down and flaps 100% and @ 145 KIAS.

Repeated actuation of the gear under full cabin pressurization is not recommended.

Accomplish the Descent checks prior to traffic entry and/or before commencing any type of approach, or it can be done during an initial penetration decent.

Ensure the radar altimeter is set to HAA/HAT prior to the final approach segment.

BEFORE LANDING

The pilot may direct lowering of the flaps and gear prior to this checklist.  Go to “Hot Mike” prior to landing.

Check the nose wheel steering indicator just prior to landing.

Higher than normal emergency brake pressure may be encountered in flight.

LANDING

Do not operate over raised cables with disc-type support in excess of taxi speed.

Brakes must be checked during landing roll.

Traffic pattern speeds are 170 until Before Landing checklist is initiated then 150 or approach speed, 140 or approach speed on base, then slow to approach speed when established on final.  Select 100% flaps if required on final and slow to threshold speed by the threshold.

Do not retard the throttles below flight idle until <115KIAS, unless it is essential to stopping within runway.  If power loss or bogdown occurs then get Mx prior to flight.

CROSSWIND LANDINGS

At light gross weights its advisable to use less than 100% flaps to touch down the main gear first.  It is not recommended to use less than 50% flaps.

In severe crosswinds, you can raise flaps before applying reverse power to prevent the upwind wing from rising.

MAXIMUM EFFORT LANDING

Normal traffic pattern will be flown unless airfield situations or mission requirements dictate otherwise. When established on final then slow to max effort threshold speed.  Adjust rate of descent to arrive over end of runway at max effort threshold speed with 500fpm descent rate.

Try to touchdown from 100 to 300 feet, but touchdown in first 500 feet is essential.

Increase threshold speed for gusts up to 10 knots, and ensure that the xwinds are in the recommended area.

After the nose wheel is down, apply max reverse and elevator back pressure.

FLAPS UP LANDING

The pitch on final closely approximates the landing attitude and rate of descent should be controlled with throttles.  Do not attempt a round out or flare (the tail skid can contact the runway), but allow the aircraft to touchdown.

TOUCH AND GO LANDING

Accomplish all normal checklists thru Before Landing Check on first touch and go.  After the first touch and go then use this checklist until departing the airport traffic or approach control area, or make a full stop landing when an immediate takeoff is not planned.

You can use this checklist for full stop taxi back operations if only the flaps, trim, throttles (including LSGI) are used.  If anything else is use or a door is opened then accomplish the After Landing, Before Takeoff, and Lineup checklists then you may use the Touch and Go checklist.

After the plane is on the runway call for “Flaps 50”, then the other pilot will accomplish the checklist and state “trim tabs set” and “throttles”.

On the Runway and After Takeoff portions do not have to be referenced, but the Before Landing Checklist must be referenced.

GO AROUND

1. Alert the Crew

2. Power as Required to establish safe airspeed and climb

3. When appropriate airspeed, altitude, and climb the check/set flaps 50%
    (>T/O )

4. Gear up when the aircraft will not touch down (two positive climbs)

5. Raise the flaps as normal (T/O + 20)

6. Accomplish the After Takeoff Checklist

AFTER LANDING

You can use the checklist for operational stops and if engines were left running then go straight to the Before Takeoff checklist.

Do not shut down engines when taxi speed, winds or combination is > 20 knots.

It is recommended to allow the engines to run in LSGI for 2 minutes before shutdown.

ENGINE SHUTDOWN

If the NTS lights do not illuminate then recheck the NTS system before next flight.

Leave the hydraulic pump switches on to allow for thermal expansion.

BEFORE LEAVING THE AIRPLANE

The propeller feather check need only be accomplished once each day (usually after the first flight).

The FE will complete a walkaround and each crew member must make sure all lights interior and exterior are off at their crewstation.

STATION KEEPING (SKE) (AN/APN-169C(V))

Make sure the SKE battery CB to be in for 15 minutes to allow the clock to stabilize.

You must have Main AC power and Radar in standby to accomplish this check.

It must be performed prior to taxi on all flight that SKE will be used.

SKE PARAMETERS

Main AC power must be available to accomplish this check.

It must be performed after the SKE checklist but prior to taxiing if SKE will be used.

If electrical power to SKE is interrupted you must reaccomplish this checklist.

SECTION III

EMERGENCY EQUIPMENT

Emergency Escape Rope - 3 (at each overhead escape hatch)

Emergency Exit Light - 7 (Ovrhd Esc Htchs, Crew Ent Dr, Rt Emer Exit, Both Troop Doors)

First Aid Kit - 23 (2 Flt Dck, 6 Fwd Rt WW, 7 Fwd Lt WW, 4 Aft Each WW)

Fire Extinguisher - 4 (Fore/Aft 245, Aft Rt Paratroop Door, & Aft Lt Wheel Well)

Liferaft - 4 (20 man rafts, 2 over each wing, 2 hndls @ FWD Esc Htch, Aft Rt Para Dr, Over Wing)

Accessory Kits (emer radio (10 hrs), 4 signals, 5 food packs, and 1 
distress mkr light)

Stanchion Ladder - Stored Foreward of Left Wheel Well (Can only go up right side)

Hand Axe - 2 (Fore or Aft 245, & Aft Rt Paratroop Door)

Life Vests - 10 (provisions for 6 Fwd 245 and 4 Aft 245)

Anti-Exposure Suits - 6 (provisions for stowage under lower bunk on flt dck)

Portable Oxygen Bottle - 4 (P, CP, Aft 245, & Aft Rt WW)

Smoke Mask/Quick Don - 4 (Same as above, Quick Dons @ P, CP)

Pyrotechnic Pistol  Mount - Over Nav Station

EMERGENCY SIGNALS

Ground Evacuation - 1 long sustained ring

Prepare to Bailout - 3 short rings

Bailout - 1 long sustained ring

Crash landing imminent - 6 short rings

Brace for impact - 1 long sustained ring

ENGINE SHUTDOWN CONDITIONS

Generator Failure (without disconnect)

Engine Fire

Turbine Overheat

Nacelle Overheat

Uncontrollable Power

Certain Propeller mallfunction

Uncontrollable rise in TIT

Uncontrollable rise in Oil Temperature

Uncontrollable drop in Oil Pressure

Unusual vibration or roughness

Throttle control failure

ENGINE SHUTDOWN PROCEDURE

Bold face  then:	LM scan the engine

Feather Overide out

Run Checklist

Radio Call: Problem, Hours of Fuel, # on board, 				Simultaneous transmissions.

If throttle control cable failure, then do not move cond lever.  Instead ESP with Fire Handle only

Isolate the wing before discharging other bottle in case of bleed air leak.

Do not hold switch for more than 1 or 2 seconds.

Discharge the other bottle only on command of the pilot.

GAS TURBINE COMPRESSOR EMERGENCY SHUTDOWN

If condition persists then discharge remain bottle.

DIRECTIONAL CONTROL PROBLEMS

If directional problems occur on the ground before refusal then a takeoff shall not be attempted and if they occur on landing then a go-around shall not be attempted.

If this occurs while in the ground range immediately place all throttles in ground idle, then shut down the engine as soon as identified if required and reverse symmetrical engines and apply brakes if required.

The airplane will turn toward the bad engine during acceleration and away from the bad engine during deceleration.

ENGINE FIRE

Ground - Ground Idle then ESP

Flight - ESP

TAILPIPE FIRE or TORCHING

Start - Ground Stop, Motor Engine / If flames spread then ESP, notify
         ground crew

Flight (loss of oil and black smoke) - monitor then ESP prior to ldg

Flight (tailpipe fire) - ESP, If persists then accelerate till smoke gone

Engine Shutdown - continue with shutdown and notify Fire Depart

TURBINE OVERHEAT

Ground - All Ground Idle, then Ground Stop on bad engine

Flight - Retard throttle toward Flight Idle, then ESP if persists

NACELLE OVERHEAT

Ground - All Ground Idle, then ESP on bad engine

Flight - ESP

HIGH TIT

Ground - Retard throttle toward Ground Idle, and TD Null, then
       Ground Stop if persists

Flight - Retard throttle toward Flight Idle, TD Null, then ESP

HIGH OIL TEMPERATURE

See ENGINE OIL SYSTEM FAILURE

GROUND EVACUATION

Pilot - Set parking brake (one side if needed), notify crew/pax to evac
           when engines shutdown (interphone, PA, or alarm bell)

CoPilot - Notify Tower, then pull all condition levers and fire handles
           when bus tied

Engineer - Bus Tied, DC Power Off when radio call out, SCNS Off

Navigator - SCNS off

Loadmaster - Chock airplane (nose if needed)

BRAKE SYSTEM FAILURE

Select opposite brake.  You can use the hand pump to stop the aircraft by holding the pedals down while the pump is operated.

ANTI-SKID SYSTEM FAILURE

If the light comes on then turn off the Anti-Skid switch and brake with caution.  Check the emergency brake switch and the parking brake.

ANTI-SKID TEST UNSATISFACTORY

Turn off the anti-skid switch.

ENGINE FAILURE DURING TAKEOFF

If the engine fails prior to refusal speed then you must abort.

Must establish 5 bank away from the dead engine if below VMCA.  Also might have to pull the opposite engine back.

ABORT PROCEDURE

Feather the engine while the throttles are in Flight Idle if there is a directional control or propeller problem to avoid making it worse.

Continue with the ESP after safe control of the airplane is assured.

NOSE WHEEL SHIMMY

If the takeoff is continued then raise the nosewheel as soon as directional control permits to reduce the severity of the shimmy.

TAKEOFF CONTINUED AFTER ENGINE FAILURE

If using reduced power procedures then you may advance to max power as directional control permits.

Once airborne:	Raise gear

Flaps 20% @ TO + 20

Flaps Up @ 2eng VMCA

Accelerate to 3eng Climb

After takeoff Checklist

If obstacles are a factor then raise the gear within 3 seconds and feather the prop within 6 seconds, then milk the flaps up 10% per 5 knots.

ENGINE FAILURE

Two engine operation is marginal if gross weight is > 120,000 or high density altitude.

If you are going to operate on 2 engines, consider the following:

“CHEWBAG”

Configuration - Consider when to configure (gear dn and flaps up)

Hydraulics - Think about systems affected (select emer brakes/NWS)

Electrics - Monitor generator loads & consider ATM

Weight - Dumping fuel or cargo, max power

Bleed Air Valves closed

Airspeeds

Ground Idle (inboard first)

PROPELLER MALFUNCTIONS

Indicated by:	Prop Low Oil Light or visible leak

Over/Underspeed

RPM surge or fluctuation

Failure of prop to feather

Audible beat, periodic minor changes in RPM (hunting), etc. are minor adjustment problems and can be handled with normal procedures.

PROPELLER MALFUNCTION DURING TAKEOFF:

If there is no fire, then allow the engine to run until 2eng Vmca

You may have to reduce power on opposite engine

Raise the gear and if out of limits then MECH GOV

If this does not fix then accelerate to 150 KTAS

If landing then ESP, otherwise Prop Malf During Flight

PROPELLER MALFUNCTION DURING FLIGHT

If uncontrolled overspeed >105% then don’t touch throttle and slow 
    down, then analyze problem

Prop Low Oil Light

RPM in limits then monitor, must shutdown prior to ldg

If RPM is out of limits then MECH

Monitor must shut down prior to landing

If still out of limits then Pitchlock Check or ESP if ldg.

RPM Out of Limits

MECH GOV, then monitor (consider reindexing)

If still out of limits, the Pitchlock Check or ESP if ldg

Pitchlock Check Procedure

TD locked, then reduce throttle or true airspeed

If RPM does not follow then ESP

If it does follow then Pitchlocked Operation

Pitchlocked Propeller Operation

Establish 96-98% when reaching airfield then 150 True

Once at 150 True or cannot maintain 96-98% then ESP

Propeller Fails to Feather

Reset Fire handle is no fire

Hold Feather Override in for 30 seconds then pull out

Attain 150 KTAS, Land ASAP

INFLIGHT DECOUPLING OF ENGINE AND PROPELLER

Engine Failed: 	Torque, TIT, FF, and eng Oil Press near 0

RPM may increase then normal

Hyd, Gen, and Gearbox Oil Press normal

Engine Running:	Extremely low TIT and FF for throttle

Fluctuating and near zero torque

Follow ESP for both

ENGINE SYSTEMS FAILURE

THROTTLE CONTROL FAILURE

If the throttle movement not pilot initiated, throttle frozen or binding,
or power indications unrelated to throttle position then perform the ESP, do not move throttle.

MULTIPLE ENGINE POWER LOSS/RPM ROLLBACK (BOLDFACE)

Erratic FF, Torque, TIT readings.

Electrical malfunctions that introduce abnormally low voltage to ESS
  AC combined with failure of undervoltage protection systems can
  cause TD amplifiers to malfunction producing torque reduction of up  to 1600 in-lb.

Low AC voltage or other electrical malfunction can cause 
  synchrophaser to reduce RPM by 4%.

- MAIN TANK BOOST PUMPS - “ON”                        (E)

- MAIN TANK CROSSFEED VALVES - “CLOSED” (E)

- PROPELLER GOVERNING CONTROL SWITCHES -
      “MECH GOV”				           (E)

- TEMPERATURE DATUM CONTROL VALVE SWITCHES 
      “NULL”					           (E)

- Turn off generator supplying ESS AC bus

- Turn off synchrophaser

- Land ASAP

TD CONTROL SYSTEM MALFUNCTION

Sudden increase in TIT with accompanying change in Torque and FF.  
Go to NULL, and if it stabilizes then go to LOCKED and continue operation.  If it persists then monitor TIT closely during NULL.

TIT INDICATION SYSTEM MALFUNCTION

Significantly lower TIT with higher FF with throttles aligned above crossover.  Go to NULL.  If it stabilizes then maintain FF at or below others and consider shutting down the engine.  If the condition persists then ESP.  Record the duration, TIT, FF, and Torque.

SECONDARY FUEL PUMP PRESSURE LIGHT ILLUMINATION

Pull the Ignition Control CB.  If the light extinguishes then, the 65% switch failed.  Continue operation and reset CB before shutting down engine.  If the light remained illuminated then you may have possible failure of the primary pump (ESP).  No ice-detection w/ CB pulled.  If this occurs on crossfeed from aux/ext then come off crossfeed.  Indication of bad press switch, bad secondary pump check valve or paralleling valve.

SPEED SENSITIVE CONTROL FAILURE (SHEARED SHAFT)

Indicated by momentary illumination if the secondary fuel pump pressure 	light, and TIT not exceeding 830 and if throttles are above 65 then illumination of the fuel correction light.  Place TD in NULL and continue operation.  Use normal speed ground idle and reset the CB for shutdown.

ENGINE OIL SYSTEM FAILURE

High Oil Temperature can be caused by failure of the oil cooler flap to function in automatic.  Manually open or close to maintain within limits.  ESP if temp cannot be maintained within limits (5 min in flight/30 minutes ground).  If the low oil light comes on then carefully monitor instruments but you do not have to shutdown if within limits.  If the rate of loss is gradual and the mission is long enough to require a shutdown eventually, then consider shutting it down and restarting later.  If loss of Oil Pressure then ESP.  Following a negative G condition then monitor gearbox and engine oil pressure for 2 minutes.  If there is a loss and it does not return to normal within 10 seconds then ESP.  After prop stops then you can Air Start.

VISIBLE FLUID LEAK

If there is an excessive fluid leak then ESP.

TACHOMETER GENERATOR FAILURE

Can be indicated by decrease or fluctuation in RPM or Torque.

Syncrophase off and reselect master if needed.

Then Mech Gov on that engine.

FUEL SYSTEM FAILURE

FUEL BOOST PUMP FAILURE

Verify.  If verified or any phase CB out then:

Pump- Off, CB - Pulled, Setup other fuel supply.

Do not turn on or reset CB until repairs have been made except to prevent fuel starvation.

FUEL QUANTITY INDICATOR FAILURE

If the guage goes off-scale high or low or fluctuates erratically then pull the CB.

Do not reset the CB until proper repairs have been made.

EXTERNAL TANK BOOST PUMP FAILURE

Select other pump.  If light goes out then original pump failed.  If light remains on then go to main tank feed and check the pressures on both pumps.  If both pumps are inoperative then fuel in that tank is unusable.  If both pumps work then its the light switch.

EXTERNAL FUEL LEAKS (Drip or Running Type)

Consider shutting down the engine especially if leaking near and engine.  Land at an airfield that is long enough for no reverse thrust.  Declare an emergency and request fire suppression equipment.

FUEL DUMPING

If a wing fire exists then stop dumping.  If you select dumping while the aux or ext tanks are supplying fuel to engines then the crossfeed valves are closed which shuts off fuel to manifold.

If you have an inoperative gauge then select that tank to DUMP first, then verify fuel is dumping before selecting other tanks.

Do not dump 5000’, or in a circular pattern.

Clear the dump manifold prior to landing with a wing-low attitude and slight skid.

ELECTRICAL SYSTEMS FAILURE

Never use a CB as a switch.  Should only be opened during emergencies or maintenance.

GENERATOR FAILURE

Light will come on if: any phase is less than 70V, output is below 368 CPS, or if field-trip relay has opened.

If generator failure is confirmed during taxi then Ground Stop.

1.  Frequency, voltage, and load normal:  Power indicator relay or T/R unit failure.  Leave ON and monitor.  If loss of indication then step 2 or 3.

2.  Frequency, and voltage normal, no load:  Generator contactor is not energized. Turn OFF and monitor.  If loss of indication then step 3.

3.Frequency, voltage and load zero:  RESET then OFF.  If normal then resume normal ops.  If no indication then generator has failed, perform GEN DISC or ESP.

If there is an indication and voltage peaks above normal  then overvoltage or generator feeder fault is problem, perform GEN DISC or ESP.

4.  Abnormal frequency or voltage:  Turn OFF and monitor.  If loss of indication then

GEN DISC or ESP.

ILLUMINATION OF AC BUS OFF LIGHT

1.  Turn the affected generator OFF, and ensure another generator picks up the load.

2.  If frequency and volts are norm and no item on that bus failed then resume norm ops.

3.  If frequency and volts are abnormal then leave off and monitor.

4.  If indication is lost then GEN DISC or ESP.

5.  If another generator did not pick up load then bus and all components are lost.

ILLUMINATION OF ISOLATED DC BUS ON BATTERY LIGHT

This is cause by an inflight failure of the reverse current relay between ESS and ISOL DC.  Check battery voltage: if 28V then indicator failure, if 24V or less then all BAT & ISOL bus items are only good for the life of the battery.  Land ASAP and turn off non-essential items to conserve battery power.  (CP pitot heat and flood lights)

LOSS OF ESSENTIAL AC BUS

You can lose one phase with out the light.  Indication by following conditions:

NASFASH	Normal brakes failure (on ground)

Erratic Autopilot

Suction Boost Pump Light

#2 Fuel Boost Pump Light

Aux Hydraulic pump failure

Syncrophaser failure

Heading indication loss or malfunction.

Proceed as follows: MGAVIR

MECH GOV on all props

Generator (one supplying power to ESS AC) OFF & monitor

If power is restored then leave off and NORM governing

If power is not restored then you must land ASAP and:

Anti-skid - OFF

VG - on both P/CP vert ref switches

Inverter switches to Standby (DC)

Reduce Load FAST and Read 3-31

#2 Fuel Boost Pump	Aux Hydraulic Pump

Suction Boost Pump	6 T/R Cbs for ESS DC

BLEED AIR SYSTEM FAILURE

A serious bleed air system leak can be indicated by GOPETER:

GTC fire warning

Oxygen quantity zero or fluctuation

Pressure on bleed air manifold reduced

Engine instruments erratic operations

Torque lower (may be transitory followed by near normal)

Electrical equipment erratic operation

Radar inoperative or intermittent

The engineer will close the engine bleed air valves on affected wing, and close the affected wing isolation valve.  If the uncontrollable loss of bleed air cannot be isolated then close 	all engine bleed air valves.  If a bleed air valve cannot be closed (rise in toque on affected engine) and there is a leak then you may have to shutdown engine.  Do not operate GTC 	after landing.

WING/EMPENNAGE AND WHEEL WELL OVERTEMPERATURE

If wheel well overheat illuminates after takeoff and persists after isolation procedures then leave gear down, due to hot brakes.  Do not reopen bleed air valves if closed for overheat due to damage to warning systems.  	You may reopen an engine bleed valve only if anti-ice is necessary to sustain flight.

The aircraft will be depressurized if all engine or both isolation bleed valves are closed.

The wing has lights and indicators, the stab and fin has just indicators and the nose, left and right wheel well have just lights.

If you get the light then turn off the respective system and wait a minute.  If it overheat persists then:  Wing:  2 respective engine bleed, respective wing isolation and GTC valves closed.  Others:  Close all 4 engine bleed and GTC valves and if a bleed valve fails to close then isolate wing.

If it is a wheel well overheat and condition persists, you ma y have overheated brakes-	lower the gear.

Emergency Operation of the Air Conditioning System

The cargo compartment ac should keep the aircraft pressurized (70 ppm)

The flight deck ac may not depending on leakage rate.

If you cannot control temp then choose shut down or discomfort to keep pressurized

If the system is leaking hot bleed air then shut down immediately

If you cannot shutdown then you can isolate

Emergency Operation of Cabin Pressurization System

Overpressurization - 	Shutoff engine bleeds one at a time until safe increase

Control pressure with engine bleed valves

Further control when descending you can shutdown an ac unit

Underpressurization-	Don masks while starting descent to alt where OXY is n/r

Check for excessive cabin leak and bleed air leak

65 to 35 psi  10 seconds

Do not unlock window or door with pressure on aircraft

WING FIRE
	Close engine bleeds and isolation valve on that wing

Sideslip the aircraft to keep fire away from fuselage

Land ASAP

ELECTRICAL FIRE

Only shutoff electric if pilot is reasonably certain that its causing smoke or  fire

Consider closing oxygen manual shutoff valve if fire is close to any oxygen component	If able to locate source then turn off/pull circuit breakers/ or unplug

If unable:	ATM - ON & All Gens - OFF If fire stops locate bad unit and isolate

If persists: 	VG & DC, Oil Cooler - Open&Fixed, ATM - Off&Stop

If ATM inop then:  VG & DC, Oil Cooler - Open&Fixed, Gens - Off, Pull 3 Main AC 		current limiters, One Gen - On.  If fire goes out isolate bad bus

If persists then Gen - Off

BUS ISOLATION PROCEDURE

Use as a last resort if malfunctioning unit cannot be located, but you know the bus

LH AC & MAIN AC - Remove 3 current limiters at FS 245

RH - Remove 6 current limiters at FS 245 (3 RH AC and 3 Deice)

ESS AC - Pull 9 Cbs at FS 245 and 9 Cbs at pilots side CB panel

MAIN DC - Pull 6 TR Cbs on CP upper CB panel and grnd cntrl CB on CP lower

ESS DC - Pull 6 Main TR Cbs on CP upper CB panel and 6 Ess TR on pilots side

Do not tie bus on landing (Open & Fix oil cooler flaps before this)

ISOL DC - Pull 6 Main TR and 6 ESS TR and DC power off (no ADIs)

BATT - There is no way to isolate the whole bus

INS BATT - There is no way to isolate the whole bus

FUSELAGE FIRE/SMOKE AND FUMES ELIMINATION

Notify crew and pax Pilot - Oxy on, 100 percent Direct crew and check them in

If flammable fumes are present avoid turning on and off electrical equipment

If pax and not enough oxygen then descend before depressurizing

Use wet towel or handkerchief helps with smoke and fumes

IF ATM is powering ESS AC then it will stop when bleed valves close

Smoke will move forward, the forward escape hatch must be opened

Loadmaster will wear restraint harness or parachute when opening doors

If paratroop doors cannot be opened then find another opening

INFLIGHT DOOR WARNING

Notify crew and pax, everyone should fasten seat belts

Do not go near door until determined safe to do so

Direct personnel without oxygen or seatbelts to remain on flight deck until it is determined the crew entrance door is not cause

If range is critical, and all pax have sufficient oxygen, then depressurize at altitude

The airplane must be completely depressurized before making check

The FE will check the doors and use parachute or restraint harness

If unable to determine cause then partial pressurize to point where light comes on and 	continue with all personnel secured.

If cause is switch then you can fully pressurize

INFLIGHT RELEASE OF LIFERAFT

If autopilot is engaged there may be excessive elevator trim pressures when disengaged

Decrease airspeed

Lower flaps and visually inspect liferaft compartments and tail area

If lodged on tail then fishtail or shallow bank

Make an emergency landing and do not reverse propellers

WINDSHIELD AND WINDOW FAILURE

If one pane of flight deck or cargo compartment then 10 in/HG

If both panes or inner pane of flight deck then 10 in/HG

If both panes of cargo compartment then 0 in/HG

RAPID DECOMPRESSION

Oxygen and safety belts - On

Have FE inspect fuselage with a harness or parachute to determine damage

If no damage then descend up to max speed

With damage then pilot will determine safe speed and flap configuration

Consider flaps carefully since changing center of lift can cause more damage

EMERGENCY DESCENT

Without structural damage - Flight Idle, Gear & Flaps up, Max speeds

With damage - unless changing config can cause more damage then Flight Idle, Gear - 		Down, Flaps 100%, and 145 KIAS

HYDRAULIC SYSTEM FAILURE

Loss of System Pressure - Hydraulic and Suction Boost Pumps - Off (affected system)

Check Reservoir - If low then check for leak in units

If loss in MLG upline then pull LG CB to position neutral

Isolate units.  If not possible then leave switches off

Engine-Driven Pump Failure - Hydraulic Pump - Off

Check Reservoir - If normal then no further action

If level decreasing then Loss of System Pressure

If you suspect pump failure then consider shutting down

If >3450 PSI (failed compensator) then do not turn Off

If >3900 PSI (failed compensator and relief valve)

Cruise Engine Shutdown, If still high then Airstart

and ESP on other

Electric Suction Boost Pump Failure

100-200 PSI drop with light may be indicated

Suction Boost Pump - Off

Check reservoir and static pressure - If <2500 PSI or  fluid then Loss of 			System pressure, If >2500 then minimize use

Utility System Failure

Loss of Normal Gear and Flaps, Normal brakes, nosewheel steering, and half  flight controls.  Do not touch NWS or you will loose dampening.

Booster System Failure - Loss of half flight controls

Auxiliary System Failure

Loss of normal ops of ramp and door, emergency brakes, and emergency NG extension.  You can use Handpump to accomplish this.

FLIGHT CONTROL SYSTEM FAILURE

Never purposely turn off a boost pack.

If a booster pack is suspected to have failed in a hard over position, verify that the control matches the hardover experienced and then turn that pack off.  Turning off the elevator boost pack in a hardover maneuver will leave the plane in a out of trim nose-high direction.

If a known or suspected structural failure then perform a controllability check.

If a hydraulic leak develops in a boost pack, you can isolate by turning both switches off.  If the leak continues then perform Loss of System Pressure.

If you must land without hydraulic assistance then avoid crosswinds, and short or narrow runways.

1.  Reduce weight and speed

2.  Long flat approach and fly airplane onto ground

3.  Use normal trim and engine power to reduce control forces.

Rudder Failure - Reduce airspeed and use trim, ailerons and engine power for directional control.

Aileron - Reduce airspeed and use rudder, asymmetric engines and trim for roll control.

Elevator - Reduce airspeed and use power, trim and coordinated yoke inputs.  Achieve level flight @ 170 kts, using inboards for pitch and outboards for a/s.   Land with 0-50% (50 rec) flaps.

Aileron Trim Failure - Hold trim opposite and pull Aileron Trim CB on pilots side CB panel

Rudder Trim - Reduce airspeed until directional control is regained, then hold opp trim and pull CB Elevator Trim - Hold trim opposite and place elevator trim switch to off, select emergency and retrim.

If nose high then use bank, reduce airspeed and lower flaps if able.

If nose low then reduce power and airspeed.

If troubles occurs in emergency then select off.  Autopilot doesn’t work in emer or off.

FLAP SYSTEM FAILURE

If flap aileron binding occurs then stop flaps.  Only move if control cannot be maintained and you must free ailerons.  Move them 10% increments towards last position.  If ailerons free or 	binding gets worse then stop flap movement. If flaps will not operate normally then use checklist to overide selector valve or mechanically lower.  Asymmetric flap protection only works in normal.  If you get this then stop flaps and  move the controllable flap to match the uncontrollable flap

Electrical Control Failure - Pull CB, place lever in desired position, remove utility cover, depress 	lower or raise button in 10% increments

Loss of Hydraulic Pressure - A leak in the flap system is indicated by loss of pressure during 	movement and slower than normal flaps.  Follow checklist.  Takes approx 650  turns.

Shift from Manual to Hydraulic Flap Drive - follow procedure in -1

Asymmetric Flaps - If flaps stop early then emergency flap brake may have engaged.  Position the lever to match the flaps.  Do not move flaps, and do not manually overide brake inflight

Flap Position Indicator Failure - If indicator does not work and flaps are moving (this is verified by drop in hyd pressure and pitching moment) you can direct visual inspection of flaps and the CB on aft fuselage junction box.

LANDING GEAR SYSTEM FAILURE

Do not takeoff with known or suspected gear malfunction.

If malfunction occurs while lowering gear then once gear is down and locked, do not move gear.  If one or more gear will not retract, do not recycle, instead extend gear and visually inspect and land ASAPractical.

If cargo configuration permits, prior to landing depressurize, remove inspection window and confirm the bottom of ball nuts contact bumper stops.  If any gear fails to extend then identify malfunction, do not recycle.  Check CB, hydraulic pressure and quantity and for leaks.  If leak is cause or if loss of pressure occurs when lowering handle, immediately place gear up and pull CB, then check pressure, if still low then perform Loss Of System Pressure.

Alternate methods of lowering gear in order of preference:

Overriding selector valve - pull CB, place handle down, remove cover, depress down 		button and hold if required.  If the button required holding then you will not 		have nosewheel steering.  Indicators should work but also have windows.  		Always depressurize when removing pressure-sealed doors

Manual gear extension (main) - follow checklist.  Depressurizing plane can assist.

Emergency hydraulic extension (nose) - follow checklist.  Do not move the emergency extension handle from emergency position until after safety lock is  installed. Maintain pressure on system.

Nose gear manual extension after complete loss of hydraulics - follow checklist.

Do not taxi and jack plane before installing pin.

Main gear extension after normal and manual failure - follow checklist

Emergency Retraction - If handle will not move up then manually release the down lock.  If either gear will not move up then follow checklist at discretion of pilot.  Investigate problem.

Main Landing Gear Tiedown - Follow procedures in -1.  Use for broken shelf bracket or drag pins not engaged in shelf bracket.

CARGO DOOR AND RAMP SYSTEM FAILURE

Cargo Door Uplock Emergency Release - pull manual release and use normal closing procedures

Ramp Fails to Lock - Check reservoir to see if its the problem.  If not then follow procedures in -1.  If 	ramp still will not lock then do not pressurize the aircraft.

CARGO JETTISON

Depressurize the aircraft, descend below 10,000 feet, and use ramp and door in the ADS position.

Relatively light cargo can be jettisoned by hand.  Use ramp/door or paratroop door if unable.

Cargo on Rollers - Consider oversized cargo on multiple pallets as one unit.  Pallets joined in train should be disconnected if emergency allows and jettisoned one at a time.  Make sure CG 	remains in limits for landing.  Jettison malfunctioning airdrop equip with this method.

Cargo not on Rollers - Large heavy palletized or cargo resting on floor should be jettisoned as a last resort.  It can move slowly and become misaligned and wedge and place CG out of limits.

Cargo Jettison Technique - Establish 10 nose up, apply power to increase extraction force.  Move 	elevator controls slowly, smoothly and avoid exceeding structural limits.  Have Load 	compute CG. It is not recommended to jettison pallets weighing less than 2,500 lbs.

BAILOUT PROCEDURES

If time and conditions permit:

1.  Give bailout warning over PA, interphone and 3 short rings on alarm.  Use the Jump Lights if 	paratroops have been briefed.

2.  Reduce airspeed and descend to where oxygen is not required.

3.  Depressurized and go to AUX VENT.

4.  Head toward isolated area and engage autopilot.

5.  Open Ramp and Door below 150, if above 150 open cargo door.  If unable ramp/door then open air defector doors and open paratroop doors.  If unable paratroop doors then jettison crew entrance door (you may have to be less than 3.1 in/HG).

6.  Give abandon signal over PA, interphone and 1 long ring of alarm.

7.  Evacuate plane.  It is not recommended out crew entrance door above 150 knots or with gear  extended.  If out crew entrance door then bailout in squatting position at rear.  Push head first outward and downward using rear edge as leverage.

If over water then make sure IFF emergency and send distress signals.  Put on anti-exposure suits over clothes, don life jacket and parachute.  Do not inflate life jacket prior to bailout.  Make your run if able so personnel will drift onto course just ahead of ship.

LANDING EMERGENCIES

Prior to crash landing or ditching the airplane or in the event of fire around flare ejectors, eject all remaining flares.

Landing with an unsecured panel, broken HF antenna, fuel leak, or hung liferaft avoid using reverse.

CONTROLLABILITY CHECK

Check for damage and injuries and climb to 10,000 ft AGL.  Consider dumping fuel and complete the Descent checklist.  Configure for landing.  Never decrease airspeed to where full control deflection is required.  There is a possibility of split flaps.  Slow in 5 knot increments to landing speed or control problem.  If stall buffet occurs then accelerate and plan touchdown to be 1.2 times that speed.  Make a low flat landing approach.

LANDING WITH ONE ENGINE INOPERATIVE

Same as normal landing except do not extend past 50% until landing assured.  Below 110 knots during flareout the aircraft will tend to turn the plane toward the bad engine.  Reversing unsymmetrical engines may cause the airplane to veer to one side.

LANDING WITH TWO ENGINES INOPERATIVE

2eng ops above 120,000 lbs or high density altitude is marginal.  Attempt to decrease weight by dumping fuel and/or jettisoning cargo.  If 1 and 2 are inop then additional time will be required for gear and flaps.  Downwind & Base: 160 KIAS  Turn to Final: 150 or appr speed  Final: 150 or appr speed until landing assured.  If no flap then high rudder boost can be obtained by pulling flap cb and positioning lever to more than 15%.  A go-around is not recommended after flaps are lowered and do not extend full flaps until landing is assured.  Assure nosewheel contact before reversing and use only as needed.

GO-AROUND WITH ONE OR TWO ENGINES INOPERATIVE

1.  Alert crew

2.  Begin at or above Vmca and use 5 bank away from dead engine.

3.  Advance throttles toward max power as control permits.  May have to reduce power on opposite engine.  Attain 2eng Vmca ASAP

4.  When good airspeed, altitude, and climb check/set flaps 50% (T/O  or threshold).  Retracting from 100% to 50% can cause sink or stall.

5.  When aircraft will not touchdown the retract gear.

6.  Raise the flaps to 20% (T/O + 20) Then flaps up after 2eng Vmca.  Raise 10% with 5 knots inc

7.  Accelerate to 3eng climb or Vmca/160 (2eng) and set climb power. After takeoff checks.

TIRE FAILURE

Nose Gear - if one the normal, if both then hold off the ground as long as possible.

After contact then  use max reverse and min braking. Taxiing is not recommended.

Main Gear - One tire: touchdown nosewheel asap and use max reverse.  Taxiing is not

recommended. Both tires: line up on the side with the good tires, touchdown nosewheel asap.  Hold  forward pressure and use NWS.  Use opposite brakes only to assist NWS.  Use reverse  cautiously but enough to minimize landing roll.  Do not taxi.

LANDING GEAR RETRACTED

Replace inspection windows prior to landing and reduce weight as much as possible.

One or both Main Gears - raise the gear and extend the nose gear with emergency hydraulics or land  gear up.

Nose up and Mains down  - hold the nosewheel off as long as possible do not use brakes and follow -1 procedure.  Ground egress after stop

Gear-up - normal landing attitude.  Ground egress after stop.

LOSS OF NOSE WHEEL STEERING DURING LANDING/COCKED NOSEWHEEL/SHIMMY

If unable to move the steering wheel, then do not force it.  Pull back on control column to relieve pressure on nosewheel and maintain directional control.  If loss of utility hydraulics do not use NWS on landing roll.  This could cause a shimmy or turn from center.  If cocked nosewheel then request foam.

DITCHING

Minimum of 2000 lbs of fuel should remain at descent to allow for jettisoning of cargo, assessment of sea, communication with or maneuvering around surface vessels, and establishing a landing pattern to the smoothest surface.  Hold nose up for as long as possible. The plane will settle nose down and primary flooding will occur in the forward portion.  There are normally 2 or more impacts.  Keep the # of personnel to a minimum in the flight deck due to flooding.  Normally board the left inboard raft but if pax then assign crewmembers to different rafts.

Normal - 7 nose up, full flaps, gear-up, and touchdown 10 knots above stall speed

(100 ppm - 200 max).  Maintain constant back pressure on control column.  Do not 	stall plane in.  If wind < 30 knots then top of swell parallel to rows of swells.

If > 30 knots then upwind but remember possiblity of ramming nose in wave.

Partial Power - 2 out on same side then use inbd only.  If 2/4 or 1/3 then considerable 		power may be used.  Use power for flattest appr.  Hold 20 knots above stall 		until flareout then 10 knots above stall speed.

Crosswind - Wings level and crab to kill drift.  Land on downwind side of swell or

wave.

Upwind - Maintain nose-up and avoid stiking wave face.  Touchdown immediately

Behind crest of rising wave.  Hold mose up after impact.

Night - Instrument approach, 20 knots above stall.  At 500-700 feet agl set up 200 fpm 		and 10 knots above stall with full flaps.  Use lights.  Hold wings level.

Crew and pax must not leave positions until certain airplane has stopped.  Do not inflate vest until clear of aircraft.  Cannot use left side of escape ladder.  Handles must be pulled fully and if not ejected then pull handles on wings.  Secure equipment to rafts.  The swing windows may be used in an emergency but heavy flooding may occur.

ABNORMAL PROCEDURES

For Buddy Starts, Windmill Taxi Starts, Practice Takeoff  Engine Failure, Simulated 3 Engine Takeoff, and 3 Engine Takeoff read the -1 procedures ahead of time.

CRUISE ENGINE SHUTDOWN

Do not use this procedure for known prop malfunction which will terminate in shutdown.

Operating in the freezing range with visible moisture may cause icing that will prevent restart.

Accomplish NTS check on one engine at a time.  Do not shutdown engine with inop NTS.

After shutdown of first engine, allow TIT to decrease to 200 prior to shutdown of another.

The checklist is arranged so that if an immediate start is required then all you have to do is go to airstart.

AIRSTART

Do not attempt to restart an engine with an inop NTS except in the case of a greater emergency.  Prior to the airstart of an engine with inop NTS reduce the airspeed to 130 KIAS and an altitude below 5000 feet.  Before restarting an engine make sure TIT is 200 or less.  Never move the throttle below flight idle in flight.  The recommended airspeed is 180 KIAS or less.

If no NTS by 10% then feather.   Normal lightoff is 30% if not by 40% then feather.

If no NTS and after lightoff the start is interupted before on speed then decoupling and severe overspeed.

PRACTICE STALLS

Practice at 5000 agl or 5000 above clouds whichever is higher.  Do not trim the aircraft to an airspeed less than 1.4 times stall speed.  Do not readjust until recovery is completed.  Raise the nose to lose 1 knot per second.  Increase throttle above flight idle only to prevent NTS.  Discontinue clean-configuration stalls at the onset of the buffet and do not attempt power on stalls.