SECTION IV

BLEED AIR SYSTEM

Systems:	Air Conditioning		Air Turbine Motor

Cabin Pressurization	Engine air-inlet scoop anti-icing

Engine Starting		Leading Edge Anti-icing

Nacelle Preheat (not 61st)	Radome Anti-icing (not 61st)

Urinal Drain Ejectors	Windshield Defogging

Valves:	Inlet air duct and oil cool scoop a-i - press act, sol cont, energized clsd

Engine Starter - press act, sol cont, energized open, regulates press

Nacelle Preheat - press act, sol cont, energized open

Engine Bleed Air - motor driven

Lead Edge Anti-ice - press act, sol cont, energized open, regulates temp

Wing Isolation - manual open, spring clsd, sol con, energized open

ATM - press act, sol cont, energized open

GTC - press act, sol cont, energized open, regulates bleed load

AirCon & Undrflr Ht - press act, sol cont, enrgzd clsd, reg venturi flow

Radome Anti-Ice - press act, sol cont, energized open, regulates temp

When the flight station air conditioning is on the gauge reeds 6 PSI lower than actual.

AIR CONDITIONING

Principle components: venturi regulator, electrical temp cntrl, water separator, 	refrigeration unit, aux vent valves and cntrls, and distribution ducts.

Airflow Regulators:  Flight Deck - 30 ppm (15 ppm in AIR COND GTC position)

Cargo Comp - 70 ppm (27 ppm in AIR COND GTC position)

Temperature Control:  Automatic - 5 mins from each extreme to the other

Manual - 4 mins full cold to full hot, 35 secs full hot to full cold

Refrigeration:  1st stage - heat txfr between bleed air and ram air

2nd stage - turbine which drives the cooling air fan

Water Separation:  removes 80% with cone shaped bag and drain

When the temp selectors are moves all the way to cool fog may enter

Underfloor Heat:  Heats floor to approx 80 and turns on overhead duct aux fan

Aux Vent: admits ambient air under ram pressure or external air conditioning.  	Prevented from opening until diff press is 0.28 psi (0.6 inHG)

Flight Deck Refrigeration Shutoff Valve Override:  Pull - closes flow regulator

Push - opens regulator if pressurized

Neutral - normal ops

Emergency Depressurization:  Closes both a/c flow regulators and opens outflow and 	safety valves.

If the engine bleed air valves are open and AIR COND GTC the cargo compartment 	flow regulator will open fully.  There may not be sufficient air to operate 	ATM and flight deck a/c.

CABIN PRESSURIZATION SYSTEM

Outflow Valve:  Right side at the aft end of the flight deck (Navs feet)

Safety Valve:   Left side of the cargo door.  Set @ 15.9 and -0.76 inHG

Cabin Pressure Controller:  Set to -1,000 to 10,000 feet  If forced you can damage

Manual Pressure Control Switch:  Deliberately driving the outflow valve to close 	resulting in the safety valve opening is prohibited.  When switching from 	auto to manual you must depress the switch for 40 secs to gain cntrl.

Do not pressurize the aircraft during ground ops to allow for rapid egress.

The diff press will be 0 for landing.  If less than 0.5 inHG there will be no discomfort 	if you go depressurize.

Do not lock or unlock any window, door, or hatch while pressurized.  First depress 	then goto AUX VENT.

ANTI-ICING AND DEICING SYSTEM

Pneumatic:  Wing & Empennage leading edges, radome, and engine inlet air duct and 	oil cooler scoops.  Compressor inlet guide vanes are heated directly from the 	compressor.

Electric:  Windshields, pitot tubes, and the fwd part of propeller spinner and 	afterbody.  Deicing is accomplished for the prop blades and rear part of 	spinner.

Automatic Ice Detect:  Radome, inlet air duct and oil cooler scoop, compressor inlet 	guide vane, and all props.  #2 & #3 engine detectors work when in RUN and 	in AUTO.    Automatically turns on, goto RESET to turn off.

Leading Edge:  350F air with sensors to keep between 158-180.  Overtemp 200.

Do not use to remove ice on ground.  Test on the ground limit 30 secs.

Wing:  Indicators and Lights		Empennage:  Indicators only

Engine Inlet Air Duct:  If an engine is shutdown in flight this should be left on if icing 	conditions exist.  This is not possible with the fire handle pulled.

Propeller:  Fwd section of front spinner and afterbody (A) are anti-icing  and 	continuous.  Aft section of front spinner (B), fwd section of rear spinner 	(B), aft 	section of rear spinner (C), spinner plateaus (B), and leading 	edge of blades (C) are deicing and 15 secs per blade.

Do not fly into icing if below 65 amp for 15 secs in 1 min cycle.

Use on the ground only if engine running and no more than 2 cycle.

Can be used on feathered prop.

NESA Windshield:  3 front, 2 on each side of front, 2 lower on pilots side.  High only 	means more voltage to heat quicker but heats to same temp as normal.  Use 	coldstart if below 43C (45) for 5 secs on and 10 secs off.  If above 27C 	(81F) then you can delaminate the panels if NESA is used.  If turned of the 	strength of the window is reduced.  If a panel with thermistors is not 	heating,   excessive heat, or electrical arcing then turn it off.

COMMUNICATION AND ASSOCIATED ELECTRONIC EQUIPMENT

Do not use other nonsecure communications (including interphone) when using 	secure-voice.

INTERCOMMUNICATION SYSTEM - All but cargo-compartment can hear outside 	communications.  The P, CP,  N & Instructor panels are the only who can 	transmit.  On some airplanes HOT MIC will not work on INT or UHF-2.  	Using one of the switches will override HOT MIC line.

PUBLIC ADDRESS SYSTEM - 7 speakers: ALL (all but ramp), FWD (one speaker 	in front), AFT (ramp only), JUMP (2 speakers next to paratroop doors)

VHF -  AM and FM capability for 4,080 total channels.  FM - 30.000 to 87.975

AM - 116.000 to 151.975 (TX/RX)   108.000 to 115.975 (RX Only)

1 minute warmup

SECURE VOICE - KY-58: VHF & UHF   KY-75:  HF

UHF -  7,000   225.000 - 399.975.  1 minute warmup. # 1 cntrl head overrides SCNS

Do not use the auto selection on the antenna selector.

When using guard in an emergency, go to GUARD and MAIN.  If you use 	BOTH then the signal may be unintelligible.

HAVE QUICK - Tone will be heard in “AJ” if an invalid active net is selected, TOD 	has not been received, or WOD has not been entered.  Radio will 	automatically receive the first TOD it receives after power up.  Selecting 	GUARD disables “AJ” mode.

MARKER BEACON - OM:blue/400 cps IM:amber/1,300 cps Airway:white/3,000 cps

To select a narrow band width then select LO when at high altitude.

VOR/ILS/MB - 200 channels (160 VOR and 40 localizer)  108.00 - 117.95

Do not test with autopilot engaged.

TACAN - 126 channels each for x and y.    90 secs warmup.

Do not test with autopilot engaged.  When Tacan is primary navigation 	source the nav will not select tacan on either BDHI/RMI.  When P & N have 	TAC 1 and CP has TAC 2 then BDHI can have 10 error.

In air-to-air the receiver selects a channel  63 from other aircraft.  Use of y 	is recommended in a-a mode.  To prevent interference from IFF then select 	y or use x channels 11-58, 74-121.

FLIGHT DIRECTOR - Consist of computer, ADI, HSI, rate gyro, MD-1 gyro, rate of 	turn sensor, and selector switch.  Should use the Ess AC selection on the CP 	INST Bus during flight.  If you are using standby and you get jittery 	indications then select HDG and NORMAL.  If you get a failure of ADI in 	Ess AC position then got to standby power.

ADI repeat only works if P has VOR/ILS 1 or 2 selected and a valid ILS 	frequency tuned.

Do not put both Vert Ref switches in INS @ same time.

RADAR ALTIMETER - Unreliable over large depths of snow.  Does not look forward 	for terrain clearance on low-levels, just directly below aircraft.  16-32 secs 	calibrations time.

ADF - 100 to 1,750 kilocycles.  5 minute warmup.

RADAR - 3 minute warmup.  Dangerous voltages (15,000 volts) are in the radar set, 	so don’t make internal repairs or adjustments.  Before selecting Search, 	Beacon, or Warning make sure people are clear.  Don’t use or direct beam 	around aircraft being refueled.  Have stabilization off while taxiing, prior to 	landing, or inflight in STBY.  Otherwise have it on.

Leakage Test:  Momentary ON until 40 inHG.  It should hold within 2 	inHG for 10 minutes.  Don not allow it to go above 41 inHG.  Do not 	operate radar until 27-32 inHG.

AIMS - Air traffic control radar beacon, Identification friend or foe, Military 	equipment, Special systems.  Must be in NORM for self test.  The audio 	option for Mode IV is inoperative.  Both Audio and Light will give you the 	light.  HOLD will retain codes with power loss if gear is down and locked.  	IDENT:  15-30 secs plus how long you hold it.  TEST:  if you hold it the 	light will remain on and it may blink once after you release it.

AUTOPILOT - Warm up 3 mins when ESS AC & DC are powered.  It will only turn 	ON when warmed up, servos disengaged, turn knob in center, and in gyro 	pilot.  Do not use below 1,000’ agl unless on a couple ILS, then monitor 	controls continuously.  Max speed: 250 knots.  Max weight: 155,000 lbs.  	Make sure its trimmed for hands off flight before engaging.  Check the 	autopilot trim indicators have an average of 0 before engaging, and always 	monitor them.  Do not overpower controls.  Do not engage in a turn or just 	after a prolonged turn.  Prior to disengaging hold wheel and pedals.  	Passing thru 200 knots retrim the rudder axis.  Go to altitude hold with less 	than 300 fpm.  Return turn knob to center slowly or the AP can overshoot 	rollout.  With altitude hold on retrim the plane with 20-30 knots change.  If 	not on course it will turn to a 60 intercept heading.  While in the cone of 	confusion you can change course 30 and between stations 5.

N-1 COMPASS - Analog style compass.  If either  compass fails then you can get 	inaccurate info.  Erratic movement of the heading pointer is a malfunction, 	that master cannot be relied on.  Disengage AP rudder.

# 1:  P - HSI compass card, FD bnk strng bars in DOP, TAC1 or VOR ILS 1

CP - RMI/BDHI compass card, FD BSB in     “           “              “

N - Right RMI/BDHI compass card

Also autopilot, search radar, and TAC 1 and VOR/ILS 1

#2:  P - RMI/BDHI compass card, FD BSB in TAC2 or VOR/ILS 2

CP - HSI compass card, FD BSB in               “             “

N - Left RMI/BDHI compass card

Also  TAC 2 and VOR/ILS 2

5 minute warmup when Ess AC & DC is powered.  Allow for 	synchronization before use.  Reset the latitude every 2.  #1 and #2 should 	be within 2 of each other.  Each remote would be within 2 of respective 	master.  Masters should be within 1 of actual aircraft heading.  Precession 	should not exceed 3 per hour.

C-12 COMPASS - Digital type set up the same as N-1 except: #1 and #2 within 2 of 	each other 3 minutes after a turn, inside of 3 minutes its 4 and the 	precession rate is 2 per hour.

EXTERIOR LIGHTS

Landing Lights: 2 under wing, Ess DC (do not operate for prolonged time on gnd  	since there is no cooling)

Taxi: 2 on gear doors, Main DC

Wing Tip Taxi: 2 on wing tips, Main DC

Formation: 9 (3 on each wing, 3 on top aft of wing), Main DC

Navigation: 6 (red on left wingtip, green on rt wt, 2 white on tailcone, white on top of 	fuselage, white on bottom of fuselage), Ess DC

Anti-collision: 2 (white/red on top of vert stab or top of fuslge and bot of fuselage), 	Ess DC, Use in IMC is not recommended, can damage eyes within 10 feet

Leading Edge: 2 on both side of fuselage, Main DC

Signal Lamp: Main DC, Red Amber Blue and Green

INTERIOR LIGHTS

P/CP Inst and Engine Inst: Ess AC, Fuse protected @ Fwd FS 245, 6 volt light bulbs

CP Secondary:  Flood lights, Isol DC, 28 Volt bulbs

P & Eng Secondary: Ess DC, 28 Volt bulbs

SCNS: Main AC

All others are Main DC and 28 volt bulbs

OXYGEN

300 psi good for 96 manhours minimum

6 regulators (6 on flight deck, 4 in cargo compartment)

Portable chargers @ P, CP, rt side aft 245, and aft rt wheel well.

When on 100% less oxygen is consumed at higher altitudes, so to increase duration 	then increase cabin altitude.

As flow increase then temp decreases, which results in lower pressure.  You can limit 	use to warm it up again.  If drop in temp or press or a lack of flow then 	reduce crews need by change in flight environment or regulator settings.  As 	a last resort put crewmembers on portable bottles, EEBD, or POKs to 	reduce usage.  Recharging bottles can increase system degradation.  Make 	sure masks fits face or you can deplete the supply.  Unless you need 	unscheduled pressure increase leave the switch in NORMAL.

2 heat exchangers warm the oxygen.  Test:  On/100%/EMER, breath normal for 3 	cycles and watch blinker, hold breath (blinker black), NORMAL (blinker 	stays black), breath normal for 3 cycles and watch blinker, leave in 	NORM/100%/ON and connected.

GTC

Provides pneumatic air for ground ops of ATM, engine starting, nacelle preheat, and 	air conditioning.

Compressor: 2 stage centrifugal type

Power Turbine: combuster and turbine(some air turns turbine and rest goes to aircraft)

Accessory Assembly: “GOOFS”

Governor: regulates FF to keep @ 100%, shutdown if overspeed

Oil Pump: 1 main 2 scavenge 1 filter

Oil Cooler & Fan: If  25C(81F)

Fuel Pump: can be fed from any tank thru xfeed manifold.  Strainer removes 		water and acts as filter

Starter:  Door must be fully open.  When oil pressure reaches 3psi (@12%) 		then fuel and ignition circuits are energized.  Disengaged by 		centrifugal force @35%.

On Speed Light:  95% switch keeps you from opening bleed valve

Door Warning Light:  On when door is not closed

Start Light:  On whenever starter is on.

If start light does not go out after 1 min then stop.

If GTC does not lightoff then maybe lack of oil in line.  You can prime this oil line.

If DC power is interrupted then GTC will stop.

Ensure 1 min warmup before applying load.

Start: DC pwr, Opn Dr, Tie Bus, Open Xfeed, close bleed air, Start

Load:  On speed, wait 1 min, bleed air open, check pressure

Stop: Close bleed air, cool down 1 min, GTC off, close door

ATM

In left wheel well above and aft of GTC

Single stage axial flow turbine

Speed controlled by speed sensing butterfly valve

CARGO COMPARTMENT EQUIPMENT

Tiedown fitting: Floor-10,000  Ramp/Sidewall-5,000  Threaded sockets-25,000 (12 	fittings stowed in cargo door)

Cargo nets stored left and right sidewall above ramp.

Snatch blocks stored aft of 245.  Connect to threaded sockets below 245.

Wheel prybars can be used singly or pairs.  Good to 5,000 lbs but floor isn’t.

Aux ramps - 4 (2 truck & 2 ground loading) rt sidewall above ramp

Portable winch - use 10,000 tiedown.

CARGO RAMP AND DOOR

Electrically and manually controlled from aft control panel.  Electrical only 		from the ADS control panel in the cockpit (inop on the ground)

Exercise caution when using manual handle, some aircraft use down to close 		and others use up to close.

Leave controls in neutral or else they may move when aux hyd pmp is on

The uplock emergency release handle is aft of left paratroop door.

Gauges - CP is electric and the aft one is direct reading

Ramp/Door Open light - door fully open and ramp in ADS position

Cargo Door uplock indicator - flag in view when uplocked

The ramp can be stopped anywhere but the door will freefall if not uplocked

To close door - goto open then pull uplock release.  Allow to freefall.  If not 		locked then goto close.

AIRDROP SYSTEM

If you release the switch the ramp stays and the door freefalls unless locked

When the switch is in open the Door Warning light comes on regardless of 		the position of any door.

For chute release to operate the ramp/door must be in the ADS position

Ensure all people are clear between cargo and open ramp/door before 		placing the AD/TJ switch in any position other than manual.

64 paratroopers (24 inch spacing), 78 ground troops (20 inch spacing) you get an 	additional 14 is you use the wheel well wall seats for a total of 92.

70 litters with 6 attendants and 73 litters with 4 attendants

Paratroop doors - stand clear when opening or closing.  The pip pin will be in locked 	position with door closed only on the ground to secure plane.

Electric control of ramp/door is inactivated when anchor line support arms are not up.  	If door is opening it will freefall and ramp will stop.

Static line retriever will not be used to load cargo.  If you have one with a slip clutch, 	then reverse direction to reset, determine overload and continue.

The static line retriever runs for 3 secs when CDS armed and green light on

Only are CDS switch when operational check and mission requirements dictate.

Make sure you have 10 ft unwound before operational check.

Jump lights - 9(fwd/aft each paratroop door, each anchor cable support arm, fwd cargo 	compartment @ interphone panel, and next to P & CP)

Air Deflectors - 15.5 inches, provides windbreak for paratroopers.  The light comes on 	if doors are not completely closed.  They will close if any switch is on close 	regardless of position of other switch.

SINGLE POINT REFUELING

Provides refueling to all tanks through refueling manifold.  Float switch in each tanks 	stops fuel when tank is full.  Defuel thru xfeed manifold using dump pumps 	or boost pumps, then thru the ground transfer valve to refueling manifold.

If crew must refuel/defuel then refer to -2.

COCKPIT VOICE RECORDER

Records transmissions and receptions from 4 channels from P, CP, FE, and cockpit 	area remote microphone.

30 mins continuous loop.

Has a underwater locator beacon activated by water.

Remote microphone picks up cockpit voices and suppresses engine noise.

SECTION V

Minimum crew is P, CP, and FE.

Fuel:  	Recommended - JP-8 (all conditions)

Alternate - Wide Cut:  Jet B, JP-4

High Flash Pint Kerosene:  JP-5

Kerosene:  Jet A, Jet A-1

Possible loss of efficiency, increased MX, and overhaul costs, and 		reduced rate of climb, altitude, and range.

Emergency - AvGas lead grades 80/87, 100/130, 115/145 not containng TCP

Significant damage to engine and other systems.  1 time flight only

Do not use JP-8 and 5 if below -52F.

Do not use fuel with TCP.  If using emergency fuel over 50 hrs then hot 		section must be inspected.

To avoid freezing fuel, temperature should not be <6F above freezing 		point.  Fuel should be considered = to OAT.  If OAT is too low 		then move airplane to a warmer area.

When attempting to start with JP-5 and kerosene type fuel and its below -		37C (-35F) then monitor TIT and RPM for stall and overtemp 		during start.

You can have fuel boiling and venting above 125F @ start of mission.  		Check the table in section 5 if above 125F or if using AVGas.

If you mix AvGas and turbine fuels you can get foaming.

If > 2.5 aileron tab deflection to balance unsymmetrical fuel above 200 knots then 	reduce recommended speed 5 knots.

Operations between recommended and max speed is permissible for penetration from 	20,000 @ 250.

If total fuel in both ext > 9,355 with < 25,000 int wing fuel then do not exceed 290.

Flight with gear doors removed authorized by MAJCOM.  Cannot fly with forward 	nose gear removed and aft door on.

Primary Fuel Management - based on JP-8 @ 6.8 lbs/gal.

Outbd > Inbds: 500-1000 except when total usable fuel is > 1000

Mains are full except fuel used for STTO when any fuel in Aux/Ext

Fuel asymmetry within limits.

Secondary Fuel Management - anything that does not meet primary.

It is not recommended to fly with less in outbds than inbds.  Flight with outbds empty 	is considered an emergency procedure.

If you must fly in red area (E) of weight limits charts only under extreme emergency 	and MAJCOM approval.  Write it up in 781.

CG:  For sequential/multiple aerial deliveries (excluding CDS) you can load with 	forward platform only if CG does not exceed 7% forward of limits.

If you must land with CG forward use 50% flaps, maximum reverse  thrust, 	and minimum brakes.

Prohibited Maneuvers:  Aerobatics, spins, excessive nose high stalls, steep dives, and 	maneuvers resulting in excessive accelerations.  Do not make hard rudder 	kicks resulting in large yaw rates.  Although good to -1gs, if 0 or negative 	Gs you can lose hydraulic pressures and control boost.

Airdrop Limits:  42,000 max over ramp.  Leave trim near original position during 	airdrop and avoid abrupt elevator inputs.

Airspeeds:	Recommended (Vh)	Maximum (Vd)

30,000	210			240

25,000	228			266

20,000	246			291

15,000	262			318

10,000	271			322

5,000	270			320

0	269			318

Taxi limits if over 155,000lbs:  Smooth surface, 10knots taxi speed, short taxi 	distances, minimum brakes, don’t brake in turn, 20 nosewheel limit, avoid 	abrupt and uneven 	brakes, no pivoting, no towing or jacking, and 285 struts.

Substandard airfield (substandard = tires produce easily visible ruts) operations:  	285 struts, minimize nose loads with elevator during takeoff and landing, 	minimum brakes, and 10 knots taxi speed.

Max-effort operations:  130,000 lbs, 540 fpm sink rate, externals empty, outboard 	tanks with 215 struts = 6,200  285 struts = 5,000.  Conditions requiring 300 	fpm sink rates during wartime or MAJCOM approval.

Bomb-damaged airfield operations:  wartime or MAJCOM approval only, 3 inch deep 	5 ft wide craters, repairs must be flush or 3 inches high, 285 struts planned, 	130,000 lbs, 540 fpm sink rates, externals empty, 10knots taxi speed, 	minimum brakes, light brakes to assist in turns, use max thrust for takeoff 	and 100% flaps for landings, do not land outside of prepared touchdown 	zone, use reverse smoothly and brakes only to stop on available strip, 	visually inspect tires after each operation.

Porkchop fitting failure:  indicated by a slow loss of pressurization, pressurize to 10 	in/HG only, no more than 2000 hours after discovery.

Turbine spacer failure:  If OAT is below -20F(-29C) use LSGI during all ground 	operations except when higher power is required for taxi and engine runup, 	maintain > 496 TIT during all phases of flight, simulated or practice engine 	shutdowns are prohibited except for FCF (if FCF restart above 0 TIT).

SECTION VI

STALLS

In flight idle the warning buffet occurs at 4-15% above stall.

Greatest stall warning margin occurs in takeoff and approach configurations.

Least stall warning margin occurs in landing and cruise configurations.

Do not raise the flaps during recovery due to increase in sink rate and stall speed.

Recovery:  Immediately drop nose, level wings, and add symmetrical power to 	minimize loss of altitude.  Ailerons and rudder to counteract wing drop.  	Apply smooth controls and avoid diving and accelerated pull-up.

Heavy gross weight cruise configuration power on recovery:  Immediate reduction of 	rudder and elevator.  Recover by applying nose down elevator.

WAKE TURBULENCE

Greatest when HEAVY, CLEAN, and SLOW.

C-130 wake turbulence can exceed aileron authority of following C-130 @ 125 knots.

Recover:  Immediate power, and simultaneously apply coordinated flight controls.

Max power and control deflection may be required.  Back pressure to 	minimize altitude loss.

SPINS

Recovery:  flight idle, rudder opposite and aileron against spin and hold till rotation 	stops.  Elevator forward of neutral.  When rotation stops return rudder and 	aileron to neutral and perform dive recovery.

FLIGHT CONTROLS

Do not deliberately turn off functioning boost control.

Lighter stick forces in power approach configurations with aft CG.  At less that 100 	knots in power approach configuration a less positive roll stability exists.  	Landing under either of these conditions are marginal in turbulence or 	crosswinds.

FIN STALL

Fin stalls are prohibited.

Occurs at high sideslip of 15-20.

Will not result from power transients, gusts, wake turbulence, or normal flight 	maneuvers.  Most likely to occur with excessive left rudder.  Can occur 	between stall speed and 170 knots in all flap configurations with power on.

Sideslips beyond onset of buffet are prohibited.

Recovery:  rudder neutral (approx 50-100 lbs of force to return rudder), roll wings 	level, and flight idle.  If flight conditions permit pushing nose down will 	assist.

If suspect hardover then verify rudder matches and turn rudder boost off.  Use 	asymmetric power and increasing airspeed will assist.

Abrupt pushovers to negative should be avoided.  There is a time lag before recovery 	begins and considerable pull force is required.  This is due to large pitching 	moment inertia.  This could result in excessive negative Gs, uncomfortable 	nose down, and excessive positive Gs from recovery.

SECTION VII

Above crossover if the instruments are not similar in fuel flow, TIT, or Torque with throttles aligned then a propulsion system malfunction may exist.  If TIT is lower with a higher fuel flow then the TIT may be faulty.  If on the ground perform Temp Controlling Check, if in flight then TIT Indicating System Malfunction procedure.  (early stage thermocouple decay may be difficult to identify).

PROPULSION SYSTEMS CHECK

Do not run up 2 engines on the same side.

To prevent wing lift and propeller contact on ground head the aircraft within 30 of 	wind when in excess of 7,000 in/lbs and greater than 10 knots of wind.

If on slippery surfaces:  Check engines in symmetrical pairs and use reverse thrust on 	others to prevent sliding forward.  Brakes alone will not prevent movement 	if all four are > 8,000 in/lbs.  Avoid performing check around other aircraft.  	Do not make full power checks unless aircraft is lined up on runway.

Keep high power settings on the ground to a minimum.

The check is accomplished by the engineer.  The pilot will set parking brake, center 	the nosewheel and move throttles as requested by the engineer.

Improper synchrophaser operations may cause faulty indications.  This can be 	eliminated by proper setting of the master trim or by re-indexing.

If there is a low pitch stop malfunction (no 200 in/lbs increase) then record in 781 and 	get MX prior to flight.

If there is no decrease in torque when going to Ground Idle then Ground Stop.  Do not 	force the propeller with additional throttle movement.

ELECTRONIC FUEL CORRECTION

Locking the TD valve provides:  Equal power distribution during approach; may stop 	RPM and Temp fluctuation during TD control malfunction; and during 	formation flying it will prevent crossover bump when changing power.

Do not lock the TD valve during power transients, wait for power to stabilize.

TEMPERATURE CONTROLLING CHECK

Advance throttles till fuel correction lights go out (760  20).  If TIT does not 	change then set 850,  and turn on wing and empennage anti-icing (do not 	operate on ground for more than 30 secs).  The TIT should rise and return to 	previous setting.  If not then temperature controlling system has 	malfunctioned.

If there is a malfunction then goto NULL.  If the TIT stabilizes then continue ops and 	monitor TIT closely (you can exceed max TIT now).  If the malfunction 	persists then another engine system malfunction exists.  Get MX prior to 	flight.

If an engine has low TIT with high fuel flow then you may have a faulty indication.  	Goto NULL and if persists then Ground Idle and Ground Stop on that 	engine.  MX required prior to flight.  If in flight then refer to TIT Indication 	malfunction procedures.

USE  OF FUEL ENRICHMENT

Enrichment starts at 16% and lasts till fuel manifold pressure reaches ~ 50 PSIG.

If during a discontinued fuel enriched start the drip valve fails to drain the excess fuel 	then turn enrichment off and motor engine with condition lever in gnd stop.

With fuel enrichment off:  light-off occurs between 22.0 and 26.5%.

With fuel enrichment normal:  lightoff occurs between 19.0 and 27.0% with rapid 	engine acceleration.

Stalled start can be noted with lag at 40% and sharp TIT increase.

When engine is still hot stalled starts are more likely to occur with fuel enrichment.

After unsuccessful start w/o fuel enrichment, reattempt with norm fuel enrichment.

FUEL MANAGEMENT

When the airplane is parked with tanks more than ¾ full all crossfeed valves should 	be closed (otherwise low tanks may overfill by transfer of fuel).

When opening main tank crossfeed valves observe fluctuations of pressure to indicate 	the valve has opened.  When operating on crossfeed from a tank that has not 	been used monitor TIT, torque, and fuel flow for 1 minute.

Read gauges with 0 roll and 3 pitch for most reliable readings.

FUEL FLOW

Boost pump operation is recommended at all times to ensure adequate pressure.

Takeoff:  crossfeed valve closed, all main boost pumps on.

Climb and cruise: Long range - aux, fuselage, external, and main.

Short range - external, aux, and main.

If you must burn mains before aux due to sink rate limits or 		max efforts then add aux fuel to cargo weight and subtract 			from fuel for computing airspeed and weight limitations.

Externals must be used prior to mains in this case.

When operating < 6,000 lbs total in mains then crossfeed valve open 	and boost pump on for all tanks with fuel.  If any main < 1,000 lbs 	then 	place that engine on crossfeed.

USE OF WHEEL BRAKES

Unless minimum landing roll is operationally required, then max anti-skid should not be used.

TAXIING  a.  Do not drag brakes

b.  Use reverse and LSGI to control taxi speed

c.  Use brakes as little as possible for turning

d.  Do not taxi into crowded parking areas with know or suspected over -		heated brakes

LANDING a.  Use full landing roll and reverse to minimize brakes

b.  After norm ldgs when brakes are only checked allow 10 minutes before 		takeoff for brake use during taxi.

When landing with tailwind brakes are more susceptible to overheating.

If runway available exceeds CFL by 300 ft min then 10 mins can be omitted.

c.  If you must minimize ground roll:

1.  Lower nosewheel and apply steadily increasing brakes until 			max travel.  Hold this till aircraft stops.

2.  If full anti-skid used then leave gear down for 15 mins after 			immediate takeoff.

3.  Unless ops req’d do not terminate after full anti-skid ldg.

4.  If ERO (brakes used and set) after full anti-skid ldg then 			minimize brakes, keep people clear of wheel well and be 			prepared for evacuation.

Failure to cool the brakes can result in tire explosion of wheel well fire.

d.  If multiple full anti-skid ldgs then min airborne time between ldgs is 15 		mins, 130,000 lbs, gear extended, no tailwind.

e.  Partially braked ldg - smooth 3 secs steadily increasing brakes at 	90 kts 		and slight application to stop or maintain safe taxi speed

1.  Interval between stop-and-go or fullstop taxi back is a normal 			rectangular traffic pattern.

2.  If multiple partial brake ldgs then full brake application is 			limited to 3 secs, gear extended, 130,000 lbs, and no 			tailwind.

3.  If multiple touch-and-go then no min interval req’d

f.  If anti-skid inop then avoid brakes immediately after touchdown and 		before reverse or when considerable lift on wings.  There is a 		tendency to turn away from locked wheels (due to rubber scuffing 		reducing friction).  the brakes on that side must be released, not 		pressed, to correct this.

After any full anti-skid ldg above 130,000 lbs (aborted ldg, engine-out/flaps up ldg, 	etc.) allow 65 mins ground cooling time before further ops.

Three fusible plugs on each wheel will deflate the tire at a safe rate when wheel rim 	temp reaches 390F.

PARKING  a.  If brake failure, excessive brakes, or hot brakes are suspected then 		have fire department inspect brakes.

Only the fire department should be within 300 ft on both sides of the wheel well.  If 	someone must approach wheel well.  Do so only from fore or aft.

Peak temperature occurs in brakes 1-5 mins and in wheel and tire assembly 20-30 	mins after max brake ops.

If brakes must be set to evacuate airplane with hot brakes known or suspected then set 	opposite brake only.  If both sides then chock nose and do not set brakes.

Brake fires are less likely to occur if brakes are released ASAP after parked.

b.  Do not taxi or tow for at least 15 mins after brakes cool.  Record hot 		brakes known or suspected in 781.

SECTION IX

INSTRUMENT TAKEOFF

1.  Set Flight Director:  Align horizon bar, set departure course, set heading bug on 	most logical for the departure, check HSI/RMI against mag, place flight 	director in manual, and select the navaid used for departure.

Any erratic movement of oscillation of bank steering bar indicates malfunction and the 	system should not be relied upon.

2.  Align airplane on runway and check heading against runway.

3.  Rotate at normal speed to 7 nose-up on ADI.

4.  Establish climb on altimeter and VVI then retract gear.

5.  Climb initially at 300 fpm.  Retract flaps at takeoff + 20.

6.  When reaching climb airspeed, set climb power and turn on anti-ice as required.

INSTRUMENT CLIMB

1.  Complete AFTER TAKEOFF checklist.

2.  Climb speeds can be adjusted to avoid excessive nose-high, if mission permits.

3.  Limit angle of bank to standard rate or 30, whichever is less.

HOLDING

Conduct holding at 170.  If max endurance is req’d then hold at max endurance + 20.  This allows a constant power setting and allows turns with little loss of airspeed.  The airspeed will return when level again.  Bank angle is limited to 30.

PENETRATIONS

Airspeed limits are Max recommended (Vh) until 20,000 feet then 250 knots.  This is the only time you can go above Vh.

1.  Begin DESCENT checklist before reaching IAF.

2.  Begin at IAF with aircraft clean, flight idle, and atleast 4,000 FPM until reaching 	penetration speed.  Bank angle limit is 30

3.  Start level off 1,000 feet above published min inbound altitude.  Establish 170 	when at that altitude.

4.  Complete BEFORE LANDING checklist prior to FAF, reduce to approach speed.

INSTRUMENT APPROACHES

Entry into an approach is at 170 knots.  Prior to IAF or radar pattern entry complete DESCENT checklist.  Approach configuration and airspeed and BEFORE LANDING checklist prior to FAF or glideslope intercept.  For two-engine approach delay this until on final approach.  Two-engine ops above 120,000 is marginal, go-around is not recommended after flaps are lowered, and do not extend full flaps until landing assured.  Be aware of additional time required for gear and flaps if #1 and #2 engines inoperative.

|  | Three or Four-engine | Two-engine |
| --- | --- | --- |
| Outbound/Baseleg/ Transition Arc | Complete Before Landing Checks, 150 KIAS or appr speed whichever is higher | Flaps-up, Gear-as req’d, 160 KIAS min |
| Inbound/Final prior to FAF/Turn to Final if two-engine | Reduce to Appr Speed | Reduce to 150 or appr speed whichever is higher |
| Final Approach after FAF/Glideslope | Appr speed | Complete Before Landing Checks, 150 KIAS or appr speed whichever is higher until landing is assured. |
| Landing Assured | Flaps as req’d | Flaps as req’d |

Automatic ILS:  Fly to final in RANGE LOC at an angle 60 or less off runway heading.  Altitude should be so that you intercept final below glideslope.  Complete the BEFORE LANDING checklist prior to interception.   When on localizer heading you can then go to APPROACH.  Aircraft will remain at constant altitude until overshooting glideslope by 1/2 scale deflection then intercepting glideslope within 30 seconds.  Make a normal landing after visual contact and autopilot disengaged.

A glideslope warning flag will appear in the event of an unreliable or complete loss of signal.  However, certain mechanical failures in the receiver can give an “on glideslope” indication without a flag when in actuality a partial failure has occurred.  A cross check of all other available aids should be utilized.

The pitch and bank steering bars will intercept and maintain the approach, however certain malfunctions may occur without visual warning.  It is essential to monitor the CDI and glideslope during the approach.

During a back-course localizer disregard the glideslope and pitch steering bar and monitor the ILS warning flag.  The bank steering bar will guide to the selected heading.

Circling approaches:  Flown at 140 KIAS or approach speed, whichever is higher, until on final for the runway to be landed on.  Then slow to approach speed.

ICING CONDITIONS

It is recommended that altitude be changed until icing no longer exists.

Icing penetration Procedures:	1.  Select the least sever altitude consistent with mission 				objectives, traffic, or combat conditions.

2.  Ensure all anti-icing is on and operative.

3.  Use deicing as required.

If ice forms and ice detection fails, place the prop and engine anti-ice into manual.  	Delay could cause collection of ice on inlet scoop and eventual power loss.  	Avoid prolonged flight in freezing rain, particularity at low airspeed and 	high angles of attack.  This could also cause inlet scoop icing.

When the ice warning light comes on the prop and engine anti-ice comes on 	automatic.  The wing and empennage must be manually placed on.

4.  Delay gear and flap extension to avoid excessive ice 				build up on them.  Make frequent checks of 				temperature and current indicators as well as 				visual checks.

Clear air icing:  engine inlet air duct icing can occur in clear air during some 	combinations of temp and humidity.  This is indicated by falling 	torquemeter indication.  If this condition occurs then assume engine inlet air 	duct icing and perform the following:  Prop and eng anti-icing to manual and 	engine inlet air duct anti-ice on;  Increase airspeed to maximum feasible;  	and seek an altitude less likely to produce icing.

TURBULENCE AND THUNDERSTORMS

Windshield wipers are ineffective above 180 KIAS.

Prior to entering known turbulence or thunderstorms, ensure cockpit lights are set, crew and pax are briefed, safety belts fastened, and all loose equipment secured.  Set trim prior to entry and avoid adjustments to it while penetrating.

The autopilot may be used and in some case is desirable.  Disengage altitude hold and do not assist or overpower autopilot.  If it cannot control attitude then disengage.

COLD WEATHER PROCEDURES

BEFORE ENTERING THE AIRPLANE

A preheating period will be arranged with portable ground heaters or with GTC.

Do not attempt takeoff with ice, snow or frost on wings, empennage or fuselage.

Ensure all ice, snow and heavy frost is removed from entire airplane and no moisture 	is allowed in critical areas where it may refreeze.

Do not attempt to scrape or chip ice.

If anti-icing compound has not been used on the crew entrance door, then the door 	may not fully open until the telescoping rod has been heated.

Check that all fuel tank vents, drains, static ports, and pitot tubes are free of ice and 	snow.  Check that all condensation has been drained from drains.

Check for proper inflation of struts, tires, and accumulators.

Wipe struts with hydraulic-soaked cloth to remove ice and dirt.

Check that a warm well-charged battery is installed.

Ensure that all items that are to be “power-on” checked  are preheated.

BEFORE STARTING ENGINES

If isopropyl alcohol has been used to remove frost for airplane then check the interior 	for leaks.  This condition can create a fire hazard.

If external AC is available, gradually bring up temperature of NESA windshields.  	Other windows may be cleared by portable ground heaters.

When temps are < -37C (-35F) ensure engines, GTC and ATM are preheated.

Use portable ground heaters or GTC to preheat plane.  In extreme cold preheat the 	GTC.  Torching may be observed and all 4 mins to warmup.

After GTC and ATM are on and checked, then the aux hyd pmp on and operate the 	emer brake system with light pedal pressure several times and set the 	parking brake.  Inspect for brake leaks.  Seeps and moderate leaks caused by 	hardened o-rings can often be stooped with hot air.

Do not attempt to taxi if hydraulic leak in wheel well areas.  Fire and loss of brakes 	can occur if hydraulic fluid contacts hot brakes.

When ATM is operated in high humidity and shutdown below freezing.  Then start 	the ATM during preflight and if it does not start apply hot air.

Before starting engines remove all ground heater ducts.

The crew door seals may stiffen making it hard to close the door from inside.  If no 	ground crew, then deplane a crwmbr to assist, then enter thru paratrp door.

STARTING ENGINES

Leave engines in LSGI until oil temp reaches 0C.

With #3 and #4 in norm ground idle prior to starting others may cause nose to skid.

When attempting a start with JP-5 and kerosene fuels below -37C (-35F), closely 	monitor TIT and RPM for stall and overtemp.

If prop low oil light during start then GND STOP to prevent damage to prop seals.

Fuel enrichment off - lightoff  22.0 and 26.5% and several seconds will elapse till 	lightoff.  Torching may also occur.

Fuel enrichment normal - lightoff 19.0 and 27.0% with rapid acceleration.

Starts in cold weather may result in TIT lower than normal starting limits.  If all 	engine starts are similar then consider normal, no further action req’d.

BEFORE TAXI

If not already accomplished bring up NESA gradually and use wipers to remove as ice 	begins to melt.  Other windows may be cleared with defogging ducts.

When in icing conditions place anti-icing in manual, turn on inlet air duct anti-icing 	during ground ops.  Place prop anti-icing on before takeoff.

Do not operate prop anti-icing unless engines are running.

Do not taxi or move throttles out of ground idle until oil is 40C.

TAXI

At the start of taxi make sure all wheels are turning.

Make sure all instruments have warmed up to check normal operation.  Check for 	sluggishness during taxi.

NESA heat is recommended when taxiing over snow or slush covered surfaces prior to 	and during use of reverse thrust.

TAKEOFF

Prop and engine anti-icing should be on for takeoff if below freezing and visible 	moisture is present.  If only the inlet anti-icing is used expect a drop in 	torque of 750 in/lbs per engine.

If airplane starts to slide before takeoff power is reached then release brakes and 	begin takeoff run.

Monitor torque when approaching takeoff power.  Consider ram effect.  Either set 	lower and allow ram rise of continually adjust power.

After takeoff from slushy runways where ground operations were conducted in 	freezing moisture, cycle the landing gear to prevent freezing in the up 	position.

During operation of prop anti-ice there may be an indicator jitter.  If it is hard to read 	then momentarily go to reset and read indicators.

LANDING

Avoid excessive reverse at slow speeds on snow or slush covered runways.  Use 	NESA and pitot heat during ldg and be prepared to use windshield wipers.

AFTER LANDING

Allow 4 minute warmup for the GTC.

BEFORE LEAVING THE AIRPLANE

If airplane will remain for more than 4 hours in temps below -29C (-20F), remove 	the battery and store in heated area.

HOT WEATHER PROCEDURES

STARTING ENGINES

When practical position the aircraft headed into the wind.

Alternate which engine is to be started first, and do not operate the ATM if starting 	with the GTC.

Record in the 781 if lightoff doesn’t occur between 16 and 25%, especially during 	GTC starts.  This provides better turbine assist to on-speed.

If a popping noise occurs when changing to or from LSGI then return for maintenance.

TAXI

Use brakes as little as possible and use LSGI.

BEFORE TAKEOFF/LINEUP

Turn bleeds off to increase power available.

CRUISE

Watch boil off rates in Section V.

DESCENT

Avoid NTS, and during descent for landing below 15,000 ft place oil cooler flaps to 	fixed and manually control flaps to keep oil close to 60C.

LANDING

Rapid movement into reverse can cause engines to bog down.

AFTER LANDING

When practical park airplane headed into the wind.

ENGINE SHUTDOWN

ASAP after parked install chocks and release brakes to prevent damage to brakes.

DESERT PROCEDURES

Use of the GTC for airconditioning may pull in large quantities of sand and dust.  Inspect all instruments, switches and controls for freedom of sand and dust.

TAXI

Operate all flight controls thru 2 full cycles to ensure unrestricted ops.

Do not operate air conditioning on the ground.

Plan to make shallow turns, minimize ground ops, and use LSGI.

If you must use norm ground idle then don’t use reverse thrust (unless reverse taxi).

During taxi-out do not lower flaps for takeoff until lined up on runway and ready for 	takeoff.

When making prop reverse check, stop airplane and advance to above crossover to 	blow away all sand.  Perform over hard surfaces and with 2 in reverse and to 	at crossover.

REVERSE TAXI

Advance throttles to crossover before reverse to blow sand away.

After aircraft is positioned then place throttle just above ground idle until sand and 	dust are blown aft of airplane.

TAKEOFF/CRUISE

Avoid takeoff or flight thru sand and dust storms if possible.

When sand/dust in atmosphere attain and altitude above contamination ASAP.

LANDING

Turn air conditioner off prior to landing.

Compute landing performance data using max anti-skid and 4 engines in grnd 	idle.

Move throttles out of max reverse at ~60 knots and have in ground idle at 40 knots.

ENGINE SHUTDOWN

ASAP after parked install chocks and release parking brake to prevent damage.