Instructor Prep Course

POWER PLANT   (502)

3 sections - Compressor, Combustor, Turbine

Compressor

Single entry, 14 Stage, axial flow

Pressure ratio 9.5 to 1

Diffuser - after 14 stage

Directs air to combustor

Supplies air to bleed air system from 14th stage

Supplies air to Speed Sensitive Valve

Bleed valves- 5th and 10th stage.  Close at 94%.

Combustor

Through-flow, Can-annular

6 liners (chambers) - Divided into 3 sections

1.  Primary (mixer) - Mixes fuel and air

2.  Secondary (dilution) - Controls flame and cools gases

3.  Transition (tapered reducer) - Decreases pressure/increases velocity

2 ignitor plugs (plus 4 dummy plugs used to hold the combustion liners in place)

Peak combustion temp:  » 3,400° F  (most of the compressed air is used for cooling)

Drain valves-	Closed by differential air pressure during engine start ( 1-4 psi)

Opened by spring tension (when pressure drops below 8 to 10 psi)

Located at bottom of outer casing

Turbine

7 Tangential Struts allow heat expansion - rear bearing turns slightly as struts expand

Turbine thermocouples: 18 dual-element

Turbine Coupling Shaft: Connects turbine to compressor rotor.  Power is extracted and

transmitted to power driven accessories, the reduction assembly, the gear train, and the

prop.

Engine stages:	Compressor- 14 stages / Turbine- 4 stages

Engine Mounted Accessories (SSOOFF):	 Speed Sensitive Control	(electrical)

Speed Sensitive Valve		(mechanical)

Oil Pump (engine)		(power section & filter)

Oil Pump (engine)		(scavenge)

Fuel Pump

Fuel Control

Torquemeter shaft:	Outer- reference shaft to torsional deflection of inner shaft

Inner- transmits power from engine to gearbox

2 magnetic pickups show deflection for Torque Indicating System

Roller bearing at shaft midpoint prevents shaft whip and vibration

2 Tie struts maintain rigidity and alignment between gearbox and engine

Powerplant to Prop:	2 stage, step-down gear train with a 13.54 to 1 ratio

Primary stage (Spur Gear Train)		 3.125 to 1 step down

Secondary stage (Sun & 5 Planet Gears)	 4.33 to 1 step down

Reduction Gear Assembly:		Reduction Gear Train		Prop Brake

NTS System			Safety Coupling

Reduction Gear Accessories

Engine Safety Features - Prevents the engine from turning backwards

1.  Negative Torque System

2.  Prop Brake

3.  Safety Coupling

Reduction Gear Accessories (GHOST):	Generator

Hydraulic Pump

Oil Pump (prop)

Starter

Tach Generator

Fuel/Heat Strainer:	Can raise fuel temp from -70°F inlet temp to at least +34°F outlet temp.

Oil bypass- When fuel temp ³ 70°F (Thermostat controlled)

Designed to prevent fuel icing.

Strainer has 200 mesh screen with 6psid bypass valve

Low pressure switch - closes when fuel entering centrifugal pump drops below 8.5psi

Thermal relief valve:  50 psi (opens when fuel control shutoff valve is closed)

Prevents excess fuel pressure buildup due to thermal expansion

Centrifugal Boost Pump:  30 psi output

Ensures positive pressure to gear driven pumps

Secondary Fuel Pump (Gear type):  Capacity- 10.4 gpm

Secondary Fuel Pump Warning Switch:	Illuminates when secondary fuel pump pressure increases from 140 to 160 psi.  Turns on flight station warning light.

Primary Fuel Pump (Gear type):  Capacity- 11.6 gpm  (10% greater than Secondary Pump)

Fuel Pump Paralleling Valve:	Solenoid operated

Energized closed at 16% - De-energized at 65%

Fuel Enrichment:	Allows fuel to enter fuel control downstream of all metering devices

Closes when fuel manifold pressure reaches 50 psi.

Enrichment burst lasts for 2 to 3 sec.

Electrical power removed at 65% rpm.

Fuel Control responds to (TRIP):	Throttle movement and positioning

RPM Inlet Temperature (Compressor) and Pressure (Compressor Inlet)

Temperature Datum Control (TD) System:

.

Designed to compensate for:	Component variations

Atmospheric variables

Fuel density

Temp Limiting Range:	0°-65° throttle travel.  The TD system control only acts when the limiting

temperature is exceeded.

Temp Controlling Range:	65°-90° throttle travel.  The TD valve acts to control turbine

inlet temperature to a prescribed schedule corresponding to

throttle position.

Fuel Nozzles:	6 dual-tip

Inner (primary)- receives fuel all of the time

Outer- @ 67 psi, nozzle metering valve opens, allowing fuel to flow from both tips

Starting Control System (Power- Essential DC Bus):

1.  Accel. to 16% rpm-	Fuel shutoff valve in the engine fuel control is opened, the Ignition

(PIED)		relay is energized completing circuits to the ignition exciter, engine

fuel pump paralleling valve closes, the fuel enrichment valve opens,

and the manifold drip valve closes.

2.  Accel. to 65% rpm-	Ignition system de-energized, fuel pump paralleling valve opened,

(de-PIED)	manifold drip valve de-energized (but remains closed)

3.  Accel. to 94% rpm-	Electronic temp datum control switches from start limiting (830° C)

to normal limiting (TD take reduced from 50% to 20%), 5th and

10th stage bleed valves close due to air from the 14th state (Speed

Sensitive Valve).

Condition Lever- FEATHER

1.	The fuel shutoff valve on the engine Fuel Control is closed mechanically and

electrically.

2.	The propeller receives a feather signal mechanically and electrically energizes the

feather solenoid.

3.	The propeller aux motor is turned on.

Propeller feathering (2 grounds):	 1. 86° switch

2. Pressure cutout switch (600-800 psi)

Aux Feather Pump Motor- Essential AC

Fire Handle- PULLED

1.  The fuel shutoff valve (eng. fuel control) closes

2.  The engine oil shutoff valve closes

3.  The firewall fuel shutoff valve closes

4.  The firewall hydraulic shutoff valve closes

5.  The engine bleed air shuts off

6.  The engine starting control circuits are de-energized

7.  The propeller receives a feather signal and electrically energizes the feather solenoid

8.  Fire Extinguisher System control valves position

9.  Extinguishing Agent Discharge switch arms

High TIT

-If an overtemperature occurs, first retard the affected throttle toward FLIGHT/GROUND

IDLE and place the TD Switch to NULL (TD system goes to a 20% bypass state).  If this

does not correct the high TIT, proceed with the ENGINE SHUTDOWN PROCEDURE.

NOTE: High TIT is usually caused by the TD Valve putting too much fuel into the engine.

Secondary Fuel Pressure Light (In Flight)

-Pull Ignition Control circuit breaker, if the light does not go out (failure of primary pump instead of 65% switch): Consider ENGINE SHUTDOWN PROCEDURE.

-On during climbout while on crossfeed:  If it goes out when you go back to tank-to-engine

and remains off when you level off and go back to crossfeed, no problem exists.  It is a

secondary fuel pump pressure switch which is set too low.

Speed Sensitive Control Failure (Sheared Shaft)

-Momentary illumination of Secondary Fuel Pump Pressure light

-Fuel correction light ON above 65° throttle travel

-TIT limited to 830°C  (Start Limiting)

-ProcedureÞ	1)  TD valve switch- NULL  (830 TIT can now be exceeded)

2)  Pull Ignition Control circuit breaker (prevents the ignition relay from

turning on during momentary re-engagement of the sheared shaft).

NOTE:	The LOCKED position can not be used because it will energize the

TD amplifier and return the engine to Start Limiting.

FUEL SYSTEM   (504)

The main and external tanks can be filled "over the wing" without using SPR.  The Aux tanks can only be filled using SPR, or by tank to tank transfer when on the ground.

Thermal switch in Boost Pump:  opens when any phase reaches 375°F  (can't be reset in the air)

Tank shutoff valves cut off fuel so that a 3% expansion space remains in each tank.

Outboard main tanks have baffles which allow only inboard fuel flow.  They restrict surge pressure in flight and assist in maintaining lateral balance during banking maneuvers.

The offload valve must be opened BEFORE fuel will flow from the SPR adapter to the fuel tank.

After SPR refueling, the drain pump will pump fuel from the SPR manifold into the #3 tank.

Crossfeed Primer Valve:	Valve opens both the crossfeed separation valve and the crossfeed primer

valve clearing the fuel from the crossfeed manifold and placing in into

the #2 main tank.

Crossfeed valves are 28 volt Ess DC sliding gate valves which are located in their respective dry bays.

Aux Tank

-Consists of 3 interconnected bladders (the bladders are not self-sealing)

-The aux fuel boost pump also serves as a dump pump.  Fuel will not gravity feed from the

aux tank if the pump fails (use this fuel first for long flights).

Total Fuel Indicator-	Automatically subtracts the amount of fuel in a tank that has an inoperative

tank gage.  Will only display the amount of fuel contained in tanks with

operating gages.  The indicator is de-energized if the master refueling switch

on the SPR control panel is in any other position other than off.

Boost Pump Output:	Main Tank 	15-24 psi	(low pressure light at 8.5 psi)

Aux/Ext 	28-40 psi	(tank empty light at 23 psi)

Fuel System Power Sources:

-All fuel boost pumps are powered by 3 phase, 115/200 volt, 400 cycle AC motors

Pump	Power Source	CB Location

(main)	1	Left AC	Pilot Upper

2	Ess AC	Pilot Side

3	Main AC	CoPilot Upper

4	Right AC	Pilot Upper

(aux)	Left	Main AC	CoPilot Upper

Right	Main AC	CoPilot Upper

(ext)	Left Fore	 Left AC	 Pilot Upper

Left Aft	 Main AC	 CoPilot Upper

Right Fore	 Right AC	 Pilot Upper

Right Aft	 Main AC	 CoPilot Upper

Valves:	Engine	Ess DC	CoPilot Side

APU	Isol DC	Pilot Side

SPR	Main DC	CoPilot Lower

Warn:	Low Pressure	Ess DC	CoPilot Side

Lights	Tank Empty	Ess DC	CoPilot Side

Takeoff- Tank to Engine on all engines.

Approach & Landing- Main tank boost pumps should be ON and the Crossfeed Separation Valve should be CLOSED.

CAUTION:	When operating main tank crossfeed valves, observe fluctuation of fuel pressure for

indication that the valve has opened.  When operating an engine on crossfeed from

any tank that fuel has not previously been used, monitor TIT, torque, and fuel flow

approximately for 1 minute.

A normal pressure reading indicates satisfactory operation of the boost pump.

Boost Pump Failure:

1.  Place boost pump switch to OFF

2.  Pull boost pump circuit breakers

3.  Configure fuel control panel for alternate fuel supply

External Tank Boost Pump failure-  1st try other pump (see if light goes out)

NOTE:	If the pump, or any fuel boost pump, has lower than normal output, but is sufficient to supply the engines, its use may be continued.

Fuel Leak:

1.	If the leak is close to an engine, the engine may have to be shut down to reduce the risk of fire.

2.	A leaking tank may require fuel to be dumped.

3.	Do Not use reverse thrust when landing with a fuel leak- it could suck raw fuel into the

engine and cause fire in the engine nacelle.

4.	Land at the nearest airfield with enough runway to allow landing without the use of reverse thrust.

5.	Declare an Emergency to ensure that necessary emergency equipment is standing by.

Fuel Quantity Indicator Failure:  Pull circuit breaker immediately!

Fuel Dumping-	When dumping from an aux or external tank, the tank crossfeed valves will close and the respective dump valves will open (go tank-to-engine FIRST prior to dumping)

- After dumping, 7,800 lbs of fuel will be remaining

Outboard Tanks  2,100 lbs each

Inboard Tanks    1,800 lbs each

Aux/Ext Tanks   Negligible

NOTE:	To dump from a tank with an inoperative fuel quantity indicator, place the fuel

dump switch for that tank to dump FIRST.  Confirm that fuel is dumping, then

continue with dumping procedures for the other tanks.

Dump Mast Shutoff Valves-	They are wired through the Touchdown Switch and are closed on the

ground (28v Ess DC Butterfly Valves).

Primary Fuel Management:  Consider main tanks to be full when:	Outboard tank = 7,500 lbs

Inboard tank   = 6,900 lbs

Rule of Thumb:  Use Primary Fuel Management for cargo weights more than 10,000 lbs.

Use of Av-Gas:  One time flight & no more than 50 hrs use on that engine.

OIL SYSTEM   (506)

Dry Sump Oil System- No oil is stored in the bottom of either the engine or gearbox

Oil Type:    MIL-L-23699    (Emergency: MIL-L-7808 Þ amount should not exceed ½ tank capacity)

Oil Function:  Cooling, Lubricating, Fuel Anti-Icing (heating)

Oil Capacity:	Tank- 19.5 gal

Oil- 12 gal    (Air Expansion- 7.5 gal)

Warning Light- 4 gal  (28v Ess DC)

Oil Pressurization:  3.5 psi  (prevents foaming as aircraft climbs)

Oil Shutoff Circuit-	Each valve control circuit interacts with the corresponding engine starting

circuit.  The engine can be started only when the Fire Handle is pushed in

and the Oil Shutoff circuit has been closed.  (28v Ess DC)

Fuel Heat/Strainer-	It directs the oil supply around the fuel heater and is fully open when the fuel

reaches » 75°F.  (NOTE: oil is bypassed, not fuel)

Gearbox Oil lubricates the following:	1.  Pinion input gear

2.  Prop shaft bearing

3.  Sun/planetary gears

Oil Cooler Bypass:     Starts Bypassing @ temp » 57°C          Full Bypass @ temp » 75°C

Oil Cooler Flap-	If full open and the oil temperature is still too high, try closing it slightly to see if

the adjusted air flow aids in cooling.

Oil Temp. Control Thermostat-	Designed to keep oil temp at » 80°C in the oil tank by controlling

the position of the oil cooler flap.  Located in the oil return line.

Oil Overtemp:	Over 100° C Þ Oil change

Over 200° C Þ Engine change

Oil Loss Rate-	The rate loss limit is .35 gal/hr.  An excessive oil leak should be noted in the

Form 781.

Low Oil Quantity-	Continue to use the engine with the low oil quantity as long as the engine

instruments indicate normal operations.

Loss Of Oil Pressure:     1st Þ ENGINE SHUTDOWN PROCEDURE      2nd Þ Analyze malfunction

CAUTION:	If pressure loss was caused by a negative g condition and gearbox and engine oil pressures do not return to normal within 10 sec. after returning to a positive g condition, shut down the engine in accordance with the ENGINE SHUTDOWN PROCEDURE.  After the propeller stops rotating, an airstart may be attempted according to the AIR START PROCEDURE.

Oil Leak-	If the leak causes heavy smoke from the tailpipe, the engine should be closely monitored.  If a fire develops, the engine should be shut down.  If the engine is not shut down in flight (no fire),  it should be shutdown before landing.  This should prevent oilcombustion after landing.

An engine can be shut down for a gradual oil leak and then restarted prior to landing or as needed.

PROPELLER SYSTEM   (508)

Throttle Ranges:

Max Reverse (0° travel)-	Max reverse thrust with engine power » 60% at 100 KIAS or 30% static

Ground Idle (» 18° travel)-  Ground starting position, blades set for minimum thrust

Flight Idle (34° travel)-	Transition point between throttle control of pitch blades and prop governing

Takeoff (90° travel)-  Max thrust

Propeller sections:

Spinner Assembly-	Streamlines airflow and allows cooling air to flow across propeller components

Dome Assembly- Contains the pitch changing mechanism and low pitch stop assembly.

Barrel Assembly-	Mounts the propeller to the engine, holds the blades, and houses the pitch lock regulator.

Control Assembly- Contains the pump assembly and valve housing.

Synchrophaser- Controls relative propeller speeds and phasing.

Dome Prop Fluid:	Decrease Blade Angle- Rear side of the piston

Increase Blade Angle- Front side of the piston

Prop Anti-Ice/De-Icing  (Anti-Ice: continuous /De-Ice: timed on and off (15 sec/min)  Rt. Hand AC)

-Spinner front section (1st 7") & afterbody:  Anti-Ice (f A)

-Aft section of front spinner, forward section of rear spinner, spinner plateaus: De-Ice (f B)

-Aft section of rear spinner, propeller blades: De-Ice (f C)

NOTE: Deicing Timer is 28v Ess DC and is located on the aft side of FS 245

Brush Blocks:	The brush block and deicer contact ring assembly are responsible for receiving and transmitting the aircraft electrical signals to the components of the propeller that are anti-iced and deiced.

Centrifugal Twisting Moment (CTM)-  Constantly tries to flatten the blade angle due to the blade trying to align with the relative wind during its rotation.  Because of the CTM effect, the blade angle is actually decreased by reducing the amount of hydraulic pressure on the front of the piston.

Aerodynamic Twisting Moment- Tries to increase blade angle due to forward motion.

NOTE:  CTM is 20 times greater than aerodynamic twisting moment.

Propeller Safety Features - Controls blade angle

1.  Low Pitch Stop

2.  Pitchlock

3.  Prop Feathering

Low Pitch Stop-	Keep blade angle from dropping below 23°.  The normal hydraulic pressure available for

decreasing the blade angle during flight operations is » 100 psi Þ too low to unseat the servo valve.  When the throttle is moved into the ground range, the control assembly provides a pressure of 240 to 280 psi which will unseat the servo valve and retract the low pitch stop.

Propeller Brake-	Gearbox oil pressure above 23% rpm will hold it disengaged.  Below 23% rpm, braking surfaces are brought into contact.  The brake also engages to stop reverse rotation of the propeller.  When the prop is feathered, it will have a blade angle of  92.5° and will result in the prop spinning counter-clockwise which will lock the prop.  Unlocking the prop is accomplished by the aux feather motor (or the starter if the aux feather motor does not release the prop brake).

Feather Latches-	These latches lock the propeller in the feather position (92.5 blade angle) after feathering.  The latches will be overcome by hydraulic pressure when the propeller is unfeathered.

Safety Coupling-	Decouples the power section from the reduction gearbox if negative torque exceeds » -6,000 in lbs.  	Do not operate an engine when it decouples.

Pitchlock Assembly-	Prevents CTM from decreasing blade angle in the case of an overspeed or loss of hydraulic 	pressure.  The pitchlock assembly will allow an increase in blade angle to allow prop feathering.

Pitchlock Range:	25-55° blade angle

Above 55°- allows unfeathering

Below 25°- allows ops in ground range

Pitchlock Regulator-	dumps hyd. fluid at 103.0% rpm

(Fuel control takes fuel at 103.5% rpm)

Control Assembly- 2 parts  (pump assembly and valve housing)

Prop Oil- 26 qts total  (6.5 qts in the pressurized sump)

Main Scavenge Pump-	Pumps oil from atmospheric sump to the pressurized sump at

20 qts/min.  This keeps the pressurized sump at 6.5 qts.

Pump Assembly- 5 pumps

Pressurized:	Main Pump- 20 qts/min	Un-pressurized:	Main Scavenge Pump

Sump	Stand-by Pump- 40 qts/min	Sump	Aux Scavenge Pump

Aux Pump

Propeller Hydraulic Fluid Flow:

1.  Feather Valve

2.  High Pressure Relief Valve (opens at 1300 psi)

3.  Feather Solenoid (electrical feathering)

4.  Feather Actuating Valve (mechanical feathering and NTS operations)

On Speed Condition: 	» 5 qts of fluid supplied through the pilot valve assembly to the high pitch line.

Compensates for CTM and internal leakage.

Overspeed Condition: 	Flyweights position the pilot valve assembly to meter more fluid pressure

to the high pitch line (pilot valve moves towards the speeder spring).

Underspeed Condition:	The speeder spring moves the pilot valve assembly to meter pressure to the low pitch line and to port the high pitch line to the pressurized sump (pilot valve moves towards the flyweights).

RPM Changes:	For rpm changes of 1% or more, the output of the standby pump is added to the

main pump (standby check valve opens at 12-15 psi)

Alpha Shaft-	Throttle and Condition Lever control it through the engine coordinator.  Allows NTS

plunger to move the feather valve in flight.  Cammed out in the ground range.

Beta Shaft-	Controlled by position of the #1 prop blade.  It indicates the actual blade angle on a

protractor in the gearbox and provides the follow up necessary for propeller control in

the ground range.

Alpha Shaft Components:

Manual Feather Cam-	Positions the manual feather linkage to allow NTS operation in the

flight range and not in the ground range.  When the condition lever

is placed in the FEATHER position, the manual feather cam opens

the feather actuating valve and pulls the feather valve into the

feather position.

Speed Set Cam-	In ground range, increases tension on speeder spring to prevent flyweights from

overcoming it until prop reaches overspeed of 110-113% rpm.  Now the pilot  valve, blade angle, and engine rpm are controlled by throttle position.

Beta Set Cam-	Positions the pilot valve in the ground range.  The prop is now a controllable pitch

propeller in the ground range.

Back-Up Valve Cam-	Allows additional pressure to the low pitch lines between 0-28° throttle travel to allow retraction of the low pitch stop.  This higher pressure also provides a more rapid response while in the ground range.

Anticipation Potentiometer- Allows the propeller to respond directly to throttle movement.

Beta Shaft Components:

Beta Follow-Up Cam-	Re-centers pilot valve in the ground range after movement of the beta set cam on 	the Alpha Shaft.

Back-Up Valve Cam-	Allows higher pressure when blade angle is greater than 35° for decreasing blade pitch when blade angle is high.  Also for retracting feather latches when moving the prop out of feather.

86° Switch Cam-	It opens a back-up ground for the propeller electrical feather circuit when blade angle is greater than 86°.  This prevents initial pressure surges when feathering from activating the Pressure Cutout Switch (600-800 psi).

Propeller Feather position:  92.5° blade angle

Pressure Cutout Switch:  600-800 psi switch at the valve housing (valve housing pressure)

Feather Relay-	The Feather Relay remains energized as long as the Condition Lever is in 				FEATHER.  This keeps the feather cycle from re-initiating when the pressure in the 		valve housing drops and Pressure Cutout Switch closes again (pressure drops below 			600-800 psi) (28v Ess DC).

NTS System-	The NTS plunger moves the Feather Actuating Valve that shifts fluid to move the

Feather Valve.  Actuates at » -1,260 (±600) in lbs of torque.

Synchrophaser functions:	1. Throttle Anticipation (anticipation potentiometer)

2. Speed Stabilization (tach. generator)

3. Synchrophasing

-It makes corrections to the blade angle by signaling the speed bias motor to adjust the

speeder spring tension through a gear (clutch, brake, and speed rack) assembly.  The

throttle anticipation circuit allows the propeller to respond directly to throttle

movement.  The tach. generator provides an actual rpm signal for the speed bias motor to

use (more precise speed control reference).

-Speed stabilization is accomplished by converting signals form each Tach generator to DC

voltage and charge a capacitor.  As RPM varies, the fluctuation in capacitor charge is

sensed by the synchrophaser amplifier, corrected to a useable signal, and sent to the

speed bias motor, which adjusts speeder spring tension.  The speeder spring then

repositions the pilot valve to correct blade angle and RPM.

-The synchrophaser can adjust propeller speeds in relation to a master RPM and #1 blade

position for all slave propellers.  This is done by pulse generators that supply signals that

correspond to the position of the #1 blade on each prop.  The master trim box

potentiometers use these signals to adjust blade position in order to reduce noise and

vibration.

Re-Indexing (on ground)- All throttles above 8,000 in lbs.  This will ensure that all propellers are in

the flight range being governed by speeder springs and fly weights.

Re-Synching Switch- When the propeller Resynchrophaser Switch is held in the RESYNC position,

two circuits are completed in each slave propeller.  The brake and clutch are powered, and

the synchrophaser receives a signal which causes it to ignore all pulse inputs.

Re-Indexing vs. Re-Synching:  The difference between re-synch and re-index is that re-synch only 	centers the speed bias motor while re-indexing centers both the motor and the lever arm.

Tach. Generator Failure (continues to supply a diminished rpm signal)- A low rpm signal will cause 	the synchrophaser to reduce the blade angle in order to increase rpm.  This lightens the load on the torque shaft and causes the indicated torque to decrease.  The synchrophaser can change the blade angle only by a small amount, but this reduced torque condition will continue until corrective action is taken.  Go Mechanical Govern to solve this problem.

Failure of Low Pitch Stop to Retract:

1.  Bad propeller seals which slow the build-up of pressure.

2.  Failure of the low pressure relief valve.

3.  Failure of the back-up valve.

Propeller Oil Leaks- With the propeller is a static condition or running, the maximum acceptable

oil leakage from the propeller control shall be 1 drop every 45 seconds (4 oz. in 24 hrs.)

((Prop leak limits are found in T.O. 1C-130B-2-11)

Prop Low Oil Light- 2 qts. low in pressurized sump.

Prop Failure To Feather- A coupled prop to the power section while windmilling produces  6 times

more drag than a decoupled prop.

-When slowing to 150 KTAS, if you:	Can maintain 96% RPM 	 blade angle low

Can’t maintain 96% RPM 	 blade angle high

NOTE:	A low or flat blade angle at airspeeds between 150 and 180 KTAS with a decoupled

propeller and reduction gearbox may produce overspeeds as high as approximately

140 to 180% RPM.

ELECTRICAL SYSTEM I   (510)

External AC Ready Light-	You have DC power control and AC power is in the correct phase

rotation.

External AC Power Switch-	It will remain in the AC EXT PWR position until one of the following

“Rattlesnake Switch”	conditions occur:

1. An operating engine driven generator is place on line

2. The ATM/APU Gen. Control Switch is placed to the ON position

3. External AC power is removed from the aircraft

Frequency Sensing Relay-  Output frequency drops below » 368 cycles per second.

Getting Power at Start-up With External Power:

1.  DC Power Switch- BATTERY  	Þ  all bus off lights- ON

2.  DC Bus Tie Switch- TIED		Þ  DC bus off lights- OUT

3.  External Power- Ext. AC/DC	Þ  all lights- OUT

(check Ext. volts & freq. 1st)

Bus Transfer Rules:

1.  Wing-Wing

2.  Symmetrical-Symmetrical

3.  1 Gen- Essential & Main

4.  APU- Essential only

Differential Current Relay-  A current differential of  » 30 amps on any one phase of the generator 	will energize the differential current relay.  This completes a circuit to the generator 	control relay and removes excitation from the generator.

Field Trip- Completes a DC circuit which energizes a relay, opening the generator exciter field,

reducing the generator output to zero.

Reset- Completes the circuit through the generator control relay to the generator exciter field.

This excites the generator field in the event residual voltage is insufficient for excitation.

Co-Pilot's AC Instrument Bus:	115v/400 Hz  3f   (Note: No automatic changeover)

Powers the pilot’s and copilots ADI’s

AC Instrument & Engine Fuel Control Bus:  115v/400 Hz  1f

-DC back-up:  1,500 v-a, 115v AC inverter from the Essential DC bus.

(soon to be replaced by a 2,500 v-a AC inverter)

-Auto switchover Þ DC to AC only

ELECTRICAL SYSTEMS II   (512)

Battery-	Hot wired to the Battery Bus

Min. volts for use: 21v

Min. volts to close the Battery Relay: 18v

Min. volts batter will accept charge: 12v

Normal voltage: 24 v

Ext. DC Light-  Correct polarity

Ext. DC-  Powers Main, Essential, and Isolated Buses

TR Unit:	115v/ 3f AC Þ 28 v DC  (rated at 200 amps)

Protected by bleeder resistors connected between the positive output terminal and

ground.  Circuit breakers for the bleeder resistors are located near the TR units on the

electrical control and supply rack.

Reverse Current Relay:

-The APP terminal allows control of reverse current flow through the RCR.

-The reverse current coil has little effect on the differential and bias coils unless the

voltage of the battery terminal exceeds the voltage of the generator terminal by approx.

1 volt.  This will cause the removal of the TR from the bus.

Reverse Current Relay Ratings:

Between TR units and Buses:	200 amps

Between Main & Essential DC:	600 amps

Between Essential & Isolated DC:	300 amps

Isolated DC on Battery Light-	Illuminate when DC power switch is in the BAT or EXT. DC PWR

position.  Also indicates failure of the RCR between Essential and

Isolated buses.  Follow these steps for an RCR failure:

1.  Conserve battery power

2.  Land as soon as possible

3.  Check out Battery Bus Þ voltmeter should read 24 volts or less

NOTE:	The relay for the ISO DC ON BAT indicator light receives power from the

battery bus through the indicator terminal on the reverse current relay

located between the isolated and essential DC buses.

Electrical Malfunctions:

Generator Out Light-  any phase below 70 volts

Bus Off light-  average phase below 90 volts

Generator Out Light:

1) Freq/Volt/Load normal:	Possible power indicator relay or TR failure. Leave running, a

failed TR will not allow a reset of the generator.

2) Freq/Volt normal:	Possible "K" relay malfunction

No load		Gen. Switch-OFF (monitor Volt/Freq)

3) Freq/Volt/Load zero:	Gen. Switch- RESET, OFF

a) If freq & voltage normal- resume normal ops

b) If freq & voltage is not indicated on all three phases or voltage 				 peaks then returns to zero- Gen. Disconnect

AC Bus Off Light:

1)  Affected generator- OFF

2)  Check Frequency & VoltageÞ	a) Normal (and bus off light remains ON)  with no item on

that bus failed- resume normal operations.

b) Abnormal- Leave generator OFF- monitor it

3) If other generator does not pick up the load- the bus is lost

Bus Isolation/Fire:

1.	If source is found- isolate by turning off/pulling the circuit breakers, or by removing the

electrical plugs.

2.  If unable to locate source:

a.  APU and APU Generator- ON

b.  All Generator switches- OFF

(You now have Ess. AC, ESS. DC, Isol. DC and Battery only)

If fire persists:

c.  Co-Pilot's inverter- DC Bus

d.  Oil Cooler Flap switches- OPEN/FIXED

e.  APU and Generator- OFF, STOP

3.  If APU is inoperative and you have an electrical fire:

a.  Co-Pilot's Inverter- DC Bus

b.  Oil Cooler Flap Switches- OPEN/FIXED

c.  All engine generators- OFF

d.  Remove 3 MAIN AC Bus current limiters at Station 245

e.  One engine generator switch- ON  (Essential AC Bus will be powered now)

f.  If fire persists: Generator switch- OFF

Main DC Bus Isolation- Pull 6 Main TR circuit breakers and the DC Main Bus Ground Control

circuit breaker.

Ess DC Bus Isolation- First pull the 6 Main DC TR circuit breakers, then pull the 6 Essential DC TR

circuit breakers along with the two ground control circuit breakers.

NOTE:  When pulling the TR unit circuit breakers, pull all three circuit breakers in A-B-C

order for one TR, then pull all three circuit breakers in A-B-C order for the other

TR unit.

Touchdown Circuits:

Touchdown Switch:	-Engine condition lever GROUND STOP (inop in flight)

-Dump mast shutoff valves (closed on the ground, open in flight)

Touchdown Relay:	-SCNS

(left MLG)		-Airdrop Release (inop on the ground)

-Cockpit controls for ramp and cargo door (inop on the ground)

-Wheel brakes/anti-skid (brakes inop in flight)

-Landing gear control handle lock (unlock in flight)

Aux Touchdown Relay:  -DC Bus Tie Control switch (inop in flight)

(right MLG)

HYDRAULIC SYSTEMS I   (514)

Utility Hydraulics:	Total Capacity- 12.5 gal

Reservoir-  3.2 gal

Booster Hydraulics:	Total Capacity-  9.3 gal

Reservoir-  2.0 gal

Auxiliary Hydraulics:	Total Capacity- 6.6 gal

Reservoir-  3.4 gal

Suction Boost Pump:	 3f, 400 cps, AC motor (Ess AC)

-11 amp thermal breaker (self resetting if heat dissipates)

-Up to 20 gpm flow

-Pressure:  70-110 psi

-Warning Light:  20 psi  (out at 30 psi)

-Failure will result in a 100 to 200 psi drop in system pressure

Engine Driven Pumps:	Flow-  8.6 to 9.6 gpm  (depending on type of pump)

Pressure: regulated to 3,000 psi

Restrictor:	This bypass circuit cools the suction boost pump when there is no demand for

hydraulic fluid.  The restrictor limits flow to 1 gpm of fluid through the bypass.

Run-around Circuit:  » 1 gal  (low pressure relief valve opens at 20 psi)

24 Hr Check Valve- Keeps hydraulic fluid in the wings from draining into the reservoir.

High Pressure Relief Valve:

Starts to open-  3450 psi

Full open-  3850 psi  (allows fluid flow of 16 gpm)

Fully closed-  3100 psi

Differences between Booster and Utility systems:

1.  Capacity:   Booster- 9.3/2.0    Utility- 12.5/3.2

2.  Booster:  No check valve to hold pressure on emergency flap brake

3.  No ground test valve on the Booster system

4.  Only one return line on the Booster system (2 on the utility system)

Hydraulic Pressures:

Utility & Booster:   2,900-3,200 psi

Low Speed GI:   2,550 psi minimum

NOTE:	Normal pressure is based on the engine at 100% RPM, all flight controls neutral,

suction boost pump on, and no system actuation.

Run-around Circuit Valves-	Consist of the motor driven firewall shutoff valve and the high pressure

solenoid shutoff valve. Both valves are powered by Essential DC and are

shut off by the Engine Driven Hydraulic Pump Switches and the Fire

Handles.  The warning light is wired into the pump switches and will

not illuminate if the fire handle is pulled.

CAUTION:	Engine pump switches activate the firewall valves and pump shutoff valves. They DO

NOT shut off the engine driven pumps.  The engine pump switches are to be left in the

ON position after engine shutdown.  If the switch is left in the OFF position, pressure

build-up due to thermal expansion of the hydraulic fluid may cause the suction line

motor driven hydraulic firewall shutoff valve to fail.

Hydraulic Low Pressure Warning Light:  1,000 psi  (range: 900-1,100 psi) or turning off and engine

driven pump switch.  It is not wired to the fire handles though and the light will only come

on when the pressure trapped between the solenoid valve and isolated valve is bled off.  Light

turns off between 1,300 and 1,400 psi.

Suction Boost Pump:	» 100 psi rise when turned on.

Warning Light:  20 psi  (goes out at 30 psi)

CAUTION:	Starting an engine with an inoperative suction boost pump may result in

damage to the engine driven hydraulic pump.

Hydraulic Accumulators (4):	Þ  3 at 1,500 ± 100 psi  and Emergency Brake is 1,000 ± 100 psi

Þ  3 at 50 cu-in and Normal Brakes is 100 cu-in

Accumulator Functions:

1. When the system demand is greater than output capacity, the accumulator will help

maintain system pressure.

2. When a pressure surge is felt in the system, the compressed air provides a cushioning

effect to dampen out the surge.

Hydraulic Gauges (power sources)

Utility System	  #2 Instrument Transformer  (Pilots Lower)

Booster System	  #1 Instrument Transformer  (Pilots Lower)

Malfunctions:

LOSS OF SYSTEM PRESSURE:	2 Engine Driven Pump Switches- OFF

Suction Boost Pump Switch- OFF

NOTE:	If loss of utility fluid, one place to check is the ground test valve by looking

through the left main wheel well view ports.

Failed Engine Driven Pump internal pressure regulator:	Pressure between 3,200-3,900 psi

Keep Engine Running!

Failed High Pressure relief valve:	Pressure is greater than 3,900 psi

Shut Down Engine (Cruise Engine Shutdown).

CAUTION:	Do Not Isolate (using the Engine Driven Pump Switches) and engine that

has a failed internal pressure regulator or a high pressure relief valve.  If

the hydraulic pump shutoff valves are closed, the pressure line between the

hydraulic pump and the shutoff valve is isolated from its means of pressure

control.  Excess pressure may build up until the hydraulic pump or line

ruptures, dumping hydraulic fluid into the engine nacelle.

Hydraulic Low Pressure Warning Light:

1.  Shut off engine pump switch- affected engine

2.  Check reservoir

a.  Normal- No other action is necessary

b.  Decreasing- LOSS OF SYSTEM PRESSURE

NOTE:	It is normal for a large hydraulic pressure drop while actuating landing gear

and flaps during single utility pump operation.

Suction Boost Pump Light:  20 psi (usually accompanied by a 100 to 200 psi pressure loss)

1.  Suction Boost Pump Switch- OFF

2.  Check pressure and reservoir

a.  Pressure is static  and greater than 2,500 psi- Continue

b.  Fluid is decreasing or pressure is less than 2,500 psi- LOSS OF SYSTEM

PRESSURE

Isolation Check Valve Failure:  Both lights extinguish when you start 1 engine.   MX  may ask

that you motor the other engine to back-flush and reset the check valve.  First turn off

the operating engine's engine driven hyd. pump and drain off the pressure before you

motor the other engine.

CAUTION: This may cause debris to be flushed down the line.

Loss of the Utility System will result in loss of the following:

-Normal landing gear extension and retraction

-Normal flap operation

-Operation of the normal brakes and anti-skid

-Nosewheel steering

-One half of the boost to the flight controls

HYDRAULIC SYSTEMS II   (516)

Auxiliary Pump:	-115/200v 3 AC motor (Essential AC)

-No warning lights

-Pressure- Normal: 2,900-3,300 psi

Max:  3,500 psi

-Operates:	Cargo ramp and door

Emergency extension of the nose gear

Emergency brakes

Utility hydraulics by use of the ground test valve

Handpump Pressure:	It can be read at the gage located at the ramp and door panel or on the

emergency brake pressure gage on the Hydraulic Control Panel.

ADS Arms- Connected throughout flight

ADS "OPEN" Light

1.  Left ADS arm microswitch is actuated

2.  Door is in the uplock

Ramp And Door Operations:

NOTE: 	If the aft anchor cable support arms are not in the stowed position, the door will

hit them during manual operations of the cargo door.  During electrical operations

of the cargo door, contacts will prevent ramp and door from operating electrically

if the aft anchor cable support arms are not stowed.

Manual Operations (handpump):

Opening:	1. Move the door control lever to the open position and open door

2. Ramp selector valve to #1 to unlock ramp latches (3000 psi)

3. Ramp selector valve to #2 to lower ramp

4. Ramp selector valve to #3  NEUTRAL

Closing:	1. Ramp selector valve to #4 to raise ramp

2. Ramp selector valve to #5 to engage locks (3000 psi)

3. Ramp selector valve to #6  NEUTRAL

4. Move the door control lever to the close position and close door

NOTE: Ramp Selector valve- Rotate clockwise only.

Electrical Operations:	Use the door and ramp switches on the Ramp Control Panel.  Make

sure that the ramp selector valve is in the 6N (NEUTRAL) position.

Ramp-	3,000 psi is necessary to engage or disengage locks.  There are 5 locks on each side of

ramp.  In flight, use 4100 psi to close and lock ramp manually.  If ramp can not be locked

in flight, the loadmaster will turn the manual control knob to the #4 position and turn the

aux pump on.

HYDRAULIC SYSTEMS III   (518)

Flap Movement:	0-15% 	  Þ Lift is increased

15-50%  Þ Lift and drag is increased

50-100%Þ Drag is drastically increased

Flap Extension:	Fully extended- 35° flap angle

Extension time-  8-15 sec

Retraction time-  10-15 sec

Flap Surge Damper:	Bring flap retraction, the surge damper prevents overspeeding caused by flight

air loads.  The damper has no function during flap extension.

NOTE:	The landing gear warning horn is interconnected with the flap system.  When the flap lever

is set at » 70% or more with the landing gear up, the landing gear warning horn will sound.

It cannot be silenced until the landing gear is down and locked or the flap lever is

retracted above 70%.

Flap Motor:  Utility hydraulic pressure drives the motor which positions the flaps.

Flap Brake:	Spring loaded, disc type flap brake is released with hydraulic pressure.  When applied,

the brake prevents movement of the flaps by aerodynamic loads.

Flap Control Valve:  Main DC, Co-Pilot's lower CB panel (powers the emergency flap brake valve)

Asymmetrical Sensing Switches:  Located at the ends of the torque shaft which drives the flap

segments.  Should the torque shaft malfunction, the timing of these switches will be

disrupted and the Emergency Flap Brake will engage.

Emergency Flap Brake:  Hydraulically applied and spring released.

WARNING:	Do not release the manual override on the emergency flap brake valve while

the airplane is in flight.  This manual override is for ground use only.

Flap Position Indicator:  Transmitter is mounted on the flap drive control unit located on the aft

face of the wing rear beam.  It is powered by 28v Main DC from the Tabs and Flaps Position

Indicators CB on the Aft Fuselage Junction box.

Flap Malfunctions:

WARNING:	If an outboard flap fails, it is possible that contact between the flap and aileron will

result in binding, and restriction of movement of the aileron.  Under these conditions,

if it is possible to control the airplane, NO attempt should be made to move the flaps.

If movement of the flaps must be attempted, return them toward the position last

selected before failure.  During flap movement, check aileron control constantly.  If

aileron control is freed, or if it is noted that binding increases, stop flap movement

immediately.

NOTE:  Anytime flap rubbing is suspected, do a Controllability Check.

WARNING:	Protection against asymmetrical operation is provided only during normal hydraulic

flap operation.  (Not override or manual ops)

If flaps fail to operate normally, alternate methods are (in order of preference):

1. Override the flap selector valve located on the left hydraulic panel

2. Mechanical operation of the flaps

Electrical Flap Failure:

1.  Pull Wing Flap CB

2.  Place flap lever in the desired position (This assures proper rudder boost)

3.  Remove Utility Hydraulic panel cover

4.  Lower or raise flaps to desired position

(Technique:  Have flap operator depress override button for 1 sec. at a time » 10%)

Loss Of Hydraulic Pressure:

1.  Pull Wing Flap CB

2.  Place flap lever in the desired position (This assures proper rudder boost)

3.  Turn off #1 & #2 engine driven hydraulic pumps and Utility suction boost pump.

Deplete Utility hydraulic pressure.

4.  Engage handcrank on input shaft

5.  Hold handcrank firmly to prevent rotation

6.  Remove input shaft pin

7.  Engage the manual extension system (rotate counterclockwise to its stop and pull out

 2 inches)

8.  Move the flaps to the desired position  (Full extension » 650 turns)

9.  Replace input shaft pin

10. Remove crank

11. Leave manual shift handle out

12. Return pressure to the utility hydraulic system (if able)

NOTE:	A slipclutch is provided in the manual gearbox to prevent the operator from

overloading the drive system.

Asymmetrical Flap brake-  If flap movement stops before the flaps have reached the desired

position, failure of the flaps to move in either direction may be due to engagement of the

emergency flap brake.  The flap handle should be positioned to correspond to the position

of the flaps, and no further in-flight movement of the flaps should be attempted.

Flight Control Boost Packs:  There are check valves within the boost packs that will allow fluid

from the return line back into the supply line when there is no system pressure.  This

serves two purposes: it provides a dampening effect on the flight controls when the

airplane is static and it also allows fluid transfer when one side of the boost pack is OFF.

Otherwise, the side not working would restrict the operation of the side with pressure.

Rudder Boost- Loss of electrical power will cause the diverter valves to de-energize to the low side

and route fluid through the pressure reducer.  (28v Essential DC)

Pressure:	1,100 to 1,400 psi  Flap lever set 0-15%

2,900 to 3,200 psi  Flap lever set 15-100%

Aileron Pressure Reducer-  Reduces aileron booster pressure from 3,000 psi to 2,050 psi.  This

reduces the roll rate of  aircraft with external fuel tanks.

WARNING:	If a control booster assembly is suspected of having failed in a hardover position, verify

that the cockpit control matches the hardover maneuver being experienced and then

place the respective control boost switches to OFF.  Expect that greatly increased

forces will be required to move the control for which the hydraulic assistance have

been turned off.  Turning the Elevator Control Boost switches OFF will alleviate the

hardover situation, leaving the airplane out of trim in the nose down direction.

Immediate correction of trim will be required after the control boost switches are

placed to the OFF position. (Booster shutoff valves are 28v Ess DC/Copilots Lower)

Flap Position Indicator Failure:  Check Tabs And Flaps Position Indicator circuit breaker located

on the Aft Fuselage Junction Box  (Main DC)

Landing With Complete Loss Of Hydraulic Assistance For A Flight Control:

1.  reduce weight and speed of the airplane as much as possible

2.  Make a long, flat approach to reduce the amount of flare necessary and fly the airplane

onto the ground.

3.  Use normal trim and engine power to reduce control forces

4.  Land with 0-50% flaps  (50% flaps recommended)

Controllability Check:

1.  Conduct a preliminary check for airplane damage and personnel injuries

2.  Attempt to climb to 10,000 ft AGL

3.  Consider dumping fuel to lighten load

4.  Complete the Descent Checklist

5.  Configure the airplane for landing

6.  Gradually slow the airplane in 5 kt increments while evaluating the control capabilities

in turns and simulated landing approaches.  Any impending control problems are

indicated by an excessive rolling, yawing, or pitching moment.  The airspeed is

decreased until the desired landing speed is attained or an undesirable control problem

is approached.

7.  If stall buffet occurs, immediately accelerate the airplane to a safe flying speed.  If a

stall buffet occurs, plan touchdown speed to be 1.2 times speed at buffet for the

selected flap setting.  The landing flare requirement can be decreased by making a low,

flat landing approach.

WARNING:	Do not slow to a point where full flight control deflection is necessary to control

the aircraft

HYDRAULIC SYSTEMS  IV   (520)

The main gear doors are connected by the linkage to each aft main strut.

Landing Gear Control Selector Valve- Essential DC

Extension Time:  Main landing gear- 19 sec. or less

Restrictor Valve- The struts strike this valve as it approaches the UP position, at which time the

restrictors slow the flow of hydraulic fluid and slow gear retraction the remaining distance.

NOTE:	When the gear is fully extended, the landing gear selector valve remains in the down

position and is held there by spring detents.  This provides a source of hydraulic pressure

for nose wheel 	steering.  Hydraulic pressure is not required to hold down the main gear.

When the weight is on the gear, a friction washer tightens on the bottom of the jack screw,

preventing rotation and inadvertent gear retraction on the ground.

Landing Gear Break Locks- Spring applied and hydraulically released

Brake Pressure Selector Valve-  De-energized open when normal brakes are selected (Emergency-

energized closed by Ess DC).  When the landing gear is in the up position, the normal brake

selector is energized closed by the touchdown relay through the landing gear control CB on

the Ess. DC bus.  When emergency brakes are selected, the normal brake selector valve is

energized closed (Main DC through the Anti-Skid circuit breaker) and the emergency brake

selector valve is de-energized open.

Brake Pressure Reducer:  Reduces pressure from 3,000 psi to 2,030 psi.

Parking Brake Pressure:  » 1,400 psi

Parking Brake Handle-  There is a switch in the linkage of the parking brake handle that will turn 	the anti-skid system off when the parking brake handle is pulled.  This in turn illuminates 	the anti-skid inop. light.

Min. speed for anti-skid operations:  15 kts.

Shuttle Valve- Purpose of the valve is to separate the hydraulic systems from each other.  In the

event of an electrical power failure, allowing both brake selector valves to open, the

system with the highest pressure would be directed to the brake assemblies by the shuttle

valve.

Hydraulic Fuses-  Hydraulic fuses are installed in the emergency brake lines of each brake, up

stream from the shuttle valves.  If a hydraulic line fails, this fuse will stop fluid flow and

prevent complete loss	of the system.

Nose Wheel Steering-  When the steering control valve is in the neutral position, the shimmy

dampening orifices in the steering actuators provide snubbing action to dampen

oscillations of the nose wheel, thus preventing nose wheel shimmy.  Torque arms transmit

steering action to the strut piston.

Centering Cam- Centers the nose wheel after it leaves the ground.

Nose Wheel Turn Limit:	60° (20° when weight exceeds 155,000 lbs or taxi speed is 20 kts or

higher)

If any landing gear fails to extend after normal operations of the landing gear lever, check:

1.  Circuit breakers (2) (Co-pilot's Ess. DC)

2.  Utility hydraulic pressure

3.  Utility hydraulic fluid

Alternate Gear Extension Methods (in order of preference):

1. Override the landing gear selector valve

2. Manual gear extension of main gear

3. Emergency hydraulic extension of the nose gear

4. Nose landing gear manual extension after complete loss of hydraulic pressure

5. Main gear extension (free fall) after normal and manual extension methods fail

NOTE:	When all other normal and emergency procedures fail to extend the main landing

gear, and it does not free fall, lower the gear by rotating the jackscrews.  Rotate

aft strut first (due to gear doors being attached to the aft strut).

CAUTION:	If a hydraulic leak is the cause of a malfunction, or if hydraulic pressure was lost after

repositioning the landing gear lever, IMMEDIATELY place/check the landing gear lever

in the UP position.  Then pull the landing gear control CB to center the landing gear

selector valve.  If the gear handle is left in the down position, the selector valve will

not move when the Landing Gear circuit breaker is pulled due to the down detent.

NOTE:	If you have to hold the manual override button in when overriding the main landing gear control

valve, it means the control valve might not be in the down detent.  If the gear continues to extend

when the button is released, you will still have downside hydraulics and nose wheel steering.

CAUTION:	With complete utility hydraulic system failure, do not use nose wheel steering on

landing roll.  Inadvertent use of nose wheel steering may allow hydraulic fluid to be

bled from the shimmy dampener and allow the wheel to shimmy or turn from center.

Main gear handcrank:  » 330 turns

NOTE: To aid in manual extension of the main landing gear, depressurize the aircraft.

Main gear position check:  The main gear should be down and locked if the 4 hex nut bolts are aligned

with the 4 half moon cutouts in the inspection window.

Nose Gear Free Fall-  This method does not ensure that the gear will move to the down & locked position.

Slow to 120 KIAS or below, then after the handle is pulled, accelerate as rapidly as possible but

not above 165 KIAS.  It may take 30 to 45 seconds to extend the nose gear.  On landing, keep the

nose wheel off the ground as long as possible, then smoothly lower it to the runway.  DO NOT

TAXI!  Do not enter the nose wheel well with an unsafe gear

indication prior to supporting the nose of the airplane.

CAUTION:	If a malfunction is encountered in lowering the landing gear, once the landing

gear is down and locked, it will not be removed from this position.  If one or

more landing gear will not retract, do not attempt to obtain an up and locked

condition by recycling the gear.  Extend the gear and attempt to obtain a down

and locked condition.  Visually confirm all the gear is down and locked and land

as soon as practical.

Nose Gear Tire Failure:

1 Tire- Normal landing

2 Tires- Keep nose wheel off ground as long as possible.  After nose gear contact, use max

reverse and min. nose wheel loading.  Taxiing is not recommended.

Main Gear Tire Failure-  If a MLG tire is flat, touch down the nose gear as soon as possible and use 	max reverse thrust.  Taxiing is not recommended.  If both tires are flat, line up on the side 	of the runway corresponding to the good tires.  Use wheel brakes of good tires only.

Unsafe Nose Gear Indication- Look through the inspection window to ensure that the band of

fluorescent paint on the actuator is visible, verifying the gear is down and locked.

Landing With Nose Gear Up:

1. Give warning over PA and give 6 short bells

2. Secure all cargo.  If cargo can be moved, shift it into a maximum allowable aft center of

gravity location.

3. Depressurize the aircraft and close all engine bleed valves

4. Open and secure the forward, center, and aft escape hatches and the paratroop doors.

Turn off all unnecessary electrical equipment.

5. Pull the landing gear warning light circuit breaker

6. Don helmets and gloves; lock shoulder harnesses

7. Request foam on the runway (3000’ by 30’)

8. Make a normal approach and assume a normal landing attitude

9. Give 1 long ring and a PA announcement prior to landing

10. After landing, keep aircraft as level as possible.  After nose contact, use reverse thrust

but do not let the nose rise off the ground.  Do not use wheel brakes.

Landing With All Gear Up:  Same as landing with nose gear up except consider:

1. Jettison of cargo

2. Dump or consume all unnecessary fuel.

Use Of Tiedowns:  Tiedowns may be used if:

1. A main landing gear shelf bracket is broken (preventing drag pin engagement)

2. A main landing gear has not completely extended (preventing drag pin engagement)

3. Crew determines the existence of an unsafe condition

Immobile Nose Wheel steering-  Do Not turn the nose wheel or try to force it.

Anti-Skid Failure- If one wheel tests unsatisfactory on the anti-skid test, you should turn the

anti-skid system OFF to prevent uneven braking and a tendency to swerve (and even blow a

tire).

Nose Wheel Shimmy-  Indicates an imbalance in one or both of the nose wheel tires or a failure of 	the Nose Wheel Steering System Þ

If takeoff is continued- raise nose wheel as soon as directional control permits.

If aborting or landing- use up elevator and max reverse.

PNEUMATIC SYSTEMS   (522)

Engine Bleed Air-  158 ppm/ 125 psi/ 635° F  (@ sea level and standard day)

Engine Diffuser Section-	The power section component that feeds bleed air to the bleed air

manifold.

Divider Valve-  Separates the flight deck and cargo compartment air conditioners.

Duct Compensator-	Provides slip joint (bellows) to allow the ducts to expand and contract while

maintaining an airtight seal.

APU Bleed Air Valve-	Will only open after APU reaches 95% rpm.  Should not be opened till after a

1 minute warm-up.  (4 minutes for Cold WX ops.)

Engine Bleed Air Valves:	ON- Pressure is regulated to 45 psi in the manifold.

OVRD (Override)- Valve is full open.

"A" Solenoid Failure:	Valve will go to the closed position and can not be opened.

"B" Solenoid Failure:	Valve will be in the regulating position (regulating to 45 psi).

Reduce manifold pressure below 45 psi  to open the engine bleed air

valve (use APU bleed air for start- min of 35 psi vs engine bleed air of

70 psi).

Bleed Air Pressure Gage-	Any time the Flight Station air conditioner is on, the gage will read 6 psi

lower than the actual pressure in the main bleed air manifold.

APU: Min. bleed air pressure- 35 psi

NOTE:	Failure to attain this minimum pressure may be an indication of an open valve, a

leaking duct, or low output from the APU.

Engine:  Min bleed air pressure- 70 psi.

Bleed Air System Check:	APUÞ 30 to 15 psi in no less than 8.5 seconds.

EngineÞ 65 to 35 psi in no less than 10 seconds.

NOTE:	If the pressure had not dropped when the bleed air valve closed, it would have

indicated that the bleed air valve was stuck in the ON/OVRD position.  If the

pressure had dropped too rapidly, it would have indicated a possible duct

failure/malfunction of another valve.

The Engine Inlet Air Duct Anti-Icing System-  Provides protection for the engine air inlet scoop,

the oil cooler inlet scoop, the inlet guide vanes, and the extension shaft.  If electrical

power is lost, the inlet guide vanes, inlet housing, and the extension shaft are

automatically anti-iced.  The engine inlet guide vanes and extension shaft can only be

anti-iced if the engine is operating.  (28v Ess DC)

There are 6 valves for the wing and empennage anti-icing system.  4 for the wing and 2 for the

empennage.  These valves are electrically controlled (28v Ess DC) and pneumatically

operated.

Piccolo Tube- Bleed air temp (600° F) is mixed with ambient air bringing the temperature down to

350° F and is then channeled evenly throughout the leading edge surfaces.  Pressure in the

piccolo tube is » 125 psi.  After passing through the venturi, this pressure is exhausted into

the engine dry bay and then exits through ducting at the rear of the compartment.

Anti-Icing Temp Control-	Keeps the temperature of the wing and empennage anti-icing between

158° F and 180° F.

Air Conditioner Water Separator-  Removes 70 to 85% of the moisture.

Water Separator Anti-Icing Control Box- It prevents the water separators from icing by maintaining water

separator exit temperature at 37 (±3)°  F.

Aux Vent- Closes the airflow valve and allows ram air into the air conditioning ducts.  A diverter

valve in the Flight Station A/C system prevents ram air from entering the defogging vents.

This is the position of the AC Master selector switch when an external conditioned air

source is used.

Manual Temperature Control:		Hot to Cold- 35 seconds

Cold to Hot- 4 minutes

Cooling Turbine Fan-	Moves air through the ram air inlet when the aircraft is on the ground,

ensuring proper operation of the heat exchanger.

Underfloor Heating-	Will not work when A/C Master switch is in the AUX VENT or OFF position.  It

maintains a temperature of » 80° F.  Whenever the underfloor heat is turned

on, an overhead duct auxiliary fan operates, which ensures proper circulation

of air entering the cargo compartment.  A underfloor heating fan switch will

operate this fan independently of the underfloor heating as long as underfloor

heating is turned off.

Flight Station Airflow Switch (with both air conditioner systems operating):

MIN (Diverter valve full open):	Flight Station- 30%	Cargo Compartment- 70%

NORMAL (Diverter valve partially open):	Flight Station- 60%	Cargo Compartment- 40%

INTMED (Diverter valve partially open):	Flight Station- 80%	Cargo Compartment- 20%

MAX (Diverter valve closed):	Flight Station- 100%	Cargo Compartment- 0

NOTE:	This FLT STA Airflow Switch may be used to provide some airflow from the cargo

compartment system to the flight station when the flight station system is not

operating.  The Flight station airflow switch should be positioned to MIN to obtain

maximum flow to the flight station for this condition.

Pressurization Limit:  15.8 in Hg  (structural limit)

Safety Valve:  -.76 in Hg to 15.9 in Hg

Pressure capable with 15.8 in Hg differential pressure:

Cabin Altitude	Aircraft Altitude

-1000 ft	16,500 MSL

Sea Level	18,500 MSL

1000 ft	20,000 MSL

5000 ft	28,000 MSL

8000 ft	35,000 MSL

10,000 ft	41,000 MSL

Cabin Pressure Controller  (Controls outflow valve):

1.  Maintains constant cabin pressure

2.  Keeps the airframe within differential pressure limits

3.  Controls rate of change during climb and descent

Rate Knob:  Rate of cabin altitude change

Min-  30 to 200 fpm

Max-  1,600 to 2,900 fpm

NOTE:	Ideally, the rate should be set so the aircraft altitude and the desired cabin altitude

are reached at the same time.

Emergency Depressurization Switch:

1.  Outflow and Safety valves open

2.  Both air conditioning flow regulator valves close

Depressurizing (using the cabin pressure controller)

£ 10,000 ft:  Set the pressure controller to the same altitude as indicated on the altimeter

> 10,000 ft:   1. Master Switch- A/C Man Press. 2.  Manual Pressure Control Switch- Decrease

Once depressurized Þ Go A/C No Press

NOTE: In both cases, you are depressurized when the differential pressure gage reads zero.

APU Fire Light (with APU not running)- Might indicate a bleed air leak in the area of the left wing 	root.

Primary Bleed Air Leak Indicators:	1.  Reduced bleed air pressure

2.  Loss of engine torque

If bleed air is being lost from the system:

1.  Engine Bleed Air Valve Switches (on the affected wing)-  CLOSED

CAUTION:	If an engine bleed air valve cannot be closed (valve closure is

determined by observing torque increase on the affected engine) and

the bleed air system is leaking, it may be necessary to shut down the

engine.

Þ If closing the engine bleed air valves does not contain the problem

2.  Bleed Air Divider Valve Switch-  CLOSED

Þ If the uncontrollable loss of bleed air cannot be isolated

3.  All Engine Bleed Air Valve Switches-  CLOSED

Failure Of Outflow Valve (overpressurization)-  Close Engine Bleed Air Valves until differential 	pressure is within limits.

Window Pane Cracks:	Flight Deck (1 or both panes):  10 in Hg

Cargo Compartment-	1 pane:	10 in Hg

2 panes:	0 in Hg

MISCELLANEOUS SYSTEMS   (524)

Turbine Overheat Warning System-  Consists of thermal switch detectors, keyer units and indicator

lights.  4 thermal switches mounted in the hot section of the nacelle, aft of the firewall.  One

detector can fail and the rest should provide overheat warning (wired in parallel). (Essential DC)

Turbine Thermal Detectors:  Set to » 700° F  (371° C)

Nacelle Overheat Warning System- Consists of thermal detectors and warning panel/indicator

lights.  6 thermal detectors connected in parallel.  One detector can fail and the rest

should provide overheat warning.  (Essential DC)

Nacelle Thermal Detectors:  Set to  300 F (149 C)

Fire Warning System- Consists of a sensing element (Inconel tubing) and an amplifier (control) unit

There is 55 ft of Inconel tubing in each engine nacelle and 15 ft in the APU compartment.

There will be no effect on the fire detection system unless there are multiple breaks in the

Inconel tubing.  The system operates by means of a control unit which detects changes in

the resistance of the Inconel tubing.

Fire Detection:  Essential DC (memory aid: It is ESSENTIAL to DETECT)

Fire Protection:  Battery Bus  (memory aid: It takes BATTERY to ASSAULT (extinguish) the fire)

Fire Extinguisher Bottle- Holds 19 lbs of extinguishing agent with nitrogen under 600 psi of 	pressure.  A serviceable bottle will have a pressure of 600 to 640 psi @ 70

NOTE:	Agent will not extinguish with the discharge switch activated if a fire handle has

not been pulled.  Directional control valve in the left aft wing root is directed to

the left wing unless the #3 or #4 engine fire handle is pulled (route set to the #2

engine initially).  When two fire handles are pulled, the agent is directed to the

engine for the last handle pulled.

Oxygen System:

Filler-Buildup Vent-	Provides a means of filling the lox converter and allows pressure build-

up.  When the lox cart servicing hose is connected to the filler-buildup

vent valve, the valve automatically moves to the FILL position.  When

in the BUILDUP position, the combination filler-buildup vent valve

allows gas to flow back into the converter and enables pressure to

increase.  the valve automatically moves to the BUILDUP position

when the servicing cart hose is disconnected.

Weight Limitations Chart:	Gives us how much fuel we can load for a given cargo weight (primary &

secondary fuel management).

NOTE:  Not a factor until cargo weight approaches 30,000 lbs.