KY-58 SECURE RADIO LOADING

LOADING:

1.  Radio (UHF/VHF) - “ON”

NOTE:  In order to hear the radio in the back, turn “PA” on and select the 	radios you are loading

2.  Control Head (Co-Pilot’s side shelf) - “C” and “ON”

3.  KY-58 (cargo compartment, VHF & HF, or under flight deck, UHF) - 	“C” and “ON” (should hear short beeping tone)

4.  Ensure KYK-13 is “OFF”

5.  Connect KYK-13 to KY-58

6.  Hold REM/LOC  Switch to LOC and Hold

7.  Push PTT, beeping should stop

8.  Release REM/LOC switch

9.  Place KY-58 Mode switch to “LD”, should hear a steady tone

10.  KYK-13 - “ON”

11.  KY-58 “Pull” switch to “1” and KYK-13 to “1”

12.  Push PTT, should hear one beep

13.  Repeat step 10 for all positions to be loaded

14.  KYK-13 - “OFF”

15.  Remove KYK-13 from KY-58

16.  Turn Mode switch on KY-58 to “C”

17.  Check filter settings

“P”		- Permits transmission of text in the clear

“C”	- Permits trans of ciphered text, can receive ciphered or plain text

“CO”	- Can transmit & receive only ciphered text (i.e.- you can’t hear anyone 			   unless they are keyed with your code, and they can’t hear you)

“LD”	- Permits loading

CIPHER MODE OPERATION:

1.  Contact receiving station to switch the KY-58 to “C”

2.  Select “C” on Co-Pilot’s side shelf remote control head

NOTE:  Ensure the remote control heads and KY-58’s are cleared, set to “P” and turned “OFF” after flight.