KY-58 SECURE RADIO LOADING

This guide assumes that you are familiar with the operation of the DTD.  Prior to starting this checklist, set up the DTD to send the secure radio code and proceed until the Send prompt before the “Press initiate on receiving station”

LOADING:

1.  Radio (UHF/VHF) - “ON” thru SCNS or manual control head

2   RCU  (Co-Pilot’s side shelf) - “C”, “ON”, and fill position as desired

NOTE:  Plug into IP cord and set intercom panel or PA to listen to radio

3.  KY-58 – “C” , ON” (short beeping tones), and fill position as desired

4.  Set  REM/LOC  Switch to LOC and release

5.  Push PTT, beeping should stop

6.  Ensure the DTD is ready to send the codes and connect to KY-58

7.  Press enter on DTD – should get message “Press initiate to begin transfer”

8.  Place KY-58 Mode switch to “LD”, should hear a steady tone

9.  Set  REM/LOC  Switch to LOC and release

10.  Push PTT, hear one beep, tone stops and DTD will say “1 key transferred”

11.  If more than one fill position is desired

a. Move fill position to new desired location

b. Hit “Retransmit” on DTD

c. Repeat steps 9 and 10 but you will hear no steady tone, only one beep

d. Repeat step 11 for subsequent fill positions

12.  Place mode switch to “C” and adjust volume on KY-58

13.  Remove DTD from KY-58

14.  Ensure RCU fill position set and set RCU to “P” for clear transmission

“P”		- Permits transmission of text in the clear

“C”	- Permits trans of ciphered text, can receive ciphered or plain text

“CO”	- Can transmit & receive only ciphered text (i.e.- you can’t hear anyone 			   unless they are keyed with your code, and they can’t hear you)

“LD”	- Permits loading

CIPHER MODE OPERATION:

1.  Contact receiving station to switch the KY-58 to “C”

2.  Select “C” on Co-Pilot’s side shelf remote control head

NOTE:  Ensure the remote control heads and KY-58’s are zeroized, set to “P” and turned “OFF” after flight.