MLS Self-Test (Automatic if power interrupted 3 seconds)

- Autopilot Disengaged

- Flight Director Switch to NORMAL

- Select MLS on FDMS

- Dial in Airplane Heading on HSI CSW

- Activate Self Test on IDCU

MLS Approach Guide

- MLS and TAC – ON

- Tune MLS – Desired channel

- Tune TAC – MLS TAC frequency if req’d (Warning 4-74) [Only if prior to SCNS OFP 40.07]

- MLS 1-2 Page – Verify AUTO mode selected (Default)

- MLS 1-2 Page – Select NON-COMP/COMP mode

- Select COMP for COMPUTED MLS approach

(Warning 4-74)

- Select NON-COMP for MLS approach (Default)

- MLS 2-2 Page- Verify 4 letter ident (Mxxx)

- MLS 2-2 Page- Verify OVRD not selected

- Select MLS on FDMS

- Select approach heading on HSI  CSW

- Verify – TAC not selected on either FDMS

- MLS 2-2 Page – Confirm TAC on MLS frequency

- Flight Director Switch – NORMAL

- Fly the Approach

- Intercept – MLS course

- Monitor – Ident on MLS 2-2 page

- Missed Approach – Retune TAC as required