LOADING IFF/SIF MODE IV

TERMS:

KIT-1C:  Gray box underneath the flight deck you plug the KYK-13 into

KOI-18:  Small green device you run the “strips” through to load KYK-13

KYK-13:  Small green device used to load codes into the aircraft

Register: Memory storage space in KYK-13 (also called “fill”)

LOADING THE KIT-1C:

1.  Load KYK-13 with MODE IV codes for the applicable zulu day (see IFG)

a.  Before loading the tapes, zero all fills

b.  Load A & B tapes under two separate fills

c.  After loading, check all fills in KYK-13

2.  Ensure power applied to aircraft

3.  Turn the IFF/SIF control head to STBY or ON

a.  MODE IV lever ON

b.  IFF/SIF caution light on

4.  Press the zeroize button on the KIT-1C

NOTE - Don’t zeroize the KIT-1C with the KYK-13 connected to it (it will 	zeroize the fill selected on the KYK-13)

5.  Attach the KYK-13 to the KIT-1C

a.  Ensure the KYK-13 is off

b.  Remove KIT-1C dust cover

c.  Holding the KYK-13 with the control knobs at the 10 O’clock position, 		attach the female connection on the KYK-13 to the fill port on the KIT-1C 	and rotate it clockwise until securely fastened

6.  Load the MODE IV codes

a.  Select the register that contains the MODE IV “A” code

b.  Watching the red LED on the KYK-13, turn on the KYK-13 (red light 	should flash, if not, wiggle the KYK-13 and try again)

c.  Turn KYK-13 off

d.  Select the register that contains the MODE IV “B” code, repeat 6b

e.  Watch KIT-1C , green LED should flash

f.  Turn KYK-13 off

g.  Load only 1 set of fills (A & B) are loaded at one time

7.  Disconnect the KYK-13 from the KIT-1C and replace dust cover

8.  Check that IFF/SIF caution light is extinguished

9.  Have MODE IV checked on ground before takeoff (IAW with FCIF file)

NOTE - In-flight, change from “A” to “B” after zulu day changes (A is for the day on the tape, B is the same as the next zulu day