| # Oil Temp |  | ## DC Electrical |  | Windshield Anti-Ice |  |
| --- | --- | --- | --- | --- | --- |
| Max taxi & time | 85-100C / 30 min | Voltage Range | 25-30v | Cold Start Procedure | 5s on, 10s off, til -43C |
| Max inflight & time | 85-100C / 5 min | Max DC load | 1.03 | Max OAT operations | 81F (27C) |
| Normal range | 60-85C | DC power sources | Batt, 4TRs, Ext | ## Nesa temp / # of windws | 80-100F / 9 |
| Oil Cooler flaps maintain | 78-82C | Batt. voltage norm / min | 24v / 21v | ## Prop De & Anti Ice |  |
| Min for take off | 40C and increasing | AC&EFC inverter pr fm | ess dc or ess av bs | Timing cycle (DeIce) | 15 sec on / 45 sec off |
| Min (start & warmup) | -40C | V to close/open batt relay | 18v –close/ 12v-open | Max Gnd ops time (De) | 2 cycles (2 minutes) |
| ## Engine Oil Pressure |  | # Amp-hr rating of battery | 31 (1hr) / 36 (5hr dis) | Deicing amp load range | 65-90 Amps |
| Max (start & warmup) | 100 psi | Battery – min temp / time | -29F for >=4 hrs | Anti-Ice amps | 20 Amps |
| Normal range / fluct | 50-60 psi 10 psi | # AC Electrical |  | ## Leading Edge Anti-Ice |  |
| Min (LSGI) | <50psi if nml@100% | Frequency range | 380-420 cps | Max ground ops time | 30 sec |
| ## Gearbox Oil Pressure |  | Voltage range | 110-125 v | Normal operating range | 75-200C |
| Max (start & warmup) | >250 psi | Max AC load | 1.05 | Overheat range | >200C |
| Normal range / fluct | 150-250 psi  20 psi | APU generator capacity | 40 kva | Control valve open/close | 158-180C |
| Mission continuation | 130 psi | APU generator load | 1.05 | Norm temp of anti-ice air | 350C |
| Min (LSGI) range | 50-250 psi | Gen cau (out) light illum | any ph =< 95 for 4 s. | Clr leading edge alt / tmp | >5000’ or < 20C |
| # Propellers |  | Bus cau (off) light illum | av ph<96v/any<100v | ## Speeds, Angles, & Feet |  |
| Prop brake disengage | 23% or starter app | Generator capacity | 40 kva | ## Max airspeed (mach) | 0.64 mach |
| Cyclical variation |  0.5 | # Oil Quantity |  | ## T-storm penetration | 65+ p off stall <=180 |
| Feather blade angle | 92.5 deg | Capacity range | 4 to 12 gal | ## Windmill Taxi Start | 100k til 4000’ / 40% |
| Pitchlock range | 25-55 deg | Expansion space | 7.5 gal | ## Flaps 10 / 20 / 30 / 40 /50 | 220/210/200/190/180 |
| Pitchlock RPM | 103% | Low light at | 4 gal | ## Flaps 60/ 70 / 80 /90 /100 | 165/155/150/145/145 |
| In pitchlock, mtn. RPM | 96-98% | Allowable burn rate | 1 qt/hr (.25 gal/hr) | ## Door Only | 185k |
| To unlock pitchlock | Increase 2 deg | # After –G, mon. eng for | 2 minutes | ## Ramp and Door | 150k |
| Lo pitch stop blade angle | 23 deg | -G: Press shd rtn within | 10 sec (if not do Esp) | ## Air deflector / troop door | 150k |
| Low pitch stop test | >200”lb increase | -G: Loss of oil when? | 30 to 90 sec | ## Lights | 250k |
| Prop oil quantity (total) | 26 quarts (6.5 gal) | # Hydraulic System |  | ## Gear | 165 k |
| Prop oil sump capacity | 6.5 qts in press side | Engine Pump Norm/Max | 2900-3200 / 3500psi | ## Wipers ineffective | 180k |
| Prop low oil light on at | 2 qts low in press side | Aux & Emer Norm/ Max | 2900-3300 / 3500psi | ## Troop door open / locked | 250k |
| Torque to decouple | -6000”lb | Shutdown if press over | 3900 psi | ## Crewdoor bailout | 150k |
| Prop aux pump limit | 1 min on, 1 min off, | RPM for indication | on speed | ## Paintd surface w/o stencil | 250k |
|  | not > 2 min in 30 min | RPM for norm press | 30 sec after on speed | ## Tire rotation (nose/main) | 139kgs / 174kgs |
| Stat. fethr ck tme (R - F) | 25 sec | LSGI minimum press | 2550 psi | ## Crosswind taxi norm/max | 30k / 60k |
| Pull out if not pop out in | 6 sec | Aileron boost pressure | 2050 psi | ## Ditching | 10+ stall w/ full flaps |
| NTS operates at | -1260 600”lb | Low press light on / off | 1000 psi / 1350 psi | ## Max bank angle | 60 deg |
| # Blade angle in reverse | -6 to -8 degrees | Suction boost lite on | 20 psi | ## Max bank angle w/ flaps | 45 deg |
| # Throttle Positions |  | Thermal CB on suc boost | 11 amps | ## Min space for max turn | 169’ 7” |
| Reverse | 0 - <18 deg | Utility reservoir capacity | 4.3 gal | ## Lowest vert clearance | 5’ 9” (inbrd prop) |
| LSGI range | 9 - 30 deg | Booster reservoir cap. | 3.1 gal | ## Max spd w/ 60d nosewhl | 5 kts |
| Ground idle / beta range | 18 deg / 0 - <34 deg | Aux hyd reservoir cap. | 5.3 gal | Max turn angle >=20 kts | 20 deg max |
| Flight idle / alpha range | 34 deg / 34 – 90 deg | Norm - 2 brake app left | 2900 - 3200 psi | ## Max spd/angle >= 155M | 10 kts / 20 deg |
| Crossover for TD | 65 deg | Norm – 1 brake app left | 2250 psi | ## Angle of flaps at full | 35 deg |
| # Bleed Air System |  | Emer – 1 brake app left | 2900 psi | Max reverse best at | 60 to 90 kts |
| Diffuser air temp/press | 600F / 125psi | Emer accum preload | 1000 100 psi | Wing span / length / high | 132’7”/ 97’9” / 38’3” |
| Eng min out press (ovrd) | 70 psi | Util/boost/brake preload | 1500 100 psi | Hold / Inst app entry spd? | 170k |
| Ground check for engine | 65–35psi >= 16 sec | Loss of sys press cklist | <2500 psi | # Inbound on approach spd | 150k |
| GTC min output press | 35 psi | Do not shut off pump if > | 3450 | 2 eng inst app entry spd | 170k |
| Ground check for APU | 30–15 psi >= 16 sec | Flaps 0-15: norm/caution | 1100-1400 / 1600 psi | 2 eng inbnd app speed | 160k |
| # Regulated output (Norm) | 40 - 50 psi | Flaps 15-100: norm/cau. | 2900- 3200/ 3500 psi | 2 eng final speed | 150k or approach (>) |
| # Regulators must be w/in | 3 psi of each other | # Amt of fluid in isol circ | 1 gal | ## Air Start with no NTS | 130kias below 5000’ |
| # Pressurization System |  | # Fluid quantity on board | 21 quarts (5 gal) | Rec airstart speed | 180 kias |
| Gauge range | -1.2 to 15.8” Hg | Brake press | appx 3000 psi | ## Airspeed for prop malf | 150 ktas |
| Safety valve open | < -0.76 & > 15.9” Hg | # Propulsion Check |  | ## Elev trim tab – elec stop | 6 deg dn / 25 deg up |
| Controller nominal press | 15.16” Hg | Wind note at > 7000” lbs | W/in 30 of 10k wind | ## Elev trim tab – mech stop | 8 deg dn / 27 deg up |
| W.shield cracked (1 or 2) | 10” Hg max | Icy – plane will move w/ | > 8000’lb on 4 eng | ## Cargo ramp (full clsd) | 21 deg abv horizontal |
| Cargo cracked (1 pane) | 10” Hg max | Min RPM for check | 8000’lb | ## Cargo ramp (full open) | 11.5 deg blow horiz |
| Cargo cracked (2 panes) | 0” Hg max | Rev ck – max diff | > 1000” lbs | ## Eng strt cycle (1 on, then) | 1 off, 5 off, 30 off |
| Rate ctl min / max rates | 30-200 / 1600-2900 | # APU |  | ## Starting TIT |  |
| Crew door jettison press | < 3.1”Hg | Normal temp range | 260-650 C | ## Max with enrich on (gnd) | 100C |
| # Pres to open pres reg vlv | 12 psi | Max EGT | 650 C | ## Max with out enrichment | 200C |
| Press regulated at | 45  5 psi | Normal RPM range | 95 - 105% | ## Max for airstart | 200C |
| Press flow sharing limit |  15% of each eng | Max RPM | 106% | ## Normal start limits | 750 – 830 C |
| # Engine Torque |  | Stop switch | 110% | ## TIT < 720 C: Do what? | write up, see MX |
| Oil temp < 0 C | minimum | Starter cycle | 1 min on, 4 min off | ## TIT 720 – 749 C: | write up, temp ctl ck |
| Oil temp 0 to 40 C | 4500”lb max | Warm up | 1m for bleed air load | ## TIT 830 – 850 C: | write up |
| Max allowable - takeoff | 19600”lb (for 5 min) | Start and operate below | 20,000’ | ## TIT 850 – 965 C: | stop, write up, 1 retry |
| Max allowable in flight | 19600”lb (30 min) | Door open ground / flight | 35 deg / 15 deg | ## TIT >965 C: | stop, write, see MX |
| Max continuous | 19600”lb | APU fuel burn | Appx 400 lbs/hr | ## Abnormal torch | stop, write, see mx |

| ## Engine TIT |  | # Fuel System |  | ## Miscellaneous |  |
| --- | --- | --- | --- | --- | --- |
| ## Normal continuous range | 200 – 1010 C | Main tank pump press | 15 – 24 psi | Prop fail to fthr, hold for | 30 sec |
| ## Takeoff range / time | 1067 - 1083C / 5 min | Aux & ext tank pump | 28 – 40 psi | Accel bleed valve open | below 94% RPM |
| ## Military limit / time | 1049 C / 30 min | Low fuel press lite on | 8.5 psi | Ramp/door open/cls time | 15 to 45 sec (E mdl) |
| ## Climb / max continuous | 1010 C | Aux & ext tank empty lt. | 23 psi | rmp/dr: air drop to op/cls | 7 to 30 sec (E mdl) |
| ## Accel peak limits | >1083(5s)/1175 max | Rated output - all pumps | 6000 pph | Time to extend full flaps | 8 to 15 sec |
| ## Crossover | 820C 20 C | Undump outbrd/in/ext/tot | 2100/1800/60/ 7800# | Time to retract flaps | 10 to 15 sec |
| ## Normal Limiting | 1083 C | Min dump altitude | 5000’ | Gear extension/retraction | 19 sec or less |
| ## Max downspeed TIT | 850 C | Pri fuel mgt per 55-130 | 7500outbrd/6900inbd | Preheat eng & apu below | -37 C |
| ## 55-130 continuous limit | 970 C | # Limits:wings/tanks/out-n | 1500/1000/500-1000 | Cold WX ops temp | <= 0C (32F) |
| ## # of thermocouples | 18 (2 elements each) | ## Cargo Compartment |  | Hot WX ops temp | > 35C (95F) |
| thermocoup bad – range | 3.5 to 22 degrees ea | Milkstool req’d for ramp | >2000 lbs across rmp | Amnt of oil in starter | 4 ounces, 175 cc |
| ##### Engine RPM |  | # of VHF radio presets | 20 | Hot brake peak tmp time | 1 to 5 minutes |
| Oil pres / ign by | 35% | # of HF radio presets | 30 | Peak hot tmp n wheel/tire | 20 to 30 min |
| Normal flt & Takeoff | 98 – 102% 0.5% | Buf stp asy max w/o rstnt | 26,650 lbs | Hot brakes: no tow for | 15 min |
| Norm flight idle (on gnd) | 92.5 – 100.5% | Buf stp asy max w/ rstnt | 37,248 lbs | Full anti-skid; no retr. for | 15 min |
| Normal gnd idle | 94 - 102% | # Ground Troops | 92 | Norm land; time bfre TO | 10 min |
| Max reverse | 96 – 106% | # Paratroops with equip | 64 | Disregard above if | Rwy > CFL by 300’ |
| Low speed ground idle | 69 – 75.5% | Litters (2 attend/6 attend) | 74 / 70 | Partial braked lndgs def | 3 sec app at 90 kts |
| Ignition Relay on / off | 16% / 65% | 2 loadmasters required | > 40 passengers | Full antiskid at > 130k | cool brake for 65min |
| Fthr on airst if no nts by | 10% | # Lightest crgo jettisonable | 2500 lbs | Fusible plugs in tire melt | 390deg |
| Airstart lite off nrm/mx | 30% / 40% max | Crgo rmp cap. w/o dul rls | 5000 lbs | Tmp for nacel/turb oheat | 300d / 700d |
| ## Speed sens. valve actuate | 94% | Crgo rmp cap. w/ dual rls | 4664 lbs | # of nacel/turb oheat sens | 7 nacelle / 4 turbine |
| ## Speed of Eng / Prop | 13,820 / 1021 | CDS A-22 max | 2328lbs (128lb rigng) | Cold wx: lsgi til oil tmp | 0 deg or more |
| # Gearbox reduction ratio | 13.54 to 1 | CDS nylon gate (1 ply) | <= 13000 lbs | Cold wx: in gnd idle till | 40 deg oil temp |
| # Oxygen & Life Support |  | ## CDS nylon gate (2 ply) | <=25000 lbs | Underfloor heat temp | 80 F |
| First aid kit on brd / statn | 6 / 23 stations | ## CDS nylon gate (3 ply) | <=37,248 lbs | Gnd ck pt – max cdi error | 4 deg |
| Oxy ports flt deck / total | 6 / 10 | CG limits | 15-30% w/i com lims | Gnd ck pt – max dme err | ½ mi or 3% of dist(> |
| ## Min lox per 55-130 | 5 liters or ETP | Crew entrance door lim | 1200 lbs | HF radio hazard range | 5 ft |
| ## Fully serviced quantity | 25 liters | ## Dual rail LH locks | 20k# forwd, 10k# aft | Start: prop rotation time | 5 sec max |
| ## Low light on | 2.5 liters on gage | ## Dual rail RH locks | 20k forwd, 4k vrb aft | Start: st. vlv lite on in | 5s max (2-3 psi to st) |
| ## Press rng (no flow/cont) | 270-455 / 270-340 psi | EFTC limit | 42,000 lbs | Start: st vlv opn lite out in | 15s after start release |
| ## Full sys – min supply -hr | 96 manhours | # of Tiedowns – 10k | 175 (7 colms of 25) | Seat inertial reel lock | 2 to 3 G’s |
| # of life rafts / capacity | 4 / 20 men | # of Tiedowns – 25k | 12 (2 avail w/ dul rls) | Pitot tubes for pilot / co | 1&3 / 2&4 |
| Fire extinguishers | 4 | # of Tiedowns – 5k | 55-11 ea wal 33 rmp | SCNS Reference waypts | 200 |
| Fire ext. press-halon1211 | 125  50 psi | ## Cargo area height | 9’ | SCNS Waypoints | 60 |
| Emer lights illum | -2.5g’s or Ess DC off | ## Cargo area length | 50’ | SCNS Markpoints | 10 |
| # of Emer exit lights | 8 | ## Cargo area space | 41’ x 10’3” | SCNS DZs / TOAs / LZs | 8 / 3 / 2 |
| POK time of oxygen | 10 to 14 min | ## Cargo compt flr pres lim | 50 psi | ## Max Symm / Unsym G’s | 2.0 Gs / 1.5 Gs |
| EEBD time of oxygen | 14 to 16 min | ## Cargo cmpt max load | Pos. 1-4=10k, 5=8.5k | ## Most reliable fuel reading | 0 deg roll,  3 d pitch |
| ELT / ULB range | 1000 mi / 2k-4k yds | ## Truck loading rmp limit | 25,000 lbs / pair | ## PA mics / plug ins/ spkrs | 3 / 2 / 7 |
| Life Preserver Units # | 1 per crewmember | ## Winch cable / pull / pwr | ¼ in / 4000# / 28v dc | ## Gally hot liquid containrs | Two 2 gal containers |
| ## Portable oxygen bottles | 4 | # of ladders | 2 (1 step, 1 escape) | ## Galley sink capacity | 1.5 gal |
|  |  | #### Boldface |  |  |  |
| ### FUSELAGE FIRE/SMOKE AND FUMES ELIMINATION. |  |  | AXUILARY POWER UNIT EMERGENCY SHUTDOWN (GROUND/INFLIGHT). |  |  |
| 1. OXYGEN “ON, 100 PERCENT” (P) |  |  | 1. FIRE HANDLE “PULLED” (E) |  |  |
|  |  |  | 2. AGENT “DISCHARGED” (FOR FIRE) (E) |  |  |
| MULTIPLE ENGINE POWER LOSS/RPM ROLLBACK |  |  |  |  |  |
| 1. MAIN TANK BOOST PUMPS – “ON” (E) |  |  | ENGINE SHUTDOWN PROCEDURE. |  |  |
| 2. MAIN TANK CROSSFEED VALVES – “CLOSED” (E) |  |  | 1. CONDITION LEVER “FEATHER” (CP) |  |  |
| 3. PROPELLER GOVERNOR CONTROL SWITCHES – “MECH” (CP/E) |  |  | 2. FIRE HANDLE “PULLED” (CP) |  |  |
| 4. TEMPERATURE DATUM CONTROL VALVE SWITCHES – “NULL” (E) |  |  | 3. AGENT “DISCHARGED” (FOR FIRE) (CP) |  |  |
| (5. Synchrophaser master switch - “off” (E) ) (6. Synchrophaser ess avionics and ess dc circuit breakers - “pulled” (E) ) (7. Land as soon as practical. ) |  |  |  |  |  |

| # Aircraft Performance |  | ## 55-130 |  | ## SKE / AWADS |  |
| --- | --- | --- | --- | --- | --- |
| ## Max wt for windmill txi | 135,000 lbs |  |  | PPI range | 10 NM |
| Vmca2 inc by ? when lvl | up to 20 kts |  |  | TWS range | 4 NM |
| Nml tkeoff / obs clnc spd | 1.1 / 1.2 x p. off stall |  |  | Max airplanes | 36 (34 w/ zone mark) |
| Meto / ME obs clnc spd | 1.2 / 1.3 x p. on stall |  |  | Standby (warm up) time | 2 min |
| Nml Thld / Tochdn spds | 1.35/1.2 x p. off stall |  |  | ## Antenna rotation speed | 40 rpm |
| ME thld / touchdn spds | 1.28/1.2 x p. off stall |  |  |  |  |
| RCR for no brakes | 2 |  |  |  |  |
| Recalc told w/ chng of | 5000#, 5d, 1000’ PA |  |  |  |  |
| TO weight/ max EWP | 155000# / 175000# |  |  |  |  |
| Stall buffet abve stall by | 4 to 15% above stall |  |  |  |  |
| Fin stall poss. up to spd? | 170 kts |  |  |  |  |
| Lbs for fin stall recovery | 50 to 100 lbs |  |  |  |  |
| Max land wt / max vvi | 155000lbs / 300 fpm |  |  |  |  |
| Norm land wt / max vvi | 130000lbs / 540 fpm |  |  |  |  |
| Overtorque tiers | 19600, 21500, 23000 |  |  |  |  |

| ## General Memory Aids |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ## Prop safety features | Pitchlock | Battery Bus | Fire agent |  |  |
|  | Low Pitch Stop |  | Alarm bells |  |  |
|  | Feather |  | AC external power |  |  |
| ## Gearbox safety features | NTS |  | Jump lights |  |  |
|  | Decouple |  | ELT |  |  |
|  | Prop brake |  | Voltmeter |  |  |
| ## Fire Handle (ess dc) (9) | Engine fuel control |  | Emer exit lites exting |  |  |
|  | Engine oil control |  | Emer depress |  |  |
|  | Firewall fuel |  |  |  |  |
|  | Firewall hydraulics |  |  |  |  |
|  | Engine bleed air |  |  |  |  |
|  | Eng start ctl circuits |  |  |  |  |
|  | Prop feathered |  |  |  |  |
|  | Positions fire ext vlvs |  |  |  |  |
|  | Arms agent discharge |  |  |  |  |
| ## CB Panel – Pilot side | LH AC |  |  |  |  |
|  | RH AC |  |  |  |  |
|  | ## AC inst & eng ful ctl |  |  |  |  |
|  | Fuses |  |  |  |  |
|  | Battery bus |  |  |  |  |
|  | Isolated DC |  |  |  |  |
|  | Essential AC |  |  |  |  |
| CB Panel – copilot side | Main |  |  |  |  |
|  | Radio |  |  |  |  |
|  | Start (ESS DC) |  |  |  |  |
|  | Engine (ESS DC) |  |  |  |  |
|  | Airframe(Ess DC) |  |  |  |  |
|  | Lights (Main DC) |  |  |  |  |
| ## Engine Accessory Drive | Oil pump |  |  |  |  |
|  | Fuel pump |  |  |  |  |
|  | Fuel filter |  |  |  |  |
|  | Speed sensitive ctl |  |  |  |  |
|  | Speed sensitive vlv |  |  |  |  |
| ## Gearbox Accessry Drive | Generator |  |  |  |  |
|  | Hydraulic pump |  |  |  |  |
|  | Oil pump |  |  |  |  |
|  | Starter |  |  |  |  |
|  | Tach generator |  |  |  |  |
| Sec AC - 115 v ac, 1 phz | Torque |  |  |  |  |
|  | Fuel flow |  |  |  |  |
|  | TIT |  |  |  |  |
|  | Fuel Quantity |  |  |  |  |
|  | LOX Quantity |  |  |  |  |
| Sec AC – 26 v ac, 1 phz | #1 stepdown xformer |  |  |  |  |
|  | ## Fuel pressure ind. |  |  |  |  |
|  | Emer hydraulic press |  |  |  |  |
|  | Booster hyd press |  |  |  |  |
|  | #3 eng&grbx oil pres |  |  |  |  |
|  | #4 eng&grbx oil pres |  |  |  |  |
| Sec AC – 26 v ac, 1 phz | #2 stepdown xformer |  |  |  |  |
|  | Aux hyd press |  |  |  |  |
|  | Utility hyd press |  |  |  |  |
|  | Normal brake press |  |  |  |  |
|  | Antiskid test |  |  |  |  |
|  | #1 eng&grbx oil pres |  |  |  |  |
|  | #2 eng&grbx oil pres |  |  |  |  |