C-130 OXYGEN PRE-FLIGHT

(Ref Sect IV, Dash-1)

Pressure limits: static 270 - 455 psi, flow 270 - 340 psi

(below 50 psi, 781 entry req’d)

LOX Limits: 25L max, 2.5L min (5L/300 psi min, IAW 55-130)

1.  Supply Lever - ON

2.  Diluter Lever - 100% OXYGEN

3.  Emer Toggle Lever - EMERGENCY

4.  3 Breathing Cycles - Black in, White out

5.  Hold breath - Blinker Blk

Emer Toggle Lever- NORMAL, Blinker Blk

6.  3 Breathing Cycles - Black in, White out

7.  Leave in NORMAL, 100%, ON, Oxygen Hose - Connected

COCKPIT INSTRUMENT CHECK

(Ref p. 4-105)

1.  ADI Mode Select Switch - NORM (SKE Acft)

2.  Flt Dir Switch - NORM

3.  Mode Select Switch - HDG

a.  Pointers & Flags - gone

b.  Course arrow - slave up

c.  Course window - compass hdg

d.  Attitude indicator alignment

4.  Flt Dir Switch - MANUAL

a.  Hdg set mkr - checked, top (rotate)

b.  Hdg set knob - checked

c.  Pitch trim - checked

d.  Nav instruments - checked

5.  Flt Dir Switch - NORM

TACAN TEST

(Ref p. 4-88)

90 sec warm-up, ESS AC/DC

Tune usable TACAN channel - Monitor ident

1.  Ensure BP points to station

a.  Check against other TAC (+/- 4)

b.  Check BDHI

2.  Center CDI

3.  Rotate Course

a.  Check for proper displacement

b.  Check 90 out (TO - FROM change)

4.  HSI - 180

5.  Function Switch - T/R

6.  Press test button & check:

a.  Bearing pointer - 270

b.  No flags (after 7 sec)

c.  Bearing pointer - 180 (+/- 3)

d.  DME - 0 (+/- 0.5)

e.  CDI - centered (+/- 1/2 dot)

f.   TO - FROM pointer - TO

VOR TEST

(Ref p. 4-86)

ESS DC

YOK Use Ch 116.2 - Monitor ident

1.  Ensure BP points to station

a.  Check against other TAC (+/- 4)

b.  Check BDHI

2.  Center CDI

3.  Rotate Course

a.  Check for proper displacement of CDI

b.  Check 90 out (TO - FROM change)

4.  HSI - 315

5.  Press the test button (10 sec)

a.  Bearing pointer - 315 (+/- 3) (airborne +/- 4)

b.  CDI - centered

c.  TO - FROM - TO

d.  RMI pointer - 315 (+/- 3)

e.  Marker beacon lights - ON (#1 system only)

f.  Warning flags - gone

6. Press the test button

ADF TUNING

(Ref p. 4-105)

5 min warm-up, ESS AC/DC

Tune in a usable NDB channel – monitor ident.

1.  Function Switch - ANT

2.  Function Switch - LOOP

a.  Turn loop switch  L-R to check for control

b.  If BP does not move, go to CONT, then LOOP

c.  Turn loop switch  L-R to check for control

3.  Function Switch - ANT

4.  CW/VOICE Switch - VOICE

a.  Rotate mixer to tune

6. CW/VOICE Switch - CW

a.  Tune for null tone

7. CW/VOICE Switch - VOICE

a.  Recheck ident

8.  Function Switch - ADF

a.  Ensure BP points to station