PERSONNEL

Pre Slowdown (LM)

- Red Light - On

- Aux hydralic pump on for tailgate drops (co-pilot)

- Alert Jumpmaster

- CDS arming switches (245) in normal position

- Troopers hook up and jumpmaster puts chute on (prior to opening door)

- ADS arms connected (tailgate), attaches anchor cable to A frame (door)

- Don’s restraint harness & helmet

Slowdown

Paratroop door drop

- Flaps - 50% , co-pilot opens air deflector door

- LM opens doors, visually checks for open air deflector doors

- Jump platfroms locked in place, Jumpmaster assumes control of the exit

Tailgate drop

- Ramp and door opened by engineer

- Aft anchor arms lowered

- Jumpmaster assumes control of the exit

Drop and Completion of Drop

- LM alerts personnel of 1-minute and 5 seconds, Green Light - On

- LM calls “Load Clear” or the condition (visually checks outside)

- Red Light - On (LM regains control of exit)

- Jump platform in,  static lines retreived, chutes dearmed, doors closed

- + Raise aft anchor cables supports, clear ramp & door for closing

No-Drop

- LM Acknowledges, accomplishes completion of drop checklist

NOTE: If drop reattempted, begin with pre slowdown checklist

EMERGENCY (towed parachutist)

- LM notifies Jumpmaster, Pilot

- Red Light - On

- Maintain drop airspeed and minimum AGL for chute used

- Avoid flying over or upwind of water or built up areas

- Jumpmaster/Loadmaster makes recommendation to pilot

- Paratroop doors

- Lower landing gear, 100% flaps (WRNG: ¯ aircraft performance)

- Avoid turning into the towed jumper

- Turns should be shallow and coordinated

- First priority is to retrieve

- Ramp and Door

- First priority is to cut if concious

- Rest flaps to 50% & raise landing gear

- Completion of drop checklist

NOTE: conciousness indicated with tight body position/hand on reserve