# PTU FIRE: 3-31

- INDICATIONS WARNING HORN, FIRE LIGHTS, ZONE ALARMS

- WILL NOT GET WARNING HORN IF ANY SWITCH IS IN RUN OR

## AIR START

- CALL FOR “ PTU FIRE CREW IN PLACE CHECKLIST

- CO-PILOT CALL FOR FIRE EQUIPMENT

1. BRAKE SWITCH EMERGENCY

2. PARKING BRAKE SET

3. GROUND EXITS OPEN (P)

-TECHNIQUE – DIRECT WHICH EXITS TO OPEN

4. REST OF CHECKLIST STEPS ARE CONDITIONAL

- FIRE FIGHTING CAPABILITY

- 1-2, 3-4 PTU PROTECTED BY LN2

- 3-4 PTU, HAND HELD HALON EXTINGUISHER

- KEEP ENGINE RUNNING (OUTBAORD PREFERABLY # 4) FOR POWER UNTIL EVACUATION DIRECTED OR UNTIL AN APU IS STARTED / RUNNING

- INFLIGHT ACTIONS FOR FE (NOT PILOTS); DIRECT CREW TO FIGHT FIRE

BLEED AIR SMOKE ELIMINATION (3-72)

- SMOKE FROM ENGINE BLEED AIR SYSTEM OR THE AIR

## CONDITIONING PACKS

- BOLD FACE: OXYGEN MASKS – ON

- FE RUN TROUBLE SHOOTING PROCEDURES

- MAY NEED TO DEPRESSURIZE AND GO AUX VENT

- ALTITUDE, PASSENGERS, OXYGEN REQUIREMENTS