HC/MC-130P

PLASTIC

BRAINS

TABLE OF CONTENTS

Conversion Tables

SM - RVR - Meters

Coordinate Conversion

Pressure Millibars to Pressure Altitude

Standard Temperature Lapse Rate

Hour & Tenth Table

Math & Conversion Table

Fahrenheit to Celsius Table

Millibars & Hectopascals to Inches

General Planning

Departure Planning

Destination Alternate Requirements

Fuel Planning Form

Minimum Runway Requirement

Overwater Planning

Overwater Checklist

Inflight Fuel Management Example

Airspeed Calibration Example

Airspeed - Compressibility Correction

Fuel Basics

Low Level Planning

EW Frequencies

Timing Triangles

Low Level Airspeed/Timing Adjustment Tables		Turn Time Adjust Table

Diameter/Radius of Turn Table

TABLE OF CONTENTS (Con’t)

Tactical Planning

Illumination Conditions

DMA Map/Chart Accuracies

Joint Planning Term Reference				Runway Definitions

Threat Data

Man Portable SAMs

Towed/Mobile SAMs

AAA Systems

Fighter Aircraft

Air-To-Air Missiles

GOB Symbols

Objective Area Planning

SCA/Airdrop Slowdown Timing Chart

Deceleration Distance

200-Yard Wind Circle Chart

Airdrop Load Data

Parachute Data

DZ Overlay

Sight/Offset Angle Table (Nav Plotter)

Airdrop Timing Table

Window Sight Angle Guide

Airspeed Adjust Distance. to Min. S/D Distance

SCA Planning Form

SCA Briefing Guide

SCA Glideslope Planning Chart

Descent Rate Chart

SCA Formulas

HAR Emergency Procedures

HAR Light Signals

HAR Timing (Trombone Technique)

TABLE OF CONTENTS (Con’t)

Search and Rescue

SCNS Tips for Delivery Patterns

Sweep Width

Parachute Drift Distance (Zero Glide Ratio)

Sector Search Pattern

Expanding Square Search Pattern

Creeping Line Search Pattern

Parallel Search Pattern

Parabundle/Freefall Delivery Pattern

Navigator Assisted MA-1/2 Delivery Pattern

MA-1/2 Kit Delivery Pattern-Upwind

MA-1/2 Kit Delivery Pattern-Downwind

International Distress/SAR Frequencies

MARS Listings

Formation

Systems

Acronyms

Operations Limits

Start Matrix

Prop Low Oil Light

HAR System

Emergency Equipment

Aircraft Emergency Equipment

Circuit Breakers

TABLE OF CONTENTS (Con’t)

Emergency Procedures

Engine Malfunctions

Ground Evacuation

Two Engine Out

Propeller Malfunction

Secondary Fuel Pump Pressure Light

Multiple Engine Power Loss

Generator Out Light

AC Bus Off Light

Miscellaneous

Training Restrictions

Joint Nomenclature System

SCNS Degraded Ops

Briefings

NVG/Assault Take-Off

SCA Briefing

Navigator Tactical Briefing

Useful INTERNET Sites

Conversion Tables

SM - RVR - METERS

Statute Mile	RVR	Meters

1/8	12	370

1/4	16	490

3/8	20	610

1/2	24	730

5/8	32	970

3/4	40	1220

7/8	45	1370

1.0	50	1520

1 1/4	60	1830

- COORDINATE CONVERSION

(Seconds to decimal minutes)

| SEC | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | .00 | .02 | .03 | .05 | .07 | .08 | .10 | .12 | .13 | .15 |
| 10 | .17 | .18 | .20 | .22 | .23 | .25 | .27 | .28 | .30 | .32 |
| 20 | .33 | .35 | .37 | .38 | .40 | .42 | .43 | .45 | .47 | .48 |
| 30 | .50 | .52 | .53 | .55 | .57 | .58 | .60 | .62 | .63 | .65 |
| 40 | .67 | .68 | .70 | .72 | .73 | .75 | .77 | .78 | .80 | .82 |
| 50 | .83 | .85 | .87 | .88 | .90 | .92 | .93 | .95 | .97 | .98 |

NM - KM - SM CONVERSION FACTORS

1 NM = 1.8520 KM = 1.15078 SM

1 KM = 0.53996 NM = 0.62137 SM

1 SM = 0.86898 NM = 1.60934 KM

Conversion Tables (Con’t)

PRESSURE MILLIBARS TO PRESSURE ALTITUDE
Pressure Millibars (mb)	Approx Pressure Alt.

850						5,000 ft

700						10,000 ft

500						18,289 ft

400						23,500 ft

300						30,065 ft

STANDARD TEMPERATURE LAPSE RATE TABLE

- Altitude		Std Temp C

| 10,000 | -4.8 |
| --- | --- |
| 11,000 | -6.8 |
| 12,000 | -8.8 |
| 13,000 | -10.8 |
| 14,000 | -12.7 |
| 15,000 | -14.7 |
| 16,000 | -16.7 |
| 17,000 | -18.7 |
| 18,000 | -20.7 |
| 19,000 | -22.6 |
| 20,000 | -24.6 |
| 21,000 | -26.6 |
| 22,000 | -28.6 |
| 23,000 | -30.6 |
| 24,000 | -32.5 |
| 25,000 | -34.5 |
| 26,000 | -36.5 |
| 27,000 | -38.5 |
| 28,000 | -40.5 |
| 29,000 | -42.5 |
| 30,000 | -44.4 |

- ALGEBRAIC FORMULA TO DETERMINE STANDARD TEMPERATURE

-2 (Altitude) + 15 = Std Day Temperature C

1000

Conversion Tables (Con’t)

HOUR & TENTH CONVERSION TABLE

00 - 02	Minutes	.0 Hour

03 - 08 	Minutes	.1 Hour

09 - 14 	Minutes	.2 Hour

15 - 20 	Minutes	.3 Hour

21 - 26 	Minutes	.4 Hour

27 - 33 	Minutes	.5 Hour

34 - 39	Minutes	.6 Hour

40 - 45	Minutes	.7 Hour

46 - 51	Minutes	.8 Hour

52 - 57	Minutes	.9 Hour

58 - 60	Minutes	Next whole hour.

Math and Conversion Table

O

p

p

o

s

i

t

e

Hypotenuse



Adjacent

Sinq=oppositehypotenuse	Cosq=adjacenthypotenuse

Tanq=oppositeadjacent	Secq=hypotenuseadjacent

Cotq=adjacentopposite	Cscq=adjacenthypotenuse

Law of Sines

aSinA=bSinB=cSinC

a

A

b

B

c

C

Conversion Tables (Con’t)

Law of Cosines

a2 = b2 + c2 – 2bc cos A

b2 = a2 + c2 – 2ca cos B

c2 = a2 + b2 – 2ab cos C

1 m = 39.37 in = 3.281 ft	1 mile = 1.609 km

1 cm = .3937 in	1 mile = 5280 ft

1 ft = .3048 m	1 radar mile = 12.4 s

1 mile = 1609 m = 5280 ft	1 NM/hr = 1.1516 MPH

1 day = 1440 min	1 MPH = .86839 Knots

1 m/s2 = 3.281 ft/s2

1 ft/s2 = .3048 m/s2	1 degree = 17.45 mils

1 MPH = .4470 m/s = 1.467 ft/s	1 mil = .05729 degrees

1 mile/min = 88 ft/s	1 NM = 6,080 ft

1 in = 2.54 cm	1 rad = 57.296 degrees

1 yd = .9144 m	1 HP = 550 ft·lb/s

1 km = .621 miles	1 nm/hr= 1.6878 ft/sec

Conversion of Fahrenheit  Celsius

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 110 | 43.3 | 43.9 | 44.4 | 45.0 | 45.6 | 46.1 | 46.7 | 47.2 | 47.8 | 48.3 |
| 100 | 37.8 | 38.3 | 38.9 | 39.4 | 40.0 | 40.6 | 41.1 | 41.7 | 42.2 | 42.8 |
| 90 | 32.2 | 32.8 | 33.3 | 33.9 | 34.4 | 35.0 | 35.6 | 36.1 | 36.7 | 37.2 |
| 80 | 26.7 | 27.2 | 27.8 | 28.3 | 28.9 | 29.4 | 30.0 | 30.6 | 31.1 | 31.7 |
| 70 | 21.1 | 21.7 | 22.2 | 22.8 | 23.3 | 23.9 | 24.4 | 25.0 | 25.6 | 26.1 |
| 60 | 15.6 | 16.1 | 16.7 | 17.2 | 17.8 | 18.3 | 18.9 | 19.4 | 20.0 | 20.6 |
| 50 | 10.0 | 10.6 | 11.1 | 11.7 | 12.2 | 12.8 | 13.3 | 13.9 | 14.4 | 15.0 |
| 40 | 4.4 | 5.0 | 5.6 | 6.1 | 6.7 | 7.2 | 7.8 | 8.3 | 8.9 | 9.4 |
| 30 | -1.1 | -0.6 | 0.0 | 0.6 | 1.1 | 1.7 | 2.2 | 2.8 | 3.3 | 3.9 |
| 20 | -6.7 | -6.1 | -5.6 | -5.0 | -4.4 | -3.9 | -3.3 | -2.8 | -2.2 | -1.7 |
| 10 | -12.2 | -11.7 | -11.1 | -10.6 | -10.0 | -9.4 | -8.9 | -8.3 | -7.8 | -7.2 |
| 0 | -17.8 | -17.2 | -16.7 | -16.1 | -15.6 | -15.0 | -14.4 | -13.9 | -13.3 | -12.8 |
| -10 | -23.3 | -22.8 | -22.2 | -21.7 | -21.1 | -20.6 | -20.0 | -19.4 | -18.9 | -18.3 |
| -20 | -28.9 | -28.3 | -27.8 | -27.2 | -26.7 | -26.1 | -25.6 | -25.0 | -24.4 | -23.9 |
| -30 | -34.4 | -33.9 | -33.3 | -32.8 | -32.2 | -31.7 | -31.1 | -30.6 | -30.0 | -29.4 |
| -40 | -40.0 | -39.4 | -38.9 | -38.3 | -37.8 | -37.2 | -36.7 | -36.1 | -35.6 | -35.0 |

Conversion Tables (Con’t)

Barometric Conversion:

Millibars & Hectopascals Inches

| Hecto-Pascals or Milibars | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Inches |  |  |  |  |  |  |  |  |  |
| 940 | 27.76 | 27.79 | 27.82 | 27.85 | 27.88 | 27.91 | 27.94 | 27.96 | 27.99 | 28.02 |
| 950 | 28.05 | 28.08 | 28.11 | 28.14 | 28.17 | 28.20 | 28.23 | 28.26 | 28.29 | 28.32 |
| 960 | 28.35 | 28.38 | 28.41 | 28.44 | 28.47 | 28.50 | 28.53 | 28.56 | 28.59 | 28.61 |
| 970 | 28.64 | 28.67 | 28.70 | 28.73 | 28.76 | 28.79 | 28.82 | 28.85 | 28.88 | 28.91 |
| 980 | 28.94 | 28.97 | 29.00 | 29.03 | 29.06 | 29.09 | 29.12 | 29.15 | 29.18 | 29.21 |
| 990 | 29.23 | 29.26 | 29.29 | 29.32 | 29.35 | 29.38 | 29.41 | 29.44 | 29.47 | 29.50 |
| 1000 | 29.53 | 29.56 | 29.59 | 29.62 | 29.65 | 29.68 | 29.71 | 29.74 | 29.77 | 29.80 |
| 1010 | 29.83 | 29.85 | 29.88 | 29.91 | 29.94 | 29.97 | 30.00 | 30.03 | 30.06 | 30.09 |
| 1020 | 30.12 | 30.15 | 30.18 | 30.21 | 30.24 | 30.27 | 30.30 | 30.33 | 30.36 | 30.39 |
| 1030 | 30.42 | 30.45 | 30.47 | 30.50 | 30.53 | 30.56 | 30.59 | 30.62 | 30.65 | 30.68 |
| 1040 | 30.71 | 30.74 | 30.77 | 30.80 | 30.83 | 30.86 | 30.89 | 30.92 | 30.95 | 30.98 |
| 1050 | 31.01 | 31.04 | 31.07 | 31.10 | 31.12 | 31.15 | 31.18 | 31.21 | 31.24 | 31.27 |

General Planning

DEPARTURE PLANNING

CONDITION					RULE

T

A				The gross weight must never exceed

K				that which would result in a critical

E				field length in excess of runway

O				available.

F

F

| CLIMBOUT IMC |  |
| --- | --- |
| No minimum climb rate specified in the departure procedure: | Aircraft must be capable of climbing 152 feet per nautical mile on three engines. |
| A minimum climb rate is specified in the departure procedure: | Aircraft must be capable of climbing at the minimum climb rate specified in the departure procedure on four engines plus clear vertically all obstacles in the climb out path on three engines. |
| CLIMBOUT VMC |  |
| If climbout cannot be made using the above criteria and the weather is VMC (3 miles visibility minimum) the AC may elect to use a see and avoid concept for obstruction clearance. Do not use see and avoid at night unless in the AC’s opinion adequate illumination exists to positively identify the obstacle(s). Even when obstacles are not a factor, the aircraft must be capable of climbing at a rate of at least 200 feet per minute on three engines at obstacle clearance speed or Vmca. |  |

NOTE:  AF SIDS assume zero feet altitude at the departure end of the runway and then 200 feet per nautical mile as the minimum climb gradient unless otherwise specified.  All other departure procedures are based on 35 feet altitude at the departure end and then 200 feet per nautical mile unless otherwise specified.

General Planning (Con’t)

DESTINATION ALTERNATE REQUIREMENTS

1.  When to designate an alternate:

a.  Radar required to fly the planned approach.

b.  For ETA (+/- 1 hour), the worst weather (TEMPO or prevailing) is forecast to be less than 3000 feet ceiling, or visibility less than 3 SMs or 2 SMs more than the lowest compatible published landing minimum visibility, whichever is greater.

c.  Destination is outside the CONUS.  Intra theater flights outside CONUS exceeding 3 hours.

d. Regardless of WX, pilots will file an alternate when destination does not have weather reporting capability.

e. MAJCOMs may authorize holding at a remote or island destination will prescribe weather criteria and recovery procedures.

f. For all IFR flights, the approach facility upon which are based must be operational at the departure alternate, and approach equipment in the aircraft must be operational. Use runway visual range (RVR), when available, to determine the minimum visibility required for takeoff.

2.  File two alternates if the forecast visibility, prevailing or intermittent, is less than published for an available approach, or the forecast surface winds, prevailing or intermittent, exceed limits corrected for RCR.

3. Remote or island destinations, with no alternate available, add holding fuel of 6500 lbs or 1+15 at TFF.  The forecast weather at the remote/island destination must meet the following criteria:

a.  The prevailing surface winds, corrected for RCR, must be within limits at ETA and remain so for two hours thereafter.

b.  The prevailing ceiling and visibility are equal to or greater than published minimums for an available nonprecision approach (excluding ASR) for ETA plus two hours:  or if a precision approach is available, the ceiling or visibility may be temporarily or intermittently below nonprecision approach minimums (excluding ASR) but not below precision approach minimums.

General Planning (Con’t)

c. Regardless of weather, pilots must designate an alternate airport on all IFR flight plans when filing to a destination requiring any of the following to fly the approach:

1) Radar

2) GPS is the only available NAVAID

3) An unmonitored NAVAID

ALTERNATE AERODROME REQUIREMENTS

The worst weather (TEMPO or prevailing) for the ETA (+/- 1 hour) of 1000 feet, or 500 feet above the lowest compatible published landing minimum, whichever is higher, and a visibility of 2 SMs or 1 SM above the lowest compatible published landing minimum, whichever is higher.

1. With a published instrument approach procedure.

a) A ceiling of at least 1000 feet

2. Without a published instrument approach procedure

a) Forecast Wx for ETA (+/- 1 hour) must permit a VFR descent from IFR enroute altitude to a VFR approach and landing.

General Planning (Con’t)

FUEL PLANNING

| OPERATING WT |  | CALL SIGN |  |
| --- | --- | --- | --- |
| CARGO/PAX WT |  | ACFT NO. |  |
| RAMP FUEL |  | MSN NO. |  |
| RAMP WT |  | TIME | FUEL |
| TAKEOFF WT |  |  |  |
|  | AD: |  |  |
| 1. ENROUTE |  |  |  |
| 2. RESERVE | TFF: |  |  |
|  | TD: / |  |  |
| 3. ENROUTE + RESERVE |  |  |  |
| 4. ALTERNATE + MISSED APPROACH |  |  |  |
| 5. HOLDING | 2.0 |  |  |
|  | 3.5 |  |  |
|  | 1.0/4.0 |  |  |
| 6. APPROACH /LANDING |  |  |  |
| 7. IDENTIFIED EXTRA |  |  |  |
| 8. TOTAL TAKEOFF (3+4+5+6+7) |  |  |  |
|  | 1.3 |  |  |
| 9. TAXI |  |  |  |
| 10. REQUIRED RAMP |  |  |  |
| 11. ACTUAL RAMP |  | ENDURANCE |  |
| 12. UNIDENTIFIED EXTRA |  |  |  |
| 13. MIN DIV OR (4+5+6) REQ OVHD DEST |  |  |  |

EXTRACT OF AF FORM 4116

General Planning (Con’t)

FUEL PLANNING

Figure 11.1. AF Form 4116 Fuel Load Components.

| 1. ENROUTE | Fuel for flight from departure/EAR to EAR/overhead destination or initial penetration fix at cruise altitude (including time for planned search, recovery, approach, climb, when applicable) |
| --- | --- |
| 2. ENROUTE RESERVE | 10% of flight time over a Category I route or segment, not to exceed 1+00 fuel at normal cruise. For search missions, 10% of flight time for that portion with inadequate NAVAIDS from search point to a Category II route or segment. Compute at terminal fuel flow. |
| 4. ALTERNATE AND MISSED APPROACH | Alternate: Fuel for flight time from overhead destination of EAR to alternate/abort airfield, or most distant alternate when two are required. Compute at terminal fuel flow. Add 10% reserve when time to an alternate exceeds 1+30. Missed Approach: 2200 pounds. Required if destination is below ceiling minimums, but above visibility minimums. |
| 5. HOLDING | Used when alternate not available, or located in Alaska, or at latitudes greater than 59 degrees N/S. All aircraft except EC-130E Commando Solo use 3500 pounds (EC-130E may use 2000 pounds). May compute using terminal fuel flow for 45 minutes. |
| 6. APPROACH/ LANDING | Approach: 1000 pounds. Entry always required. Minimum Landing fuel: 6000 pounds. Entry always required. This fuel will not be included in BURNOFF. |
| 7a. Pressurization Loss | Additional fuel for pressure loss at ETP, used when pressurized, carrying passengers, & aircraft oxygen not available to the passengers. Compute at 1000 pounds/hour for “T” time. If computed fuel is less than item 2, no entry is required here. If computed fuel exceeds item 2, add the difference here. |
| 7b. Stored Fuel | Ramp fuel for succeeding legs without refueling. |
| 7c. Off-Course Maneuvers | Fuel for anticipated off-course maneuvering for terrain, thunderstorm avoidance, ATC requirements. Compute at 100 pounds per minute for departure, 50 pounds per minute for en route fuel. |
| 7d. Icing | 500 pounds for each hour of anticipated icing. |

General Planning (Con’t)

FUEL PLANNING

| 7e. Known Holding Delays | Fuel for anticipated or planned holding time. Compute at terminal fuel flow. |
| --- | --- |
| 7f. Insufficient or Unreliable NAVAIDS | 1000 pounds maximum. Add for insufficient or unreliable NAVAIDS at destination and /or on the IFR track. |
| 7g. Helicopter Offload | Fuel planned for helicopter air refueling offload. |
| 9. TAXI AND RUNUP | Normally 1300 pounds. For known taxi delays or additional engine-running ground time in excess of 20 minutes, add 50 pounds per minute. |
| 12. UNIDENTIFIED EXTRA | Difference between required ramp and actual ramp fuel. Should not exceed 2200 pounds. (for fuel conservation purposes) |
| 13. REQUIRED OVER DESTINATION | Total of items 4, 5, and 6. Will never be less than 7000 pounds. |
| BURNOFF | Total en route fuel (#1) plus 1000 pounds (approach fuel) plus taxi and runup (39) plus identified extra fuel (#7) (except stored fuel). Entry Optional. |

General Planning (Con’t)

Minimum Runway Requirements

PEACETIME RESTRICTIONS: Compute landing performance with two-engine reverse, two-engine ground idle, and full brakes.  Do not use runways less than 3,000 feet long.

# NORMAL TAKEOFF & LANDING

| Conditions | Takeoff | Landing |
| --- | --- | --- |
| RVR (VIS) less than 40 (¾) | Critical Field Length | Landing distance from 50 ft over the threshold + 1,000 ft |
| RVR (VIS) equal to or greater than 40 (¾) | Critical Field Length | Landing distance from 50 ft over the threshold + 500 ft |

NOTE: If the runway length available for landing is less than required above, basic proficiency pilots may use landing ground roll plus 1,000 feet as minimum runway length when approved by the OPG/CC.  In this case, modify the landing to touchdown in the first 500 feet of runway.

# ASSUALT TAKEOFF & LANDING

| Takeoff | Landing |
| --- | --- |
| Minimum field length for maximum effort takeoff (corrected for Vmca if necessary) | Ground Roll + 500 ft |

NOTE: Climb out at Vmca or Vmeto depending on mission requirements/runway environment.  Climb until clear of real or simulated obstacle at Vmca + 10 kts or max effort obstacle clearance speed if using Vmeto.  Decision to takeoff at Vmca or Vmeto will be the pilot’s based on a consideration of all available data including weather, runway length, TOLD, Vmeto, Vref, Vmca, Vmcg, applicable airfield survey, and a review of hazards, obstructions, and terrain.

# RUNWAY AND TAXIWAY WIDTH

| Type of Operation | Runway | Taxiway |
| --- | --- | --- |
| Normal | 80 ft/25 m | 30 ft/9 m |
| Assault | 60 ft/19 m | 30 ft/9 m |

NOTE: The OPG/CC may approve use of other than hard-surfaced runways or taxiways and may waive runway and taxiway width requirements.

Overwater Planning

OVERWATER FLIGHT PLANNING

- I. FLIGHT PLANNING

- PUBLICATIONS

a. Foreign Clearance Guide (FCG), http://www.fcg.pentagon.mil

b. Airfield Suitability Reports, https://afd.scott.af.mil

- FLIGHT INFORMATION

a. Weather (Ensure WX forecast includes enough time if have to turn around at ETP); http://www.baseops.net/metro.html, http://www.nws.noaa.gov

b. NOTAMs (Don’t forget KGPS, FIRS-Oceanic), https://www.notams.jcs.mil/

3. 	FLIGHT CHARTS

a. Departure/Approach chart-required CHUMmed ONC scale or larger within 25 NM of airfield on CAT II or Airways routes for flight following

b. Maintain a plotting chart on all Category I routes

c. Name and date is required on chart in vicinity of departure or coast out point

d. Check CHUM for all charts (GNC, JNC, etc.). Not required to write CHUM information on high-level charts

4. 	FUEL PLANNING

a. Use MAX Continuous Power for cruise altitude ceiling for planning purposes, will actually fly 970 degrees TIT to cruise altitude (Chapter 5)

b. Find Terminal Fuel Flow

c. Approach/Landing -planning purposes always 7000 lb. Chapter 4 says to land with 8,000 lbs calculated main tank fuel.

d. Identified Extra

e. ETP (required if time between LSAF and FSAF is 5 hours or more)

(1) Suitable airfield--within 100 NM of flight planned course meeting weather, fuel, and C-130 runway requirements

(2) 5,000 feet long, 60 feet wide, and must have single tandem weight bearing capacity for planned gross weight

Overwater Planning (Con’t)

- II. ENROUTE PROCEDURES

1. 	HEADING DEVIATION

a. As soon as practical after initial level off or coast out

b. Compasses need to be within 2 degrees of INS heading

c. Recommend doing compass deviation check on the runway before takeoff.

2. 	TRUE AIRSPEED CHECK

a. Done within 1 hour of initial cruise

b. and a change greater than 4,000 feet after initial check

3. 	FIX INTERVAL

a. Time between recorded positions will not exceed 1+20

b. Under normal conditions, 1 hour or less is the recommended time between recorded positions

4. 	INFLIGHT FUEL MANAGEMENT

a. Time requirement--record at departure, level off, and regular time intervals not to exceed 1 hour

b. Exception--time between level off entry and next entry maybe up to 1+30

5. 	FORM 4116 COMPLETION

a. Actual Side--only required items for a full line entry when Doppler, INS, SCNS, or GPS are providing reliable information are: Time, Position, Average TH, Average TAS, and spot winds.

b. Proposed Side

(1) Log best-known information

(2) All blocks required

(3) Exceptions:

(a.) On portions of flight designated Category II

(b.) On departure, approach, and other segments of flight under positive radar control

(c.) When numerous alters for weather, traffic, etc. make the accomplishment of the entries impractical

Overwater Planning (Con’t)

FUEL MANAGEMENT

| RETA DEST | -- | 15:44 | 15:44 |  |
| --- | --- | --- | --- | --- |
| TIME | T/O 10:01 | L/O 10:22 | 11:30 |  |
| GROSS WT. | 135.6 | 133.8 | 128.1 |  |
| PAGE/BURN RT | -- | 31 | 31 |  |
| FUEL REM | 46.6 | 44.8 | 39.1 |  |
| OH FUEL |  | -8.6 | -8.6 |  |
| DIFF |  | 36.2 | 30.5 |  |
| FUEL ETE |  | 7+12 | 6+01 |  |
| ETE DEST |  | 5+22 | 4+14 |  |
| EXT TIME |  | 1+50 | 1+47 |  |
| TEMP DEV/ALT |  | +14 | +14 |  |

AIRSPEED CALIBRATION EXAMPLE

| TIME | 10:27 |  |
| --- | --- | --- |
| ALT (PA) | FL 180 |  |
| IOAT | +2 |  |
| TOAT | -7 |  |
| IAS | 217 |  |
| CAS | 217 |  |
| EAS | 214 |  |
| TAS | 291 |  |
| -ITAS | 288 |  |
| CORR | +3 |  |

COMPRESSIBILITY CORRECTION

| TAS | IOAT CORRECTION | TAS | IOAT CORRECTION |
| --- | --- | --- | --- |
| 190-210 | -4 | 277-288 | -9 |
| 210-230 | -5 | 288-300 | -10 |
| 230-246 | -6 | 300-310 | -11 |
| 246-263 | -7 | 310-319 | -12 |
| 263-277 | -8 | 319-325 | -13 |

FUEL BASICS

FUEL TANKS

| TANKS WITH FOAM | U.S. GALLONS | JP-8 POUNDS |
| --- | --- | --- |
| TANK NO. 1 | 1222 | 8310 |
| TANK NO. 2 | 1126 | 7657 |
| LEFT AUX | 855 | 5814 |
| LEFT EXTERNAL | 1309 | 8901 |
| RIGHT EXTERNAL | 1309 | 8901 |
| RIGHT AUX | 855 | 5814 |
| TANK NO. 3 | 1126 | 7657 |
| TANK NO. 4 | 1222 | 8310 |
| LEFT FUSELAGE | 1700 | 11,560 |
| RIGHT FUSELAGE | 1700 | 11,560 |
| TOTAL | 12,424 | 84,484 |

FUEL MANAGEMENT

PRIMARY FUEL MANAGEMENT:

1. Maximum usable fuel weights for the wing tanks are those shown in table above.

2. With air refueling pods installed, tanks No. 2 & 3 always contain 0 up to 500 lbs. more fuel per tank than tanks No. 1 &4.

3. The main tanks are full, except for the fuel used for taxi and takeoff, when the external and/or auxiliary tanks contain usable fuel.

Note: Main tanks considered full for purposes of Primary Fuel Management at a minimum of 6500 lbs. of fuel in outboard tanks and a minimum of 6900 lbs. in the inboard tanks, with pods installed.

4. Fuel asymmetry is within the limits specified by the AIRSPEED LIMITATIONS and FUEL UNBALANCE LIMITS.

SECONDARY FUEL MANAGEMENT:

Defined as doesn’t meet the requirements of Primary Fuel Management. This will occur any time there is usable fuel in the external tanks and/or auxiliary tanks and main tanks are partly filled.

Caution: Airplane should not be flown with tanks No. 1 & No. 4 empty only in an emergency.

Note: Recommend the same weight difference required for Primary Fuel Management between inboard and outboard fuel tanks be observed for Secondary Fuel Management.

Low Level Planning

Low Level Planning (Con’t)

TIMING TRIANGLES

| TURN | LEGS | LOSE (SEC) | XTK (NM) |
| --- | --- | --- | --- |
| 15/30/15 | 1 MIN | 7.5 | 1.0 |
| 30/60/30 | 1 MIN | 15-17 | 1.5 |
| 45/90/45 | 1 MIN | 30-35 | 2.5 |
| 60/120/60 | 1 MIN | 60 | 3.5 |

210 IAS:  AVERAGE AIRSPEED ADJUSTMENT CHART

(Time to Adjust:Ahead/Behind)

| ##### Minutes Ahead/Behind |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Time Remaining to Initial Point | 1+20 | 2 | 5 | 8 | 11 | 13 | 16 | 19 | 21 |
|  | 1+15 | 3 | 6 | 9 | 12 | 15 | 17 | 20 | 23 |
|  | 1+10 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
|  | 1+05 | 3 | 7 | 10 | 13 | 16 | 19 | 23 | 26 |
|  | 1+00 | 4 | 7 | 10 | 14 | 17 | 21 | 24 | 28 |
|  | 55 | 4 | 7 | 11 | 15 | 19 | 23 | 28 | 30 |
|  | 50 | 5 | 9 | 13 | 17 | 22 | 26 | 30 | 34 |
|  | 45 | 5 | 10 | 15 | 18 | 24 | 28 | 33 | 38 |
|  | 40 | 5 | 10 | 16 | 21 | 26 | 32 | 37 |  |
|  | 35 | 6 | 12 | 18 | 24 | 30 | 36 |  |  |
|  | 30 | 7 | 14 | 21 | 28 | 35 |  |  |  |
|  | 25 | 8 | 17 | 25 | 33 |  |  |  |  |
|  | 20 | 10 | 21 | 31 |  |  |  |  |  |
|  | 15 | 14 | 28 |  |  |  |  |  |  |
|  | 10 | 21 |  |  |  |  |  |  |  |
|  | 5 | 40 |  |  |  |  |  |  |  |

AIRSPEED ADJUSTMENT FORMULA

1.  MB-4 	_____210	       	____Read off New IAS___

Time remaining to IP	Add minutes Behind/

Subtract minutes Ahead

Low Level Planning (Con’t)

LOW LEVEL ENROUTE TIMING CHART

| NM | 200 | 210 | 220 | 230 | 240 |
| --- | --- | --- | --- | --- | --- |
| 50 | 15:00 +1:22 | 14:17 +:39 | 13:38 | 13:01 -:37 | 12:30 -1:08 |
| 45 | 13:30 +1:17 | 12:51 +:35 | 12:16 | 11:44 -:32 | 11:15 -1:01 |
| 40 | 12:00 +1:07 | 11:26 +:33 | 10:53 | 10:26 -:27 | 10:00 -:53 |
| 35 | 10:30 +:58 | 10:00 +:28 | 9:32 | 9:08 -:24 | 8:45 -:47 |
| 30 | 9:00 +:49 | 8:34 +:23 | 8:11 | 7:49 -:22 | 7:30 -:41 |
| 25 | 7:30 +:41 | 7:09 +:20 | 6:49 | 6:31 -:18 | 6:15 -:34 |
| 20 | 6:00 +:33 | 5:43 +:16 | 5:27 | 5:13 -:14 | 5:00 -:27 |
| 15 | 4:30 +:25 | 4:17 +:12 | 4:05 | 3:55 -:10 | 3:45 -:20 |
| 10 | 3:00 +:16 | 2:51 +:07 | 2:44 | 2:37 -:07 | 2:30 -:14 |
| 05 | 1:30 +:08 | 1:26 +:04 | 1:22 | 1:18 -:04 | 1:15 -:07 |
| 04 | 1:12 | 1:08 | 1:05 | 1:03 | 1:00 |
| 03 | 0:54 | 0:51 | 0:49 | 0:47 | 0:45 |
| 02 | 0:36 | 0:34 | 0:33 | 0:31 | 0:30 |
| 01 | 0:18 | 0:17 | 0:16 | 0:16 | 0:15 |

TIME ADJUST

(For turns 30-degree bank at 220 knots ground speed)

Degrees		Minutes

000 - 045		0.0

046 - 070		0.1

071 - 082		0.2

083 - 093		0.3

094 - 103		0.4

104 - 112		0.5

113 - 121		0.6

122 - 131		0.7

132 - 139		0.8

140 - 146		0.9

147 - 154		1.0

Low Level Planning (Con’t)

DIAMETER/RADIUS OF TURN TABLE

20 DEGREE OF BANK

| SPEED | DIAM. (NM) | RADIUS (NM) | SPEED | DIAM. (NM) | RADIUS (NM) |
| --- | --- | --- | --- | --- | --- |
| 120 | 1.2 | 0.6 | 220 | 3.9 | 2.0 |
| 130 | 1.4 | 0.7 | 230 | 4.3 | 2.1 |
| 140 | 1.6 | 0.8 | 240 | 4.6 | 2.3 |
| 150 | 1.8 | 0.9 | 250 | 5.0 | 2.5 |
| 160 | 2.1 | 1.0 | 260 | 5.4 | 2.7 |
| 170 | 2.3 | 1.2 | 270 | 5.9 | 3.0 |
| 180 | 2.6 | 1.3 | 280 | 6.3 | 3.2 |
| 190 | 2.9 | 1.5 | 290 | 6.8 | 3.4 |
| 200 | 3.2 | 1.6 | 300 | 7.2 | 3.6 |
| 210 | 3.5 | 1.8 | 310 | 7.7 | 3.9 |

30 DEGREE OF BANK

| SPEED | DIAM. (NM) | RADIUS (NM) | SPEED | DIAM. (NM) | RADIUS (NM) |
| --- | --- | --- | --- | --- | --- |
| 120 | 0.7 | 0.4 | 220 | 2.5 | 1.3 |
| 130 | 0.9 | 0.5 | 230 | 2.7 | 1.4 |
| 140 | 1.0 | 0.5 | 240 | 2.9 | 1.5 |
| 150 | 1.1 | 0.6 | 250 | 3.2 | 1.6 |
| 160 | 1.3 | 0.7 | 260 | 3.4 | 1.7 |
| 170 | 1.5 | 0.8 | 270 | 3.7 | 1.9 |
| 180 | 1.6 | 0.8 | 280 | 4.0 | 2.0 |
| 190 | 1.8 | 0.9 | 290 | 4.3 | 2.2 |
| 200 | 2.0 | 1.0 | 300 | 4.6 | 2.3 |
| 210 | 2.2 | 1.1 | 310 | 4.9 | 2.5 |

Tactical Planning

Illumination Conditions

BMNT- “Beginning Morning Nautical Twilight”, Before sunrise, when the sun is 12 degrees below horizon. Illumination is poor, only vague outlines distinguishable.

BMCT- “Beginning Morning Civil Twilight”, Before sunrise, when the sun is 6 degrees below horizon. On a clear day, outdoor “visibility” activity possible.

EECT- “Ending Evening Civil Twilight”, After sunset, when the sun is 6 degrees below horizon. On a clear day, outdoor “visibility” activity possible.

EENT- “Ending Evening Nautical Twilight”, After sunset, when the sun is 12 degrees below horizon. Illumination is poor, only vague outlines distinguishable.

DMA Map/Chart Accuracies

TOPO & AERONAUTICAL PRODUCTS

| CHART | SCALE | HCE 90% | VLE 90% |
| --- | --- | --- | --- |
| City | 1: 25,000 | 50M | +/- 20M |
| Topo | 1: 50,000 | 50M | +/- 20M |
| Topo | 1: 100,000 | 50M | +/- 20M |
| JOG-A/G | 1: 250,000 | 250M | +/- 100M |
| TPC | 1: 500,000 | 1,000M | +/- 150M |
| ONC | 1: 1,000,000 | 2,000M | +/- 650M |

HCE: Horizontal Circular Error	VLE: Vertical Linear Error

Note: Charts fall into four levels of accuracy, depending on OPLAN and national interests when compiled. For security reasons, charts are not marked to indicate their level of accuracy, as it indicates areas of heightened national concern. If true accuracy levels are required, they can be acquired from DMA. When time doesn’t permit, however, the above matrices provide worst-case values.

Tactical Planning (Con’t)

JOINT PLANNING TERM REFERENCE

TYPES OF OPERATIONS

OVERT Operations: Visible ops; clearly traceable to U.S.

COVERT Operations: Visible ops; fact they occurred is clearly evident. However, they are carefully designed not to be traceable to U.S.

Clandestine Operations: Invisible ops; fact they occurred is carefully hidden. Since it “did not occur,” it is not traceable to the U.S.

TYPES OF ORDERS (In Order of Receipt)

WARNING ORDER: Directive used by commanders to advise subordinates of impending action. JCS may use as planning directive to initiate Crisis Action Procedures (see below) and Course of Action (COA) development. (JDS Users Manual, Vol. 1)

ALERT ORDER: Formal directive from JCS. Follows an NCA decision that military forces may be required and provides essential guidance for planning. (JDS Users Manual, Vol. 1)

DEPLOYMENT ORDER: Order issued by a competent authority to prepare forces for movement (e.g. increases deployment posture of units), or to actually move forces. (Joint Pub 5-02.4)

EXECUTION ORDER: Order issued by a competent authority to initiate operations. (Joint Pub 5-02.4)

CRISIS ACTION PROCEDURES

1. Situation Development		4. Course of Action Selected

2. Crisis Assessment		5. Execution Planning

3. Course of Action Development	6. Execution

DAY/HOUR CODES

C-Day: Day on which a deployment operations begin. (Joint Pub 5-02.1)

D-Day: Day on which a particular operations begin. (Joint Pub 1-02

M-Day: Day on which mobilization operations begin. (Joint Pub 1-02)

K-Hour: Specific hour on D-Day an operation begins. (Joint Pub 1-02)

L-Hour: Specific hour on C-Day a deployment begins. (Joint Pub 5-02.1)

COMMAND STRUCTURES (Ex. J2, S3, etc.)

J- Joint Command (Multi-Service)		1- Manpower & Personnel

G- General Officer lead Command		2. Intelligence

S- Non-General lead Command		3. Operations

C- Combined Command (Multi-national)	4. Logistics

N- Naval Command (Informal)		5. Plans & Policy

A- Air Command (Informal)			6. Communications

Tactical Planning (Con’t)

Runway Definitions (Non-Permanent Surface)

Graded Runway

ys: Constructed of crushed stone of gravel, these runways will sustain repeated usage. Such runways are often used as base construction for prefabricated, improved, or permanent runways.

Sod Runways: Selectively cultivated runways that are capable of withstanding pressures up to 0.48 MPa without forming ruts during takeoff, landing, and taxiing of aircraft. Sod runways, when dry, will support limited operations.

Unprepared Runways: A selected landing area, the surface of which has not been hardened, but is smooth and free of trees, brush, large rocks, bumps, and holes.

Unsurfaced Runways:  Unsurfaced (graded sod and unprepared) runways that are considered acceptable if the elastic deformation caused by aircraft does not exceed 1-1.6 inches and permanent deformation (ruts) do not exceed 2-4 inches.

Tactical Planning (Con’t)

This Tactical Information is UNCLASSIFIED, to find classified information use the ATTP 3-1 Vol. 2

THREAT DATA

Tactical Planning (Con’t)

THREAT DATA

Tactical Planning (Con’t)

THREAT DATA

Tactical Planning (Con’t)

THREAT DATA

Tactical Planning (Con’t)

GOB Symbols

Regular Infantry

Airborne Infantry

Armored

Mechanized Infantry

Airborne

Motorized Infantry

Calvary & Recon

Mountain

Naval Infantry

Amphibious

Special Forces

Irregular (Guerrilla)

Aviation

Coastal Artillery

Air Mobile (Helo)

Field Artillery

Air Defense Artillery

Missile; SSM

Missile; Anti-Tank

Radar

Signal

Electronic Warfare

Bridging

Engineer

Chemical Biological

Psyops

Medical

Maintenance

Supply

Military Police

SF

I

MP

## Unit Size Markings

XXXXX	=	Army Group	III	=	Regimental Group

XXXX	=	Army	II	=	Battalion/Squadron

XXX	=	Corps	I	=	Company/Troop/Battery

XX	=	Division	•••	=	Platoon

X	=	Brigade	••	=	Section

•	=	Squad

### GOB Unit Examples

5

MP

XXX	XX	II

# MP

II

2		1		9		2-508

5th Corps	2nd Infantry	1st Med Battalion	2nd MP Battalion

HQ	Division	9th Brigade	508th Brigade

Objective Area Planning

SCA/Airdrop Slowdown Timing Chart

| GSNM | 90 | 95 | 100 | 105 | 110 | 115 | 120 | 125 | 130 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | :40 | :38 | :36 | :34 | :33 | :31 | :30 | :29 | :28 |
| 2 | 1:20 | 1:16 | 1:12 | 1:09 | 1:05 | 1:03 | 1:00 | :58 | :55 |
| 3 | 2:00 | 1:54 | 1:48 | 1:43 | 1:38 | 1:34 | 1:30 | 1:26 | 1:23 |
| 4 | 2:40 | 2:32 | 2:24 | 2:17 | 2:11 | 2:05 | 2:00 | 1:55 | 1:51 |
| 5 | 3:20 | 3:09 | 3:00 | 2:51 | 2:44 | 2:37 | 2:30 | 2:24 | 2:19 |
| 6 | 4:00 | 3:47 | 3:36 | 3:26 | 3:16 | 3:08 | 3:00 | 2:53 | 2:46 |
| 7 | 4:40 | 4:25 | 4:12 | 4:00 | 3:49 | 3:39 | 3:30 | 3:22 | 3:14 |

| GSNM | 135 | 140 | 145 | 150 |
| --- | --- | --- | --- | --- |
| 1 | :27 | :26 | :25 | :24 |
| 2 | :53 | :51 | :50 | :48 |
| 3 | 1:20 | 1:17 | 1:14 | 1:12 |
| 4 | 1:47 | 1:43 | 1:39 | 1:36 |
| 5 | 2:13 | 2:09 | 2:04 | 2:00 |
| 6 | 2:40 | 2:34 | 2:29 | 2:24 |
| 7 | 3:07 | 3:00 | 2:54 | 2:48 |

Example:  Preplanned 	5 NM Slowdown For SCA

115 GS for 5 NM		  2:37

Rule of Thumb: Loss of 5 kt/sec

(215 GS-115 GS = 100 Kt difference5 kt/sec = 20 secs)

Subtract Fudge Factor -20 seconds 			- 0:20

Total	  2:17

Example:  Preplanned 	5 NM Slowdown For 140 GS Airdrop

140 GS for 5 NM		  2:09

(210 GS-140 GS = 70 5 kt/sec = 14 seconds)

Subtract Fudge Factor -14 seconds  			- 0:14

Total	  1:55

Objective Area Planning (Con’t)

DECELERATION DISTANCE

| ENROUTE TO 140 KIAS DIFF | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DECELERATION DISTANCE | 1.0 | 1.2 | 1.5 | 1.7 | 2.0 | 2.2 | 2.5 |

##### 200 YARD WIND CIRCLE CHART

| TTF | W/V | TTF | W/V |
| --- | --- | --- | --- |
| 8 | 45 | 35 | 10 |
| 9 | 40 | 36 | 10 |
| 10 | 36 | 37 | 10 |
| 11 | 32 | 38 | 9 |
| 12 | 30 | 39 | 9 |
| 13 | 27 | 40 | 9 |
| 14 | 25 | 41 | 9 |
| 15 | 24 | 42 | 8 |
| 16 | 22 | 43 | 8 |
| 17 | 21 | 44 | 8 |
| 18 | 20 | 45 | 8 |
| 19 | 19 | 46 | 8 |
| 20 | 18 | 47 | 8 |
| 21 | 17 | 48 | 7 |
| 22 | 16 | 49 | 7 |
| 23 | 15 | 50 | 7 |
| 24 | 15 | 51 | 7 |
| 25 | 14 | 52 | 7 |
| 26 | 14 | 53 | 7 |
| 27 | 13 | 54 | 7 |
| 28 | 13 | 55 | 6 |
| 29 | 12 | 56 | 6 |
| 30 | 12 | 57 | 6 |
| 31 | 11 | 58 | 6 |
| 32 | 11 | 59 | 6 |

Objective Area Planning (Con’t)

Airdrop Load Data

High Velocity (HVCDS)/12’, 22’, 26’ Ring Slot:

Altitudes:

100’ Plus Vertical distance for Load being dropped

Airspeed:

<120,000 lbs	130 kts

>120,000 lbs	140 kts

Winds:

High Velocity	Unlimited

Drop Zone Sizes:

To 3000 ft AGL	580 x 660 yds

Each additional row add 50 yards

Above 3000 ft AGL	+25 yds each side & 100 yards for every 1000 ft. increase

Forward Throw	550 yds

Personnel:

Altitudes:

Tactical Training	800 ft

SATB-P	500 ft

Emergency Bailout (Static Line)	400 ft

Emergency Bailout (Free Fall)	2000 ft

Airspeed:

Static Line	125*/130 kts

HALO		110 - 150 kts (130 Desired)

Winds:

AF Static Line (Land)	13 kts

AF Static Line (Water )	25 kts

AF MFF (Land)	18 kts

AF MFF (Water)	25 kts

AF Intentional Tree Jumps	22 kts

Drop Zone Sizes:

Minimum (Up to 1000 ft AGL)	600x600 yds

Additional Jumpers	+75 ft/Jumper

Forward Throw	250 yds

*-For PJ Deployment

Training Bundle:

Wind Limit	25 kts

Forward Throw	160 yds

Rescue Drops

MA-1/2 Kits	Min Alt. 300ft AWL Day/500 AWL Night

Parabundles (G-8, 7-100 lbs)	Min Alt. 300ft AWL Day/500 AWL Night

Parabundles (G13/14, 200-500 lbs)	Min Alt. 400ft AWL Day/500 AWL Night

Free-Fall Equipment	Min Alt. 150 ft AWL/500 AWL at Night

T-10C		Min Alt. 400ft AWL/500 AWL at Night

Deployment airspeed will be 130 KIAS for rescue drops.

Minimum altitude for night equipment airdrops is 500 AGL/AWL

Flap setting will be 50 percent up to 140,000 pounds aircraft gross weight, 70 percent at or above 140,000 pounds aircraft gross weight.

Objective Area Planning (Con’t)

Equipment Parachute Data

| US Army |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Description |  | Minimum Alt (AGL) | Drop Airspeed (KIAS) | Remarks |
| T- I 0 A/B/C MCI-I A/B/C | 35’ Parabolic (T-10) |  | 750 | 125 – 150 | Jump altitude providing reserve parachute capability in event of main parachute malfunction. All parachute canopies equipped with skirt anti-inversion net. |
|  | 35’ 11 Gore TU Cut-out (MCI-1) |  | 400 |  | Combat jump altitude providing no reserve parachute capability. Minimum altitude to evacuate aircraft in an airborne emergency using the main parachute. |
| MC-1 | 35’ Single Orifice |  | 3000-5000 | 110 – 150 | HALO free fall parachute with single orifice/slip riser parachute canopy and Type F- I B or Type FF- I Automatic Parachute Ripcord Release. Minimum altitude based on a reserve parachute capability in the event of main parachute malfunction. Minimum opening altitude setting for Type F- I B release is 5000 feet AGL.; for Type FF-I release, 3000 feet AGL. |
| MC-3 | 24’ Para Cmdr |  | 2500 | 110 – 150 | HALO free fall parachute with Para-Commander canopy and Type FF-2 automatic parachute ripcord release Minimum altitude based on a reserve parachute capability in the event of main parachute malfunction. Type FF-2 release imposes no minimum actuation altitude restriction. Minimum planned parachute actuation altitude providing no reserve parachute capability |
|  |  |  | 1800 |  |  |
| ## US Air Force |  |  |  |  |  |
| Type |  | Description |  | Remarks |  |
| S-10/S-11/S- 12 |  | 35' Single Orifice |  | Used for special operations missions. Static lines used. |  |
| MCI-2 |  | 35' 1 1 Gore TLJ Cut-out |  | HALO parachutes. Refer to operational directives for altitude and airspeed restrictions. |  |
| MCI-3 |  | 24' Para Cmdr |  |  |  |
| Jumbo |  | 28' Para Cmdr |  |  |  |

Objective Area Planning (Con’t)

Equipment Parachute Data

| Type | Description |  |  |  | Minimum Alt (AGL) | Drop Airspeed (KIAS) |  | #/ Wt Range (Pounds) |  |  |  | Remarks |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | G-13 | 24' |  |  |  | 400 |  | 125 - 150 |  | 200 - 500 |  | Single parachute cargo container loads |  |
|  |  |  |  |  |  |  | 400 |  |  |  | 501 - 1000 |  | Cluster of two parachutes |  |
|  |  |  |  |  |  |  | 500 |  |  |  | 1001 - 1500 |  | Cluster of three parachutes |  |
|  |  | G-14 |  | 34' |  |  | 300 |  | 125 - 150 | 200 - 500 |  |  | Single parachute cargo container loads |  |
|  |  |  |  |  |  |  | 300 |  |  | 501 - 1000 |  |  | Cluster of two parachutes |  |
|  |  |  |  |  |  |  | 400 |  |  | 1001 - 1500 |  |  | Cluster of three parachutes |  |
|  |  | 12’ High Velocity |  | 12' Ring Slot |  |  | 400 |  | 125 - 150 | 151 - 500 |  |  | Intended for high altitude airdrops |  |
|  |  | 26' High Velocity |  | 26' Ring Slot |  |  | 500 |  | 125 - 150 250 For HSLLADS | 501 - 2200 |  |  | Intended for high altitude airdrops |  |
|  |  | G-12D G-12E |  | 64 |  |  | 400 (G-12D) 300 (G-12E) |  | 125 - 150 | 501 - 2200 |  |  | Single parachute cargo container loads airdropped using the container delivery system |  |
|  |  |  |  |  |  |  | 450 |  |  | 2145 -3500 |  |  | Applies to platform, extracted cargo/equipment using cluster of two or three parachutes |  |

Objective Area Planning (Con’t)

Objective Area Planning (Con’t)

Objective Area Planning (Con’t)

SIGHT/OFFSET ANGLES

|  |  | CARP TO PI DISTANCE (YARDS) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 150 | 200 | 250 | 300 | 350 | 400 | 450 | 500 | 550 | 600 |
| DISTANCE ABOVE THE PI (FT) | 100 | 57 | 72 | 77 | 81 | 82 | 84 | 84 | 85 | 86 | 86 | 87 | 87 |
|  | 200 | 37 | 57 | 66 | 72 | 75 | 77 | 79 | 80 | 81 | 82 | 83 | 84 |
|  | 300 | 27 | 45 | 57 | 64 | 68 | 72 | 74 | 76 | 77 | 79 | 80 | 81 |
|  | 400 | 21 | 37 | 49 | 57 | 62 | 66 | 69 | 72 | 73 | 75 | 76 | 77 |
|  | 500 | 17 | 31 | 42 | 51 | 57 | 61 | 65 | 67 | 70 | 72 | 73 | 74 |
|  | 600 | 14 | 27 | 37 | 45 | 51 | 57 | 60 | 64 | 66 | 68 | 70 | 72 |
|  | 700 | 12 | 23 | 33 | 41 | 47 | 52 | 57 | 60 | 63 | 65 | 67 | 69 |
|  | 800 | 11 | 21 | 29 | 37 | 43 | 48 | 53 | 57 | 59 | 62 | 64 | 66 |
|  | 900 | 10 | 18 | 27 | 34 | 40 | 45 | 49 | 53 | 57 | 59 | 61 | 64 |
|  | 1000 | 9 | 17 | 24 | 31 | 37 | 42 | 46 | 50 | 53 | 57 | 59 | 61 |
|  | 1100 | 8 | 15 | 22 | 29 | 34 | 39 | 43 | 47 | 51 | 54 | 57 | 59 |
|  | 1200 | 7 | 14 | 21 | 27 | 32 | 37 | 41 | 45 | 48 | 51 | 54 | 57 |
|  | 1300 | 7 | 13 | 19 | 25 | 30 | 35 | 39 | 43 | 46 | 49 | 52 | 54 |
|  | 1400 | 6 | 12 | 18 | 23 | 28 | 33 | 37 | 41 | 44 | 47 | 50 | 52 |

|  |  | CARP TO PI DISTANCE (YARDS |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DISTANCE ABOVE THE PI (FT) |  | 650 | 700 | 750 | 800 | 850 | 900 | 950 | 1000 | 1050 | 1100 | 1150 | 1200 |
|  | 100 | 87 | 87 | 87 | 88 | 88 | 88 | 88 | 88 | 88 | 88 | 88 | 88 |
|  | 200 | 84 | 85 | 85 | 85 | 85 | 86 | 86 | 86 | 86 | 86 | 87 | 87 |
|  | 300 | 81 | 82 | 82 | 83 | 83 | 84 | 84 | 84 | 85 | 85 | 85 | 85 |
|  | 400 | 78 | 79 | 80 | 81 | 81 | 81 | 82 | 82 | 83 | 83 | 83 | 84 |
|  | 500 | 76 | 77 | 78 | 78 | 79 | 80 | 80 | 81 | 81 | 81 | 82 | 82 |
|  | 600 | 73 | 74 | 75 | 76 | 77 | 77 | 78 | 79 | 79 | 80 | 80 | 81 |
|  | 700 | 70 | 72 | 73 | 74 | 75 | 75 | 76 | 77 | 77 | 78 | 79 | 79 |
|  | 800 | 68 | 69 | 70 | 72 | 73 | 74 | 74 | 75 | 76 | 76 | 77 | 77 |
|  | 900 | 65 | 67 | 68 | 69 | 70 | 72 | 73 | 73 | 74 | 75 | 75 | 76 |
|  | 1000 | 63 | 65 | 66 | 67 | 69 | 70 | 70 | 72 | 72 | 73 | 74 | 75 |
|  | 1100 | 60 | 62 | 64 | 65 | 67 | 98 | 69 | 70 | 71 | 72 | 73 | 73 |
|  | 1200 | 59 | 60 | 62 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 |
|  | 1300 | 57 | 58 | 60 | 62 | 63 | 64 | 65 | 67 | 68 | 68 | 69 | 70 |
|  | 1400 | 54 | 57 | 58 | 60 | 61 | 63 | 64 | 65 | 66 | 67 | 68 | 69 |

CAUTION: Adjust table value for CDS- Subtract 7 degrees.

NOTE: Enter table with Altitude above PI.

NOTE: For FLIR values subtract table value from 90 degrees.

Objective Area Planning (Con’t)

WINDOW SIGHT ANGLES

NORMAL

75	Bottom of Latch

71	Bottom of Windscreen

67	Top of window

65	Center of window

All angles based on eyes level with bottom of handhold

62	Bottom of window

61	Top of lower window

58	Center of lower window

55	Bottom of lower window

AIRSPEED ADJUSTMENT DISTANCE TO MIN SLOWDOWN DISTANCE

| DIFF +/- FROM 210 KIAS | 10 | 20 | 30 | 40 |
| --- | --- | --- | --- | --- |
| DISTANCE ADJUSTMENT | 0.3 | 0.5 | 0.8 | 1.0 |

DISTANCE PER AIRSPEED (1 MIN / 30 SEC)

| GS | 110 | 120 | 125 | 130 | 135 | 140 | 145 | 150 | 155 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 SEC | 1.0 | 1.0 | 1.1 | 1.1 | 1.2 | 1.2 | 1.3 | 1.3 | 1.3 |
| 1 MIN | 1.9 | 2.0 | 2.1 | 2.2 | 2.3 | 2.4 | 2.5 | 2.5 | 2.6 |

Objective Area Planning (Con’t)

Objective Area Planning (Con’t)

SCA BRIEFING GUIDE

1.  Field / LZ

2.  TDZE

3.  Authentication / Markings

4.  Route to Final / Go Around Pattern / Altitude

5.  MSA / ESA

6.  Altimeter Setting

7.  Final Approach:  	Course

Altitude

Hazards/Obstacles

8.  Slow to Threshold Speed (NM to TDZ)

9.  RN - FLIR usage and update coordination (Pilot/LN)

10.  Glideslope

11.  Descent Point (NM to TDZ)

12.  Descent Rate

13.  Glideslope 1/2 NM Points:  MSL / AGL

14.  Minimum Descent Altitude:  HAT / MSL

15.  Missed Approach Point

16.  Missed Approach Procedures

17.  Crew Duties:

GLIDESLOPE PLANNING CHART (HAT) ALTITUDE

| NM GS | 0.5 | 1.0 | 1.5 | 2.0 | 2.5 | 3.0 | 3.5 | 4.0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.0 | 106 | 212 | 318 | 424 | 530 | 636 | 742 | 848 |
| 2.5 | 133 | 266 | 399 | 532 | 665 | 798 | 931 | 1064 |
| 2.83 | 150 | 300 | 450 | 600 | 750 | 900 | 1050 | 1200 |
| 3.0 | 159 | 318 | 477 | 636 | 795 | 954 | 1133 | 1272 |
| 3.5 | 186 | 372 | 558 | 744 | 930 | 1116 | 1302 | 1488 |
| 4.0 | 212 | 424 | 636 | 848 | 1060 | 1272 | 1484 | 1696 |
| 4.5 | 239 | 478 | 717 | 956 | 1195 | 1434 | 1673 | 1912 |
| 5.0 | 265 | 530 | 795 | 1080 | 1325 | 1590 | 1855 | 2120 |

Example: Glideslope 2.0, Distance 1.0 NM = 209 feet plus Landing Zone Elevation = MSL

MSL minus 1.0 NM spot terrain = Altitude AGL

Objective Area Planning (Con’t)

DESCENT RATE (PER MINUTE) CHART

| Gnd SP Gld SL | 105 | 110 | 115 | 120 | 125 | 130 | 135 | 140 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.0 | 366 | 383 | 401 | 418 | 435 | 453 | 470 | 488 |
| 2.5 | 459 | 480 | 502 | 524 | 546 | 568 | 590 | 611 |
| 2.83 | 518 | 543 | 567 | 592 | 617 | 641 | 666 | 691 |
| 3.0 | 550 | 576 | 602 | 628 | 654 | 680 | 707 | 733 |
| 3.5 | 641 | 671 | 701 | 732 | 762 | 793 | 824 | 854 |
| 4.0 | 733 | 768 | 803 | 838 | 873 | 908 | 943 | 978 |
| 4.5 | 824 | 863 | 903 | 942 | 981 | 1020 | 1060 | 1099 |
| 5.0 | 917 | 961 | 1004 | 1048 | 1092 | 1135 | 1179 | 1223 |

DESCENT RATE FORMULA

1.  MB-4	Glideslope	      =	  	(FT per NM)

57.3				  60

2.  MB-4	Glideslope 		= 	(FT per Min)

(Drift window)		 	Groundspeed

START DESCENT POINT FORMULA

1.  MB-4	Feet per NM	=	   Low Level AGL

1.0			(Start Descent Point)

Objective Area Planning (Con’t)

HAR Emergency Procedures

NO VISUAL CONTACT

Visual contact not established by 1 NM, but

Electronic contact is maintained

- Turn right 450 of track heading

- Flaps 70%

- Slow to refueling airspeed

- Call “No Visual Contact”

- After 30 seconds, turn left to track heading

and remain 1-2 NM behind

# CONTACT LOST

Visual and electronic contact is lost after tanker is

within 3 NM of receiver

- Turn right 450 of track heading

- Maintain refueling altitude

- Call “ Contact Lost”

- After 30 seconds accomplish Post Air

Refueling Checklist

# LOST VISUAL CONTACT

After tanker assumes formation lead

1.     Lead Tanker:

- Calls “Execute” with base heading and MSA

- Accelerate to cruise airspeed and climb to MSA

- Spare Tanker:

- Turns right 450

- Accelerate to cruise airspeed and climb to MSA + 1000 ft

## ALDIS LIGHT SIGNALS

TANKER ----------------------------RECEIVER

(*) Green ----------------------- clear to contact/crossover

(*) White ----------------------- go to observation position

(**) White ---------------------- crossover to other hose

(*) Amber ----------------------- prepare to turn

(**) Amber --------------------- unable to refuel, wait for spare

(********) Red ---------------- breakaway

RECEIVER ------------------------- TANKER

(*) White ------------------------ reset reel response

(********) White -------------- require more fuel than planned

(********) Red ----------------- breakaway

Objective Area Planning (Con’t)

AIR REFUELING TIMING

TROMBONE

1.  Enter ARCT time (20:00)

2.  Enter time to be abeam ARIP 3:00 Min (19:57)

3.  Enter actual time at ARIP (19:38)

4.  Compute time to turn to arrive abeam ARIP.

a.  Subtract step 3 from step 2 (19:57 - 19:38 = 19 min)

b.  Subtract one minute for 180 degree turn (19 - 01 = 18 min)

c.  Divide number of minutes by two (18 / 2 = 09 min)

d.  Add number of minutes to actual time at ARIP (19:38 + 09

= 19:47)

5.  Hack watch over IP (19:38)

6.  Turn towards ARIP at time computed in step 4.d.

Note:  Adjust for groundspeed variations or adjust airspeed to match groundspeed (outbound GS= inbound GS).

Search and Rescue

###### SCNS TIPS for Delivery Patterns

- Mark the target as the aircraft flies overhead.

- Designate the new markpoint as target waypoint (Ex. Waypoint 60)

  - Load final course into LZ information

    - Upwind drops final course = Wind direction minus 90 degrees

    - Downwind drops final course = Wind direction plus 90 degree

    - LZ data: HAT/HAA = 300 day/500 night, LZ ELEV = 10, MAG COURSE = Reciprocal of run-in heading, DIST TO MAP = 3.0, MAP = 0.0, GLS = 3.0

- Create a waypoint (Ex. Waypoint 59), designate as IP

  - Update IP offset bearing and range (ex. 270/005)

    - Insure bearing is the reciprocal of the run-in course, range of 5 NM to 10 NM

- Re-sequence IP to LZ (ex. 59 to 60)

Note- for MA-1/2 Kit drops using SCNS (No Smokes) drop kit when ETE to target reads 5 seconds.

### MA-1 CONTENTS

5 Bundles

1. & 5.: 6-7 Man raft with sea anchor

2. & 4.: Life Support Equipment (Distillation kit, 2 blankets, tarp, signal mirror, etc.)

3.: Medical equipment

MA-2 CONTENTS

#### 5 Bundles

1. & 5.: 20 Man raft with sea anchor

2. & 4.: Life Support Equipment (Distillation kit, 2 blankets, tarp, signal mirror, etc.)

3.: Medical equipment

#### PARABUNDLE

It could be any equipment needed by the survivor. For instance, Bundle 2 or 4 with the life support equipment because it has a radio if no radio contact with survivor. Normally, dropped with a G-8 Chute which supports equipment weighing 7 to 100 pounds.

Search and Rescue (Con’t)

SWEEP WIDTH

SRU: Search Rescue Unit that can consist of an aircraft, boat, helicopter, dogsled team, etc.

SWEEP WIDTH: Sweep width is the distance on both sides of the SRU where the probability of detecting a target outside of the sweep width is equal to the probability of missing a target inside that distance. It is a measure of detection capability based on target characteristics, weather, and other factors. Sweep width is less than twice the maximum detection range, which is the farthest range at which the target can be detected. See below:

Factors affecting sweep width are:

1. Target characteristics			2. Meteorological Visibility

3. Terrain/Sea Conditions		4. Cloud Cover

5. Search Altitude			6. Search Speed

7. Visual Distress Signaling

Search and Rescue (Con’t)

| PARACHUTE DRIFT DISTANCE (ZERO GLIDE RATIO) (DISTANCE IN MILES OF LANDING POSITIONS DOWNWIND FROM POSITION OF PARACHUTE OPENING) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PARACHUTEOPENING HEIGHT | WNDS IN KNOTS |  |  |  |  |  |
|  | 10 | 20 | 30 | 40 | 50 | 60 |
| 30,000 FT (9000M) | 3.7 | 7.4 | 11.1 | 14.7 | 18.4 | 22.1 |
| 20,000FT (6000M) | 2.7 | 5.3 | 8.0 | 10.7 | 13.3 | 16.0 |
| 14,000 FT (4300M) | 1.9 | 3.8 | 5.7 | 7.7 | 9.5 | 11.4 |
| 10,000 FT (3050M) | 1.4 | 2.8 | 4.2 | 5.7 | 7.0 | 8.3 |
| 8,000 FT (2400M) | 1.2 | 2.3 | 3.5 | 4.6 | 5.8 | 6.9 |
| 6,000 FT (1800M) | 0.9 | 1.7 | 2.6 | 3.5 | 4.4 | 5.2 |
| 4,000 FT (1200M) | 0.6 | 1.2 | 1.8 | 2.4 | 3.0 | 3.5 |
| 2,000 FT (600M) | 0.3 | 0.6 | 0.9 | 1.2 | 1.5 | 1.8 |

Search and Rescue (Con’t)

## SECTOR SEARCH

NOTE: ALL TURNS TO THE RIGHT

| START |  | # SIGHTINGS |  |
| --- | --- | --- | --- |
| TIME (Z) |  | MARK POINT |  |
| LAT |  | MARK POINT |  |
| LONG |  | MARK POINT |  |
| IAS |  | MARK POINT |  |
| ALTITUDE |  | MARK POINT |  |
| TRACK SPACING |  | MARK POINT |  |
| INITIAL HEADING |  | MARK POINT |  |
| TURN TYPE |  | MARK POINT |  |

Search and Rescue (Con’t)

NOTE: ALL TURNS TO THE RIGHT

| START |  | # SIGHTINGS |  |
| --- | --- | --- | --- |
| TIME (Z) |  | MARK POINT |  |
| LAT |  | MARK POINT |  |
| LONG |  | MARK POINT |  |
| IAS |  | MARK POINT |  |
| ALTITUDE |  | MARK POINT |  |
| TRACK SPACING |  | MARK POINT |  |
| INITIAL HEADING |  | MARK POINT |  |
| TURN TYPE |  | MARK POINT |  |

Search and Rescue (Con’t)

NOTE: TURNS TO THE RIGHT OR LEFT DEPENDING ON ENTRY

| START |  | # SIGHTINGS |  |
| --- | --- | --- | --- |
| TIME (Z) |  | MARK POINT |  |
| LAT |  | MARK POINT |  |
| LONG |  | MARK POINT |  |
| IAS |  | MARK POINT |  |
| ALTITUDE |  | MARK POINT |  |
| TRACK SPACING |  | MARK POINT |  |
| INITIAL HEADING |  | MARK POINT |  |
| TURN TYPE |  | MARK POINT |  |

Search and Rescue (Con’t)

NOTE: ALL TURNS TO THE RIGHT

| START |  | # SIGHTINGS |  |
| --- | --- | --- | --- |
| TIME (Z) |  | MARK POINT |  |
| LAT |  | MARK POINT |  |
| LONG |  | MARK POINT |  |
| IAS |  | MARK POINT |  |
| ALTITUDE |  | MARK POINT |  |
| TRACK SPACING |  | MARK POINT |  |
| INITIAL HEADING |  | MARK POINT |  |
| TURN TYPE |  | MARK POINT |  |

Search and Rescue (Con’t)

Search and Rescue (Con’t)

Navigator Assisted MA-1/2 Kit Drops

500 Foot AGL

(CARP= 20 yards RIGHT/ 350 yards SHORT)

Release Kit 5 Seconds short of Target crosswind.

Sight Angle= 65 		FLIR/IDS ANGLE

-25 Degrees Down

-  3 Degrees Left

300 Foot AGL

(CARP= 20 yards RIGHT/ 275 yards SHORT)

Release Kit 4 Seconds short of Target crosswind

Sight Angle= 70		FLIR/IDS ANGLE

-20 Degrees Down

-  5 Degrees Left

Search and Rescue (Con’t)

Navigator Assisted MA-1/2 Kit Drops

Step 1: As the aircraft flies over the target on the initial pass, the navigator will mark this position and store it. The wind (e.g. 180/15) should be noted and recorded at this time. The SCNS will allow the navigator to direct the pilot perpendicular to the wind on the final run-in. Upwind drop final course= Wind -90 degrees. (SCNS= Wind+ 90 for IP setup) Downwind drop final course= Wind + 90 degrees. (SCNS = Wind - 90 for IP setup)

Step 2: Go to the plan page. Make a dummy waypoint (e.g. 49) using Mark point of target and define as a IP using range and bearing. Use bearing reciprocal of the desired run-in heading (e.g. 090 for downwind IP, 270 for upwind IP) and a range of 5 to 10 NM and verify. The pilot must confirm the final run-in heading before the navigator can load the bearing.

Step 3: Now go to next waypoint and make this waypoint (e.g. 50) the target, which is your saved mark point. Go to mode control and lock SCNS manually from waypoint 49 to 50. Insert these waypoints on Modify sequence page if these are newly added waypoints.

Note: Keep in mind that the target will drift. This is basically a visual maneuver: the navigation systems are used as a backup. The pilot should make every attempt to keep the target in sight.

Step 4: The pilot will use the system to help maintain situational awareness and line up on the target for the delivery pass. During the run-in, the pilot will offset upwind or downwind as necessary.

Note: For a SCNS assisted drop at 500 feet AGL drop kit 5 seconds from target; at 300 feet AGL drop kit 4 seconds from target.

REFERENCE: AFI 11 2-MC130 Vol. 3, Chapter 20.10.1.5 & AFI 11 2-HC130 Vol. 3, Chapter 17.10.6.2

Search and Rescue (Con’t)

MA-1/2 KIT UPWIND DELIVERY PATTERN

(For 1-Man rafts)

ALTITUDES

ALTITUDES

FINAL RUN-IN = WIND - 90 DEGRESS

DAY- 300’ AWL, 130 KIAS

Wind

NIGHT- 500’ AWL, 130 KIAS

Search and Rescue (Con’t)

Search and Rescue (Con’t)

MA-1/2 KIT UPWIND DELIVERY PATTERN

(For 1-Man rafts)

FINAL RUN-IN = WIND - 90 DEGRESS

SCNS IP set-up = Wind + 90 degrees (Ex. 090 degree bearing/006 NM)

Wind

5. “30 seconds to target”, altitude/airspeed stabilized (130KIAS)

6. Advise “10 seconds” when abeam second timing smoke.

7. Command “Ready, Drop” when abeam first timing smoke.

1.

50’

Displacement

15 knots

50 feet

25 knots

75 feet

35 knots

100 feet

45 knots

125 feet

2.

4.

5.

6.

7.

3.

1. Abeam Target- Pilot calls “1,000 one, 1,000 Drop”.

Note- Drop timing smokes on altitude, heading and airspeed.

2. Maintain Heading for 10 seconds and drop second timing smoke,

Navigator calls.

3. Turn right 45 degrees and maintain heading for 15 secs (desired)

30 seconds gives a better run-in (technique only).

4. Maintain heading until final turn to pass proper distance from target.

Search and Rescue (Con’t)

MA-1/2 KIT DOWNWIND DELIVERY PATTERN

(For 6,7, OR 20-Man rafts)

ALTITUDES

DAY- 300’ AWL, 130 KIAS

Deployment

Distance for

Downwind

Drops is

Always 50

Feet

NIGHT- 500’ AWL, 130 KIAS

FINAL RUN-IN = WIND +- 90 DEGRESS

Search and Rescue (Con’t)

(For 6,7, OR 20-Man rafts)

Turning final for

timing smokes

Hack over

target- Mark

Set up

SCNS

SCNS IP set-up = Wind - 90 degrees

(Ex. 270 degree bearing/006 NM)

MA-1/2 KIT DOWNWIND DELIVERY ENTRY

Wind

Time outbound for

22 seconds

Left 90 degrees

Left 90 degrees

Left 90 degrees

DAY- 300’ AWL, 130 KIAS

NIGHT- 500’ AWL, 130 KIAS

with smoke,

FINAL RUN-IN = Wind + 90 degrees

Search and Rescue (Con’t)

SCNS IP set-up = Wind - 90 degrees

1. Abeam Target- Pilot calls “1,000 one, 1,000 Drop”.

Note- Drop timing smokes on altitude, heading and airspeed.

2. Maintain Heading for 10 seconds and drop second timing smoke, Navigator calls.

3. Turn right 45 degrees and maintain heading for 15 seconds (desired)

.   30 seconds gives a better run-in (technique only).

4. Maintain heading until final turn to pass proper distance from target.

FINAL RUN-IN = Wind + 90 degrees

Wind

MA-1/2 KIT DOWNWIND DELIVERY PATTERN

(For 6,7, OR 20-Man rafts)

4.

1.

2.

3.

5.

6.

7.

Deployment

Distance for

Downwind

Drops is

always 50 Feet

50’

ALTITUDES

DAY- 300’ AWL, 130 KIAS

NIGHT- 500’ AWL, 130 KIAS

5. “30 seconds to target”, altitude/airspeed stabilized (130KIAS)

6. Advise “10 seconds” when abeam second timing smoke.

7. Command “Ready, Drop” when abeam first timing smoke.

Search and Rescue (Con’t)

International Distress/SAR Frequencies

| FREQUENCY | SERVICE | FUNCTION |
| --- | --- | --- |
| 500 KHz 2182 KHz | Aeronautical & Maritime Mobile Distress | Distress |
| 8364 KHz | Lifeboat, Liferaft and Survival Craft Stations | Search and Rescue |
| 121.5 MHz | International Voice Aeronautical | Emergency & Safety |
| 156.8 MHz | International Voice Distress, Maritime | Call, Reply, & Safety |
| 243.0 MHz 282.8 MHz | Military Aeronautical (See Note Below) | Emergency & Survival |
| 406 MHz | Mobile-Satellite | Search & Rescue |
| 27.065 KHz | Citizens Band Channel 9 | Emergency |
| 138.78 MHz | US Military Voice On-Scene | Search & Rescue |
| 123.1 MHz 3.023KHz 5.680 KHz | International Voice On-Scene/CW SAR On-Scene | Search & Rescue |

NOTE: 243.0 is primary voice & beacon freq.; 282.8 is alternate voice.

| MARS LISTINGS |  |
| --- | --- |
| Primary Contact Freq: 13.927 KHz | Secondary Frequency: 4.557 KHz |
| ALL CALL is ANY MARS, ANY MARS |  |

Formation

BRIEF

Requirements are in 11-2MC-130V3, Attach 4 (Mission Planning Guide)

Where/when to Start & Stop radio silence

Times - 	Stations/ Form Check-in: T/O – 35 min

Start: T/O – 30 min

Taxi: T/O – 20 min

LIGHT SIGNALS

Echelon – out PT Door	Trail – Tail (nav lights)	HAR – Right scanner window

| FLASH (ES) | # FORM | HAR | WING TO LEAD |
| --- | --- | --- | --- |
| 1 | I assume lead | Fuel Flowing |  |
| 2 | #2 assumes lead | Unable to refuel | Nav problem |
| 3 | #3 assumes lead |  | Power problem |
| 4 | Go to trail |  | Aborting |
| 5 | Crossover/ echelon |  |  |
| 6 | IMC trail |  |  |

TAXI

Min taxi interval = 1 aircraft length, usually use 2

No offset, just see nose gear under belly, tip of horizontal stab touch inside edge of OB engines

LIGHTING

Echelon/ close trail, only last plane has lights on

Lead turns top rotating beacon/strobe off 1 min prior to turning ext lights off. LE lights up 1 min prior to lights back up.

LINEUP

Center of your half, 2 beaver tail lights merge – or –

Feed-up, take RWY in sequence, start T/O roll same loc as lead

TAKEOFF

Power to flight idle T/O – 5 sec

Brake release = 15 sec spacing

Join-up RIGHT echelon unless OTW briefed, Lead 180 KIAS, 970 TIT until join-up

Formation (Con’t)

CLOSE-TRAIL (Mountains)

200-500’ horizontal spacing – belly to tail

200’ - 	Split fuselage w/ vertical stab

Horizontal stab touching inside of OB engine

50% exhaust pipes visible (1/2 donut on NVG’s)

500’ - 	Horizontal stab touching inside of ext tank

EXTENDED TRAIL (threat)

Same 500’ except can be anywhere behind lead’s 3-9 line

ECHELON (join-up, AR, recovery)

Normally RIGHT side, same alt, nose to tail and wingtip clearance

All turns away from 2

AR pod aft end should just touch front end of tank

Should see bottom of scanner window

Climbs > 500’ use 970 TIT max, adjust pitch for speed

Descents >500’ use 5000 in-lbs min

No. 2 increases airspeed 10 kts for lead change, move R/L for wingtip clearance

CROSSOVER

- Drop back

- Climb up

- Heading change

- Power up to stop

- Drop down

- Forward into position

Formation (Con’t)

WX PEN

IMC TRAIL

MSA w/in corridor, ESA outside

1 NM in trail, use Air to Air TACAN and radar

500’ Alt sep (#2 = base alt + 500’, etc…)

Lead transmit heading, altitude, speed changes

20˚ bank

Climbs < 2500’ @ enroute airspeed, 970 TIT - OTW use –1

airspeed

Descents – use enroute airspeed and 1000 fpm

PREPLANNED

Lead enroute airspeed +20 kts & base altitude (1000 fpm to get

there if necessary)

#2 enroute airspeed & base altitude + 500’

#3 enroute airspeed –20 kts & base altitude + 500’

Hold for 3 min then resume enroute airspeed, should have

established 1 NM separation

INADVERTENT

Designed for 3-ship or less

Wing loses lead:

- “CS, IMC BREAK” x3 on primary

- Immediately initiate climbing turn

- Lead: “CS, hdg, alt (MSA/ESA)”

- All aircraft turn Nav lights to bright, Modes 1 & 3 to normal (permissive environment)

- Lead: 1010 TIT, climb @ 180 KIAS to base alt or until encountering VMC and maintain 180 KIAS.

- #2: RIGHT turn >10˚, 1010 TIT, climb @ 170 KIAS to base alt + 500’ or until encountering VMC then maintain 180 KIAS.  Resume original heading after 30 sec.

- #3: LEFT turn >10˚, 1010 TIT, climb @ 160 KIAS to base alt + 1000’ or until encountering VMC then maintain 180 KIAS.  Resume original heading after 30 sec.

Formation (Con’t)

HAR

Start slowdown 2 mi in trail, flaps on speed, or 1.5 mi for PA SL – 3000’

Lead slows, spare 500 – 2000’ RIGHT and slightly behind, configures at same time as lead

Spare turns anti-collision lighting on when lead turns off

2 retracts hoses when lead gives signal fuel is flowing, OTW signals spare & receivers and lead accelerates, climbs to 500’ when clear, and turns to rejoin to the spare position.

Spare

___________________________________________________________________

OR			#1		After fuel’s flowing,

Spare retracts hoses &

Moves to #1 spare position

Spare

#2

No visual contact or contact lost: spare stays with lead.

Spare loses sight of lead: 10˚ RIGHT and same steps as before

After rendezvous, receiver calls “lost visual contact”: Lead transmits “ Execute, base heading, MSA.”  then goes to base hdg, MSA, and cruise speed.  Spare turns RIGHT 10˚ and climbs to MSA + 500’.

Formation (Con’t)

RECOVERY

200 KIAS trail or echelon

OVERHEAD
1500A/ Traffic Pattern Altitude (higher)

Break at approach end, 45˚ bank, throttles toward flight idle

Level turn to Downwind, 150 KIAS, Before Landing Checklist

Complete 300’, ¼ nm

DOWNWIND

1000A/ Traffic Pattern Altitude (higher)

Break ½ nm past approach end, 45˚ bank, flight idle, configure,

Before Landing checklist

Level turn until 140 KIAS/ Approach (higher)

Complete 300’, ¼ nm

LANDING

Centerline, 50% flaps

Min 15 sec interval, normal (desired) 20 sec

Ground Idle @ 3000’ remaining

Brakes & Reverse @ 2000’

Systems

ACRONYMS

#1 INST TRANSFORMER

F uel pressure indicator

E mergency brake hydraulic indicator

B ooster hydraulic indicator

3 & 4 Engine & Gearbox oil press indicators

ENGINE OUT

Ö Engine Shutdown Ö-List

Ö Before Landing Ö-List

T rim

A irspeeds (App,Vmca,3eng Climb)

S imultaneous Radio Back-up

R eversing

GO Around

2 ENGINES OUT

C B (For Hi-Rudder Boost)

B leed Air Off

C onfiguration

H ydraulics

E lectrics

W eight (120,000 lbs Critical)

FIRE HANDLE ACTIONS

B leed air valve closes

E xt agent control valves pos

E xt agent discharge switch arms

F uel SO valves (fuel cont & firewall) close

S tart control circuit de-energized

H yd shutoff valve (firewall) closes

O il shutoff valve closes (engine)

P rop rec fx signal & elec energ fx solenoid

Systems

ACRONYMS

AUX PUMP

C argo dor

R amp

E mergency nose gear extension

E mergency brakes

P ressure (Util) on gnd (gnd test valve)

LOSS OF ESS AC ACTIONS

M ech Govern (all 4)

G enerator #2 off

A nti-skid off

V G (no INS) ADI

I nverters on

REDUCE LOAD, CB's reset - no reset?

Isolate ess ac bus

F uel boost #2 off

A ux boost pump off

T ransformer rectifier for #2 (CB's)

A utopilot

S uction boost

S ynchrophaser

BATTERY BUS

F ire Extinguishing Agent

A C Ext Pwr switch solenoid

A larm Bells

J ump lights

I solated DC bus on battery light

V oltmeter

E LT

E mergency depressurization

E mergency exit light ext

S ke Battery

Systems

ACRONYMS

CONSERVE BATTERY

C P Floods Off

U HF #1 Off

V oltmeter not in Battery

I sol Bus on Battery Light Off

I nterphone (minimize)

C P Pitot Off

ISOLATED DC BUS

P itot Heat (CP & Nav)

I nterphone

C P Inverter

A TM Start Control (H Model)

B us Tie Switch

U HF #1 Manual

G TC Start Control (E Model)

T D Relay

RIGHT HAND LOCKS

C heck

N ormal

E mergency

L oad

LEFT HAND LOCKS

S towed

O perational

A ft Restraint Release

S imultaneous Release

REDUCTION GEAR

G enerator

H ydraulic pump

O il pump

S tarter

T ach generator

Systems

ACRONYMS

ENGINE ACCESSORIES

S peed sensitive control

S peed sensitive valve

O il pumps

F uel pump

F uel control

Systems

H/MC-130P OPS LIMITS

STARTING TIT	OIL QUANTITY

Max Normal Start	200° C		Maximum	12 gal

Max Enrichment	100° C		Minimum	4 gal

781 Write Up	830-850° C		Expansion	7.5 gal

Stop Start (1 restart)	850-965° C		Engine Low Oil Light	4 gal

Overtemp Inspection	>965° C		Prop Oil Quantity	26 qts

Prop Low Oil Light	2 qts low

ENGINE TIT

Max TIT	1083° C	OIL TEMPERATURE

Military	1047° C® 30 min		Normal	60-85° C

Take Off Pwr	1067-1083°C®5m		Maximum	85-100° C

(or 19,600 in/lbs, whichever first)		Ground	30 min

Norm Continuous	200-1010° C		Air	5 min

T/O Reduced Power	900° C		Min Allowable	40° & inc

Climb	 1010° C		Min For Start	-40° C

Normal Limiting	 1083° C

Crossover	65 deg/820 +20° C	OIL PRESSURE (PSI)

Gearbox Max Start	>250

ENGINE RPM			Eng. Max Start	100

Oil Pressure Indicator	35% 		Gearbox Normal	150-250

Low Speed GI	69-75.5%		Engine Normal	50-60

Normal GI	94-102%		Gearbox LSGI	50

Flight Idle (ground	92.5-100.5%		Engine LSGI	50

Max Reverse	96-106%		Gearbox Flux	+20

Normal RPM	98-102%		Engine Flux	+10

Allowable Flux	+0.5%		Gearbox Mission Min	130

Prop Pitchlocks At	103%

Fuel Control Limits	103.5%	GTC LIMITS

Overspeed	105% 		Starting Oil Press	3 psi

Pitchlock Operations	96-98%		Start Light Goes Out	~35%

Igniters Energized	16%		On Speed Light On	95%

Warm Up (No Load)	1 min

ENGINE TORQUE (IN-LBS)			Oil Quantity	4 qts

Oil Temp < 0° C	Minimum

Oil Temp 0-40° C	4,500	MAXIMUM WEIGHTS

Max ContinuousTorque	19,600		Max. Takeoff 	155,000 lbs.

NTS Actuation	-1,260 ±600		Max. Wt. EWP	175,000 lbs.

Decouple	-6,000		Normal Landing 	130,000 lbs.

Prop Reversing (max diff)	1000		Max. Effort 	130,000 lbs.

Torque Comparison Check	200		No Flap 	120,000 lbs.

Systems (Con’t)

FUEL SYSTEMS	HYDRAULICS (AUXILIARY)

Imbalance Limits			Normal	2,900-3,300 psi

Otbd>Inbd	500 lbs		Max	3,500 psi

Symmetricals (Excl Aux)1000 lbs		Reservoir	6.22 gal

Wings (Excl. Aux)	1500 lbs

Fuel Tank Pressure		HYDRAULICS (BRAKES)

Main	15-24 psi		Normal Brakes

Aux/Externals	28-40 psi		    2 Appl	2,900-3200 psi

Max Usable	60,112 lbs		    1 Appl	2,250 psi

Main Low Press Lt	<8.5 psi		Emergency Brakes

Aux/Ext Empty Lt	<23 psi		   1 Appl	2,900 psi

Fuel Dumping

Total Left	6,200 lbs	BLEED AIR SYSTEM

Outboards	1,600 lbs		14th Stage Temp	600° F

Inboards	1,500 lbs		                  Press	125 psi

Assault Limits (Outboard Fuel)			Min Eng Output	70 psi

250/450 psi Struts	5,200 lbs		Min APU Output	35 psi

285 psi Struts	4,000 lbs		Bleed Air Checks

Max Aircraft Wt	130,000 lbs		    Eng (65-35 psi)	10 sec

No Fuel In External Tanks			    APU(30-15 psi)	8.5 sec

HYDRAULICS (UTIL/BOOST)	 	PRESSURIZATION

Indication By	On Speed		Cabin Diff	-1.2-15.8 inHg

Normal By	30 sec		Safety Valve	-.76-15.9 inHg

Normal	2,900-3,200 psi		Normal Pressure	15.16 inHg

Max	3,500 psi		Press Cont	-1000-10,000 ft

LSGI	2,550 psi		Man Control takes........40 sec

Aileron Boost	2,050 psi		Cracked Window (both panes)

Utility Reservoir	7.0 gal		    Flt. Station	10 inHg

Booster Reservoir	2.0 gal		    Cargo Compart	0 inHg

Low Press Lt	1,000 psi

Suction Boost Lt	20 psi	THROTTLE RANGE

Rudder Boost Pressure			Ground Idle	18°

0-15% Flap	1100-1400 psi		Flight Idle	34°

Max	1600 psi		LSGI	9-30°

15-100%	2900-3200 psi		Temp Limiting	0-65°

Max	3500 psi		Temp Controlling	65-90°

Loss Sys. Press	<2500 psi

Compensator Fail	>3,450 psi	DUTY CYCLES

Relief Valve Fail	>3,950 psi	Eng	1on/1off/1on/5off/1on/30off

Suction Boost CB	11-12 amp	GTC Starter	1 on/ 4 off

Feather	1 on/1 off/ 1on/30 off

Systems (Con’t)

ELECTRICAL SYSTEM (AC)		TAXI OPERATIONS

Voltage	110-125 V		Normal X-Wind Taxi	30 kts

Frequency	380-420 Hz		Max X-Wind Taxi	60 kts

Load Range	1-1.05		Taxi Speed w/Wt £155,000 lbs

AC Gen Rating	115/200 v/3 f		    5 kts	60° Nose Wheel

AC Gen Capacity	40 kva		    20 kts	20° Nose Wheel

Taxi Speed With Wt> 155,000

ELECTRICAL SYSTEM (DC)			    10 kts	20° Max Turn

Voltage	25-30 v/28 V		Max Tire Rotation

Battery Voltage	24 v		    Nose	139 kts

Min. for Takeoff	21 v		    Main	174 kts

Load Range	0-1.03

Min to Close Relay	18 v 	AUTOPILOT OPERATIONS

Accept Charge	.³12 v		Min Altitude	1000 ft

Bus Off Lt (Avg V)	<90 v		Max Airspeed	250 kts

Gen Out Lt (Any f)	<70 v		Max Weight	155,000 lbs

# Gen to Power all buses	2		Retrim Rudder	200 kts

Altitude Hold	<300 fpm

ANTI-ICING

Prop/Spinner

Prop/Spinner	65-90 amp 	G-LOADING

Cycle	15 on/45 off		Sym (0 Flap)	3.0

Max Gnd Cycles	2		Assym (0 Flap)	2.33

Num of Ice Probes	2		Sym (Flaps)	2.0

Leading Edge			Assym (Flaps)	1.5

Normal Range	75-200° F		Max Bank (0 Flap)	60°

Overheat Light	>200° F		Max Bank (Flaps)	45°

Max Gnd Time	30 sec

NESA		AIRSPEED LIMITS

Max OAT	27° C (81° F) 		T-Storm Pen	180 kts Max

Cold Start	<-43° C			(65 kts above poweroff stall)

Ops	5 sec ON/10 sec OFF		Gear & Lights	165 kts

Don't  Use Auto	<-43° C		Ramp & Door	150 kts

Door Only	185 kts

OXYGEN SYSTEM			Troop Door (moving)	150 kts

LOX Quantity	50 L		Air Deflectors	150 kts

LOX Low Light	2.5 L		Repainted Surface	250 kts

System Pressure			Recom Airstart	180 kts

No Flow	270-455 psi		Bailout	150 kts

Continuous	270-340 psi

Man Hours Avail	192 hrs

Systems (Con’t)

FLAP SPEEDS		SHUTDOWN CONDITIONS

10%	220 kts		1.  Engine Fire

20%	210 kts		2.  Turbine Overheat

30%	200 kts		3.  Nacelle Overheat

40%	190 kts		4.  Uncontrollable Power

50%	180 kts		5.  Uncontrollable Rise In TIT

60%	165 kts		6.  Uncontrollable Loss Oil Press

70%	155 kts		7.  Uncontrollable Rise Oil Temp

80%	150 kts		8.  Throttle Cont. Cable Failure

90%	145 kts		9.  Certain Propeller Malfunction

100%	145 kts		10.  Unusual Vibration/Roughness

11.  Generator Failure (No Disconnects)

12. Visible Fluid Leak

EMERGENCY EQUIPMENT

First Aid Kits	23	ENGINE SHUTDOWN PROCEDURE

Fire Ext	4	1. Condition Lever - FEATHER (CP)

Oxygen Bottles	10	2. Fire Handle - PULLED (CP)

Regulators (Flt. Stat.)	6		    (for fire, nacelle overheat,

Regulators (Cargo)	13		    or visible fluid leak)

Hand Axes	2		3. Agent - DISCHARGED

Life Rafts	2		 (for fire or nacelle overheat) (CP)

Emer. Exit Lts	8

Escape Ropes	3	SMOKE & FUME ELIMINATION

1. Oxygen - ON,100% (P)

CARGO EQUIPMENT

Tiedown Ratings		3 ENGINE OPERATIONS

Wall	5,000 lbs	ü - Emergency Checklist (ESP)

Floor	10,000 lbs	ü - Normal Checklists (Before lndg)

Capped	25,000 lbs	T - Trim (Neutral rudder before lndg)

Ramp	5,000 lbs	A - Airspeeds (approach, landing)

Vmca 2eng & 3 eng. climb)

FLARE LAUNCHER SYSTEM   		S - Simultaneous transmissions

Normal Ops must have ESS Power	R - Reversing (Symm. good engines)

& Utility Hyd Press 30 seconds to 		GO - Go Around

ensure tubes clear (green light

indicates successful launch)

Systems (Con’t)

Start Matrix

-No Ignition

No---16% Switch or Ignition Control CB

Fuel Flow?

Yes---“LM, when did fuel drain?”

Continuous---Ignition Relay or Drip ValveMX

Fuel Drain

After Stop Start---Bad Igniter or Spray Pattern

Try Enrichment

-No Secondary Fuel Pump Pressure Light

Yes---Bulb, CB, Pressure Switch

TIT Drop?

No---Secondary Pump or ValveMX

-Secondary Fuel Pump Pressure Light Remains On

Yes---Pressure SwitchMX

TIT Drop?

No---Pull Ignition Control CB

Light Out---65% SwitchFLY

Light On---Primary Pump or ValveMX

Systems (Con’t)

HAR SYSTEM

SYSTEM OPERATION

Pressure warning light 1700 psi

Reel latch release cylinder releases latch @1000 psi

Preset trail position is 81’

Tanker ready light on 5 sec after in preset trail position

Hydraulic pressure shutoff solenoid valve actuated by the hydraulic pressure control switch.

Reel power control valve open when switch is in TRAIL.

Servo valve position assembly moved by mechanical linkage in drum assembly (jackscrew).

REEL RESPONSE CHECK

- Emergency trail switch – EMER TRAIL:

Response test valve opens

Pressure to sensing cylinder and shifts valve piston to rewind

Tanker ready light goes out (refueling range) 76 – 56’,

Light back on (<56’).

- Emergency trail switch – NORMAL:

Returns to preset trail.  If not, momentarily place reel response switch to RESET (light on 5 sec after preset trail position.)

- Reel Response Switch – REWIND:

Rewind solenoid valve closes

Reference pressure solenoid opens pressure (servo valve piston) to rewind port of drum motor and retracts hose.

Drogue position switch maintains power on rewind valve which prevents inadvertent release if drogue stop switch prematurely opened by hose whip then closes 1’ prior to stowed position.

Switches close reel control valve by removing power < 400 psi in reel latch, engaged & locks drum, stowed & locked light is on.

- Hyd pressure control switch – OFF

- Reel pressure switch – AUTO (so guillotine switch isn’t armed)

- Reel control power switch – OFF (secures panel)

Systems (Con’t)

NORMAL HAR PROCEDURES

ARCP – point/ coordinates tanker (T) abeam receiver (R) & assumes lead

ARCT – time “  “, T arrive @ or later, R arrive @ or earlier

ARIP – Upstream of initial rendezvous point minimum 6 NM

AREP – End of AR track

ARAP – Abort point where cannot take R any further

RENDEZVOUS

Random – VMC, T @ AR alt NLT 3 NM from R, maneuver

to ½ nm trail & join-up

Head on offset –

R

Track							   ARCT-2min	 heading

ARIP

3 NM

offset

Reciprocal track heading			   ARCT-3min

T

Min trail may be as little as 2 nm, if turn back to track based on R position, en route timing may end abeam ARIP.  Arrive abeam ARIP 1 min earlier for HAR formation options & rendezvous.

Tanker Orbit – (hardly ever do) VMC, T slows prior to join-up

Enroute Overtaking – T approaches R from rear by Electronic/ Vis. T @ AR altitude 3 NM from R.  Maneuver to ½ NM trail & join-up

EMERGENCY EQUIPMENT

AIRCRAFT EMERGENCY EQUIPMENT

| Hand-Op Fire Extinguishers |  | 4 | Fw & Aft 245, Aft LWW, Aft RPTD |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Halon or BCM, 125 +/- 50 psig. |  |  |  |  |  |  |  |  |
| Smoke Masks |  | 3 | 2FD, 21Cargo: 6 Fw RWW, 7Fw LWW |  |  |  |  |  |  |
| First Aid Kits |  | 23 | 4Fw R & L PTD |  |  |  |  |  |  |
| Hand Axes |  | 2 | Aft 245, Aft RPTD |  |  |  |  |  |  |
| Emergency Exit Lights |  | 8 | Crew entrance door, 2 PTDs, 2 side EEs, 3 Overhead hatches |  |  |  |  |  |  |
|  | Battery operated. Turned on by 2.5G deceleration, loss of Ess DC, and when disconnected or switched on. |  |  |  |  |  |  |  |  |
| Life Rafts |  | 4 | Wing roots |  |  |  |  |  |  |
|  | 20-man, Each has accessory kit w/ emer radio, 4 distress signals, 5 food kits, and 1 marker light. |  |  |  |  |  |  |  |  |
| Life raft release handles |  | 6 | 2 FD Escape Hatch, 2 Aft RPTD, 1/wing. |  |  |  |  |  |  |
| Life vests, Anti-exposure suits |  | 16 | Ind. Equip bins in RWW area |  |  |  |  |  |  |
| Flare gun |  | 1 | Ceiling over Nav station |  |  |  |  |  |  |
| Emergency escape exits |  | 7 | 3 overhead, 2 side exit panels, 2 chopping locations |  |  |  |  |  |  |
| Ropes |  | 3 | One per hatch |  |  |  |  |  |  |
| Rope Ladder |  | 1 | Center OH escape hatch, T-handle Aft RWW bulkhead. |  |  |  |  |  |  |
| Quick Don |  | 4 | P, CP, N, FE |  |  |  |  |  |  |
| Portable O2 |  | 10 | P, CP, Aft L&R 245, over L Scanner, Fw L MG, Fw&Aft R&L PTD |  |  |  |  |  |  |
| Converters |  | 2 | Under CP, RWW |  |  |  |  |  |  |
|  | 25L, 2.5 min/ tank, 5L min operate. 192 man-hours @ 25K’ |  |  |  |  |  |  |  |  |
| Regulators |  | 19 | 6 FD, 1 |  |  |  |  |  |  |

EMERGENCY EQUIPMENT

CIRCUIT BREAKERS

Little Rock Air Force Base Is Essential MR. SEAL

Left AC			Main AC

Right AC			Radios

AC Instr & Fuel Contr	Start

Fuses				Engines

Battery Bus			Aircraft Systems

Isolated DC			Lighting

Essential DC

L      R				 M     R

Is/

A     F    B     ESS	       S      E     A     L

Emergency Procedures

ENGINE EMERGENCIES:

Engine Fire:	Ground- 	1) All throttles- GROUND IDLE

2) ENGINE SHUTDOWN PROCEDURE

In Flight- 	1) ENGINE SHUTDOWN PROCEDURE

Tailpipe Fire:	Ground- 	1) Condition Lever- GROUND STO

Or Torching					2) Motor engine with starter

3) If flames spread- ENGINE SHUTDOWN

In Flight- Smoke with loss of engine oil quantity

1) Monitor during flight

2) ENGINE SHUTDOWN PROCEDURE

(prior to landing)

Tailpipe Fire

1) ENGINE SHUTDOWN PROCEDURE

2) If fire continues, accel. to max airspeed

Turbine: 		Ground-	1) All throttles- GROUND IDLE

Overheat					2) Affected engine- GROUND STOP

In Flight-	1) Retard throttle towards FLIGHT IDLE

2) If light does not go out,  ESP

Nacelle:		Ground-	1) All throttles- GROUND IDLE

Overheat					2) ENGINE SHUTDOWN PROCEDURE

In Flight- 1) ENGINE SHUTDOWN PROCEDURE

High TIT:		Ground-	1) All throttles- GROUND IDLE

2) TD switch- NULL (affected engine)

3) If overtemp continues- GROUND STOP

In Flight-	1) Affected throttle toward FLIGHT IDLE

2) TD switch- NULL (affected engine)

3) If overtemp continues- ESP

GROUND:			1) Set parking brake (P)	5)	Notify all crew/pax

EVACUATION		2) Notify tower (CP)			to evacuate  (P)

3) Bus tie switch- Tied (E)	6)	DC power switch

4) Place all condition levers		OFF (E)

to Feather, pull all Fire	7) Chock airplane (LM)

handles (CP)

TWO ENGINES OUT OPERATIONS:

(Considerations if engine #1 & #2 are affected)

NOTE:	2 eng. ops above 120,000 lbs or high altitude/low density

altitude is marginal. Downwind/base leg- 160 kts min.

Emergency Procedures (Con’t)

1. Gear down prior to shutdown	6. No nosewheel steering

2. Flap Cont. CB- High rudder		7. Watch generator loading

boost			8. Watch turn direction-

3. Emergency brakes selected		     right turns only

4. All bleeds OFF						9. Dump fuel

5. No flap landing vs. 50% flaps	10. No anti-skid

(#1 & 2 vs. #3 & 4 out)

PROPELLER MALFUNCTIONS:

Note:	If a prop malfunctions during takeoff (after Refusal Speed),

when fire is not indicated, it is recommended that the engine

be allowed to run until  Vmca (2 eng. out), then go Mech Gov.

Prop Malf  (low prop oil): Check RPM

WITHIN LIMITSÞ Continue  (Shutdown prior to landing)

OUT OF LIMITSÞ Go Mechanical Governing

Mech Gov:	WITHIN LIMITSÞ Continue (Shutdown prior to landing)

OUT OF LIMITSÞ Pitchlock Check

Pitchlock:	PITCHLOCKEDÞ Continue (Pitchlock Ops)

Check		NOT PITCHLOCKEDÞ ENGINE SHUTDOWN

Pitchlock Operations

TD Valve - Locked

Engine Bleed Air Valve - Closed

Maintain 96-98% RPM by adjusting throttle

ESP when you reach 150 KIAS or cannot maintain 96%

SECONDARY FUEL PUMP PRESSURE LIGHT:

Above 65% RPMÞ Pull Ignition Control CB (Copilot's Side CB shelf)

Light?	ON:   Bad primary pump, shut down engine may be advisable.

Reset CB

OFF: Bad 65% switch, leave running with CB pulled. After

landing shutdown engine by resetting CB and place

condition lever to “GROUND STOP”

Emergency Procedures (Con’t)

MULTIPLE ENGINE POWER LOSS/RPM ROLLBACK:

1) Main tank Boost Pumps						“ON” (E)

2) Main tank Crossfeed Valves				“CLOSED (E)

3) Prop Gov Control Switches				“MECH” (CP/E)

4) Temp Datum Control Valve Switches			“NULL (E)

5) Gen supplying ESS AC Bus					“OFF” (E)

6) Synchrophaser Master Switch				“OFF” (E)

7) Synchrophaser ESS AC and ESS DC

Circuit breakers								“PULLED”(E)

8) Land as soon as practical

GENERATOR OUT LIGHT:  Check It Out

1) Freq/Volt/Load normal:  Possible power indicator relay or TR

failure. Leaving running, a failed TR

will not allow a reset of the generator.

2) Freq/Volt normal: Possible "K" relay malfunction

No load			   Gen. Switch-OFF (monitor Volt/Freq, loss of

indication -go to set 3)

3) Freq/Volt/Load zero:  Gen. Switch- RESET, OFF

a) If freq & voltage normal - resume normal ops

b) If freq & voltage is not indicated on all three phases

or voltage peaks then returns to zero- Gen. Disconnect

4) Abnormal Freq/Volts:

a) Gen switch OFF- monitor

b) If indicator lost-Gen Disconnect

AC BUS OFF LIGHT:  Turn It Off

1) Affected generator- OFF (Ensure Load is Picked Up)

2) Check Freq & VoltageÞ a) Normal (and bus off light remains ON)

with no item on that bus failed- resume

normal operations.

b) Abnormal- Leave gen. OFF- monitor it

c) If indications lost- Gen. Disconnect

3) If other gen. does not pick up the load- the bus is lost

Miscellaneous

TRAINING RESTRICTIONS

| SIM. ENGINE FAIL |  | 1. Direct IP supervision required (N/A if IP candidate |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | is in seat during initial/requal upgrade evaluation to |  |  |  |
|  |  | IP and is under supervision by EP not in seat) |  |  |  |
|  |  | 2. Authorized for ACs/AC upgrades/CPs w/ 300 hrs |  |  |  |
|  |  | 3. WX required: Day - circling mins; Night - VMC |  |  |  |
|  |  | 4. Crosswinds w/in recommended zone |  |  |  |
|  |  | 5. Retard one throttle to FLIGHT IDLE not less than |  |  |  |
|  |  | 1 engine VMCA (out of ground effect) or less than |  |  |  |
|  |  | 300 ft AGL |  |  |  |
|  |  | 6. NO acft systems emergencies during TAC training |  |  |  |
|  |  | 7. No passengers or aero medical personnel on board |  |  |  |
|  |  | 8. Minimize turns into the dead engine |  |  |  |
| NO FLAP LANDING |  | 1. Direct IP supervision required |  |  |  |
|  |  | 2. Authorized for AC candidates and above |  |  |  |
|  |  | 3. Engine out no-flap approaches authorized for ACs |  |  |  |
|  |  | and above; no-flap circling approaches authorized for |  |  |  |
|  |  | AC upgrades and above. Do not combine no-flap |  |  |  |
|  |  | circling approaches w/ any other simulated EPs |  |  |  |
|  |  | 4. WX required: Day - Circling mins; Night - VMC |  |  |  |
|  |  | 5. Crosswinds w/in recommended zone |  |  |  |
|  |  | 6. Max GW is 120,000 lbs |  |  |  |
|  |  | 7. Check no-flap landing distance vs. rwy available |  |  |  |

Miscellaneous

TRAINING RESTRICTIONS

| TOUCH-AND-GOs |  |  | 1. Authorized for any pilot in either seat when an EP/IP |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | or IP candidate on initial or requalification IP eval |  |  |
|  |  |  | occupies a pilot's seat; OR PIC is T & Go certified |  |  |
|  |  |  | 2. Min WX: 300 and 3/4 mile (RVR 40) |  |  |
|  |  |  | 3. Crosswinds (corr. for RCR) w/in recommended zone |  |  |
|  |  |  | 4. Min rwy length (50 flap flight idle T & Go): 5000 ft |  |  |
|  |  |  | Min rwy length (0/100/gnd idle T & Go): 6000 ft |  |  |
|  |  |  | 5. No passengers; T & Gos may be performed w/ MEGP |  |  |
|  |  |  | 6. Do not perform no-flap, ground idle touch-and-gos |  |  |
| GO-AROUND, MISSED |  |  | 1. Min altitude: 500 ft AGL when aircraft, equipment, or |  |  |
| APPROACH |  |  | personnel on runway |  |  |
|  |  |  | 2. Initiate VFR go-around no lower 200 ft AGL when |  |  |
|  |  |  | practicing simulated EPs |  |  |
|  |  |  | 3. Initiate practice instrument missed approaches no |  |  |
|  |  |  | lower than minimum altitude for the approach |  |  |
| MAX EFFORT T/O |  |  | 1. Authorized for AC/IP/EP |  |  |
|  |  |  | 2. Should be performed from main runway when avail. |  |  |
|  |  |  | 3. T/o from assault zone auth at FTU, or appr. by OG/CC |  |  |
| WINDMILL TAXI START |  |  | 1. Direct IP supervision required |  |  |
|  |  |  | 2. Daylight only |  |  |
|  |  |  | 3. Crosswinds must be w/in recommended zone |  |  |
|  |  |  | 4. Runway must be dry, hard-surfaced, 147 ft wide; |  |  |
|  |  |  | Dash-1 recommendations are mandatory |  |  |
| SIM ONLY |  |  | 1. Full stalls/spins |  |  |
|  |  |  | 2. Rudder force reversals (fin stalls) |  |  |
|  |  |  | 3. Simulated runaway trim malfunctions |  |  |
|  |  |  | 4. Simulated loss of hydraulics |  |  |
|  |  |  | 5. Simulated two-engine out, approach, or landing |  |  |
|  |  |  | 6. Simulated engine-out takeoffs |  |  |

Miscellaneous

TRAINING RESTRICTIONS

| INSTRUMENT STEEP |  |  |  | 1. WX required: Day VMC w/ maximum 60o of bank |  |
| --- | --- | --- | --- | --- | --- |
| TURNS |  |  |  | 2. Restricted to at or above 5000 ft AGL for bank |  |
|  |  |  |  | angles in excess of 45o |  |
|  |  |  |  | 3. Check stall speed prior to making instrument steep |  |
|  |  |  |  | turns (stall speed for 60o bank is 1.4Vs for 0o bank) |  |
| APPROACH TO STALLS |  |  |  | 1. Direct IP supervision required |  |
|  |  |  |  | 2. Authorized during formal upgrade training |  |
|  |  |  |  | 3. WX required: Day - VMC |  |
|  |  |  |  | 4. Minimum 5000 ft above ground or cloud deck |  |
| SLOW |  |  |  | 1. Direct IP supervision required |  |
|  | FLIGHT DEMO |  |  | 2. Authorized at or above 5000 ft AGL |  |
|  |  |  |  | 3. Fly at approach, threshold, and 1.2Vs (power off) |  |
|  |  |  |  | w/ gear-dn, flaps-70%/100%; do not exceed 15o bank |  |
|  |  |  |  | 4. Do not exceed 30o bank at PJ MOS or normal air |  |
|  |  |  |  | refueling speeds; do not use any bank at AR MOS |  |
| ENGINE SHUTDOWN |  |  | 1. Direct IP supervision required |  |  |
| AND AIRSTART |  |  | 2. Authorized at or above 2500 AGL in day, VMC |  |  |
| STOP-AND-GOs |  |  | 1. Authorized for any qualified C-130 pilot |  |  |
|  |  |  |  |  | 2. Rwy remaining must be sufficient for RS = T/O |
|  |  |  |  |  | 3. Min WX: 300 and 3/4 mile (RVR 40) |
|  |  |  |  |  | 4. Crosswinds (corr. for RCR) w/in recommended zone |
|  |  |  |  |  | 5. No passengers; may be performed w/ MEGP |
|  |  |  |  |  | 6. Do not perform S & Gos in conjunction with no-flaps |

Miscellaneous

Joint Electronics Type Designation System

| INSTALLATION (1ST Letter) | EQUIPMENT (2ND Letter) | PURPOSE (3RD Letter) |
| --- | --- | --- |
| A – Piloted Aircraft B – Underwater Mobile, Submarine C – Blank D – Pilotless Carrier E – Blank F – Fixed Ground G – General Ground Use K – Amphibious M – Ground Mobile P – Portable S – Water T – Ground Transportable U – General Utility V – Ground Vehicular W – Water, Surface, And Underwater Z – Pilot and Pilotless Airborne Vehicle Combination | A – Invisible Light, Heat Radiation C – Carrier D – Radiac G – Telegraph- Teletype I – Interphone and Public Address J – Electromechanical or Inertial Wire Covered K – Telemetry L – Countermeasure M – Meteorological N – Sound in Air P – Radar Q – Sonar and Underwater Sound R – Radio S – Special Types T – Telephone V – Visual and Visible Light W – Armament X – Facsimile or TV Y – Data Processing | A – Auxiliary B – Bombing C – Communications D – DF/ Recon, Surveillance E – Ejection/Release G – Fire Control H – Record-Reproduce K – Computing M – Maintenance/Test N – Navigation Aids Q – Special Purpose R – Receiving S – Detect/Range Bearing Search T – Transmitting W – Automatic Flight or Remote Control X – ID & Recognition Y – Surveillance and Control |

# EXAMPLE

AN/ALQ-131

AN = Standard Prefix for Military Equipment

A = Piloted Aircraft

L = Countermeasure

Q = Special Purpose

131 = Design Number (Sequential)

Note: “AN” comes from the old “Army-Navy” designation.

SCNS Degraded Operations

| Inoperative Component: |  |
| --- | --- |
| BICU | OUTPUT 2 (Set B) Not available (CP’s FDMS Selected for SCNS outputs) OUTPUT 1 (Set A) - Failure detection not available Flight Plan - Lost Altitude - Baro-altitude -Lost Airdrops - Lost Update Capability - Not Available INU “Battery Low” message not available Auto-release airdrop - Not Available Ref Pt Library - No Access Fail Hist Info - No Access Radio Presets - No Access (except for HF radios with ACP) TAS Computations - Not Available Cel Computations- Not Available |
| DVS | I-DOP Mode is invalid and solution is blanked |
| GPS | After 4 minutes, integrated SCNS submode degrades to D-I GPS - mode invalid Baro-altitude used for altitude system SOFI Aircraft: IDS boresight calibration function inhibited |
| INU | OUTPUT 1 (Set A) - Failure occurs/not available Integrated SCNS mode automatically switches to GPS or DOP GPS altitude used for system altitude SOFI Aircraft: Visual & Runway Altitude Update capability LOST |
| SCP (SCNS Control Panel) | SCNS On/Off switches not available--use Bypass |
| NAV IDCU | OUTPUT 1 (Set A) - Not Available OUTPUT 2 (Set B) - Fail Detection Not Available VHF 1 & UHF 2 - Not Available SCP Annunciator Lights - Inoperative VOR 1 & 2 Backup ON/Off capability - Not Available NOTE: If Essential DC Bus power fails, NAV IDCU is only IDCU capable of being powered from INS Battery Bus See -4, figure 1-2 |

SCNS Degraded Operations

| PILOT IDCU | OUTPUT 2 (Set B) - Not Available (CP’s FDMS Selected for SCNS outputs) TACAN 1 - Control Not Available HF 1 - Control Not Available UHF 1 - Control Not Available VOR 1 Backup ON/Off capability - Not Available Pilot’s Flight Director annunciator - Lights Inop Updates - TACAN 1 Position Updates Not Available See -4, figure 1-2 |
| --- | --- |
| COPILOT IDCU | OUTPUT 1 (Set A) - Not Available TACAN 2 - Control Not Available HF 2 - Control Not Available VHF 2 - Control Not Available VOR 2 Backup ON/Off capability - Not Available Copilot’s Flight Director annunciator - Lights Inop Updates - TACAN 2 Position Updates Not Available See -4, figure 1-2 |
|  | LOSS OF POWER |
| MAIN AC BUS | IDCU - Keyboard Lighting Inoperative |
| ESS AC BUS | BICU - Inoperative DVS - Inoperative INU - Fan Inoperative & INS VERT REF not valid OUTPUT 1 & 2 (Set A&B) - May not be Synchronized Panel Lighting - FDMS, AD/TJ CMPTR, & Master Volume Control Inoperative |
| MAIN DC BUS | SCNS Control Panel - and annunciator lights Inop |
| ESS DC BUS | SCNS relays LH & RH - Inoperative HSI Distance - Inoperative Pilot IDCU - Inoperative Copilot IDCU - Inoperative BICU - Inoperative FDMS - Inoperative |
| INS BATT BUS | Nav IDCU - Inoperative INU - Inoperative SCNS Control Panel - Inoperative |

BRIEFINGS

NVG / ASSAULT TAKE-OFF BRIEFING

1.  TOLD

Required torque (bleeds closed / open)

Ground run (corrected for Vmca?)

Stop & Go Distance

Refusal

Speeds			no obstacle		obstacle

T.O.			Vmca			V meto

Climb			Vmca + 10			Vmeobs

Min flap		Vmca + 20			Vmeobs + 10

Normal takeoff speed

Acceleration check time

3-eng climb performance

2.  RUNWAY ENVIRONMENT

Runway surface -- braking action (TOLD assumes dry, paved)

Obstacles

Takeoff continued -- Height/Distance / MSA / 3 eng performance?

Takeoff aborted - - Area past departure end

3.  RADAR ALTIMETER

50’ or Low-level

4.  CREW DUTIES

P	-- call “hack” @ brake release; Nose wheel steering

E	-- monitor engine inst / power; Bleeds - closed

CP	-- yoke forward / wings level

-- “Reject time” if speed fails (-3 knots)

-- “Go” at VR  (guard yoke)

-- “Rotate” at T/O speed

-- Ready for ESP / emer. brakes / ready to T/O

N	-- “Timing” at brake release

-- “Time” (pilot checks speed +/- 3 kts)

-- Monitor departure / obstacles

6.  EMERGENCY PROCEDURES

Directional control problems only

-- Prior to refusal - “reject”; after refusal - advisory

All other emergencies will be handled airborne enroute to big runway

7.  DEPARTURE

First course, Next event

BRIEFINGS

NVG LANDING (SCA) BRIEFING

1.  PRE-BRIEF ITEMS

- Threats / Weather

- Obstacles / Terrain / MSA

- Location of slowdown, descent point

- Glideslope, expected VVI, MDA, go-around heading

- LZ -- Dimensions / Surface / Markings / Lighting (Amp)

- Aircraft Lighting (Covert / Overt, Taxi / Landing Lights)

- Go-Around Point (visual point / timing from threshold)

- Taxi procedures to On load/offload point

2.  TOLD

- Assault Procedures?—Landing / T/O (Fuel, gross weight)

- 2 TOLD Cards (Offload/On load), 3-eng T/O ground run

- Ground roll / Stop & Go Distance

- Normal / Max Effort approach, threshold, touchdown speeds

- Actual METO speed or Vmca

# 3.  CREW DUTIES

Al l -- NVG preflight/battery check (extra batteries)

E	-- Run checklist, flaps and trim / monitor engines, then clear

-- SD [W]: “Flaps-50% on-speed” (P), Flaps-50%, then verbalizes;

“165” (E) / “Gear Down” (P), then challenge;

“Flaps 100%” (P), Flaps-100%, then verbalize

-- Close bleeds when “Flaps - 100%” (simulated or actual)

-- On RWY- Flaps 50% and trim set once out of reverse

CP	-- B/U SD calls—flaps and gear

-- AIRSPEED--3 slow / 5 hot   // VVI +/- 200 OR > 1000 ft/min

-- B/U Nav on aimpoint, glidepath and centerline

-- IR taxi light on @ 200’ (all lights extended for B/U)

-- Landing - short / long  // P will shift to zone at 25’, flare at 10’

-- Expect yoke right after touchdown -- keep backpressure

-- EPs--emergency brakes // ESP // loss of NVG’s on ground

-- Go-around – Rad Alt set, B/U Flaps 50% > 50% threshold speed

N	-- Std call plus 200’

-- Distance, L/R, Profile until descent point (confirm SCNS course)

-- “Begin Descent,  on profile, Target VVI” //  “Descending” (P)

-- Distance, Altitude/Glideslope, L/R

-- Go-around – reset Rad Alt to enroute, call every 100’; “Light out”

LM -- Engines in reverse = clear on ramp & door

RO -- Ensure Landing and T/O clearance

-- Confirm Comm. plan

4.  REMINDERS

SECURE LOOSE ITEMS // COCKPIT LIGHTING/LIP LIGHTS

BRIEFINGS

Navigator Briefing Guide

Mission

- Overview (Route/Flying Time)

# Weather

- Sun (Rise/Set – Into/Away), EENT, BMNT etc.

- Moon (Illumination / Azimuth / Elevation)

# Fuel

- Required/Extra

## Enroute Procedures

- ESA/Location

- Takeoff Time / Entry/ No Later than Times

- Departure Procedures

- Altitude/ Airspeeds

- Preflight Winds

- Waypoint Numbering

- Mag Course/MSA/Leg Distance/Recommended Altitudes

- Checkpoints/Turnpoint Description

- Waypoint Brief (5 NM DTG)

- New HDG / ALT / MSA/Deviation Heading

- On Roll-Out Drift / MH / Time Status

- Restricted Areas/MOA’s/No Fly Areas

- Warning Locations

- 20 Min/ 10 Min/ 1 Min / 10/5 sec

- Threats/Locations/Counter Tactics

- IR SAM -appropriate “Call”

- Radar SAM- run away

- AAA- Turn away/Jink

- FENCE/FLOT

- DFENCES Checklist

- Authentication

- Considerations

- Aircraft Shadow/Wingman Position

- Set up Bulls-eyes in SCNS/Reference Points

BRIEFINGS

Navigator Briefing Guide

- Time Control

- Airspeed alone

- Cut-off Points

- Off Course (HDG / New MSA/New Recommended)

- Lead changes

- Wingman Consideration - Twrs/ Turns / Threats

- Crew Coordination

- Radar Terminology

- Ex.- ridge, perpendicular ridge, peak, knoll, etc.

- Clock position, Distance, What to do with that terrain

- Pilot always respond with “In Sight” or “Not in Sight”

- Not painting past _____ NM

- Pilot response-“Terrain in sight” or “Climbing”

- Work at least 7 NM in front of aircraft

- Flight Progress - “On Course/On Dev”

- Terrain Trends-upcoming ridges or contours

- Emergency Procedures

- Ensure three/two engine climb capability

- Climb to MSA

- Emergency Airfields

#### Drop Zone

- SCNS Set-Up

- TP Location

- CDI Information (CARP or centerline)

- Altitude Gate On / Off

- Ballistic / % Altitude Wind

- Range Control Clearance (If required)

- IP Location

- Visually Identified

- Roll out on Magnetic Course/Run-In Heading

- Time Status at IP

- GPS Status at IP (GPS tight)

- Slowdown (Visual Reference/SCNS)

- Lock SCNS-Manual Mode TP to DZ

- Double the Drift

BRIEFINGS

Navigator Briefing Guide

- Drop Altitude/Airspeed

- On Drop Altitude and Airspeed

- Recomputed / Restate CARP

- Visual Run In Update Points

- 2 to 3 course update points

- “I Have/ I Want”

- DZ Recognition

- DZ Markings

- Pilot, Copilot, and Navigator -“Have the Drop Zone in Sight”

- TOT

- 1 Min Warning Location

- 10/5 Second Location

- Green Light Location

- Red Light Location

- No Drop Box

- Escape Heading/Altitude/Airspeed

- Terrain/Obstacles

- Emergency Procedures

#### Helicopter Air Refueling

- HAR Track

- Track Heading

- Altitude

- Airspeed

- Type Rendezvous

- Head-On Offset

- Enroute Overtaking

- Random

- Helicopter Type Formation

- Option 1/2

- Receiver’s Approach

- TACAN Usage

- Air-to Air Frequency

- When to turn on

- Radar Terminology

- “Radar Contact, __ NM”

- Distance calls: every NM from 10, inside 3 NM every ½ NM

BRIEFINGS

Navigator Briefing Guide

- ARIP Location

- ARCP Location

- ARCT

- AREP/Abort Point/Abort Bases

- Emergency Airfields

Landing Zones

- SCNS Set-up

- Glideslope

- TDZE

- Magnetic Course/Run-in

- Missed Approach

- Range Control Clearance (If required)

- IP Location

- Visually Identified

- Roll out on Magnetic Course/Run-In Heading

- Time Status at IP

- GPS Status at IP (GPS Tight)

- Slowdown (Visual Reference/SCNS)

- Lock SCNS-Manual Mode TP to LZ

- Kill the Drift

- Intercept Altitude/Threshold Airspeed

- Intercept Distance

- Glideslope

- Visual Run In Update Points

- 2 to 3 course update points

- Distance Calls Every Mile

- Radar/FLIR Update Procedures

- LZ Recognition

- LZ Markings

- Pilot & Copilot, -“Have the LZ in Sight”

- TOT

- Descent Minimums

- 100 ft, 50 ft, 25 ft, & 10 ft Calls

- Go Around Procedures

- Headings/Altitude

- Box Pattern

- 3 NM Final

- Terrain/Obstacles

- Emergency Procedures

Useful Internet Sites

EXCELLENT SITE: www.baseops.net

FOREIGN CLEARANCE GUIDE: http://www.fcg.pentagon.mil/

HQ AMC HOME PAGE: http://amc.af.mil

NOTAMS:  http://notams.jcs.mil

FAA HOMEPAGE: www.faa.gov/nap

ARINC:  http://www.arinc.com

NOAA AVIATION WEATHER HOMEPAGE: http://www.noaa.gov

WEATHER FORECASTS:  	http://adds.awc-kc.noaa.gov

NATIONAL RADAR SUMMARY: http://intellicast.com/weather/us/cities

WEATHER:  http://www.afwin.afwa.af.mil/index.html: http://aviationweather.noaa.gov

NIMA (National Imagery and Mapping Agency) HOMEPAGE: http://www.nima.mil/

ELECTRONIC CHART UPDATING MANUAL (ECHUM): http://164.214.2.69/products/webchum/index.cfm

US RESCUE COORDINATION CENTER: http://www2.acc.af.mil/afrcc/

AIR FORCE ADVANCED INSTRUMENT SCHOOL: http://www.randolph.af.mil/12ftw/12og/ais/

FEDERATION OF AMERICAN SCIENTISTS: www.fas.org

AVWEB (Aviation Magazine and News Service): http://avweb.com

AIR FORCE PUBLICATIONS:  http://www.afpubs.hq.af.mil

GPS Predictions:  http://sirius.chinalake.navy.mil/satpred/