ASSAULT TAKEOFF BRIEFING GUIDE

1.  RUNWAY ENVIROMENT

a.  Departure Conditions (Weather, Pressure Altitude, Temperature)

b.  Climbout Obstructions - Hazardous Terrain (Navigator

c.  Elevtion, Length, Width, Type Surface (RCR)

2.  TOLD

a.  Brake Cooling Time

b.  Bleed Air Valves - Closed

c.  Charted MFLMETO (corrected for Vmca if applicable)

d.  Refusal Speed

e.  Takeoff Speed - Higher of

(1)  Max Effort Takeoff or

(2)  Vmca (in gnd effect)

f.  Acceleration Time Check (takeoff speed > refusal speed)

(1)  Acceleration Time Reached - Navigator State “Time”

(2)  Refusal Speed - Co-Pilot State “Go”

(3)  Takeoff Speed - Co-Pilot State “Rotate”

g.  Obstacle Clearance Speed

(1)  Max Effort Obstacle Clearance Speed or

(2)  Vmca + 10 kts

3.  EMERGENCY PROCEDURES

a.  Engine/Propeller/System/Load Malfunction

(1)  Before Refusal Speed

(2)  Between Refusal & Takeoff Speed

(3)  After Takeoff Speed

b.  Loss of Normal Brakes

c.  Fuel Dumping/Cargo Jettison

4.  DEPARTURE PROCEDURES

a.  Departure Instructions

b.  Navaids

c.  Radar Altimeter

5.  REMINDERS

a.  Do not Rotate at “Go” unless Refusal = Takeoff Speed

b.  Raise Gear Immediately When Airborne

c.  Min Flap Retraction = Obstacle Clearance + 10 kts

d.  Secure Cockpit/Cargo Compartment