MISSION STUDY/CREW COORDINATION BRIEFING GUIDE

Introduction:

Mission Purpose

Route Study Itinerary:

NAV: Run-In, DZ, CARP, Safety Box, Routes

AC: Crew Coordination Items

Pre-Flight:

Pre-Flight Ö‘s: Instruments, SCNS, SKE, Have Quick, Secure

CP: Check Flight Plan

NAV: Check Loads/Chutes

Takeoff:

CP: T/O in SKE, Check PPI, Advise P

Abort: CP on Primary, NAV on Interplane

Phantom Timing

Combat Entry/Exit Checklist:

Simulated/Actual

Non Flying Pilot Brief Drop Card, Safety Box

SCNS/Radios/SKE:

Flying Pilot: SCNS/Other Pilot in TAC 2 (if available)

IDCU Page for NAV

Radio Plan

E-SKE:

P/CP/N Call Out Every Command

PPI Main Reference for Turns, Timing Back-Up

Turns, Timing:

Turn Point Briefs: Non-Flying Pilot Verify

SKE: Non-Flying Pilot Call Out In-Track Distance

“Ready, Ready, TURN”

On Rollout, Check Time Status, Drift and SCNS

Running Time, Full Leg Timing

Mid-Leg Update Points: “Ready, Ready, HACK”

Off Course Maneuvering:

Marginal Weather - Look Ahead!

Terrain Masking/Box Canyons

Airspeed Above 200 kts

Restricted Areas/Uncontrollable Airfields: Climb or Avoid

Threats:

CP Call in to AWACS/ABCCC ASAP

Authentication: CP (use ENG if busy)

NAV Plot, then have CP Plot, Compare, Brief P

Avoidance Plan

Run-In:

SKE: When Will SCNS go to Sensitive Steering?

SKE Offset Reset Inside IP

Memorize 3 Key Terrain Features

CP and NAV  be Assertive, Direct Pilot to Visual Reference

Concentrate on Visual References for Alignment

Recheck Drift and Winds after Slowdown

Positive Drop Zone/RAM/Shape Des Identification: Direct Others to it

NAV: Remind P of Drop AGL

Drop Altitude Corrected for 2nd/3rd Element SKE (50’/100’)

If New CARP, Brief New Safety Box (CP verify)

Drop:

“NO-DROP” if any doubt

All Drops 2-Handed

Review Malfunction Checklists

Airspeed ± 5 kts, Altitude ± 50 ft

CDS: Reset Flaps by 1 min Warning

50% @ Load Clear/Red Light on Pilots Command

70% (out of horn) for Malfunction

Personnel: Gear Down, Flaps 100% for malfcn out Paratroop Door

HE: CP Back Up Airspeed and Power

Salvo & Racetrack Procedures

Recovery:

SKE: ADI Mode Sel Switch - Normal with Course Guidance

NAV: T.I.M., Approach Timing

Visual:  Start Checklist Early

CP: 	Do Not Punch off Horn, 160 kts, 20° to Rollout on Overheads

Flaps & Gear ASAP on Pilots Command on Downwinds

Rt Hand Patterns, Watch Lead

NAV:	Back-Up Timing (12-14 OH/10-12 DW), 10,15,20 sec on Landing

Misc:

NAV: RADAR, “What if Lead”, Drift, Time Status

All:  Know Safety Box Cold