| TYPE DROP | SURFACE WIND LIMITS | DROP AIRSPEED | FWD TRAVEL DISTANCE | DRIFT EFF (10kt WIND) | TOTAL TIME OF FALL |
| --- | --- | --- | --- | --- | --- |
| PERSONNEL |  | 130 | 250 (650 AGL) | 230 (650 AGL) | 40.0 (650 AGL) |
| ARMY | DZCO |  |  |  |  |
| AIR FORCE | 13 (17 Water Jumps) |  |  |  |  |
| HALO/HAHO | 17 | 130 Desired (110-150) |  |  |  |
| EQUIPMENT |  | 140 | 500 (400 AGL) | 210 (400 AGL) | 23.0 (400 AGL) |
| ARMY | DZCO |  |  |  |  |
| AIR FORCE |  |  |  |  |  |
| w/ Para Release | 17 |  |  |  |  |
| w/o Para Release | 13 |  |  |  |  |
| CDS |  | 130/140* | 550 | 100 | 12.0 |
| G-13/14 | 20 | (*over 120,000 lbs) |  |  |  |
| G-12D/E | 13 |  |  |  |  |
| HI-V/Free Fall | Unlimited |  |  |  |  |
| SATB | 25 | Same as Simulated | 160 | TBP-P 200 | 35.0 |
|  |  | Drop |  | TBP-H 260 | 47.0 |
|  |  |  |  | TBP-C 150 | 26.0 |

RUN-IN RULES OF THUMB:

- 5 kts IAS error = 45 yds scoring error 	- Stay on the upwind side of the DZ		- Speed over the DZ = 75 yds/sec

- 2 degrees drift: 				- 60 yds drift for 10 kt wind & 10 sec TTF	- Drift doubles after slowdown

Offset 100 yds (HE/PER), 50 yds (CDS)