WINDMILL TAXI START BRIEFING GUIDE

1.  PREPARATION

a.  After Landing Checklist (if Req’d)

b.  Ground Stop Downwind Engine (simulated)

c.  Review Dash-1 (Notes, Warnings, Cautions)

2.  BRIEFING

a.  Pilot/Co-Pilot Call: 	“Off Nosewheel Steering”

“90 kts”

“100 kts”

“4000¢ Remaining”

b.  Engineer Call:		“Rotation”

“Light Off”

“40 %”

“On Speed”

c.  Any Crewmember	“Reject” (as Req’d)

(1)  Less than 40% - Feather Starting Engine First

(2)  Greater Than 40% - Allow Starting Engine to Run

(3) Starting Engine Malfunction - Feather

3.  EXECUTION

a.  Before Takeoff Checklist

b.  Lineup Checklist

c.  Windmilling Engine Throttle - Flight Idle

d.  Assymetrical Throttle - Start of Torque Curve

e.  Symmetrical Throttles - Max Power

f.  Rudder - Toward Good Engine - Release Brakes

g.  Rudder Effectiveness - Off Nosewheel Steering

h.  Throttle on Assymetric Engine - Advance as Req’d

i.   90 kts - Retard Throttles to Maintain 100 kts

j.   40% or 4,000¢ Remaining - Throttles Gnd Idle

k.  Reverse Symmetrical On-Speed Engines as Req’d

l.   Starting Engines Checklist